Table 1: Instrument and chemical

| Name | Specification |
| --- | --- |
| HPLC System | Agilent |
| Software | Empower 2 |
| Pump | Binary pump |
| Detector | UV detector |
| Column | Waters reliant C18, 250 mm x 4.6mm, 5 μm. |
| UV Spectrophotometer | Shimazdu COPR |
| Analytical balance | Mettler Toledo |
| pH meter | RD/EQ-0872 |
| Ultra-sonicator | Labtek |

Table 2: Finalised HPLC system specification

| Parameter | Specification |
| --- | --- |
| Mobile Phase | Buffer: ACN (45:55) |
| Column | Waters Reliant C18, 250 Mm X 4.6mm, 5 mm |
| Flow Rate | 1.5 ml/Min |
| Wavelength | 210 nm |
| Injection Volume | 10 µl |
| Retention Time Of Racecadotril | About 8.5 Minutes |
| Column Oven Temperature | 30°C |

Table 3: multiple concentration levels of Racecadotril

| Concentration | 80% | 100% | 120% |
| --- | --- | --- | --- |
| Amount of racecadotril to be added (mg) | 16.0 | 20.0 | 24.0 |
| Amount of placebo to be added(mg) | 1980 | 1980 | 1980 |

Table 4: Linearity for RP-HPLC Method

| Sample | Concentration level | Wt of RAC to be added(mg) | Wt of placebo to be added(mg) | RAC concentration(mg/ml) |
| --- | --- | --- | --- | --- |
| Linearity-1 | 25% | 5.0 | 1980 | 0.1 |
| Linearity-2 | 80% | 16.0 | 1980 | 0.32 |
| Linearity-3 | 100% | 20.0 | 1980 | 0.4 |
| Linearity-4 | 120% | 24.0 | 1980 | 0.48 |
| Linearity-5 | 150% | 30.0 | 1980 | 0.6 |

Table 5: Optimization of parameters for analysis

|  | Factor 1 | Factor 2 | Response 1 | Response 3 |
| --- | --- | --- | --- | --- |
| Run | A: flow rate | B: mobile phase composition | R.T. | T.P. |
| R1 | 1.5 | 55 | 8.50 | 3990 |
| R2 | 1.0 | 60 | 14.0 | 1995 |
| R3 | 1.0 | 50 | 12.7 | 1945 |
| R4 | 0.8 | 55 | 16.0 | 1940 |
| R5 | 2.0 | 50 | 7.40 | 3500 |
| R6 | 1.5 | 50 | 7.65 | 2840 |
| R7 | 2.2 | 55 | 6.57 | 3998 |
| R8 | 1.5 | 60 | 8.45 | 2980 |
| R9 | 2.0 | 60 | 7.85 | 3665 |

Table 6: Desirability of Numerical

| Number | Flow rate | MP composition | RT | TP | Desirability |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.782 | 53.108 | 7.55 | 3993.94 | 1.000 | Selected |
| 2 | 1.00 | 60.0 | 14.0 | 1950.81 | 1.000 |  |
| 3 | 2.00 | 55.0 | 6.90 | 3374.44 | 1.000 |  |
| 4 | 1.50 | 55.0 | 8.05 | 3816.79 | 1.000 |  |
| 5 | 2.00 | 60.0 | 7.37 | 3550.28 | 1.000 |  |
| 6 | 1.50 | 50.0 | 8.075 | 3020.57 | 1.000 |  |
| 7 | 1.50 | 60.0 | 8.939 | 3138.90 | 1.000 |  |
| 8 | 1.20 | 55.00 | 11.35 | 2607.52 | 1.000 |  |

.

Table 7: Results of Linearity study of racecadotril

| Sr.No. | Concentration (mg/ml) | Peak Area |
| --- | --- | --- |
| 1 | 0.1 | 30343539 |
| 2 | 0.32 | 96983579 |
| 3 | 0.4 | 120144534 |
| 4 | 0.48 | 144165550 |
| 5 | 0.6 | 178532808 |
| r2 |  | 0.999 |
| Slope |  | 296743308.1 |
| Y-intercept |  | 1271544.9233 |

Table 8: Comparative data of method precision and intermediate precision

|  | Mean % assay of racecadotril |
| --- | --- |
| Method precision | 100.1 |
| Intermediate precision | 100.1 |
| % difference | 0.0 |

Table 9: Results for Accuracy

| Levels |  | Amount added (mg) | Amount recovered (mg) | % recovery | Mean % recovery | % RSD |
| --- | --- | --- | --- | --- | --- | --- |
| 80% | 1 | 16.24 | 16.21 | 99.8 | 99.7 | 0.3 |
|  | 2 | 16.03 | 15.99 | 99.8 |  |  |
|  | 3 | 16.32 | 16.18 | 99.1 |  |  |
|  | 4 | 15.99 | 15.93 | 99.6 |  |  |
|  | 5 | 16.09 | 16.05 | 99.8 |  |  |
|  | 6 | 16.33 | 16.32 | 99.9 |  |  |
| 100% | 1 | 20.87 | 21.03 | 100.8 | 99.8 | 0.9 |
|  | 2 | 20.57 | 20.40 | 99.2 |  |  |
|  | 3 | 20.07 | 19.96 | 99.5 |  |  |
| 120% | 1 | 24.28 | 24.44 | 100.7 | 99.1 | 1.0 |
|  | 2 | 24.27 | 24.12 | 99.4 |  |  |
|  | 3 | 23.97 | 23.68 | 98.8 |  |  |
|  | 4 | 23.94 | 23.71 | 99.0 |  |  |
|  | 5 | 23.97 | 23.61 | 98.5 |  |  |
|  | 6 | 24.05 | 23.55 | 97.9 |  |  |
| Mean |  |  |  | 99.5 |  |  |

Table 10: Result of robustness

| Sample (flow rate) | % assay of racecadotril |  |
| --- | --- | --- |
|  | Flow rate of 1.35ml/min | Flow rate of 1.65ml/min |
| Test 1 | 98.4 | 98.6 |
| Test2 | 97.9 | 98.6 |
| Test3 | 98.2 | 98.6 |
| Mean | 98.2 | 98.7 |
| %RSD | 0.3 | 0.1 |
| Sample (pH) | % assay of racecadotril |  |
|  | pH at 2.3 | pH at 2.7 |
| Test 1 | 100.4 | 100.2 |
| Test2 | 100.8 | 100.1 |
| Test3 | 102.1 | 99.8 |
| Mean | 101.1 | 100.0 |
| %RSD | 0.9 | 0.2 |
| Sample (MP composition) | % assay of racecadotril |  |
|  | MP Composition(39:61) | MP Composition(51:49) |
| Test 1 | 100.0 | 100.5 |
| Test2 | 100.0 | 100.3 |
| Test3 | 99.9 | 99.7 |
| Mean | 100.0 | 100.2 |
| %RSD | 0.3 | 0.4 |
| Sample (Temperature) | % assay of racecadotril |  |
|  | Temperature at 250C | Temperature at 350C |
| Test 1 | 98.7 | 99.2 |
| Test2 | 98.1 | 98.7 |
| Test3 | 99.2 | 99.4 |
| Mean | 98.7 | 99.1 |
| %RSD | 0.6 | 0.4 |

Table 11: Result for forced degradation study

| Stressed conditions | Duration of exposure | Rt (min) | % assay | Peak area | % Degradation | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Control | - | 8.650 | 99.0 | 6808444 | - |  |
| Acid degradation ((0.1M HCl) | 60 min | 8.650 | 91.8 | 6312330 | 7.3 | Degradation observed |
| Base degradation (0.1M NaOH) | 60 min | 8.667 | 85.7 | 5892726 | 13.4 | Degradation observed |
| Oxidative degradation (30%H2O2) | 60 min | 8.650 | 93.9 | 6451766 | 5.2 | Degradation observed |
| Thermal degradation (600C) | 24 hr. | 8.667 | 99.5 | 6663380 | -0.5 | No Degradation observed |