Appendix 1: Traffic Movements across times of various movements

i)	Through movements along the intersection;

Fig. 7 Plot of total number of vehicles flow against time in through movement

Through vehicles in the major roads (Gaindakot and Shahidchowk) are prevalent compared to that of minor roads. The number of vehicles in the through movement tends to peak around 10 am, indicating a significant increase in traffic during this time. Moreover, when comparing traffic patterns throughout the week, the flow on weekends remains consistent with the flow observed on other days, showing that there is no substantial decrease in vehicle numbers during weekends as one might expect.

ii)	Right turning movements;

Fig. 8 Plot of total number of vehicles flow against time in right turning movements

The right-turning movement at 7 (from Aaptari to Gaindakot) sees the heaviest traffic flow during the evening hours, making it the peak time for this direction. However, when comparing the traffic patterns across the week, Sunday shows a slightly reduced flow for this right turn compared to other days, indicating a minor dip in the number of vehicles making this turn on weekdays, Sunday.

iii)	Left turning movements

Fig. 9 Plot of total number of vehicles flow against time in right turning movements

For left-turning movements, the traffic from Gaindakot to Aaptari is particularly high, reaching approximately 560 PCUs (Passenger Car Units) between 10 and 11 AM. This indicates a significant concentration of vehicles making this turn during this time period. Additionally, when comparing traffic patterns across different days of the week, Wednesday consistently shows the highest flow for left turns, except for movement 3, which has a different pattern.

Classified 15 minutes traffic volumes were converted into total hourly PCUs based on the Equivalency factor value suggested by Indo-HCM and the peak hourly volume observed in the movements is given in Table 1.

# Appendix 2: Manual Calculation and Validation

## 1.	Conflicting flow, Critical gap and Capacity calculation by Indo-HCM

i)	Conflicting flow and Critical Gap Calculation

Conflicting flow in each movement of the intersection were calculated following the standard equation provided by Indo-HCM. The heavy vehicles proportions were calculated in the conflicting stream. These parameters resulted in the calculation of critical gap [2,3] The calculated conflicting flow per the equation Indo-HCM provided is shown below.

Table 9 Calculation of conflicting flow

| Priority Rank | Movement | Conflicting flow equation | Conflicting flow |
| --- | --- | --- | --- |
| 1 | Movement 2 |  |  |
|  | Movement 3(in case of 4-legged intersection) |  |  |
|  | Movement 5 |  |  |
|  | Movement 6 |  |  |
| 2 | Movement 1 | v5 (Total Volume of Movement no. 5 from table 1) | 1684.52 |
|  | Movement4(in case of 4-legged intersection) | v2 | 715.9 |
| 3 | Movement 7 | v4+v5+v1+0.5*v2 | 2819.11 |
|  | Movement 10(in case of 4-legged intersection) | v1+v2+v4+0.5*v5 | 2334.8 |
| 4 | Movement 8 | v4+v5+v1+v2+v3+v10 | 4022.97 |
|  | Movement 9 |  |  |
|  | Movement 11(in case of 4-legged intersection) | v1+v2+v4+v5+v7 | 4831.64 |
|  | Movement 11 | v4+v2+v4+v5+v6+v7 |  |
|  | Movement 12 |  |  |

From the table 13, it is found that through Movements; movement 8 (Aanptari to Newroad) and movement 11 (Newroad to Aanptari) have the maximum conflicting flow along the intersection whereas the right turning movement 4(Gaindakot to Newroad) has the least conflicting flow.

After determining the conflicting flow, we used standard base critical gap values provided by Indo-HCM to determine the critical gap which is shown in the table below:

Table 10 critical gap values

| Movement | Base Critical Gap tc,base | Adjustment factor for light and heavy commercial vehicles fLV | Proportion of heavy vehicles in the conflicting traffic stream PLV | Critical Gap tc,x =tc,base+fLV*lnPLV |
| --- | --- | --- | --- | --- |
| Movement 1 | 2.7 | 0.457 | 59.54 | 4.568 |
| Movement 4 | 2.7 | 0.457 | 27.07 | 4.207 |
| Movement 7 | 3.8 | 0.885 | 41.79 | 7.103 |
| Movement 10 | 3.8 | 0.885 | 27.07 | 6.719 |
| Movement 8 | 6.8 | 0.583 | 35 | 8.873 |
| Movement 11 | 6.8 | 0.583 | 23.99 | 8.653 |

Sample calculation

For movement 1:

tc,base =2.7 seconds

fLV= 0.457 seconds

PLV= 59.54

tc,x =tc,base+fLV*lnPLV= 2.7+ 0.457*ln(59.54) =4.568 seconds

The maximum critical gap of 8.873 was observed in the movement 8 which means drivers are waiting for larger gaps in traffic before proceeding which leads to reduction in capacity of intersection and potentially lower the level of service (LOS) whereas the minimum critical gap of 4.207 is seen in movement 4.

ii)	Capacity Calculation

sThe capacity of turning movements was calculated from the given formula mentioned below and the capacity thus estimated is presented in the table below:[2,3]

Cx=a*Vc,xe-vc,x(tc,x-b)/36001-e-Vc,xtf,x/3600

Table 11 Capacity Calculation

| Movement | Critical gap (s) | Follow-up time=0.6*col-2 | Conflicting flow | Adjustment factor "a" | Adjustment factor "b" | Capacity (PCU/h) |
| --- | --- | --- | --- | --- | --- | --- |
| col-1 | col-2 | col-3 | col-4 | col-5 | col-6 | col-7 |
| Movement1 | 4.568 | 1.541 | 1684.52 | 0.8 | 1.3 | 568.44 |
| Movement4 | 4.207 | 1.324 | 715.9 | 0.8 | 1.3 | 1391.59 |
| Movement7 | 7.103 | 3.062 | 2819.11 | 1 | 2.16 | 64.631 |
| Movement10 | 6.719 | 2.831 | 2334.8 | 1 | 2.16 | 215.31 |
| Movement8 | 8.873 | 4.124 | 4022.97 | 0.9 | 5.04 | 56.06 |
| Movement11 | 8.653 | 3.992 | 4831.64 | 0.9 | 5.04 | 38.037 |

From the Table 11 of capacity of the various movements of intersection we can say that the movement 4 (Gaindakot to Newroad) have the maximum capacity of 1391.59, following to its lower critical gap and small conflicting flow, whereas due to highest conflicting flow (4831.64 pcu) and the high critical gap (8.653s), movement 8 (Newroad to Aaptari) has the least capacity 38.037

2.	Capacity Calculation by Indonesian- HCM

Table 12 Various Parameter of Indoneisan-HCM

| Parameter | Value | Remarks |
| --- | --- | --- |
| WA | 3.5 m | Entry width of Newroad |
| WB | 10 m | Entry width of Shahidchowk |
| WC | 7 m | Entry width of Aaptari |
| WD | 10 m | Entry width of Gaidakot |
| WAC | 5.25 m | The average width of A and C |
| WBD | 10 m | The average width of B and D |
| WE | 15.25/2=7.625 | Intersection entry width |
| QP | 5865 | Total traffic (pcu/h) |
| LT% | 33.4 | LT%=QLTQv*100% |
| RT% | 19.50 | RT%=QRTQv*100% |
| SP% | 40.3 | SP%=QMIQv*100% |

Table 13 Capacity Calculation by Indonesian-HCM

| C0 | 3400 | Intersection type 424 having four major and two minor road lanes |
| --- | --- | --- |
| Fw | 1.174 | Fw=0.61+0.0740We |
| Fm | 1 | Narrow major road median type |
| FCS | 0.94 | Bharatpur city has a population of around 0.6M so it is a medium-sized city. |
| FRF | 1 | Roads have commercial-type road environments and low-side friction |
| FLT | 1.38 | 0.84+0.0161LT% |
| FRT | 1 | For four-legged intersection, the right turning correction factor =1 |
| FSP | 0.85 | split correction factor was calculated with the help of the graph in Figure 13. |
| Capacity | 4402.157 |  |

Calculation:

Capacity = C0*Fw*Fm*FCS*FRF*FLT*FRT*FSP= 3400 *1.174 *1 *0.94 *1 *1.38 *1 *0.85 = 4402.157 PCU/h

A.	Degree of Saturation

The degree of saturation was calculated after determining the capacity of the intersection,

DS=Qp/C

= 5865.8/4402.157

= 1.332

B.	Delay

Finally, the delay time was calculated by using the formula,

D=1.05040.2742-0.2042 DoS

D=1.05040.2742-0.2042*1.332

= 523 sec

After determining the delay time of 523 sec we concluded that the intersection is operating with LOS F. The delay time of 523 sec looks very unreasonable due to very high degree of saturation observed. [4]

3.	Data Validation

From Indonesian HCM too, we concluded that the level of service is in worst condition, i.e. LOS of F. (Table 8)

The result from Indonesian HCM and Indo HCM seem to align with the low level of service of the intersection. This has to be improved with the traffic control devices.

4.	Phase Planning By webster Method

Saturation flow was determined using the formula:

S=525W

Where W=width of the road in m

Sample calculation:

For aaptari road, S=525* 7 =3675m

[12] (IRC 93: 1985)

Where W=width of the road in m

The total critical flow(Y) was obtained from the summation of the critical flows of each intersection. Y=y1+y2= 0.4541+0.2994= 0.7535

Critical flow Ratio= Traffic Volume Saturation flow  (Source: IRC 93:1985)

Where y1= greater critical flow ratio of Aaptari and Newroad =0.4541

y2= greater critical flow ratio of Shahidchowk and Gaindakot =0.2994

Total lost time per phase= startup time + movement time =2+2.5 =4.5 seconds

Where, startup time=2 second

Movement time = 0.5*amber time =0.5*5 =2.5 seconds (amber time is taken as 5sec) 
 (Agarwal, et al., May-June-2021)

Total lost time per cycle =2n+R =9+19 =28 seconds.

► Then the optimum cycle length, (co) was computed by using the formula:

Optimum cycle lengthco=1.5L+51-Y =1.5*28+51-0.7535 =185 seconds

Where L= total lost time per cycle

►Now the effective green time= optimum cycle time-pedestrian time- lost time =185-19-9 =157 seconds.

Where pedestrian time is taken as 19 sec.

Phase green time (G1,G2) were calculated as:

G1=y1*effective green time/Y =0.4541*157/0.7535 =95 seconds

G2=y2*effective green time/Y = 62 seconds

Phase red time (R1,R2) were calculated as:

R1=Co-(G1+amber time) = 185-(95+5) =85 seconds

R2=Co-G2+amber time = 185-(62+5) =118 seconds

In this way, signal timing was designed using Webster`s method.[10]