### Supplementary Information (SI)

Decoupling of inter-regional functional connectivity and regional neural activity in Alzheimer’s Disease– a simultaneous PET/MRI study

Methods:

Section1. Imaging data acquisition and preprocessing

Data acquisition

All imaging data were scanned on an integrated Siemens Biograph mMR scanner (Siemens, Erlangen, Germany) capable of simultaneously acquiring PET and MRI data using the vendor-supplied 12-channel phase-array head coil. Magnetization-prepared rapid-acquisition gradient echo (MPRAGE) T1-weighted anatomical images, T2-weighted echo planar imaging MRI data and PET images were acquired using the following scanning parameters. MP-RAGE: repetition time/echo time/α  = 2.300 ms/2.98 ms/9°; 160 slices (gap 0.5 mm) covering the whole brain; field of view (FOV) 256 mm; matrix size 256 × 256; voxel size 1.0 × 1.0 × 1.0 mm3. Echo planar imaging: repetition time/echo time/ α  = 2.000 milliseconds (ms)/30 ms/90°; 35 slices (gap 0.6 mm) aligned to AC/PC covering the whole brain; FOV 192 mm; matrix size 64 × 64; voxel size 3.0 × 3.0 × 3.0 mm3. Each measurement consists of 240 acquisitions in interleaved mode with a total scan time of 8 minutes, 08 seconds. PET: list-mode acquisition, 30 minutes after injection, 15 minutes acquisition time, 128 slices (gap 0.5 mm) covering the whole brain; FOV 450 mm; matrix size 192 × 192; voxel size: 3.7 × 2.3 × 2.7 mm3.

Preprocessing of imaging data

Preprocessing of multimodal imaging data started with realigning and coregistering of the multimodal imaging dataset to create subject-specific multimodal datasets. Imaging data were preprocessed using Statistical Parametric Mapping (SPM12) and the Data Processing Assistant for Resting-State fMRI toolbox (DPARSF, V5.1) [1].

To do so, raw DICOM imaging data were converted to 3D  nifti volumes and three volumes of each subject’s functional images were then removed for blood oxygen level dependent (BOLD) signal counterbalance, and the remaining volumes were realigned to the same subject’s mean functional image using a least squares approach with a 6 rigid body parameter spatial transformation to estimate head motion of subjects during fMRI scan. Excessive head motion (cumulative motion translation or rotation > 3mm or 3°) was used as an exclusion criterion. No subject was excluded under this criterion. Then, mean PET image and the mean functional image and all the functional volumes of the same subject were realigned to the structural T1-weighted image using rigid-body transformation to create subject-specific multimodal datasets.

MRI images: The subject-specific T1-weighted images were segmented to tissue-probability maps of grey matter (GM), white matter (WM), and cerebrospinal fluid (CSF) based on affine regularization procedure. The EPI and T1-weighted images were then spatially normalized to a standard template provided by the Montreal Neurological Institute (MNI) template using the Statistical Parametric Mapping (SPM12) unified segmentation on T1-weighted routine. The MNI tissue-probability maps of GM, WM, and CSF were warped onto the single-subject T1-weighted images and stored as masks for later use during the nuisance covariate regression. All the EPI images were resampled to an isotropic voxel size of 3 × 3 × 3 mm3. The EPI images were smoothed by the use of Gaussian filter (full-width at half-maximum (FWHM) Gaussian kernel of 4 mm). In order to prepare the rs-fMRI data for functional analysis, they were detrended to remove linear trend [2]. As low frequency (0.01~0.1 Hz) fluctuations of BOLD signal reflects spontaneous neural activity [3], bandpass filtering was also applied on rs-fMRI data to extract physiological signal frequency. Finally, the nuisance variables, such as motion parameters using the Friston-24 model, as well as WM and CSF signals, were regressed out to eliminate their confounding effects [4, 5]. We opted out global signal regression corrections, since this issue is currently challenging in the rs-fMRI field and introduce anti-correlations to data [6].

FDG-PET images: The subject-specific PET images were corrected for partial volume effect by the use of PMOD software package (PMOD Technologies Ltd., Adliswil, Switzerland), as suggested previously [7]. This software uses the segmented the individual T1-weighted image in GM, WM, and CSF for partial volume correction (PVC). The PVC-PET images were normalized to the MNI template and resampled to an isotropic voxel size of 3 × 3 × 3 mm3 using SPM12. Finally, the PET images were smoothed by the use of Gaussian filter (FWHM Gaussian kernel 12 × 12 × 12 mm3).

Section2: Inter-regional FC topological metrics

To quantify the organization of the whole brain functional connectivity (FC), we used the topological metrics, which were calculated by graph theory analyses. These included weighted CC [8] and weighted DC [9] metrics, reflecting regional segregation [10] and centrality [11] of FC patterns, respectively. Weighted DC represents the sum of the weights of all edges that are directly linked to a node [9], and is defined as:

DCiw=j=1Nwij		(1)

where wij is the weight of connection between node i and j, while N is the number of nodes in the graph. The weighted CC for a given node i is equivalent to the fraction of the node neighbors that are also neighbors of each other and is calculated as following:

CCiw=j,h∈N(wijwjhwih)1/3DCiw(DCiw-1)		(2)

where DCiw is the weighted degree of node i, while N is the number of nodes in the graph.

Section 3: Hierarchical moderated multiple regression (HMMR)

To understand the moderating effect of disease on the associations between rFDG and the inter-regional FC topological metrics, we used hierarchical moderated multiple regression (HMMR). In this model, the moderation variable is simply the third variable (M) which has an effect on the link between a predictor variable (X) and outcome variable (Y). The term "moderation effect" is equal to the interaction effect in regression analyses. Put differently, a moderator variable potentially changes the strength of an association between two other variables. SI-Figure 1 (A and B) illustrates two most common forms of moderation effects: conceptual diagram and statistical diagram. In particular, hierarchical moderated multiple regression (HMMR) analysis is an analytical approach to investigate the effect of moderator variable M on the association between hypothetical variables X and Y, which can be formulated as the following:

Y=b0+b1X+b2M+b3X*M+e			(3)

# In this model, b0 is an intercept of the regression of Y on X that depends on the specific value of M. There is a different line with different slope and intercept for every individual value of M (SI-Figure 1C). The moderation (interaction) effect is modelled by the (X*M) term, which is the product of X and M. The b3 coefficient reflects the interaction between X and M only if the lower order terms b1X and b3M are included in the equation. To evaluate whether a two-way interaction is significantly present, we first explored whether the increment in the squared multiple correlation (∆R2) given by the interaction is significantly greater than zero. Then, we tested whether the coefficient b3  differs significantly from zero.

Section 4:  Control analysis to investigate the moderator effect of disease on the link between regional glucose metabolism and the inter-regional FC topological metrics

### As the control analysis, we also applied the HMMR analysis to test whether and how clinical diagnosis and FC derived topological measures (CC/DC) predict rFDG. To do so, FC derived topological measures (CC/DC) and clinical groups (i.e., AD, MCI, or HC) were entered as main predictor variables to a model to rFDG as response variable. Then, the interaction as the product term of the main predictor variables " rFDG × clinical diagnosis " were added to the prediction model. Of note, we removed the effect of age and sex from the response variables, before applying the HMMR analyses.

To evaluate whether the interaction between predictor variables ("FC × clinical diagnosis ") is meaningful, we tested whether adding the interaction term increased the variance explained by the model in successive regression steps (∆R2). When a statistically significant interaction emerged, it was interpreted according to the available guidelines [12, 13]. The results were corrected for multiple comparisons by using Family wise error rate.

Results:

Section1 : The moderator effect of disease on the link between regional glucose metabolism and the inter-regional FC topological metrics in non-overlapping regions

In the non-overlapping regions, the HMMR analysis included 126 tests of exploring interaction effect between rFDG and clinical diagnosis in the model predicting changes in CC and 90 tests in the model predicting changes in DC. We also corrected the results for multiple comparisons using family wise error rate correction (p-value < 0.05 / number of all interaction tests in overlap regions = (0.05 /216) <= 0.0002).  Our results showed that in AD vs. HC, there is a two-way interaction between “rFDG × clinical diagnosis” in the right supplementary motor area (SMA) was able to predict CC changes (R2=0.57, F (4,55) = 23.9, p(model) < 0.0001) (Figure 5B). Also, a two-way interaction between “rFDG × clinical diagnosis” in the right planum temporale was able to predict DC changes (R2=0.30, F (4,55) = 7.94, p(model) = 0.00017) (Figure 5C). Adding the interaction term (rFDG × clinical diagnosis), significantly improved the model in right supplementary motor area (∆R2= 0.05, pinteraction= 0.016) and in right planum temporale (∆R2= 0.08, pinteraction= 0.013). None of the model was able to predict changes in CC or DC, when comparing AD vs. MCI or MCI vs HC in non-overlapping regions. Figures 5 (B and C) illustrates the interactive effects of rFDG and clinical diagnosis in predicting FC topological metrics of right SMA and right planum temporale respectively. We found that in right SMA CC decreases as a function of rFDG in HC subjects, but increases in AD patients and in right planum temporale DC decreases as a function of rFDG in HC subjects, but increases in AD patients.

Section2 : The findings of control analysis to investigate the moderator effect of disease on the link between regional glucose metabolism and the inter-regional FC topological metrics in the non-overlapping regions

In non-overlapping regions, we also explored the interaction effect between CC/DC and clinical diagnosis using HMMR analysis to predict changes in rFDG while correcting for multiple comparisons by using family wise error rate correction (p-value < 0.05 / number of all interaction tests in overlap regions = (0.05 /216) <= 0.0002). Results showed that in AD vs. HC, a two-way interaction between “CC × clinical diagnosis” in the left middle temporal gyrus (R2=0.58, F (5,53) =25.1, p(model) < 0.0001), the left anterior supramarginal gyrus (R2=0.34, F (4,55) = 9.45, p(model) < 0.0001) and the left posterior supramarginal gyrus (R2=0.53, F (4,55) = 20.7, p(model) < 0.0001) were able to significantly predict changes in rFDG . SI-Figure 1 illustrates the interactive effects of CC and clinical diagnosis in predicting rFDG.  Adding the interaction term (CC × clinical diagnosis), significantly improved the model for the left middle temporal gyrus (∆R2= 0.05, pinteraction = 0.018), in left anterior supramarginal gyrus (∆R2= 0.06, pinteraction = 0.025) and in left posterior supramarginal gyrus (∆R2= 0.04 pinteraction=0.037). In all these three regions, we found that rFDG and CC changes were congruent in HC subjects, but this pattern was incongruent in AD patients (i.e., increased rFDG, decreased CC).

Moreover, comparing AD vs. HC group comparison, a two-way interaction between “DC × clinical diagnosis” in the left middle temporal gyrus (temporooccipital part) (R2=0.6, F (4,55) = 27.9, p(model) < 0.0001), the left posterior supramarginal gyrus (R2=0.55, F (4,55) = 22.9, p(model) < 0.0001) and the left posterior parahippocampal gyrus (R2=0.3, F (4,55) = 7.88, p(model) = 0.0002) were able to significantly predict changes in rFDG. SI-Figure 2 indicates the interactive effects of DC and clinical diagnosis in predicting rFDG. Adding the interaction term (DC × clinical diagnosis) interaction, significantly improved the model for the left middle temporal gyrus (temporooccipital part) (∆R2= 0.04, pinteraction = 0.025), in left posterior supramarginal gyrus (∆R2= 0.04, pinteraction = 0.026) and in left posterior parahippocampal gyrus (∆R2= 0.06, pinteraction = 0.034). We also found that in all identified regions rFDG and DC changes were congruent in HC subjects, but incongruent in AD patients (i.e., increased rFDG, decreased DC).

# SI-Figure 1. Schematic diagrams of moderation are illustrated as A) conceptual diagram and B) statistical diagram. C) The effect of moderator variable on the association between variable X and Y. The slop and intercept of the regression of Y on X that depends on the specific value of M (High, Medium, or Low).

SI-Figure 2. AD moderated the effect of the inter-regional FC topological metric, clustering coefficient (CC), on predicting the regional glucose metabolism (rFDG) in non-overlapping regions. Results demonstrated that only in A) Left middle temporal gyrus-temporooccipital part, B) Left anterior supramarginal gyrus and C) Left posterior supramarginal gyrus defined by group comparison between AD and HC, HMMR analysis predicted rFDG significantly with the two-way interaction (CC × clinical diagnosis).  Figures in the first row indicate that rFDG increases as a function of CC in HC subjects, but decreases in AD patients. In the second row, we showed changes in the rFDG in both HC and AD groups, when CC is fixed at three low, medium and high levels. The lines for HC and AD are crossing, indicating an interaction between CC and clinical diagnosis. AD: Alzheimer’s disease; HC: healthy control; HMMR: hierarchical moderated multiple regression.

SI-Figure 3. AD moderated the effect of the inter-regional FC topological metric, degree centrality (DC), on predicting the regional glucose metabolism (rFDG) in non-overlapping regions. Results demonstrated that only in A) Left middle temporal gyrus-temporooccipital part, B) Left posterior supramarginal gyrus and C) Left posterior parahippocampal gyrus defined by group comparison between AD and HC, HMMR analysis predicted rFDG significantly with the two-way interaction (DC × clinical diagnosis). Figures in the first row indicate that rFDG increases as a function of DC in HC subjects, but decreases in AD patients. In the second row, we showed changes in the rFDG in both HC and AD groups, when DC is fixed at three low, medium and high levels. The lines for HC and AD are crossing, indicating an interaction between DC and clinical diagnosis. AD: Alzheimer’s disease; HC: healthy control; HMMR: hierarchical moderated multiple regression.

SI-Table 1. Analysis of covariance on regional glucose uptake for each region in Harvard-Oxford Atlas. Age and sex were considered as the covariates of no-interest. The results were corrected for multiple comparisons using N-region statistical comparison as described previously [14, 15]. The significance threshold for ANCOVA was calculated using 1/number of ROIs, as: p-value <= 1/112=0.009.  Post-hoc test: permutation test (100,000 permutations, the significance threshold for post-hoc test was calculated using 1/number of tests, as p-value =1/(31×3) <=0.01).

| Brain Regions | F-test | HC | MCI | AD | Post-hoc |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | p-value |  |  |  | Group comparison | p-value |
| Precentral Gyrus (L) | 0.0006 |  |  |  | HC<AD MCI<AD | 0.007 0.0004 |
| Precentral Gyrus (R) | <0.0001 |  |  |  | HC<AD MCI<AD | <0.0001 0.0003 |
| Superior Temporal Gyrus, posterior (L) | 0.001 | 1.09±0.07 | 1.08±0.1 | 1.03±0.14 | HC>AD | 0.0002 |
| Middle Temporal Gyrus, posterior (L) | <0.0001 |  |  |  | HC>AD MCI>AD | 0 0.005 |
| Middle Temporal Gyrus, temporooccipital (L) | <0.0001 |  |  |  | HC>AD MCI>AD | 0 0.002 |
| Middle Temporal Gyrus, temporooccipital (R) | 0.0003 | 1.01±0.12 | 0.98±0.12 | 0.9±0.16 | HC>AD | <0.0001 |
| Inferior Temporal Gyrus, posterior (L) | 0.004 |  |  |  | HC>AD | 0.0002 |
| Inferior Temporal Gyrus, temporooccipital (L) | <0.0001 |  |  |  | HC>AD MCI>AD | 0 0.0002 |
| Inferior Temporal Gyrus, temporooccipital (R) | 0.0006 |  |  |  | HC>MCI HC>AD | 0.007 <0.0001 |
| Superior Parietal Lobule(L) | 0.007 | 0.94±0.12 | 0.86±0.15 | 0.82±0.18 | HC>AD | 0.0007 |
| Supramarginal Gyrus, posterior (L) | <0.0001 | 1.08±0.12 | 1.04±0.12 | 0.98±0.14 | HC>MCI HC>AD | 0.01 0 |
| Angular Gyrus (L) | <0.0001 |  |  |  | HC>MCI HC>AD MCI>AD | 0.003 0 0.002 |
| Angular Gyrus (R) | <0.0001 |  |  |  | HC>AD MCI>AD | 0 0.008 |
| Lateral Occipital Cortex, superior (L) | <0.0001 |  |  |  | HC>AD MCI>AD | 0 0.0008 |
| Lateral Occipital Cortex, superior (R) | 0.0004 |  |  |  | HC>AD | <0.0001 |
| Lateral Occipital Cortex, inferior (L) | 0.0009 | 0.9±0.11 | 0.89±0.1 | 0.82±0.16 | HC>AD MCI>AD | 0.0002 0.01 |
| Intracalcarine Cortex (R) | 0.008 | 1.41±0.1 | 1.47±0.17 | 1.53±0.21 | HC<AD | 0.0004 |
| Precuneous Cortex (L) | 0.003 |  |  |  | HC>AD | 0.0008 |
| Precuneous Cortex (R) | 0.0005 |  |  |  | HC>AD | <0.0001 |
| Parahippocampal Gyrus, posterior (L) | 0.007 |  |  |  | HC<AD | 0.0007 |
| Parahippocampal Gyrus, posterior (R) | 0.001 |  |  |  | HC<MCI HC<AD | 0.001 0.0002 |
| Lingual Gyrus (L) | 0.0001 |  |  |  | HC<MCI HC<AD | 0.002 <0.0001 |
| Lingual Gyrus (R) | 0.0002 |  |  |  | HC<MCI HC<AD | 0.002 <0.0001 |
| Temporal Occipital Fusiform Cortex (L) | 0.0007 |  |  |  | HC<MCI HC<AD | 0.0008 <0.0001 |
| Temporal Occipital Fusiform Cortex (R) | 0.0005 |  |  |  | HC<MCI HC<AD | <0.0001 0.0003 |
| Occipital Fusiform Gyrus (L) | <0.0001 |  |  |  | HC<MCI HC<AD | <0.0001 <0.0001 |
| Occipital Fusiform Gyrus (R) | 0.001 |  |  |  | HC<MCI HC<AD | <0.0001 0.001 |
| Brain-Stem(L) | 0.0009 |  |  |  | HC<AD MCI<AD | 0.0003 0.008 |
| Brain-Stem(R) | 0.002 |  |  |  | HC<AD | 0.0005 |
| Putamen (L) | 0.0006 |  |  |  | HC<AD MCI<AD | <0.0001 0.005 |
| Hippocampus (R) | 0.001 |  |  |  | HC<AD | 0.0002 |

AD: Alzheimer’s disease; HC: healthy control; MCI: mild cognitive impairment; SD: standard deviation; L: left; R: right.

SI-Table 2. Analysis of covariance on clustering coefficient as a inter-regional FC topological metric for each region in Harvard-Oxford Atlas. Age and sex were considered as the covariates of no-interest. The results were corrected for multiple comparisons using N-region statistical comparison as described previously [14, 15]. The significance threshold for ANCOVA was calculated using 1/number of ROIs, as: p-value <= 1/112=0.009.  Post-hoc test: permutation test (100,000 permutations, the significance threshold for post-hoc test was calculated using 1/number of tests, as: p-value =1/(43×3) <=0.0077).

| Brain Regions | F-test | HC | MCI | AD | Post-hoc |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | p-value |  |  |  | Group comparison | p-value |
| Insular Cortex (R) | 0.0004 |  |  |  | HC>AD MCI>AD | 0.0001 0.004 |
| Middle Frontal Gyrus (L) | 0.002 |  |  |  | HC>AD MCI>AD | 0.0005 0.005 |
| Middle Frontal Gyrus (R) | 0.0009 |  |  |  | HC>AD MCI>AD | 0.0004 0.002 |
| Inferior Frontal Gyrus, pars triangularis (L) | 0.003 |  |  |  | HC>AD MCI>AD | 0.001 0.002 |
| Inferior Frontal Gyrus, pars triangularis (R) | 0.002 |  |  |  | HC>AD MCI>AD | 0.0007 0.006 |
| Inferior Frontal Gyrus, pars opercularis (L) | 0.007 |  |  |  | HC>AD MCI>AD | 0.003 0.004 |
| Inferior Frontal Gyrus, pars opercularis (R) | 0.002 |  |  |  | HC>AD | 0.0002 |
| Precentral Gyrus (L) | 0.0001 |  |  |  | HC>MCI HC>AD | 0.004 <0.0001 |
| Precentral Gyrus (R) | 0.0002 |  |  |  | HC>AD | <0.0001 |
| Temporal Pole (R) | 0.003 |  |  |  | HC>AD MCI>AD | 0.0009 0.006 |
| Superior Temporal Gyrus, anterior (L) | 0.0007 |  |  |  | HC>AD | 0.0002 |
| Superior Temporal Gyrus, posterior (L) | 0.0009 |  |  |  | HC>AD MCI>AD | 0.0003 0.003 |
| Superior Temporal Gyrus, posterior (R) | 0.0004 |  |  |  | HC>AD | <0.0001 |
| Middle Temporal Gyrus, anterior (R) | 0.0002 |  |  |  | HC>AD | <0.0001 |
| Inferior Temporal Gyrus, temporooccipital (L) | 0.002 |  |  |  | HC>AD | 0.0002 |
| Inferior Temporal Gyrus, temporooccipital (R) | 0.001 |  |  |  | HC>AD | 0.0002 |
| Postcentral Gyrus (L) | <0.0001 |  |  |  | HC>AD MCI>AD | 0 0.007 |
| Postcentral Gyrus (R) | 0.0001 |  |  |  | HC>AD MCI>AD | <0.0001 0.002 |
| Superior Parietal Lobule (L) | <0.0001 |  |  |  | HC>MCI HC>AD | 0.007 <0.0001 |
| Superior Parietal Lobule (R) | 0.0003 |  |  |  | HC>AD | <0.0001 |
| Supramarginal Gyrus, anterior (L) | 0.0008 |  |  |  | HC>AD | 0.0002 |
| Supramarginal Gyrus, anterior (R) | 0.001 |  |  |  | HC>AD | 0.0002 |
| Supramarginal Gyrus, posterior (R) | 0.003 |  |  |  | HC>AD | 0.0005 |
| Angular Gyrus (L) | 0.008 | 0.3±0.12 | 0.29±0.07 | 0.23±0.07 | HC>AD MCI>AD | 0.004 0.003 |
| Lateral Occipital Cortex, superior (R) | 0.0007 |  |  |  | HC>AD | 0.0002 |
| Lateral Occipital Cortex, inferior (R) | 0.0006 |  |  |  | HC>AD MCI>AD | 0.0002 0.002 |
| Frontal Medial Cortex (L) | 0.006 |  |  |  | HC>AD | 0.0007 |
| Supplementary Motor Area (L) | 0.005 |  |  |  | HC>AD | 0.001 |
| Supplementary Motor Area (R) | <0.0001 |  |  |  | HC>MCI HC>AD MCI>AD | 0.005 0 0.002 |
| Frontal Orbital Cortex (R) | 0.0017 |  |  |  | HC>AD | 0.0003 |
| Temporal Occipital Fusiform Cortex (R) | 0.0002 |  |  |  | HC>AD | <0.0001 |
| Occipital Fusiform Gyrus (R) | 0.0023 |  |  |  | HC>AD | 0.0009 |
| Frontal Operculum Cortex (R) | 0.0003 |  |  |  | HC>AD MCI >AD | <0.0001 0.002 |
| Central Opercular Cortex (L) | <0.0001 |  |  |  | HC>AD | <0.0001 |
| Central Opercular Cortex (R) | <0.0001 |  |  |  | HC>MCI HC>AD | 0.003 0 |
| Parietal Operculum Cortex (L) | 0.0002 |  |  |  | HC>AD | 0.0001 |
| Parietal Operculum Cortex (R) | 0.0018 |  |  |  | HC>AD | 0.0004 |
| PlanumPolare (L) | 0.0008 |  |  |  | HC>AD MCI>AD | 0.0001 0.007 |
| PlanumPolare (R) | 0.0005 |  |  |  | HC>AD | <0.0001 |
| Heschls Gyrus (includes H1 and H2) (L) | 0.001 |  |  |  | HC>AD | 0.0002 |
| Heschls Gyrus (includes H1 and H2) (R) | <0.0001 |  |  |  | HC>MCI HC>AD | 0.004 0 |
| PlanumTemporale (L) | 0.0005 |  |  |  | HC>AD | 0.0002 |
| PlanumTemporale (R) | 0.0001 |  |  |  | HC>AD | <0.0001 |

AD: Alzheimer’s disease; HC: healthy control; MCI: mild cognitive impairment; SD: standard deviation; L: left; R: right.

SI-Table 3. Analysis of covariance on degree centrality as a inter-regional FC topological metric for each region in Harvard-Oxford Atlas. Age and sex were considered as the covariates of no-interest. The results were corrected for multiple comparisons using N-region statistical comparison as described previously [14, 15]. The significance threshold for ANCOVA was calculated using 1/number of ROIs, as: p-value <= 1/112=0.009.  Post-hoc test: permutation test (100,000 permutations, the significance threshold for ANCOVA was calculated using 1/number of test, as: p-value =1/(22×3) <=0.015).

| Brain Regions | F-test | HC | MCI | AD | Post-hoc |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | p-value |  |  |  | Group comparison | p-value |
| Insular Cortex (R) | 0.0001 | 20.49±8.69 | 12.74±5.07 | 11.79±5.81 | HC>MCI HC>AD | 0.004 0.0006 |
| Precentral Gyrus (L) | 0.003 | 25.6±9.23 | 21.64±7.37 | 17.11±7.57 | HC>AD MCI>AD | 0.001 0.01 |
| Precentral Gyrus (R) | 0.005 | 24.77±8.93 | 20.25±6.32 | 16.85±7 | HC>MCI HC>AD | 0.004 0.001 |
| Postcentral Gyrus (R) | 0.002 | 23.72±9.09 | 19.07±5.05 | 16.07±7.09 | HC>AD | 0.001 |
| Superior Parietal Lobule (R) | 0.0008 | 18.4±6.95 | 13.44±6.21 | 11.07±6.4 | HC>AD | 0.0002 |
| Supramarginal Gyrus, anterior (R) | 0.008 | 16.36±5.87 | 13.4±6.5 | 12.1±4.87 | HC>AD | 0.0006 |
| Lateral Occipital Cortex, inferior (L) | 0.0009 | 22.43±10.05 | 14.96±5.13 | 14.41±6.81 | HC>MCI HC>AD MCI>AD | 0.007 0.0005 0.002 |
| Lateral Occipital Cortex, inferior (R) | 0.0002 | 19.78±8.51 | 12.81±6.83 | 13.63±6.76 | HC>MCI HC>AD | 0.008 0.0003 |
| Supplementary Motor Area (L) | 0.0002 | 21.22±7.81 | 16.27±5.46 | 13.48±7.51 | HC>MCI HC>AD | 0.002 <0.0001 |
| Cuneal Cortex (R) | 0.005 | 19.93±8.2 | 12.57±6.18 | 13.1±6.07 | HC>MCI HC>AD | 0.005 0.003 |
| Parahippocampal Gyrus, anterior (R) | 0.008 | 6.12±4.8 | 10.63±3.51 | 10.29±5.98 | HC<MCI HC<AD | 0.002 0.004 |
| Parahippocampal Gyrus, posterior (L) | 0.006 | 5.03±5.96 | 6.24±8.34 | 7.32±4.58 | HC<AD | 0.0007 |
| Central Opercular Cortex (R) | 0.0004 | 24.81±9.75 | 17.31±6.79 | 14.62±6.8 | HC>AD | <0.0001 |
| Parietal Operculum Cortex (L) | 0.0004 | 22.57±7.3 | 18.45±6.35 | 15.2±5.89 | - | - |
| Parietal Operculum Cortex (R) | <0.0001 | 22.67±7.74 | 14.81±6.11 | 13.68±6.33 | HC>MCI HC>AD | 0.0007 <0.0001 |
| PlanumPolare (L) | 0.008 | 15.17±7.7 | 11.61±7.18 | 10.46±4.92 | HC>AD | 0.001 |
| PlanumPolare (R) | 0.0005 | 18.85±7.5 | 11.8±7.28 | 11.83±5.06 | HC>MCI HC>AD | 0.002 0.0002 |
| Heschls Gyrus (includes H1 and H2) (L) | 0.0008 | 20.34±9.1 | 14.06±7.68 | 12.12±4.74 | HC>AD | 0.0001 |
| Heschls Gyrus (includes H1 and H2) (R) | 0.004 | 18.44±9.2 | 13.62±7.27 | 11.13±6.27 | HC>AD | 0.0006 |
| PlanumTemporale (R) | 0.001 | 21.55±9.88 | 15.74±4.17 | 12.12±4.84 | HC>AD | 0.0001 |
| Brain-Stem(L) | 0.007 | 6.04±6.5 | 5.67±3.86 | 9.78±5.6 | MCI<AD | 0.0005 |
| Brain-Stem(R) | 0.006 | 6.4±6.27 | 5.42±3.7 | 10.01±4.99 | HC<AD MCI<AD | 0.0003 |

AD: Alzheimer’s disease; HC: healthy control; MCI: mild cognitive impairment; SD: standard deviation; L: left; R: right.

References:

1.	Yan CG, Wang XD, Zuo XN, Zang YF. DPABI: Data Processing & Analysis for (Resting-State) Brain Imaging. Neuroinformatics. 2016;14:339-51. doi:10.1007/s12021-016-9299-4.

2.	Lowe MJ, Russell DP. Treatment of baseline drifts in fMRI time series analysis. J Comput Assist Tomogr. 1999;23:463-73.

3.	Lu H, Zuo Y, Gu H, Waltz JA, Zhan W, Scholl CA, et al. Synchronized delta oscillations correlate with the resting-state functional MRI signal. Proceedings of the National Academy of Sciences of the United States of America. 2007;104:18265-9. doi:10.1073/pnas.0705791104.

4.	Kelly AM, Uddin LQ, Biswal BB, Castellanos FX, Milham MP. Competition between functional brain networks mediates behavioral variability. NeuroImage. 2008;39:527-37. doi:10.1016/j.neuroimage.2007.08.008.

5.	Satterthwaite TD, Elliott MA, Gerraty RT, Ruparel K, Loughead J, Calkins ME, et al. An improved framework for confound regression and filtering for control of motion artifact in the preprocessing of resting-state functional connectivity data. Neuroimage. 2013;64:240-56. doi:10.1016/j.neuroimage.2012.08.052.

6.	Murphy K, Fox MD. Towards a consensus regarding global signal regression for resting state functional connectivity MRI. NeuroImage. 2017;154:169-73. doi:10.1016/j.neuroimage.2016.11.052.

7.	Mevel K, Desgranges B, Baron JC, Landeau B, De la Sayette V, Viader F, et al. Detecting hippocampal hypometabolism in Mild Cognitive Impairment using automatic voxel-based approaches. Neuroimage. 2007;37:18-25. doi:10.1016/j.neuroimage.2007.04.048.

8.	Saramäki J, Kivelä M, Onnela JP, Kaski K, Kertész J. Generalizations of the clustering coefficient to weighted complex networks.

. Physical review E, Statistical, nonlinear, and soft matter physics. 2007;E 75:027105.

9.	Rubinov M, Sporns O. Weight-conserving characterization of complex functional brain networks. Neuroimage. 2011;56:2068-79. doi:10.1016/j.neuroimage.2011.03.069.

10.	van den Heuvel MP, Hulshoff Pol HE. Exploring the brain network: a review on resting-state fMRI functional connectivity. Eur Neuropsychopharmacol. 2010;20:519-34. doi:10.1016/j.euroneuro.2010.03.008.

11.	Sporns O. Network attributes for segregation and integration in the human brain. Curr Opin Neurobiol. 2013;23:162-71. doi:10.1016/j.conb.2012.11.015.

12.	Aiken LS, & West, S. G. Multiple regression: Testing and interpreting interactions. Thousand Oaks, CA,US: Sage Publications, Inc.; 1991.

13.	Whisman MA, McClelland GH. Designing, testing, and interpreting interactions and moderator effects in family research. J Fam Psychol. 2005;19:111-20. doi:10.1037/0893-3200.19.1.111.

14.	Lynall ME, Bassett DS, Kerwin R, McKenna PJ, Kitzbichler M, Muller U, et al. Functional connectivity and brain networks in schizophrenia. The Journal of neuroscience : the official journal of the Society for Neuroscience. 2010;30:9477-87. doi:10.1523/JNEUROSCI.0333-10.2010.

15.	Meng C, Brandl F, Tahmasian M, Shao J, Manoliu A, Scherr M, et al. Aberrant topology of striatum's connectivity is associated with the number of episodes in depression. Brain : a journal of neurology. 2014;137:598-609. doi:10.1093/brain/awt290.