# Supplementary information

Efficient PFAS detection in water associated with ice rinks: On-line SPE and LC-QTOF mass spectrometry for targeted and suspect screening in a single run

Yanan Lia, Roxana Sühringa*

a Department of Chemistry and Biology, Toronto Metropolitan University, Toronto Ontario, M5B 2K3, Canada

*Corresponding author:

E-mail address: roxana.suehring@torontomu.ca (Roxana Sühring)

# Table of contents

Table S1. Chemical details of the 24 targeted PFAS	3

Table S2. Quantifier and qualifier ions of 24 targeted PFAS by on-line SPE LC-QTOF	7

Table S3. Recovery of isotope-labelled PFAS passing PES filter	8

Table S4. Modified L16(46) array of MS ionisation parameters for optimisation	8

Table S5. Rapid suspect screening results of PFAS for ice rink water samples	11

Fig. S1. Results of 28 targeted PFAS from the optimisation of six MS parameters	9

Fig. S2. Results of 28 targeted PFAS from the optimisation of five MS parameters .	10

Table S1. Chemical details of the 24 targeted PFAS

| Full names | Acronym | CAS | Chemical structure | Log P | pKa |
| --- | --- | --- | --- | --- | --- |
| Perfluoropentanoic acid | PFPeA | 2706-90-3 |  | 3.414a | 0.400±0.10a 2.271 |
| Perfluoro(4-methoxybutanoic) acid | PFMBA | 863090-89-5 |  | 4.021a | 0.507±0.10a |
| Perfluorobutanesulfonic acid | PFBS | 375-73-5 |  | 1.946a | –3.574±0.50a 0.142 −5.5–0.143 |
| Perfluoro(2-ethoxyethane) sulfonic acid | PFEESA | 113507-82-7 |  | 2.595a | –3.250±0.50a |
| Perfluoro-3,6-dioxaheptanoic acid | 3,6-OPFHpA | 151772-58-6 |  | 5.141a | 0.198±0.10a |
| 4:2 Fluorotelomer sulfonic acid | 4:2 FTS | 757124-72-4 |  | 1.166a | 1.249±0.50a |
| Perfluorohexanoic acid | PFHxA | 307-24-4 |  | 3.906 a | 0.417±0.10a 2.171 −0.162 −0.16–0.93 0.43–0.644 |
| Perfluoro-1-pentanesulfonic acid; | PFPeS | 2706-91-4 |  | 2.429a | –3.561±0.50a |
| 2,3,3,3-Tetra-2-(1,1,2,2,3,3,3-heptafluoropropoxy)propanoic acid | HFPO-DA | 13252-13-6 |  | 5.099a | –1.356±0.10a 2.845 |
| Perfluoroheptanoic acid | PFHpA | 375-85-9 |  | 4.870a | 0.470±0.10a 2.261 −0.19–−0.153 0.74–0.924 |
| Perfluorohexanesulfonic acid | PFHxS | 355-46-4 |  | 2.976a | –3.339±0.50a 0.142 –5.5–0.143 |
| Dodecafluoro-3H-4,8-dioxanonanoic acid | DONA | 919005-14-4 |  | 5.782a | 0.567±0.10a |
| Perfluoro-1-(perfluoroethyl)cyclohexanesulfonic acid | PFECHS | 646-83-3 |  | 3.457a | –3.565±0.70a |
| 6:2 Fluorotelomer sulfonic acid | 6:2 FTS | 27619-97-2 |  | 2.501a | 1.305±0.50a |
| Perfluorooctanoic acid | PFOA | 335-67-1 |  | 5.576a | 0.504±0.10a 2.86 2.57 1.31–2.88 <1–3.89 −3.3210 |
| Perfluoro-1-heptanesulfonic acid | PFHpS | 375-92-8 |  | 3.617a | −3.290±0.50a |
| Perfluorohexanesulfonamide | FHxSA-I | 41997-13-1 |  | 3.676a | 6.939±0.60a |
| Perfluorooctanesulfonic acid | PFOS | 1763-23-1 |  | 4.175a | −3.272±0.50a −3.272 −4.2010 −5.5–0.143 |
| Perfluorononanoic acid | PFNA | 375-95-1 |  | 6.598a | 0.516±0.10a 2.581 −0.21–0.173 |
| 9-Chlorohexadecafluoro-3-Oxanone-1-Sulfonic Acid | 9Cl-PF3ONS | 756426-58-1 |  | 5.706a | −3.172±0.50a |
| Perfluorononanesulfonic acid | PFNS | 68259-12-1 |  | 4.854a | −3.265±0.50a |
| 8:2 Fluorotelomer sulfonic acid | 8:2 FTS | 39108-34-4 |  | 3.658a | 1.316±0.50a |
| Perfluorodecanoic acid | PFDA | 335-76-2 |  | 7.389a | 0.520±0.10a 2.611 −0.42–2.63 |
| Perfluoroundecanoic acid | PFUnDA | 2058-94-8 |  | 7.995a | 0.522±0.10a 3.131 −0.39–2.73 |

| a | Predicated values were extracted from SciFinder database and calculated using Advanced Chemistry Development (ACD/Labs) Software (© 1994-2025 ACD/Labs) |
| --- | --- |

Table S2. Quantifier and qualifier ions of 24 targeted PFAS by on-line SPE LC-QTOF

| Acronym | Formula | Quantifier ion | Qualifier ion | Isotope labelled internal standard |
| --- | --- | --- | --- | --- |
| PFPeA | C5HF9O2 | 262.9778 | 263.9811 | 13C5-PFPeA |
| PFMBA | C5HF9O3 | 278.9704 | 280.9839 | 13C3-PFBS |
| PFBS | C4HF9O3S | 298.9448 | 299.9476 | 13C5-PFPeA |
| PFEESA | C4HF9O4S | 314.9400 | 315.9428 | 13C3-PFBS |
| 3,6-OPFHpA | C5HF9O4 | 294.9667 | 295.9700 | 13C5-PFHxA |
| 4:2 FTS | C₆H₅F₉O₃S | 326.9750 | 327.9779 | 13C2-4:2FTS |
| PFHxA | C6HF11O2 | 312.9753 | 313.9785 | 13C5-PFHxA |
| PFPeS | C5HF11O3S | 348.9419 | 349.9445 | 13C3-PFHxS |
| HFPO-DA | C6HF11O3 | 284.9797 | 285.9831 | 13C3-HFPO-DA |
| PFHpA | C7HF13O2 | 362.9725 | 363.9757 | 13C4-PFHpA |
| PFHxS | C6HF13O3S | 398.9404 | 399.9433 | 13C3-PFHxS |
| DONA | C7H2F12O4 | 376.9723 | 377.9755 | 13C4-PFHpA |
| PFECHS | C8HF15O3S | 460.9364 | 461.9392 | 13C3-PFHxS |
| 6:2 FTS | C8H5F13O3S | 426.9698 | 427.9730 | 13C2-6:2FTS |
| PFOA | C8HF15O2 | 412.9684 | 413.9714 | 13C8-PFOA |
| PFHpS | C7HF15O3S | 448.9355 | 449.9382 | 13C3-PFHxS |
| FHxSA-I | C6H2F13NO2S | 397.9527 | 398.9570 | 13C8-PFOA |
| PFOS | C8HF17O3S | 498.9328 | 499.9359 | 13C8-PFOS |
| PFNA | C9HF17O2 | 462.9651 | 463.9684 | 13C9-PFNA |
| 9Cl-PF3ONS | C8ClF16O4S | 530.8966 | 532.8935 | 13C8-PFOS |
| PFNS | C9HF19O3S | 548.9306 | 549.9336 | 13C3-PFHxS |
| 8:2 FTS | C10H5F17O3S | 526.9646 | 527.9680 | 13C2-8:2FTS |
| PFDA | C10HF19O2 | 512.9653 | 513.9684 | 13C6-PFDA |
| PFUnDA | C11HF21O2 | 562.9571 | 563.9604 | 13C7-PFUnDA |

Table S3. Recovery of isotope-labelled PFAS passing PES filter

| PFAS | Peak area of PES filtered (n = 4) |  |  | Peak area of unfiltered (n = 4) |  | Mean recovery (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean | RSD%a | Mean |  | RSD% |  |
| 13C5-PFPeA | 5692636.5 | 14% | 6328132.5 |  | 4% | 90% |
| 13C3-PFBS | 7672025.0 | 8% | 7775087.3 |  | 6% | 99% |
| 13C5-PFHxA | 6161302.0 | 8% | 6138953.3 |  | 4% | 100% |
| 13C3-HFPO-DA | 2237683.3 | 9% | 2393161.0 |  | 5% | 94% |
| 13C4-PFHpA | 5171868.0 | 9% | 5614932.5 |  | 6% | 92% |
| 13C3-PFHxS | 6201562.5 | 10% | 6003007.3 |  | 6% | 103% |
| 13C8-PFOA | 4151644.5 | 15% | 4568418.8 |  | 6% | 91% |
| 13C8-PFOS | 3005866.3 | 18% | 3724659.8 |  | 4% | 81% |
| 13C9-PFNA | 865045.0 | 7% | 785689.8 |  | 5% | 110% |
| 13C6-PFDA | 1988354.3 | 8% | 2250570.5 |  | 4% | 88% |
| 13C7-PFUndA | 1531756.8 | 13% | 1338739.0 |  | 7% | 114% |
| 13C2-PFDoA | 506757.5 | 23% | 528589.5 |  | 16% | 96% |

| a | RSD = relative standard deviation |
| --- | --- |

Table S4. Modified L16(46) array of MS ionisation parameters for optimisation

| Experiments number | Capillary voltage (V) | Drying gas temperature (°C) | Drying gas flow rate (L min−1) | Sheath gas temperature (°C) | Nozzle voltage (V) | Nebulizer pressure (psi) |
| --- | --- | --- | --- | --- | --- | --- |
| Exp 1 | 2000 | 175 | 4 | 250 | 0 | 25 |
| Exp 2 | 2500 | 215 | 7 | 275 | 700 | 35 |
| Exp 3 | 3000 | 165 | 10 | 325 | 1500 | 45 |
| Exp 4 | 3500 | 300 | 13 | 350 | 2000 | 55 |
| Exp 5 | 2000 | 215 | 10 | 325 | 2000 | 35 |
| Exp 6 | 2500 | 265 | 13 | 350 | 0 | 45 |
| Exp 7 | 3000 | 300 | 4 | 250 | 700 | 55 |
| Exp 8 | 3500 | 175 | 7 | 275 | 1500 | 25 |
| Exp 9 | 2000 | 265 | 7 | 325 | 1500 | 55 |
| Exp 10 | 2500 | 300 | 10 | 350 | 2000 | 25 |
| Exp 11 | 3000 | 175 | 13 | 250 | 700 | 35 |
| Exp 12 | 3500 | 215 | 4 | 275 | 1500 | 45 |
| Exp 13 | 2000 | 300 | 13 | 350 | 0 | 25 |
| Exp 14 | 2500 | 175 | 4 | 250 | 700 | 35 |
| Exp 15 | 3000 | 215 | 7 | 275 | 1500 | 45 |
| Exp 16 | 3500 | 265 | 10 | 325 | 2000 | 55 |

Fig. S1. Results of 28 targeted PFAS from the optimisation of six MS parameters (nozzle voltage, sheath gas temperature and flow rate, drying gas temperature and flow rate, nebulizer pressure and capillary voltage).

Fig. S2. Results of 28 targeted PFAS from the optimisation of five MS parameters (sheath gas temperature and flow rate, drying gas temperature and flow rate, nebulizer pressure and capillary voltage) while nozzle voltage kept at 0 V.

Table S5. Rapid suspect screening results of PFAS for ice rink water samples

| Ice rink water types | PFAS detectiona |
| --- | --- |
| Source water (n = 9) | 2-(Perfluooctyl)ethanthiol)*, 3H-Perfluoro-2,2,4,4-tetrahydroxypentane*, Octafluoroadipamide*, 2,2,2-Trifluoroethyl perfluorobutanesulfonate*, PFHxA, PFHpA, N-Methylperfluorooctane sulfonamidoacetic acid* |
| Reverse osmosis treated water (n = 6) | NDb |
| Melt ice water (n = 5) | PFOA, PFHxA, 1-Pentafluoroethylethanol* |

| a | The PFAS outside of the target list used in the quantitative analysis were indicated by asterisk (*) |
| --- | --- |
| b | ND = No detection |

References

1.	R. Čabala, K. Nesměrák and T. Vlasáková, Dissociation constants of perfluoroalkanoic acids, Monatshefte für Chemie - Chemical Monthly, 2017, 148, 1679-1684.

2.	A. Maimaiti, S. Deng, P. Meng, W. Wang, B. Wang, J. Huang, Y. Wang and G. Yu, Competitive adsorption of perfluoroalkyl substances on anion exchange resins in simulated AFFF-impacted groundwater, Chemical Engineering Journal, 2018, 348, 494-502.

3.	H. Chen, Y. Yao, Z. Zhao, Y. Wang, Q. Wang, C. Ren, B. Wang, H. Sun, A. C. Alder and K. Kannan, Multimedia Distribution and Transfer of Per- and Polyfluoroalkyl Substances (PFASs) Surrounding Two Fluorochemical Manufacturing Facilities in Fuxin, China, Environmental Science & Technology, 2018, 52, 8263-8271.

4.	B. Yang, Y. Han, G. Yu, Q. Zhuo, S. Deng, J. Wu and P. Zhang, Efficient removal of perfluoroalkyl acids (PFAAs) from aqueous solution by electrocoagulation using iron electrode, Chemical Engineering Journal, 2016, 303, 384-390.

5.	S. A. Gannon, W. J. Fasano, M. P. Mawn, D. L. Nabb, R. C. Buck, L. W. Buxton, G. W. Jepson and S. R. Frame, Absorption, distribution, metabolism, excretion, and kinetics of 2,3,3,3-tetrafluoro-2-(heptafluoropropoxy)propanoic acid ammonium salt following a single dose in rat, mouse, and cynomolgus monkey, Toxicology, 2016, 340, 1-9.

6.	A. Prior, R. C. Moldovan, J. Crommen, A. C. Servais, M. Fillet, G. J. de Jong and G. W. Somsen, Enantioselective capillary electrophoresis-mass spectrometry of amino acids in cerebrospinal fluid using a chiral derivatizing agent and volatile surfactant, Analytica Chimica Acta, 2016, 940, 150-158.

7.	A. H. Karoyo and L. D. Wilson, Investigation of the Adsorption Processes of Fluorocarbon and Hydrocarbon Anions at the Solid–Solution Interface of Macromolecular Imprinted Polymer Materials, The Journal of Physical Chemistry C, 2016, 120, 6553-6568.

8.	Y. Gong, L. Wang, J. Liu, J. Tang and D. Zhao, Removal of aqueous perfluorooctanoic acid (PFOA) using starch-stabilized magnetite nanoparticles, Science of The Total Environment, 2016, 562, 191-200.

9.	M. Guo, Y. Lyu, T. Xu, B. Yao, W. Song, M. Li, X. Yang, T. Cheng and X. Li, Particle size distribution and respiratory deposition estimates of airborne perfluoroalkyl acids during the haze period in the megacity of Shanghai, Environmental Pollution, 2018, 234, 9-19.

10.	O. Svahn and E. Björklund, Increased electrospray ionization intensities and expanded chromatographic possibilities for emerging contaminants using mobile phases of different pH, Journal of Chromatography B, 2016, 1033-1034, 128-137.