Supplementary information

Effect of soluble phenolic compounds from hydrothermally pretreated wheat straw on cellulose degrading enzymes: Compositional analysis and inhibition experiments data

Anna S. Borisova1, Ville Pihlajaniemi1,3, Riin Kont2, Klaus Niemelä1, Taru Koitto1,4, Atte Mikkelson1, Priit Väljamäe2, Kristiina Kruus1,5, Kaisa Marjamaa1

1VTT Technical Research Centre of Finland Ltd, P.O. Box 1000, 02044 VTT, Finland 2Institute of Molecular and Cell Biology, University of Tartu, Riia 23b - 202, 51010 Tartu, Estonia

3current address: eniferBio Oy, Peuraniitty 5 B 25, 02750 Espoo, Finland

4current address: Aalto University, School of Chemical Technology, Department of Bioproducts and Biosystems, Espoo, Finland, P.O. Box 16300, FI-00076 AALTO, Finland.

5current address: Aalto University School of Chemical engineering, P.O. Box 16100. FI-00076 AALTO, Finland

Corresponding author: Anna S. Borisova (anna.borisova@vtt.fi)

Abstract

The data provides additional information on inhibition experiments of purified cellulases and commercial cellulolytic cocktails by hydrothermally pretreated wheat straw liquor and its fractions, described in [1]. The data includes information on enzymes used in experiments and compositional analysis of raw material: chopped wheat straw, pretreated wheat straw solids and pretreatment liquor (PL), analysed with HPLC. Calculation data of correlation factor for isolated phenolic fraction (Phe-Fr) and effect of phenolics in reducing sugar assay method (PAHBAH) [2] are shown, as well as chromatograms for separation on a DB-5 column of the Phe-Fr trimethylsilylated compounds, used in GC/MS analysis [1]. Data on hydrolysis of Phe-Fr carbohydrates by Cellic Ctec2 is presented, quantified with PAHBAH assay. Data on inhibitory power of PL and its fractions, including isolated Phe-Fr and enzymatically pretreated fraction CTec-PL, is presented as IC50 values for the inhibition of Celluclast 1.5L and Cellic Ctec2, and two purified CBHs (TrCel7A and TrCel6A) acting on 14C-BMCC substrate. Data on tannic acid inhibition of purified cellulases and CellicCtec2 represents effect of high molecular weight phenols in Phe-Fr and is quantified with PAHBAH assay.

Table S1. Molar extinction coefficients and masses of the enzymes used in screening for inhibitory effects of phenolic fraction (Phe-Fr).

| Enzyme | MW (kDa) | Molar extinction coefficient, M−1⋅cm−1 |
| --- | --- | --- |
| TrCel7A (CBH1) | 55.8 | 83 000 |
| TrCel6A (CBHII) | 56.7 | 104 000 |
| TrCel7B (EGI) | 51.9 | 70 000 |
| TrCel5A (EGII) | 48.2 | 91 000 |
| TrCel12A (EGIII) | 25.1 | 73 500 |
| TrCel45A (EG5) | 24.4 | 48 900 |
| TrCel11 (Xyn2) | 20.8 | 58 330 |
| AnCel3A | 115.6 | 180 000 |

Table S2. Compositions of chopped wheat straw, pretreated wheat straw solids and pretreatment liquor (PL). Chopped wheat straw was pre-soaked with water to 50% DM content and loaded into a 30-L pressure reactor with a batch size of 1.71 kg (DM). The material was heated to 195 °C with direct steam injection and a pressurized water jacket, and the temperature was maintained for 15 min. After the treatment, the material was quickly cooled to 80 °C with the water jacket, and dissolved material was extracted by pumping 80 °C water through the material bed. The first 6 L of extract was collected and this material is hereafter referred to as pretreatment liquor (PL). The pretreated wheat straw solids were collected manually.

|  | Wheat straw solids, % DM | Pretreated wheat straw solids, % DM | Combined PL1, g/L |
| --- | --- | --- | --- |
| Glucose total1 | 32.24 | 41.47 | 1.42 |
| Glucose monomeric |  |  | 0.19 |
| Xylose total | 14.58 | 7.30 | 12.23 |
| Xylose monomeric |  |  | 2.10 |
| Arabinose total | 1.64 | 0.48 | 0.94 |
| Fructose total |  |  | 0.66 |
| Galactose total | 0.56 | 0.23 | 0.69 |
| Mannose total | 0.40 | 0.23 | 0.41 |
| Total carbohydrates | 49.75 | 49.81 | 15.76 |
| Lignin | 20.30 | 24.50 |  |
| Extractives | 0.88 | 1.15 |  |
| Ash | 6.36 | 5.79 |  |
| 5-Hydroxymethylfurfural |  |  | <0.04 |
| Furfural |  |  | 1.65 |
| Soluble phenolics (UV) |  |  | 4.55 |
| Acetic acid |  |  | 3.99 |

1 Pretreatment liquor from 6 wheat straw batches pretreated in conditions described in the legend and in the Materials and methods [1].

Figure S1. Correlation coefficient between total solids freeze-dried phenolic fraction (Phe-Fr) and UV-phenolics. In combined PL the concentration of UV-phenolics was 4.5 g/L, which corresponds to 9 g/L total solids concentration of freeze-dried phenolic fraction. The correlation factor was calculated by linear regression 0.48 ± 0.07 (g UV-phenolics/g total solids).

Figure S2. Effect of phenolic fraction (Phe-Fr) on the reducing sugars assay (PAHBAH). The PAHBAH assay was carried out for several dilutions of glucose in presence of Phe-Fr total solids 0-0.5 g /L. The values presented in the figure are averages of 3 measurements with standard deviation. In performed experiments phenolic concentration was not greater than 0.2 g/L total solids and the slope was varied insignificantly within a range of standard deviation (see equations next to the legends). Intercept value was correlated well with blank measurement. The PAHBAH assay was thus considered suitable for assays including Phe-Fr within above mentioned limitations.

Figure S3. Separation on a DB-5 column of the trimethylsilylated compounds in the isolated phenolic fraction (Phe-Fr): 1, oxalic acid; 2, 2-furancarboxylic acid; 3-4, isomeric hydroxypyridines; 5, dodecamethylpentasiloxane (from column); 6, guaiacol; 7, benzoic acid; 8, 4-vinylphenol; 9, a cyclohexanedione (dienol); 10, succinic acid; 11, catechol; 12, fumaric acid; 13, 4-hydroxybenzaldehyde; 14, 3-methylcatechol; 15, 4-ethylguaiacol; 16, 4-methylcatechol; 17, syringol; 18, hydroquinone; 10, a hydroxy-methoxyacetophenone (tentative); 20, 4-vinylguaiacol; 21, reductic acid; 22, 2-methylhydroquinone; 23, 1-(4-hydroxyphenyl)ethanol; 24, salicylic acid (internal standard); 25, pyroglutamic acid; 26, vanillin; 27, a hydroxy-dimethoxyacetophenone (tentative; methoxy derivative of peak 19); 28, 3-hydroxybenzoic acid; 29, eugenol (+ unknown); 30, 1-guaiacylethanol; 31, unknown (m/z 281, 100%); 32, unknown phenolic (MW, 282); 33, 3,4,5-trimethoxyphenol (tentative); 34, 3,4-dihydroxybenzaldehyde; 35, acetovanillone; 36, 4-hydroxybenzoic acid; 37, guaiacylacetone; 38, two unknowns co-eluting; 39, 3,4-dihydroxyacetophenone; 40, syringaldehyde; 41, unknown (m/z 169, 100%); 42, 2,3-dihydroxybenzoic acid; 43, unknown phenolic (MW, 296); 44, unknown (m/z 359, 100%); 45, unknown (m/z 373, 100%); 46, vanillic acid; 47, acetosyringone; 48, homovanillic acid; 49, a dihydroxybenzoic acid; 50, a dihydroxyacetophenone (enol); 51, azelaic acid; 52, a dihydroxybenzeneacetic acid (2,3-isomer?); 53, 3,4-dihydroxybenzoic acid; 54, coniferyl alcohol + unknown phenolic; 55, a guaiacyl compound; 56, acetosyringone (enol); 57, unknown catecholic compound (MW, 398); 58, syringic acid; 59, homosyringic acid; 60, a guaiacyl compound; 61, 3-(2,3-dihydroxyphenyl)propanoic acid; 62, 1-guaiacyl-3-hydroxyacetone (keto form); 63, p-coumaric acid; 64, unknown phenolic (MW, 380); 65, sinapyl alcohol; 66, 3,8-dihydroxy-2-methylchromone (alginetin); 67, unknown (m/z 335, 100%; MW, 426); 68, 1-hydroxy-3-syringylacetone (keto form); 69, 1-guaiacyl-3-hydroxyacetone (enol form); 70, syringyl compound (m/z 327, 100%); 71, ferulic acid; 72, 1-hydroxy-3-syringylacetone (enol form); 73, dilignol (m/z 223, 100%); 74, p-coumaric acid ester (m/z 219, 100%; m/z 351, 38%; MW, 454); 75, unknown [m/z 492 (MW), 100%]; 76, dilignol (MW, 566); 77, disaccharide; 78, ferulic acid ester (m/z 249, 100%; m/z 381, 22%; MW, 484); 79, dilignol (MW, 522); 80, syringyl dilignol (m/z 327, 100%); 81, syringyl dilignol (m/z 327, 100%); 82, dilignol (m/z 235, 100%; MW, 510); 83, unknown (m/z 463, 100%; MW, 668); 84, 5-O-p-coumaroylpentose; 85, 4,4´-dihydroxy-3,3´-dimethoxystilbene; 86, dilignol (m/z 265, 100%; MW, 540); 87, p-coumaric acid ester; 88, ferulic acid ester; 89, p-coumaric acid ester; 90, ferulic acid ester; 91, 5-O-feruloylpentose; 92, 5-O-feruloylpentose; 93, 4,4´-dihydroxy-3,3´,5-trimethoxystilbene; 94, dilignol (MW, 708); 95, ferulic acid ester (MW, 678); 96, feruloylpentose (m/z 249, 100%; m/z 381, 29%; MW, 614); 97, feruloylpentose (m/z 249, 100%; m/z 338, 62%; MW, 614); 98, dilignol (MW, 690); 99, ferulic acid ester (m/z 338, 100%; MW, 676); 100, ferulic acid ester (m/z 338, 100%; MW, 588); 101, ferulic acid ester (m/z 338, 100%); 102, 4,4´-dihydroxy-3,3´,5,5´-tetramethoxystilbene; 104-105, syringyl dilignols (m/z 327, 100%); 106, dilignol (m/z 498, 100%); 107, syringyl dilignol; and 108, syringaresinol.

Figure S4: Hydrolysis of phenolic fraction (Phe-Fr) carbohydrates by Cellic Ctec2. Reducing sugars were quantified with PAHBAH assay. Reactions were performed at 45 ˚C in 50 mM NaAc buffer pH 5.0 in duplicates  for 4 or 24 or 48 h. Phe-Fr concentration was 0.25-5 g/L UV-phenolics. Concentration of Cellic Ctec2 was 0.1 g/L.

Table S3. IC50 values for the inhibition of Celluclast 1.5L and Cellic Ctec2, and two purified CBHs (TrCel7A and TrCel6A) acting on 14C-BMCC substrate.

| Enzyme | IC50 value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | volume fraction (%)1 |  |  |  | phenolics (g/L)2 |
|  | PL | Phe-Fr | CTec-PL |  | Phe-Fr |
| TrCel7A | 1.0 ± 0.1 | 3.8 ± 0.4 | 51.2 ± 5.1 |  | 0.17 ± 0.02 |
| TrCel6A | 0.5 ± 0.2 | 2.2 ± 0.3 | 51.9 ± 7.9 |  | 0.10 ± 0.01 |
| Celluclast 1.5L | 2.0 ± 0.4 | 8.0 ± 1.2 | 20.2 ± 1.8 |  | 0.36 ± 0.05 |
| Cellic CTec2 | 5.0 ± 0.8 | 15.3 ± 0.6 | 22.0 ± 7.1 |  | 0.70 ± 0.03 |

1 The volume fractions of Phe-Fr and Ctec-PL were calculated based on their concentration of the UV-phenolics (Table 1). In doing so the concentration of UV-phenolics in PL (4.55 g/L) was taken equal to 100% volume fraction.

2 IC50 value for Phe-Fr expressed in the units g/L UV-phenolics.

Figure S5. Effect of tannic acid on the enzymatic hydrolysis of cellulose. All hydrolysis reactions were performed at 45 ˚C in 50 mM NaAc buffer pH 5.0 with Avicel concentration 10 g/L in duplicates. Tannic acid concentration was 0.1 g/L, corresponding to 10 % w/w HMW phenolics (MW>1700 Da) in Phe-Fr. Concentrations of Cellic Ctec2 and purified enzymes (Tr Cel7A, Tr Ce6A, Tr Cel5A, Tr Cel12A, Tr Cel45A, Tr Xyn11) were 0.1 g/L. The reactions with purified enzymes were supplemented with β-glucosidase An Cel3A (5000 nkat/L). Reducing sugars were quantified with PAHBAH assay.

References

[1] A. S. Borisova, V. Pihlajaniemi, R. Kont, K. Niemelä, T. Koitto, A. Mikkelson, P. Väljamäe, K. Kruus, K. Marjamaa, Effect of soluble phenolic compounds from hydrothermally pretreated wheat straw on cellulose degrading enzymes, Biotechnology for Biofuels. In Press.: 
[2] Lever, M. (1972) ‘A new reaction for colorimetric determination of carbohydrates’, Analytical Biochemistry, 47(1), pp. 273–279. doi: 10.1016/0003-2697(72)90301-6.