Supplementary materials (for Huang et al. “Lake sediment DNA as a proxy in fish population studies: analytical challenges and opportunities”)

ddPCR assays and the data analysis

The ddPCR assays were performed using QX200TM ddPCRTM EvaGreen Supermix (Bio-Rad, CA, USA), following the manufacturer’s instruction. Each ddPCR reaction contains 1x ddPCR EvaGreen Supermix, 0.2 μM teleo_F and teleo_R primers, and DNA isolate of 3 μL in 20 μL reaction. For droplet generation, the 20 μL reaction mix was loaded into a sample well of a DG8TM Cartridge (Bio-Rad) followed by 70 μL of QX200 Droplet Generation Oil for EvaGreen (Bio-Rad) into the oil well, and the mix was partitioned into approx. 15,000-20,000 nL droplets by the QX200 Droplet Generator (Bio-Rad). Droplets were transferred to a well of a 96-well PCR plate followed by sealing, and the thermal cycling. The PCR program for the ddPCR assay was: denature at 95°C for 5 min, followed by 35 cycles of 30s at 95°C and 1 min at 56°C with ramp rate of 2°C per sec (note that ddPCR with EvaGreen supermix does not require elongation process), and finally 5 min at 4°C and 5 min at 90°C for signal stabilization and then hold at 4°C. We ran triplicate PCR reactions for each DNA extract. After amplification, the PCR plate was scanned by the QX200 Droplet Reader (Bio-Rad) to read fluorescence from each droplet. The Bio-Rad QuantaSoft software version 1.7.4 was used to quantify the detection of positive droplets (with target DNA in them), and the fraction of positive droplets was used to calculate target DNA concentration. The PCR triplicates were averaged and the final DNA count was shown as number of copies per μL in a PCR reaction.

The analytical software QuantaSoft combined the positive and negative droplets from the triplicates and analyzed these data together as a single experiment. Random partitioning of target molecules into droplets will lead to some droplets with 2 or even more copies, and the partitioning statistics (Poisson statistics) tells us exactly how many droplets to expect. The results shown in Table S3 are the merged data from PCR triplicates with teleo primers, confidence intervals and final DNA counts in initial DNA extracts. Each ddPCR run had at least three PCR negative controls (NTC), and NTCs from each PCR run were merged and listed in Table S3. Results from extraction blanks (Ex bk) are also presented in Table S3.

Fig S1. Examples of fluorescence amplitude (RFU) of droplets in PCRs with sediment fishDNA in a series of dilutions and with a reference DNA (extracted Salvelinus alpinus DNA). The pink line indicates the threshold to detect fishDNA at 13368 RFU according to the reference; the blue and black dots represent positive and negative signals, respectively. The red numbers are the calculated number of copies of fishDNA per μL in the PCR reaction.

Fig S2. Linear correlations between sedimentary fishDNA counts obtained from ddPCR assays and selected water and sediment chemistry. Pearson’s correlation coefficient (r) and pair-end t-Test are shown.

Table S1. Geochemical properties of surface sediments collected from study lakes

|  |  |  | -------------------------------------------------------- Geochemistry ---------------------------------------------------------- |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Depth | Na |  | Mg | Al | Si | P | S | Cl | K | Ca | Ti | Mn | Fe | Zn | Br | Zr | Pb | LOI |
|  | m | % |  | % | % | % | ppm | % | ppm | % | % | ppm | ppm | % | ppm | ppm | ppm | ppm | % |
| ZF08-1 | 14 | 0.130 |  | 0.149 | 2.462 | 11.929 | 965 | 0.439 | 340 | 0.272 | 1.106 | 1098 | 257 | 3.25 | 62 | 131 | 87 | 94 | 51.5 |
| ZF08-2 | 4 | 0.138 |  | 0.053 | 1.371 | 15.971 | 592 | 0.274 | 203 | 0.128 | 0.455 | 524 | 70180 | 22.99 | 275 | 109 | 62 | 52 | 22.9 |
| ZF09 | 8 | 0.403 |  | 0.418 | 3.107 | 13.617 | 1113 | 0.327 | 236 | 0.688 | 0.757 | 1743 | 877 | 6.98 | 153 | 74 | 129 | 96 | 38.6 |
| ZF10-1 | 23 | 0.227 |  | 0.214 | 1.770 | 15.350 | 870 | 0.262 | 119 | 0.362 | 0.521 | 1293 | 190582 | 3.36 | 113 | 96 | 98 | 97 | 22.7 |
| ZF10-2 | 7.5 | 0.194 |  | 0.127 | 1.784 | 14.269 | 629 | 0.232 | 175 | 0.289 | 0.543 | 949 | 28540 | 16.26 | 365 | 120 | 97 | 74 | 27.4 |
| ZF11-1 | 14 | 0.277 |  | 0.418 | 2.460 | 10.953 | 933 | 0.334 | 269 | 0.720 | 0.751 | 2281 | 2836 | 6.23 | 195 | 46 | 110 | 51 | 44.9 |
| ZF11-2 | 4 | 0.177 |  | 0.084 | 0.786 | 4.546 | 802 | 0.114 | 99 | 0.199 | 0.468 | 681 | 24447 | 35.24 | 273 | 66 | 83 | 32 | 26.4 |
| ZF12-1 | 9 | 0.422 |  | 0.633 | 3.201 | 11.252 | 591 | 0.200 | 128 | 0.877 | 0.751 | 2504 | 1355 | 7.05 | 170 | 37 | 118 | 39 | 32.1 |
| ZF12-2 | 4 | 0.402 |  | 0.460 | 2.650 | 11.035 | 926 | 0.207 | 157 | 0.747 | 0.672 | 1755 | 1964 | 13.85 | 174 | 51 | 106 | 46 | 33.0 |
| ZF13-1 | 9 | 0.213 |  | 0.489 | 2.508 | 10.188 | 919 | 0.391 | 348 | 0.848 | 0.996 | 2086 | 1402 | 4.99 | 192 | 82 | 107 | 83 | 51.0 |
| ZF13-2 | 3 | 0.199 |  | 0.072 | 0.728 | 3.873 | 891 | 0.116 | 99 | 0.197 | 0.671 | 537 | 123909 | 28.56 | 283 | 28 | 67 | 26 | 22.8 |
| ZF14-1 | 8 | 0.468 |  | 0.656 | 3.171 | 16.256 | 714 | 0.275 | 209 | 0.819 | 0.938 | 2769 | 1923 | 5.15 | 199 | 25 | 168 | 29 | 26.3 |
| ZF14-2 | 2 | 0.705 |  | 0.732 | 3.532 | 15.565 | 862 | 0.207 | 82 | 0.830 | 1.046 | 2898 | 2059 | 6.66 | 159 | 41 | 206 | 24 | 29.1 |
| ZF15-1 | 3.5 | 0.097 |  | 0.065 | 0.641 | 4.153 | 2588 | 0.469 | 224 | 0.156 | 1.580 | 473 | 886 | 25.01 | 153 | 77 | 64 | 27 | 44.5 |
| ZF15-2 | 2.5 | 0.295 |  | 0.284 | 1.906 | 17.848 | 1099 | 0.319 | 219 | 0.601 | 1.014 | 1720 | 576 | 4.55 | 96 | 63 | 130 | 31 | 37.3 |
| ZF16-1 | 4.5 | 0.461 |  | 0.425 | 2.760 | 14.306 | 1007 | 0.343 | 228 | 0.831 | 0.908 | 2361 | 584 | 3.92 | 92 | 53 | 139 | 48 | 37.5 |
| ZF16-2 | 3 | 0.286 |  | 0.282 | 1.965 | 14.411 | 764 | 0.486 | 257 | 0.680 | 0.917 | 1893 | 983 | 3.44 | 49 | 54 | 150 | 32 | 48.1 |
| ZF17-1 | 8 | 0.177 |  | 0.135 | 2.049 | 15.390 | 1153 | 0.357 | 218 | 0.284 | 0.876 | 1069 | 2444 | 7.58 | 205 | 56 | 116 | 55 | 40.7 |
| ZF17-2 | 3 | 0.232 |  | 0.203 | 1.863 | 18.477 | 605 | 0.325 | 151 | 0.349 | 1.105 | 1287 | 1635 | 5.92 | 156 | 43 | 114 | 30 | 29.9 |
| ZF19-1 | 6 | 0.179 |  | 0.125 | 1.441 | 22.084 | 758 | 0.327 | 113 | 0.259 | 0.774 | 950 | 109908 | 5.07 | 241 | 79 | 91 | 43 | 25.1 |
| ZF19-2 | 14 | 0.257 |  | 0.168 | 2.383 | 17.651 | 1275 | 0.338 | 160 | 0.354 | 0.852 | 1454 | 2096 | 9.54 | 134 | 103 | 111 | 99 | 32.9 |
| ZF20-1 | 7 | 0.234 |  | 0.153 | 1.621 | 14.542 | 896 | 0.381 | 165 | 0.341 | 0.694 | 1114 | 1493 | 8.31 | 101 | 47 | 102 | 44 | 37.5 |
| ZF20-2 | 2.8 | 0.532 |  | 0.289 | 2.566 | 19.926 | 604 | 0.353 | 129 | 0.547 | 1.004 | 1468 | 658 | 6.58 | 104 | 30 | 149 | 47 | 17.7 |
| ZF21 | 12 | 0.304 |  | 0.207 | 1.810 | 9.557 | 1475 | 0.353 | 130 | 0.378 | 0.681 | 1350 | 2656 | 18.01 | 187 | 47 | 111 | 19 | 34.9 |

Table S2. Estimated marginal means of DNA yields from sediments with varied LOI and different extraction buffers by two-way ANOVA

| Extraction_buffer | LOI | n | Mean | 95% Confidence interval |
| --- | --- | --- | --- | --- |
| buffer 1 | ca. 30% LOI | 3 | 2155.00 | 1852.24 to 2457.76 |
|  | ca. 60% LOI | 3 | 1915.00 | 1612.24 to 2217.76 |
|  | <5% LOI | 3 | 2465.00 | 2162.24 to 2767.76 |
| buffer 2 | ca. 30% LOI | 3 | 1823.33 | 1520.57 to 2126.10 |
|  | ca. 60% LOI | 3 | 2101.67 | 1798.90 to 2404.43 |
|  | <5% LOI | 3 | 2265.00 | 1962.24 to 2567.76 |
| buffer 3 | ca. 30% LOI | 3 | 0.00 | -302.76 to 302.76 |
|  | ca. 60% LOI | 3 | 0.00 | -302.76 to 302.76 |
|  | <5% LOI | 3 | 0.00 | -302.76 to 302.76 |
| PowerSoil | ca. 30% LOI | 3 | 1310.00 | 1007.24 to 1612.76 |
|  | ca. 60% LOI | 3 | 1155.00 | 852.24 to 1457.76 |
|  | <5% LOI | 3 | 2586.67 | 2283.90 to 2889.43 |

Table S3. Experimental settings for ddPCR with EveGreen Supermix, DNA counts with teleo primers from PCR triplicates, number of positive, negative and total droplets accepted for analysis. PoissonConfMax and PoissonConfMin indicate the Posisson 95% confidence intervals. Final DNA copies in extract were calculated based on volumes of PCR reaction and DNA extract.

|  | Sample ID | Experiment | Supermix | Target | Concentration (copies/ul) | PoissonConfMax | PoissonConfMin | Positives Droplets | Negatives Droplets | Accepted Droplets | Concentration (copies/ul extract) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZF08-1 | 1 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.97 | 1.27 | 0.72 | 48 | 58202 | 58250 | 743.67 |
|  | 2 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.05 | 0.15 | 0.01 | 2 | 51471 | 51473 | 38.33 |
|  | 3 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 1.2 | 1.55 | 0.9 | 52 | 51140 | 51192 | 920.00 |
| ZF08-2 | 4 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.069 | 0 | 0 | 51392 | 51392 | 0.00 |
|  | 5 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.067 | 0 | 0 | 52290 | 52290 | 0.00 |
|  | 6 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.066 | 0 | 0 | 53593 | 53593 | 0.00 |
| ZF09 | 8 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.82 | 1.11 | 0.58 | 37 | 53358 | 53395 | 628.67 |
|  | 9 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 1.63 | 2.03 | 1.28 | 72 | 52008 | 52080 | 1249.67 |
|  | 10 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 1.45 | 1.84 | 1.13 | 65 | 52617 | 52682 | 1111.67 |
| ZF10-1 | 12 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.02 | 0.12 | 0 | 1 | 47565 | 47566 | 15.33 |
|  | 13 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.05 | 0.16 | 0.01 | 2 | 45912 | 45914 | 38.33 |
|  | 14 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.10 | 0.24 | 0.03 | 4 | 46582 | 46586 | 76.67 |
| ZF10-2 | 15 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.06 | 0.16 | 0.01 | 3 | 58312 | 58315 | 46.00 |
|  | 16 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.04 | 0.14 | 0.01 | 2 | 52967 | 52969 | 30.67 |
|  | 17 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.32 | 0.51 | 0.18 | 15 | 55501 | 55516 | 245.33 |
| ZF11-1 | 18 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.07 | 0.18 | 0.02 | 3 | 51224 | 51227 | 53.67 |
|  | 19 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.04 | 0.14 | 0.01 | 2 | 54210 | 54212 | 30.67 |
|  | 20 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.06 | 0.17 | 0.02 | 3 | 55590 | 55593 | 46.00 |
| ZF11-2 | 21 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.064 | 0 | 0 | 55459 | 55459 | 0.00 |
|  | 22 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.00 | 0.071 | 0 | 0 | 49670 | 49670 | 0.00 |
|  | 23 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.021 | 0.099 | 0.001 | 1 | 56893 | 56894 | 16.10 |
| ZF12-1 | 24 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.04 | 0.13 | 0.01 | 2 | 57824 | 57826 | 30.67 |
|  | 25 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.14 | 0.28 | 0.05 | 6 | 51519 | 51525 | 107.33 |
|  | 26 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.40 | 0.63 | 0.24 | 17 | 49678 | 49695 | 306.67 |
| ZF12-2 | 27 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.96 | 1.25 | 0.71 | 49 | 60305 | 60354 | 736.00 |
|  | 28 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.02 | 0.093 | 0.001 | 1 | 60140 | 60141 | 15.33 |
|  | 29 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.10 | 0.21 | 0.03 | 5 | 59624 | 59629 | 76.67 |
| ZF13-1 | 31 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.13 | 0.28 | 0.05 | 6 | 52320 | 52326 | 99.67 |
|  | 32 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.07 | 0.17 | 0.02 | 3 | 53975 | 53978 | 53.67 |
|  | 33 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.06 | 0.17 | 0.01 | 3 | 56164 | 56167 | 46.00 |
| ZF13-2 | 34 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.06 | 0 | 0 | 58826 | 58826 | 0.00 |
|  | 35 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.04 | 0.12 | 0.01 | 2 | 60551 | 60553 | 30.67 |
|  | 36 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.065 | 0 | 0 | 54520 | 54520 | 0.00 |
| ZF14-1 | 38 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.04 | 0.12 | 0.01 | 2 | 61270 | 61272 | 30.67 |
|  | 39 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.04 | 0.13 | 0.01 | 2 | 59234 | 59236 | 30.67 |
|  | 40 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.061 | 0 | 0 | 57859 | 57859 | 0.00 |
| ZF14-2 | 41 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.06 | 0.2 | 0.01 | 2 | 38359 | 38361 | 46.00 |
|  | 42 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.25 | 0.43 | 0.13 | 11 | 52166 | 52177 | 191.67 |
|  | 43 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.32 | 0.52 | 0.18 | 14 | 51296 | 51310 | 245.33 |
| ZF15-1 | 45 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.067 | 0 | 0 | 52271 | 52271 | 0.00 |
|  | 46 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.071 | 0 | 0 | 49922 | 49922 | 0.00 |
|  | 47 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.02 | 0.12 | 0 | 1 | 48084 | 48085 | 15.33 |
| ZF15-2 | 48 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.26 | 0.45 | 0.14 | 11 | 48906 | 48917 | 199.33 |
|  | 49 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.16 | 0.31 | 0.07 | 7 | 51131 | 51138 | 122.67 |
|  | 50 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.29 | 0.48 | 0.15 | 12 | 49211 | 49223 | 222.33 |
| ZF16-1 | 52 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.26 | 0.45 | 0.14 | 11 | 49185 | 49196 | 199.33 |
|  | 53 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.10 | 0.23 | 0.03 | 4 | 48791 | 48795 | 76.67 |
|  | 54 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.49 | 0.74 | 0.3 | 20 | 48143 | 48163 | 375.67 |
| ZF16-2 | 55 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.38 | 0.59 | 0.22 | 16 | 49980 | 49996 | 291.33 |
|  | 56 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.14 | 0.29 | 0.06 | 6 | 49128 | 49134 | 107.33 |
|  | 57 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.09 | 0.22 | 0.03 | 4 | 51456 | 51460 | 69.00 |
| ZF17-1 | 59 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 1.51 | 1.93 | 1.16 | 61 | 47375 | 47436 | 1157.67 |
|  | 60 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.35 | 0.56 | 0.2 | 15 | 50396 | 50411 | 268.33 |
|  | 61 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 1.10 | 1.46 | 0.8 | 44 | 47135 | 47179 | 843.33 |
| ZF17-2 | 62 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.07 | 0.18 | 0.02 | 3 | 52493 | 52496 | 53.67 |
|  | 63 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.12 | 0.27 | 0.04 | 5 | 48131 | 48136 | 92.00 |
|  | 64 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.15 | 0.3 | 0.06 | 7 | 53901 | 53908 | 115.00 |
| ZF19-1 | 66 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.21 | 1.34 | 0.74 | 45 | 52256 | 52301 | 161.00 |
|  | 67 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.02 | 0.11 | 0 | 1 | 49797 | 49798 | 15.33 |
|  | 68 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.26 | 0.44 | 0.14 | 12 | 54522 | 54534 | 199.33 |
| ZF19-2 | 69 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 9.10 | 9.9 | 8.2 | 430 | 55591 | 56021 | 6976.67 |
|  | 70 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 6.30 | 7 | 5.5 | 263 | 49266 | 49529 | 4830.00 |
|  | 71 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 4.50 | 5.1 | 3.8 | 196 | 51598 | 51794 | 3450.00 |
| ZF20-1 | 73 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 1.12 | 1.48 | 0.83 | 46 | 48267 | 48313 | 858.67 |
|  | 74 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.44 | 0.69 | 0.26 | 16 | 43058 | 43074 | 337.33 |
|  | 75 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 1.30 | 1.67 | 1 | 58 | 52278 | 52336 | 996.67 |
| ZF20-2 | 76 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.02 | 0.11 | 0 | 1 | 53132 | 53133 | 15.33 |
|  | 77 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.18 | 0.36 | 0.08 | 7 | 45133 | 45140 | 138.00 |
|  | 78 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.05 | 0.17 | 0.01 | 2 | 45164 | 45166 | 38.33 |
| ZF21 | 80 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.07 | 0.19 | 0.02 | 3 | 50380 | 50383 | 53.67 |
|  | 81 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.05 | 0.16 | 0.01 | 2 | 47469 | 47471 | 38.33 |
|  | 82 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.03 | 0.13 | 0 | 1 | 43065 | 43066 | 23.00 |
|  | NTC1 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.03 | 0.11 | 0.01 | 2 | 67893 | 67895 |  |
|  | NTC2 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.018 | 0.084 | 0.001 | 1 | 66807 | 66808 |  |
|  | NTC3 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.088 | 0 | 0 | 39957 | 39957 |  |
|  | NTC4 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.00 | 0.075 | 0 | 0 | 47307 | 47307 |  |
|  | NTC5 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.00 | 0.073 | 0 | 0 | 48159 | 48159 |  |
|  | Ex bk 1 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.21 | 0 | 0 | 16989 | 16989 | 0.00 |
|  | Ex bk 2 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.00 | 0.23 | 0 | 0 | 15511 | 15511 | 0.00 |
|  | Ex bk 3 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.00 | 0.23 | 0 | 0 | 15291 | 15291 | 0.00 |
|  | Ex bk 4 | ABS | QX200 ddPCR EvaGreen Supermix | fish DNA | 0.00 | 0.23 | 0 | 0 | 15590 | 15590 | 0.00 |
|  | Ex bk 5 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.08 | 0.37 | 0 | 1 | 15320 | 15321 | 61.33 |
|  | Ex bk 6 | ABS | QX200 ddPCR EvaGreen Supermix | Fish DNA | 0.00 | 0.22 | 0 | 0 | 16049 | 16049 | 0.00 |