SUPPLEMENTARY INFORMATION

Conflicting objectives of energy development and water security in Africa

Ioannis Pappis1*, Vignesh Sridharan1, Mark Howells23, Hrvoje Medarac4, Ioannis Kougias5,Rocío González Sánchez5, Abhishek Shivakumar6, Will Usher1

1Division of Energy Systems, KTH – Royal Institute of Technology, Sweden

2Department of Geography, Loughborough University, UK

3 Centre for Environmental Policy, Imperial College London, UK

4 Joint Research Centre (JRC), European Commission, Petten, Netherlands

5 Joint Research Centre (JRC), European Commission, Ispra, Italy

6United Nations Department of Economic and Social Affairs

*e-mail: pappis@kth.se

Supplementary Results

## Transformation of the energy system

The total final energy demand, in the reference scenario, on the continent increases by 148% from 2015 to 2065, reaching 1,387Mtoe. Fossil fuels represent approximately 40% of the total final consumption, followed by electricity (33%) and biomass and waste (27%). Among these, electricity shows the highest increase, almost seven-fold, followed by natural gas (four-fold), coal (double), oil (double), biomass and waste (stable) by 2065 (Supplementary Figure 1). The projected high GDP and population growth in the WAPP, followed by EAPP, drive the continent´s energy transition. EAPP is one of the fastest-growing economies in the continent with aggressive electrification strategies (Ethiopia, Kenya, Rwanda). By 2065, WAPP is responsible formost of the continent´s fossil fuel (29%) demand, followed by EAPP (27%). Within the power pools, individual countries lead the growth in energy demand. Nigeria is responsible for almost 67% of the WAPP´s future energy demand by 2065, while Egypt and Ethiopia are responsible for 35% and 19% of EAPP´s respectively. Across the scenarios, these three countries are responsible for over 50% of their respective power pool demands, and the transformation of their energy consumption plays a significant role in the continent. More efficient technologies penetrate the future energy mix in Africa, replacing coal and oil with natural gas. WAPP (30%) becomes the highest consumer of natural gas by 2065, overtaking NAPP (43%), which led in 2015. In Nigeria alone, economic and population growth lead to an increase in its natural gas consumption from 4Mtoe in 2015 to 38Mtoe in 2065. And CAPP experiences the highest increase (117%) in the continent´s energy demand, mainly caused by the Democratic Republic of Congo (4th most populous country), due to its low electricity access levels (15% in 2016) and four-fold population increase by 2065.

- Supplementary Figure 1. Total final consumption (Mtoe) in Africa across the scenarios.

Low carbon electricity, along with biomass and waste, play an essential role in the transition associated with these decarbonization scenarios. On the supply side, coal and oil products are replaced by low-carbon alternatives, such as modern renewables and nuclear.The final fossil- fuel consumption decreases by 24% (2.0°C)  and 84% (1.5°C), respectively, by 2065. In both mitigation scenarios, NAPP undergoes a dramatic decrease in fossil fuel consumption, displaced by biomass and waste. On the other hand, WAPP (Cote D Ivoire, Nigeria, Gambia) and CAPP (Equatorial Guinea, Democratic Republic of Congo) are the only regions increasing their fossil fuel shares in the 2.0°C scenario. In the 1.5°C scenario, all power pools decrease their fossil fuel demand by a significant margin.

- Supplementary Figures 2-7: Water withdrawal in a continental and regional level among the scenarios

In the following figures the total water withdrawal and consumption in a continental and regional level is presented among the scenarios.

Water Withdrawal (MCM)	Water Consumption(MCM)

Supplementary Figure 2. Total water withdrawal and consumption in Africa among the scenarios.

Supplementary Figure 3. Total water withdrawal and consumption in NAPP among the scenarios.

Supplementary Figure 4. Total water withdrawal and consumption in CAPP among the scenarios.

Supplementary Figure 5. Total water withdrawal and consumption in EAPP among the scenarios.

Supplementary Figure 6. Total water withdrawal and consumption in WAPP among the scenarios.

Supplementary Figure 7. Total water withdrawal and consumption in SAPP among the scenarios.

- Supplementary Figures 8-10: Water withdrawal and consumption per type of technology in a continental level among the scenarios

The total water withdrawal and consumption per type of technology among the scenarios in Africa is presented in the following figures.The water losses for hydropower generation are not included.

Water Withdrawal (MCM)	Water Consumption(MCM)

Supplementary Figure 8. Total water withdrawal and consumption per type of technology in a continentallevel in the Reference scenario.

Supplementary Figure 9. Total water withdrawal and consumption per type of technology in a continentallevel in the 2.0oC scenario.

Supplementary Figure 10. Total water withdrawal and consumption per type of technology in a continentallevel in the 1.5oC scenario.

- Supplementary Figures 11-28: Power generation capacity in a continental andregional level among the scenarios.

The following figures present the power generation capacity in a continental and regional level among the scenarios.

Supplementary Figure 11. Power generation capacity (GW) in Africa in the Reference scenario.

Supplementary Figure 12. Power generation capacity (GW) in Africa in the 2.0oC scenario.

Supplementary Figure 13. Power generation capacity (GW) in Africa in the 1.5oC scenario.

Supplementary Figure 14. Power generation capacity (GW) in NAPP in the Reference scenario.

Supplementary Figure 15. Power generation capacity (GW) in NAPP in the 2.0oC scenario.

Supplementary Figure 16. Power generation capacity (GW) in NAPP in the 1.5oC scenario.

Supplementary Figure 17. Power generation capacity (GW) in CAPP in the Reference scenario.

Supplementary Figure 18. Power generation capacity (GW) in CAPP in the 2.0oC scenario.

Supplementary Figure 19. Power generation capacity (GW) in CAPP in the 1.5oC scenario.

Supplementary Figure 20. Power generation capacity (GW) in EAPP in the Reference scenario.

Supplementary Figure 21. Power generation capacity (GW) in EAPP in the 2.0oC scenario.

Supplementary Figure 22. Power generation capacity (GW) in EAPP in the 1.5oC scenario.

Supplementary Figure 23. Power generation capacity (GW) in WAPP in the Reference scenario.

Supplementary Figure 24. Power generation capacity (GW) in WAPP in the 2.0oC scenario.

Supplementary Figure 25. Power generation capacity (GW) in WAPP in the 1.5oC scenario.

Supplementary Figure 26. Power generation capacity (GW) in SAPP in the Reference scenario.

Supplementary Figure 27. Power generation capacity (GW) in SAPP in the 2.0oC scenario.

Supplementary Figure 28. Power generation capacity (GW) in SAPP in the 1.5oC scenario.

- Supplementary Figures 29-46: Electricity generation mix in a continental and regional-level among the scenarios

The following figures present the electricity generation mix in a continental and regional level among the scenarios.

Supplementary Figure 29. Electricity generation mix (PJ) in Africa in the Reference scenario.

Supplementary Figure 30. Electricity generation mix (PJ) in Africa in the 2.0oC scenario.

Supplementary Figure 31. Electricity generation mix (PJ) in Africa in the 1.5oC scenario.

Supplementary Figure 32. Electricity generation mix (PJ) in NAPP in the Reference scenario.

Supplementary Figure 33. Electricity generation mix (PJ) in NAPP in the 2.0oC scenario.

Supplementary Figure 34. Electricity generation mix (PJ) in NAPP in the 1.5oC scenario.

Supplementary Figure 35. Electricity generation mix (PJ) in CAPP in the Reference scenario.

Supplementary Figure 36. Electricity generation mix (PJ) in CAPP in the 2.0oC scenario.

Supplementary Figure 37. Electricity generation mix (PJ) in CAPP in the 1.5oC scenario.

Supplementary Figure 38. Electricity generation mix (PJ) in EAPP in the Reference scenario.

Supplementary Figure 39. Electricity generation mix (PJ) in EAPP in the 2.0oC scenario.

Supplementary Figure 40. Electricity generation mix (PJ) in EAPP in the 1.5oC scenario.

Supplementary Figure 41. Electricity generation mix (PJ) in WAPP in the Reference scenario.

Supplementary Figure 42. Electricity generation mix (PJ) in WAPP in the 2.0oC scenario.

Supplementary Figure 43. Electricity generation mix (PJ) in WAPP in the 1.5oC scenario.

Supplementary Figure 44. Electricity generation mix (PJ) in SAPP in the Reference scenario.

Supplementary Figure 45. Electricity generation mix (PJ) in SAPP in the 2.0oC scenario.

Supplementary Figure 46. Electricity generation mix (PJ) in SAPP in the 1.5oC scenario.

- Supplementary Figures 47-49: Water loss (MCM) of hydropower in a regional level among the scenarios

Supplementary Figure 47. Water Loss (MCM) of hydropower in a regional level in the Reference scenario.

- Supplementary Figure 48. Water Loss (MCM) of hydropower in a regional level in the 2.0oC scenario.

- Supplementary Figure 49. Water Loss (MCM) of hydropower in a regional level in the 1.5oC scenario.

- Supplementary Figures 50-52: Fuel supply costs and capital investment costs among the scenarios.

In the following figures the fuel supply costs and capital investment costs among the scenarios are presented.

Supplementary Figure 50. Fuel supply costs and capital investment costs (BUSD) in the power sector in the Reference scenario.

Supplementary Figure 51. Fuel supply costs and capital investment costs (BUSD) in the power sector in the 2.0oC scenario.

Supplementary Figure 52. Fuel supply costs and capital investment costs (BUSD) in the power sector in the 1.5oC scenario.

- Supplementary Figure 53. Renewable – energy cost development assumptions (overnight investment costs)16.

- Supplementary Figure 54. Fuel price projections16.

- Supplementary Tables 1-15: Final energy demand projections by country among the scenarios

The following tables present the fuel demand projections by country for the respective years 2015, 2030, 2050 and 2065 among the scenarios.

Supplementary Table 1. Electricity demand projections by country (Mtoe) – Reference scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 10.08 | 14.99 | 28.03 | 42.99 |
| Algeria (DZ) | 3.88 | 4.76 | 7.49 | 10.53 |
| Libya (LY) | 2.28 | 2.79 | 4.40 | 6.19 |
| Mauritania (MR) | 0.08 | 0.19 | 0.72 | 1.17 |
| Morocco (MA) | 2.44 | 4.61 | 9.80 | 15.95 |
| Tunisia (TN) | 1.40 | 2.64 | 5.62 | 9.14 |
| SAPP | 21.47 | 30.64 | 72.11 | 108.15 |
| Angola (AO) | 0.72 | 2.64 | 10.71 | 18.28 |
| Botswana (BW) | 0.30 | 0.64 | 2.45 | 4.17 |
| Lesotho (LS) | 0.06 | 0.19 | 0.64 | 0.95 |
| Malawi (MW) | 0.13 | 0.52 | 2.05 | 3.40 |
| Mozambique (MZ) | 1.16 | 2.72 | 10.83 | 18.25 |
| Namibia (NM) | 0.33 | 0.74 | 2.78 | 4.38 |
| South Africa (ZA) | 17.10 | 19.06 | 26.76 | 32.96 |
| Swaziland (SZ) | 0.10 | 0.23 | 0.84 | 1.28 |
| Zambia (ZM) | 0.98 | 2.48 | 9.80 | 16.24 |
| Zimbabwe (ZW) | 0.59 | 1.42 | 5.23 | 8.24 |
| EAPP | 14.85 | 27.02 | 82.53 | 137.19 |
| Egypt (EG) | 11.62 | 15.59 | 38.02 | 63.70 |
| Burundi (BI) | 0.02 | 0.28 | 1.14 | 1.92 |
| Djibouti (DJ) | 0.03 | 0.05 | 0.18 | 0.27 |
| Eritrea (ER) | 0.03 | 0.14 | 0.50 | 0.78 |
| Ethiopia (ET) | 0.72 | 2.87 | 10.78 | 17.25 |
| Kenya (KE) | 0.68 | 1.72 | 6.43 | 10.25 |
| Rwanda (RW) | 0.04 | 0.38 | 1.38 | 2.11 |
| Somalia (SO) | 0.03 | 0.45 | 1.87 | 3.26 |
| South Sudan (SS) | 0.045 | 0.26 | 0.96 | 1.51 |
| Sudan (SD) | 0.96 | 2.22 | 8.55 | 13.97 |
| Tanzania (TZ) | 0.45 | 1.75 | 7.14 | 12.22 |
| Uganda (UG) | 0.24 | 1.31 | 5.60 | 9.93 |
| WAPP | 4.85 | 20.24 | 79.27 | 132.29 |
| Benin (BJ) | 0.10 | 0.39 | 1.51 | 2.49 |
| Burkina Faso (BF) | 0.10 | 0.63 | 2.53 | 4.24 |
| Cote d Ivoire (CI) | 0.52 | 0.99 | 3.75 | 6.11 |
| Gambia (GM) | 0.03 | 0.06 | 0.26 | 0.45 |
| Ghana (GH) | 0.75 | 1.90 | 7.09 | 11.68 |
| Guinea (GN) | 0.08 | 0.37 | 1.44 | 2.39 |
| Guinea Bissau (GW) | 0.00 | 0.06 | 0.21 | 0.33 |
| Liberia (LR) | 0.02 | 0.14 | 0.53 | 0.86 |
| Mali (ML) | 0.59 | 1.62 | 6.77 | 11.84 |
| Niger (NE) | 0.08 | 0.66 | 3.15 | 6.08 |
| Nigeria (NG) | 2.16 | 12.32 | 47.91 | 79.09 |
| Senegal (SNG) | 0.30 | 0.63 | 2.48 | 4.11 |
| Sierra Leone (SL) | 0.01 | 0.21 | 0.75 | 1.16 |
| Togo (TG) | 0.10 | 0.24 | 0.89 | 1.45 |
| CAPP | 1.49 | 5.42 | 22.03 | 37.77 |
| Cameroon (CM) | 0.53 | 1.02 | 3.91 | 6.41 |
| Central African Republic (CF) | 0.01 | 0.12 | 0.45 | 0.71 |
| Chad (TD) | 0.02 | 0.60 | 2.45 | 4.21 |
| Congo (CG) | 0.07 | 0.40 | 1.58 | 2.62 |
| Democratic Republic of Congo (CD) | 0.62 | 2.50 | 10.61 | 18.80 |
| Equatorial Guinea (GQ) | 0.08 | 0.33 | 1.30 | 2.17 |
| Gabon (GA) | 0.16 | 0.44 | 1.72 | 2.84 |

Supplementary Table 2. Coal demand projections by country (Mtoe) – Reference scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 0.18 | 0.76 | 3.76 | 9.30 |
| Algeria (DZ) | 0.16 | 0.42 | 1.53 | 3.23 |
| Libya (LY) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mauritania (MR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Morocco (MA) | 0.03 | 0.34 | 2.23 | 6.07 |
| Tunisia (TN) | 0.00 | 0.00 | 0.00 | 0.00 |
| SAPP | 17.38 | 26.32 | 24.61 | 20.32 |
| Angola (AO) | 0.00 | 0.00 | 0.00 | 0.00 |
| Botswana (BW) | 0.05 | 0.32 | 0.49 | 0.49 |
| Lesotho (LS) | 0.40 | 8.12 | 11.26 | 10.17 |
| Malawi (MW) | 0.05 | 0.47 | 0.75 | 0.76 |
| Mozambique (MZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Namibia (NM) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Africa (ZA) | 16.53 | 16.13 | 10.15 | 6.94 |
| Swaziland (SZ) | 0.01 | 0.09 | 0.14 | 0.13 |
| Zambia (ZM) | 0.10 | 0.69 | 1.10 | 1.12 |
| Zimbabwe (ZW) | 0.26 | 0.49 | 0.73 | 0.71 |
| EAPP | 0.94 | 6.29 | 9.95 | 10.74 |
| Egypt (EG) | 0.18 | 0.30 | 0.69 | 1.42 |
| Burundi (BI) | 0.00 | 0.25 | 0.41 | 0.43 |
| Djibouti (DJ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Eritrea (ER) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ethiopia (ET) | 0.25 | 2.59 | 3.92 | 3.86 |
| Kenya (KE) | 0.35 | 1.56 | 2.34 | 2.29 |
| Rwanda (RW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Somalia (SO) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Sudan (SS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sudan (SD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Tanzania (TZ) | 0.16 | 1.58 | 2.60 | 2.73 |
| Uganda (UG) | 0.00 | 0.00 | 0.00 | 0.00 |
| WAPP | 0.36 | 2.23 | 3.48 | 3.53 |
| Benin (BJ) | 0.03 | 0.35 | 0.55 | 0.56 |
| Burkina Faso (BF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Cote d Ivoire (CI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gambia (GM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ghana (GH) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea (GN) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea Bissau (GW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Liberia (LR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mali (ML) | 0.00 | 0.00 | 0.00 | 0.00 |
| Niger (NE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Nigeria (NG) | 0.03 | 1.30 | 2.03 | 2.06 |
| Senegal (SNG) | 0.30 | 0.57 | 0.90 | 0.92 |
| Sierra Leone (SL) | 0.00 | 0.00 | 0.00 | 0.00 |
| Togo (TG) | 0.00 | 0.00 | 0.00 | 0.00 |
| CAPP | 0.00 | 0.00 | 0.00 | 0.00 |
| Cameroon (CM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Central African Republic (CF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Chad (TD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Congo (CG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Democratic Republic of Congo (CD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Equatorial Guinea (GQ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gabon (GA) | 0.00 | 0.00 | 0.00 | 0.00 |

Supplementary Table 3. Oil demand projections by country (Mtoe) – Reference scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 43.29 | 54.28 | 54.36 | 52.40 |
| Algeria (DZ) | 16.66 | 20.83 | 20.23 | 19.34 |
| Libya (LY) | 10.70 | 13.38 | 12.99 | 12.42 |
| Mauritania (MR) | 0.64 | 1.19 | 2.05 | 2.67 |
| Morocco (MA) | 10.91 | 13.48 | 13.62 | 12.82 |
| Tunisia (TN) | 4.38 | 5.41 | 5.47 | 5.15 |
| SAPP | 36.05 | 45.20 | 56.28 | 63.10 |
| Angola (AO) | 4.92 | 6.74 | 12.34 | 16.77 |
| Botswana (BW) | 1.04 | 1.74 | 2.97 | 3.90 |
| Lesotho (LS) | 0.21 | 0.23 | 0.36 | 0.43 |
| Malawi (MW) | 0.22 | 1.32 | 2.36 | 3.12 |
| Mozambique (MZ) | 1.12 | 2.29 | 4.13 | 5.54 |
| Namibia (NM) | 1.27 | 2.36 | 4.01 | 5.04 |
| South Africa (ZA) | 24.67 | 26.31 | 22.87 | 19.08 |
| Swaziland (SZ) | 0.47 | 0.86 | 1.44 | 1.74 |
| Zambia (ZM) | 0.93 | 1.95 | 3.48 | 4.60 |
| Zimbabwe (ZW) | 1.21 | 1.39 | 2.31 | 2.90 |
| EAPP | 44.06 | 61.12 | 87.26 | 103.93 |
| Egypt (EG) | 28.00 | 32.00 | 36.00 | 36.54 |
| Burundi (BI) | 0.12 | 0.72 | 1.31 | 1.76 |
| Djibouti (DJ) | 0.08 | 0.13 | 0.20 | 0.25 |
| Eritrea (ER) | 0.09 | 0.34 | 0.57 | 0.72 |
| Ethiopia (ET) | 3.04 | 7.30 | 12.41 | 15.82 |
| Kenya (KE) | 3.95 | 4.39 | 7.40 | 9.40 |
| Rwanda (RW) | 0.25 | 0.97 | 1.59 | 1.94 |
| Somalia (SO) | 0.11 | 1.15 | 2.15 | 2.99 |
| South Sudan (SS) | 0.39 | 0.67 | 1.11 | 1.39 |
| Sudan (SD) | 4.00 | 5.66 | 9.84 | 12.81 |
| Tanzania (TZ) | 2.64 | 4.46 | 8.22 | 11.21 |
| Uganda (UG) | 1.40 | 3.33 | 6.45 | 9.10 |
| WAPP | 30.74 | 49.68 | 87.66 | 115.94 |
| Benin (BJ) | 1.65 | 2.95 | 5.14 | 6.75 |
| Burkina Faso (BF) | 0.88 | 1.60 | 2.91 | 3.89 |
| Cote d Ivoire (CI) | 1.74 | 2.53 | 4.32 | 5.60 |
| Gambia (GM) | 0.10 | 0.16 | 0.30 | 0.41 |
| Ghana (GH) | 3.76 | 3.50 | 5.84 | 7.41 |
| Guinea (GN) | 0.81 | 0.95 | 1.66 | 2.19 |
| Guinea Bissau (GW) | 0.11 | 0.14 | 0.24 | 0.31 |
| Liberia (LR) | 0.30 | 0.36 | 0.61 | 0.79 |
| Mali (ML) | 0.78 | 1.64 | 3.10 | 4.31 |
| Niger (NE) | 0.49 | 1.69 | 3.62 | 5.58 |
| Nigeria (NG) | 18.06 | 31.40 | 55.17 | 72.53 |
| Senegal (SNG) | 1.17 | 1.62 | 2.86 | 3.77 |
| Sierra Leone (SL) | 0.28 | 0.53 | 0.86 | 1.07 |
| Togo (TG) | 0.61 | 0.60 | 1.03 | 1.33 |
| CAPP | 4.95 | 13.80 | 25.36 | 34.64 |
| Cameroon (CM) | 1.38 | 2.61 | 4.51 | 5.87 |
| Central African Republic (CF) | 0.10 | 0.32 | 0.52 | 0.66 |
| Chad (TD) | 0.23 | 1.52 | 2.82 | 3.86 |
| Congo (CG) | 0.77 | 1.02 | 1.81 | 2.40 |
| Democratic Republic of Congo (CD) | 0.90 | 6.36 | 12.22 | 17.24 |
| Equatorial Guinea (GQ) | 0.81 | 0.84 | 1.50 | 1.99 |
| Gabon (GA) | 0.77 | 1.12 | 1.98 | 2.61 |

Supplementary Table 4. Natural gas demand projections by country (Mtoe) – Reference scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 15.79 | 20.31 | 28.68 | 38.16 |
| Algeria (DZ) | 11.18 | 12.99 | 16.57 | 20.72 |
| Libya (LY) | 2.72 | 3.16 | 4.03 | 5.03 |
| Mauritania (MR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Morocco (MA) | 0.06 | 0.12 | 0.24 | 0.36 |
| Tunisia (TN) | 1.83 | 4.03 | 7.84 | 12.04 |
| SAPP | 3.97 | 8.24 | 17.16 | 25.82 |
| Angola (AO) | 0.63 | 2.01 | 5.45 | 8.81 |
| Botswana (BW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Lesotho (LS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Malawi (MW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mozambique (MZ) | 0.14 | 0.69 | 1.82 | 2.91 |
| Namibia (NM) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Africa (ZA) | 3.20 | 5.54 | 9.88 | 14.10 |
| Swaziland (SZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Zambia (ZM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Zimbabwe (ZW) | 0.00 | 0.00 | 0.00 | 0.00 |
| EAPP | 12.64 | 19.62 | 30.33 | 39.90 |
| Egypt (EG) | 12.50 | 18.00 | 26.00 | 32.99 |
| Burundi (BI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Djibouti (DJ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Eritrea (ER) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ethiopia (ET) | 0.00 | 0.00 | 0.00 | 0.00 |
| Kenya (KE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Rwanda (RW) | 0.00 | 0.29 | 0.70 | 1.02 |
| Somalia (SO) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Sudan (SS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sudan (SD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Tanzania (TZ) | 0.14 | 1.33 | 3.63 | 5.89 |
| Uganda (UG) | 0.00 | 0.00 | 0.00 | 0.00 |
| WAPP | 4.02 | 10.96 | 28.43 | 44.40 |
| Benin (BJ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Burkina Faso (BF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Cote d Ivoire (CI) | 0.25 | 0.76 | 1.91 | 2.94 |
| Gambia (GM) | 0.00 | 0.05 | 0.13 | 0.22 |
| Ghana (GH) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea (GN) | 0.00 | 0.28 | 0.73 | 1.15 |
| Guinea Bissau (GW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Liberia (LR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mali (ML) | 0.00 | 0.00 | 0.00 | 0.00 |
| Niger (NE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Nigeria (NG) | 3.76 | 9.39 | 24.39 | 38.11 |
| Senegal (SNG) | 0.00 | 0.48 | 1.26 | 1.98 |
| Sierra Leone (SL) | 0.00 | 0.00 | 0.00 | 0.00 |
| Togo (TG) | 0.00 | 0.00 | 0.00 | 0.00 |
| CAPP | 0.00 | 0.59 | 1.54 | 2.42 |
| Cameroon (CM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Central African Republic (CF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Chad (TD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Congo (CG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Democratic Republic of Congo (CD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Equatorial Guinea (GQ) | 0.00 | 0.25 | 0.66 | 1.05 |
| Gabon (GA) | 0.00 | 0.34 | 0.87 | 1.37 |

Supplementary Table 5 .Biofuels & waste demand projections by country (Mtoe) – Reference scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 2.63 | 5.75 | 7.05 | 8.58 |
| Algeria (DZ) | 0.01 | 0.08 | 0.12 | 0.18 |
| Libya (LY) | 0.24 | 1.81 | 2.97 | 4.25 |
| Mauritania (MR) | 0.17 | 1.43 | 1.31 | 1.17 |
| Morocco (MA) | 1.39 | 1.53 | 1.67 | 1.88 |
| Tunisia (TN) | 0.82 | 0.90 | 0.98 | 1.11 |
| SAPP | 41.61 | 68.73 | 63.21 | 57.25 |
| Angola (AO) | 6.05 | 20.27 | 19.55 | 18.31 |
| Botswana (BW) | 0.54 | 2.71 | 2.44 | 2.20 |
| Lesotho (LS) | 0.56 | 0.70 | 0.58 | 0.47 |
| Malawi (MW) | 0.70 | 3.98 | 3.74 | 3.41 |
| Mozambique (MZ) | 8.09 | 6.90 | 6.54 | 6.05 |
| Namibia (NM) | 0.17 | 2.34 | 2.09 | 1.81 |
| South Africa (ZA) | 10.92 | 11.61 | 10.03 | 8.89 |
| Swaziland (SZ) | 0.46 | 0.80 | 0.71 | 0.59 |
| Zambia (ZM) | 6.67 | 8.42 | 7.94 | 7.22 |
| Zimbabwe (ZW) | 7.44 | 10.99 | 9.61 | 8.30 |
| EAPP | 90.01 | 129.48 | 121.22 | 111.27 |
| Egypt (EG) | 1.50 | 1.00 | 2.00 | 3.12 |
| Burundi (BI) | 0.98 | 2.16 | 2.07 | 1.92 |
| Djibouti (DJ) | 0.02 | 0.38 | 0.32 | 0.27 |
| Eritrea (ER) | 0.46 | 1.04 | 0.91 | 0.79 |
| Ethiopia (ET) | 36.92 | 46.34 | 41.48 | 36.44 |
| Kenya (KE) | 10.92 | 13.20 | 11.73 | 10.27 |
| Rwanda (RW) | 1.19 | 2.92 | 2.51 | 2.12 |
| Somalia (SO) | 1.87 | 3.46 | 3.41 | 3.27 |
| South Sudan (SS) | 0.15 | 0.18 | 0.15 | 0.13 |
| Sudan (SD) | 6.82 | 7.92 | 7.79 | 6.99 |
| Tanzania (TZ) | 18.15 | 25.28 | 24.52 | 23.05 |
| Uganda (UG) | 11.02 | 14.65 | 14.91 | 14.52 |
| WAPP | 125.81 | 180.05 | 167.14 | 152.23 |
| Benin (BJ) | 1.82 | 3.00 | 2.75 | 2.49 |
| Burkina Faso (BF) | 2.30 | 4.82 | 4.61 | 4.25 |
| Cote d Ivoire (CI) | 4.27 | 7.61 | 6.85 | 6.12 |
| Gambia (GM) | 0.10 | 0.50 | 0.48 | 0.45 |
| Ghana (GH) | 2.42 | 10.53 | 9.25 | 8.10 |
| Guinea (GN) | 2.28 | 2.86 | 2.62 | 2.39 |
| Guinea Bissau (GW) | 0.53 | 0.71 | 0.63 | 0.55 |
| Liberia (LR) | 1.53 | 1.78 | 1.60 | 1.42 |
| Mali (ML) | 2.26 | 4.95 | 4.90 | 4.71 |
| Niger (NE) | 2.15 | 5.10 | 5.74 | 6.09 |
| Nigeria (NG) | 102.72 | 129.91 | 120.18 | 108.91 |
| Senegal (SNG) | 1.13 | 4.87 | 4.53 | 4.11 |
| Sierra Leone (SL) | 0.83 | 1.60 | 1.36 | 1.17 |
| Togo (TG) | 1.50 | 1.81 | 1.63 | 1.46 |
| CAPP | 31.78 | 49.49 | 48.25 | 45.69 |
| Cameroon (CM) | 5.22 | 7.86 | 7.14 | 6.41 |
| Central African Republic (CF) | 0.22 | 0.96 | 0.83 | 0.72 |
| Chad (TD) | 1.23 | 4.58 | 4.47 | 4.22 |
| Congo (CG) | 1.22 | 3.08 | 2.87 | 2.63 |
| Democratic Republic of Congo (CD) | 19.98 | 27.46 | 27.78 | 27.02 |
| Equatorial Guinea (GQ) | 0.09 | 0.65 | 0.61 | 0.55 |
| Gabon (GA) | 3.81 | 4.92 | 4.55 | 4.14 |

Supplementary Table 6. Electricity demand projections by country (Mtoe) – 2.0oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 9.96 | 12.80 | 23.48 | 37.09 |
| Algeria (DZ) | 3.81 | 3.93 | 5.94 | 8.48 |
| Libya (LY) | 2.25 | 2.31 | 3.49 | 4.98 |
| Mauritania (MR) | 0.08 | 0.18 | 0.64 | 1.06 |
| Morocco (MA) | 2.44 | 4.08 | 8.58 | 14.37 |
| Tunisia (TN) | 1.38 | 2.30 | 4.82 | 8.20 |
| SAPP | 21.52 | 27.76 | 63.39 | 99.61 |
| Angola (AO) | 0.72 | 2.52 | 9.61 | 16.57 |
| Botswana (BW) | 0.30 | 0.62 | 2.19 | 3.78 |
| Lesotho (LS) | 0.06 | 0.18 | 0.58 | 0.86 |
| Malawi (MW) | 0.13 | 0.49 | 1.84 | 3.08 |
| Mozambique (MZ) | 1.16 | 2.59 | 9.72 | 16.53 |
| Namibia (NM) | 0.33 | 0.70 | 2.49 | 3.97 |
| South Africa (ZA) | 17.14 | 16.73 | 22.72 | 31.49 |
| Swaziland (SZ) | 0.10 | 0.22 | 0.75 | 1.16 |
| Zambia (ZM) | 0.98 | 2.36 | 8.79 | 14.72 |
| Zimbabwe (ZW) | 0.59 | 1.36 | 4.69 | 7.46 |
| EAPP | 14.81 | 24.85 | 70.30 | 115.31 |
| Egypt (EG) | 0.02 | 0.27 | 1.02 | 1.74 |
| Burundi (BI) | 0.03 | 0.05 | 0.16 | 0.25 |
| Djibouti (DJ) | 11.58 | 13.97 | 30.38 | 48.72 |
| Eritrea (ER) | 0.03 | 0.13 | 0.45 | 0.71 |
| Ethiopia (ET) | 0.72 | 2.73 | 9.67 | 15.63 |
| Kenya (KE) | 0.68 | 1.64 | 5.77 | 9.29 |
| Rwanda (RW) | 0.04 | 0.36 | 1.23 | 1.91 |
| Somalia (SO) | 0.03 | 0.43 | 1.68 | 2.96 |
| South Sudan (SS) | 0.05 | 0.25 | 0.86 | 1.37 |
| Sudan (SD) | 0.96 | 2.12 | 7.67 | 12.66 |
| Tanzania (TZ) | 0.45 | 1.67 | 6.40 | 11.07 |
| Uganda (UG) | 0.24 | 1.25 | 5.02 | 9.00 |
| WAPP | 4.85 | 19.29 | 71.09 | 119.86 |
| Benin (BJ) | 0.10 | 0.37 | 1.35 | 2.26 |
| Burkina Faso (BF) | 0.10 | 0.60 | 2.27 | 3.84 |
| Cote d Ivoire (CI) | 0.52 | 0.94 | 3.37 | 5.53 |
| Gambia (GM) | 0.03 | 0.06 | 0.24 | 0.41 |
| Ghana (GH) | 0.75 | 1.82 | 6.34 | 10.58 |
| Guinea (GN) | 0.08 | 0.36 | 1.29 | 2.16 |
| Guinea Bissau (GW) | 0.00 | 0.05 | 0.19 | 0.30 |
| Liberia (LR) | 0.02 | 0.13 | 0.48 | 0.78 |
| Mali (ML) | 0.59 | 1.55 | 6.07 | 10.73 |
| Niger (NE) | 0.08 | 0.63 | 2.82 | 5.51 |
| Nigeria (NG) | 2.16 | 11.74 | 42.98 | 71.66 |
| Senegal (SNG) | 0.30 | 0.60 | 2.23 | 3.72 |
| Sierra Leone (SL) | 0.01 | 0.20 | 0.67 | 1.06 |
| Togo (TG) | 0.10 | 0.23 | 0.80 | 1.32 |
| CAPP | 1.49 | 5.16 | 19.76 | 34.22 |
| Cameroon (CM) | 0.53 | 0.98 | 3.51 | 5.80 |
| Central African Republic (CF) | 0.01 | 0.12 | 0.41 | 0.65 |
| Chad (TD) | 0.02 | 0.57 | 2.20 | 3.82 |
| Congo (CG) | 0.07 | 0.38 | 1.41 | 2.38 |
| Democratic Republic of Congo (CD) | 0.62 | 2.38 | 9.52 | 17.04 |
| Equatorial Guinea (GQ) | 0.08 | 0.32 | 1.17 | 1.97 |
| Gabon (GA) | 0.16 | 0.42 | 1.54 | 2.58 |

Supplementary Table 7. Coal demand projections by country (Mtoe) – 2.0oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 0.18 | 0.49 | 0.49 | 0.38 |
| Algeria (DZ) | 0.16 | 0.28 | 0.24 | 0.22 |
| Libya (LY) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mauritania (MR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Morocco (MA) | 0.03 | 0.22 | 0.25 | 0.17 |
| Tunisia (TN) | 0.00 | 0.00 | 0.00 | 0.00 |
| SAPP | 17.76 | 22.29 | 1.96 | 0.06 |
| Angola (AO) | 0.00 | 0.00 | 0.00 | 0.00 |
| Botswana (BW) | 0.05 | 0.28 | 0.03 | 0.00 |
| Lesotho (LS) | 0.40 | 7.05 | 0.75 | 0.00 |
| Malawi (MW) | 0.05 | 0.41 | 0.05 | 0.00 |
| Mozambique (MZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Namibia (NM) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Africa (ZA) | 16.91 | 13.44 | 0.99 | 0.06 |
| Swaziland (SZ) | 0.01 | 0.08 | 0.01 | 0.00 |
| Zambia (ZM) | 0.10 | 0.60 | 0.07 | 0.00 |
| Zimbabwe (ZW) | 0.26 | 0.43 | 0.05 | 0.00 |
| EAPP | 0.94 | 5.45 | 0.72 | 0.06 |
| Egypt (EG) | 0.00 | 0.22 | 0.03 | 0.00 |
| Burundi (BI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Djibouti (DJ) | 0.18 | 0.25 | 0.10 | 0.06 |
| Eritrea (ER) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ethiopia (ET) | 0.25 | 2.25 | 0.26 | 0.00 |
| Kenya (KE) | 0.35 | 1.35 | 0.16 | 0.00 |
| Rwanda (RW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Somalia (SO) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Sudan (SS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sudan (SD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Tanzania (TZ) | 0.16 | 1.38 | 0.17 | 0.00 |
| Uganda (UG) | 0.00 | 0.00 | 0.00 | 0.00 |
| WAPP | 0.36 | 1.93 | 0.23 | 0.00 |
| Benin (BJ) | 0.03 | 0.31 | 0.04 | 0.00 |
| Burkina Faso (BF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Cote d Ivoire (CI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gambia (GM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ghana (GH) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea (GN) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea Bissau (GW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Liberia (LR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mali (ML) | 0.00 | 0.00 | 0.00 | 0.00 |
| Niger (NE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Nigeria (NG) | 0.03 | 1.13 | 0.14 | 0.00 |
| Senegal (SNG) | 0.30 | 0.50 | 0.06 | 0.00 |
| Sierra Leone (SL) | 0.00 | 0.00 | 0.00 | 0.00 |
| Togo (TG) | 0.00 | 0.00 | 0.00 | 0.00 |
| CAPP | 0.00 | 0.00 | 0.00 | 0.00 |
| Cameroon (CM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Central African Republic (CF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Chad (TD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Congo (CG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Democratic Republic of Congo (CD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Equatorial Guinea (GQ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gabon (GA) | 0.00 | 0.00 | 0.00 | 0.00 |

Supplementary Table 8. Oil demand projections by country (Mtoe) – 2.0oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 43.39 | 48.61 | 24.59 | 11.88 |
| Algeria (DZ) | 16.74 | 18.35 | 9.19 | 4.40 |
| Libya (LY) | 10.75 | 11.79 | 5.90 | 2.82 |
| Mauritania (MR) | 0.64 | 1.13 | 1.17 | 0.87 |
| Morocco (MA) | 10.89 | 12.37 | 5.94 | 2.71 |
| Tunisia (TN) | 4.37 | 4.97 | 2.39 | 1.09 |
| SAPP | 36.03 | 43.44 | 31.21 | 19.47 |
| Angola (AO) | 4.92 | 6.42 | 7.01 | 5.46 |
| Botswana (BW) | 1.04 | 1.66 | 1.69 | 1.27 |
| Lesotho (LS) | 0.21 | 0.22 | 0.21 | 0.14 |
| Malawi (MW) | 0.22 | 1.26 | 1.34 | 1.02 |
| Mozambique (MZ) | 1.12 | 2.19 | 2.35 | 1.80 |
| Namibia (NM) | 1.27 | 2.25 | 2.28 | 1.64 |
| South Africa (ZA) | 24.65 | 25.43 | 12.21 | 5.15 |
| Swaziland (SZ) | 0.47 | 0.82 | 0.82 | 0.57 |
| Zambia (ZM) | 0.93 | 1.85 | 1.98 | 1.50 |
| Zimbabwe (ZW) | 1.21 | 1.33 | 1.31 | 0.94 |
| EAPP | 43.99 | 56.76 | 42.47 | 27.37 |
| Egypt (EG) | 0.12 | 0.68 | 0.74 | 0.57 |
| Burundi (BI) | 0.08 | 0.12 | 0.11 | 0.08 |
| Djibouti (DJ) | 27.92 | 29.00 | 13.33 | 5.44 |
| Eritrea (ER) | 0.09 | 0.33 | 0.33 | 0.23 |
| Ethiopia (ET) | 3.04 | 6.96 | 7.06 | 5.15 |
| Kenya (KE) | 3.95 | 4.18 | 4.21 | 3.06 |
| Rwanda (RW) | 0.25 | 0.93 | 0.90 | 0.63 |
| Somalia (SO) | 0.11 | 1.10 | 1.22 | 0.97 |
| South Sudan (SS) | 0.39 | 0.41 | 0.42 | 0.40 |
| Sudan (SD) | 4.00 | 5.40 | 5.60 | 4.17 |
| Tanzania (TZ) | 2.64 | 4.25 | 4.67 | 3.65 |
| Uganda (UG) | 1.40 | 3.18 | 3.66 | 2.96 |
| WAPP | 30.74 | 47.36 | 49.83 | 37.74 |
| Benin (BJ) | 1.65 | 2.81 | 2.92 | 2.20 |
| Burkina Faso (BF) | 0.88 | 1.53 | 1.65 | 1.27 |
| Cote d Ivoire (CI) | 1.74 | 2.41 | 2.46 | 1.82 |
| Gambia (GM) | 0.10 | 0.16 | 0.17 | 0.13 |
| Ghana (GH) | 3.76 | 3.34 | 3.32 | 2.41 |
| Guinea (GN) | 0.81 | 0.91 | 0.94 | 0.71 |
| Guinea Bissau (GW) | 0.11 | 0.14 | 0.14 | 0.10 |
| Liberia (LR) | 0.30 | 0.34 | 0.35 | 0.26 |
| Mali (ML) | 0.78 | 1.57 | 1.76 | 1.40 |
| Niger (NE) | 0.49 | 1.61 | 2.06 | 1.82 |
| Nigeria (NG) | 18.06 | 29.93 | 31.36 | 23.61 |
| Senegal (SNG) | 1.17 | 1.54 | 1.63 | 1.23 |
| Sierra Leone (SL) | 0.28 | 0.51 | 0.49 | 0.35 |
| Togo (TG) | 0.61 | 0.57 | 0.58 | 0.43 |
| CAPP | 4.95 | 13.16 | 14.42 | 11.27 |
| Cameroon (CM) | 1.38 | 2.49 | 2.56 | 1.91 |
| Central African Republic (CF) | 0.10 | 0.30 | 0.30 | 0.21 |
| Chad (TD) | 0.23 | 1.45 | 1.60 | 1.26 |
| Congo (CG) | 0.77 | 0.98 | 1.03 | 0.78 |
| Democratic Republic of Congo (CD) | 0.90 | 6.06 | 6.95 | 5.61 |
| Equatorial Guinea (GQ) | 0.81 | 0.80 | 0.85 | 0.65 |
| Gabon (GA) | 0.77 | 1.07 | 1.12 | 0.85 |

Supplementary Table 9. Natural gas demand projections by country (Mtoe) – 2.0oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 15.85 | 17.03 | 16.67 | 16.16 |
| Algeria (DZ) | 11.23 | 10.46 | 9.27 | 8.93 |
| Libya (LY) | 2.73 | 2.54 | 2.25 | 2.17 |
| Mauritania (MR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Morocco (MA) | 0.06 | 0.12 | 0.15 | 0.15 |
| Tunisia (TN) | 1.84 | 3.91 | 5.00 | 4.91 |
| SAPP | 3.99 | 9.35 | 13.63 | 13.36 |
| Angola (AO) | 0.63 | 2.14 | 5.13 | 6.73 |
| Botswana (BW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Lesotho (LS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Malawi (MW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mozambique (MZ) | 0.14 | 0.73 | 1.72 | 2.22 |
| Namibia (NM) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Africa (ZA) | 3.22 | 6.48 | 6.78 | 4.40 |
| Swaziland (SZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Zambia (ZM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Zimbabwe (ZW) | 0.00 | 0.00 | 0.00 | 0.00 |
| EAPP | 12.84 | 15.80 | 14.94 | 12.98 |
| Egypt (EG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Burundi (BI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Djibouti (DJ) | 12.71 | 14.39 | 11.53 | 8.48 |
| Eritrea (ER) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ethiopia (ET) | 0.00 | 0.00 | 0.00 | 0.00 |
| Kenya (KE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Rwanda (RW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Somalia (SO) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Sudan (SS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sudan (SD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Tanzania (TZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Uganda (UG) | 0.14 | 1.42 | 3.42 | 4.50 |
| WAPP | 0.00 | 0.00 | 0.00 | 0.00 |
| Benin (BJ) | 4.02 | 11.13 | 25.55 | 32.41 |
| Burkina Faso (BF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Cote d Ivoire (CI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gambia (GM) | 0.25 | 0.80 | 1.80 | 2.25 |
| Ghana (GH) | 0.00 | 0.05 | 0.13 | 0.17 |
| Guinea (GN) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea Bissau (GW) | 0.00 | 0.30 | 0.69 | 0.88 |
| Liberia (LR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mali (ML) | 0.00 | 0.00 | 0.00 | 0.00 |
| Niger (NE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Nigeria (NG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Senegal (SNG) | 3.76 | 9.98 | 22.94 | 29.12 |
| Sierra Leone (SL) | 0.00 | 0.00 | 0.00 | 0.00 |
| Togo (TG) | 0.00 | 0.00 | 0.00 | 0.00 |
| CAPP | 0.00 | 0.00 | 0.00 | 0.00 |
| Cameroon (CM) | 0.00 | 0.63 | 1.45 | 1.85 |
| Central African Republic (CF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Chad (TD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Congo (CG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Democratic Republic of Congo (CD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Equatorial Guinea (GQ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gabon (GA) | 0.00 | 0.27 | 0.62 | 0.80 |
| NAPP | 0.00 | 0.36 | 0.82 | 1.05 |

Supplementary Table 10. Biofuels & waste demand projections by country (Mtoe) – 2.0oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 2.63 | 6.56 | 12.71 | 20.53 |
| Algeria (DZ) | 0.01 | 0.10 | 0.26 | 0.48 |
| Libya (LY) | 0.24 | 2.36 | 6.23 | 11.56 |
| Mauritania (MR) | 0.17 | 1.44 | 1.28 | 1.23 |
| Morocco (MA) | 1.39 | 1.68 | 3.11 | 4.57 |
| Tunisia (TN) | 0.82 | 0.99 | 1.83 | 2.69 |
| SAPP | 41.59 | 70.02 | 67.14 | 68.06 |
| Angola (AO) | 6.05 | 20.34 | 19.09 | 19.15 |
| Botswana (BW) | 0.54 | 2.72 | 2.38 | 2.31 |
| Lesotho (LS) | 0.56 | 0.71 | 0.56 | 0.49 |
| Malawi (MW) | 0.70 | 4.00 | 3.66 | 3.57 |
| Mozambique (MZ) | 8.09 | 6.92 | 6.39 | 6.33 |
| Namibia (NM) | 0.17 | 2.34 | 2.04 | 1.89 |
| South Africa (ZA) | 10.90 | 12.74 | 15.21 | 17.50 |
| Swaziland (SZ) | 0.46 | 0.81 | 0.69 | 0.62 |
| Zambia (ZM) | 6.67 | 8.45 | 7.75 | 7.55 |
| Zimbabwe (ZW) | 7.44 | 11.01 | 9.37 | 8.67 |
| EAPP | 90.03 | 130.56 | 124.14 | 129.36 |
| Egypt (EG) | 0.98 | 2.16 | 2.02 | 2.01 |
| Burundi (BI) | 0.02 | 0.38 | 0.31 | 0.28 |
| Djibouti (DJ) | 1.51 | 1.70 | 7.71 | 16.26 |
| Eritrea (ER) | 0.46 | 1.04 | 0.89 | 0.82 |
| Ethiopia (ET) | 36.92 | 46.48 | 40.52 | 38.11 |
| Kenya (KE) | 10.92 | 13.24 | 11.45 | 10.74 |
| Rwanda (RW) | 1.19 | 2.93 | 2.45 | 2.21 |
| Somalia (SO) | 1.87 | 3.47 | 3.33 | 3.42 |
| South Sudan (SS) | 0.15 | 0.18 | 0.15 | 0.14 |
| Sudan (SD) | 6.82 | 7.95 | 7.66 | 7.36 |
| Tanzania (TZ) | 18.15 | 25.34 | 23.94 | 24.09 |
| Uganda (UG) | 11.02 | 14.69 | 14.57 | 15.19 |
| WAPP | 125.81 | 180.60 | 163.26 | 159.20 |
| Benin (BJ) | 1.82 | 3.01 | 2.69 | 2.61 |
| Burkina Faso (BF) | 2.30 | 4.84 | 4.50 | 4.44 |
| Cote d Ivoire (CI) | 4.27 | 7.63 | 6.69 | 6.40 |
| Gambia (GM) | 0.10 | 0.50 | 0.47 | 0.47 |
| Ghana (GH) | 2.42 | 10.57 | 9.04 | 8.47 |
| Guinea (GN) | 2.28 | 2.87 | 2.56 | 2.50 |
| Guinea Bissau (GW) | 0.53 | 0.72 | 0.62 | 0.58 |
| Liberia (LR) | 1.53 | 1.78 | 1.56 | 1.49 |
| Mali (ML) | 2.26 | 4.96 | 4.79 | 4.93 |
| Niger (NE) | 2.15 | 5.11 | 5.61 | 6.37 |
| Nigeria (NG) | 102.72 | 130.32 | 117.38 | 113.91 |
| Senegal (SNG) | 1.13 | 4.88 | 4.43 | 4.30 |
| Sierra Leone (SL) | 0.83 | 1.60 | 1.33 | 1.22 |
| Togo (TG) | 1.50 | 1.82 | 1.59 | 1.52 |
| CAPP | 31.78 | 49.65 | 47.12 | 47.78 |
| Cameroon (CM) | 5.22 | 7.88 | 6.97 | 6.71 |
| Central African Republic (CF) | 0.22 | 0.96 | 0.81 | 0.75 |
| Chad (TD) | 1.23 | 4.59 | 4.36 | 4.41 |
| Congo (CG) | 1.22 | 3.09 | 2.81 | 2.75 |
| Democratic Republic of Congo (CD) | 19.98 | 27.55 | 27.13 | 28.26 |
| Equatorial Guinea (GQ) | 0.09 | 0.65 | 0.59 | 0.58 |
| Gabon (GA) | 3.81 | 4.93 | 4.45 | 4.33 |

Supplementary Table 11. Electricitydemand projections by country (Mtoe) – 1.5oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 9.88 | 12.24 | 19.40 | 31.40 |
| Algeria (DZ) | 3.77 | 3.72 | 4.85 | 7.24 |
| Libya (LY) | 2.22 | 2.19 | 2.85 | 4.25 |
| Mauritania (MR) | 0.08 | 0.17 | 0.68 | 1.17 |
| Morocco (MA) | 2.44 | 3.94 | 7.05 | 11.99 |
| Tunisia (TN) | 1.38 | 2.22 | 3.97 | 6.75 |
| SAPP | 21.48 | 26.60 | 60.34 | 100.80 |
| Angola (AO) | 0.72 | 2.46 | 10.13 | 18.35 |
| Botswana (BW) | 0.30 | 0.60 | 2.28 | 4.18 |
| Lesotho (LS) | 0.06 | 0.17 | 0.61 | 0.95 |
| Malawi (MW) | 0.13 | 0.48 | 1.94 | 3.42 |
| Mozambique (MZ) | 1.16 | 2.52 | 10.24 | 18.32 |
| Namibia (NM) | 0.33 | 0.69 | 2.62 | 4.39 |
| South Africa (ZA) | 17.11 | 15.84 | 17.50 | 25.34 |
| Swaziland (SZ) | 0.10 | 0.21 | 0.79 | 1.28 |
| Zambia (ZM) | 0.98 | 2.30 | 9.27 | 16.30 |
| Zimbabwe (ZW) | 0.59 | 1.32 | 4.95 | 8.27 |
| EAPP | 14.78 | 24.09 | 65.49 | 110.13 |
| Egypt (EG) | 0.02 | 0.26 | 1.07 | 1.93 |
| Burundi (BI) | 0.03 | 0.05 | 0.17 | 0.27 |
| Djibouti (DJ) | 11.56 | 13.47 | 23.41 | 36.36 |
| Eritrea (ER) | 0.03 | 0.13 | 0.47 | 0.79 |
| Ethiopia (ET) | 0.72 | 2.66 | 10.19 | 17.31 |
| Kenya (KE) | 0.68 | 1.60 | 6.08 | 10.29 |
| Rwanda (RW) | 0.04 | 0.35 | 1.30 | 2.12 |
| Somalia (SO) | 0.03 | 0.42 | 1.77 | 3.28 |
| South Sudan (SS) | 0.05 | 0.24 | 0.91 | 1.52 |
| Sudan (SD) | 0.96 | 2.06 | 8.08 | 14.02 |
| Tanzania (TZ) | 0.45 | 1.63 | 6.75 | 12.27 |
| Uganda (UG) | 0.24 | 1.21 | 5.29 | 9.96 |
| WAPP | 4.85 | 18.81 | 74.85 | 132.75 |
| Benin (BJ) | 0.10 | 0.36 | 1.43 | 2.50 |
| Burkina Faso (BF) | 0.10 | 0.58 | 2.39 | 4.26 |
| Cote d Ivoire (CI) | 0.52 | 0.92 | 3.55 | 6.13 |
| Gambia (GM) | 0.03 | 0.06 | 0.25 | 0.45 |
| Ghana (GH) | 0.75 | 1.78 | 6.61 | 11.69 |
| Guinea (GN) | 0.08 | 0.35 | 1.36 | 2.40 |
| Guinea Bissau (GW) | 0.00 | 0.05 | 0.20 | 0.34 |
| Liberia (LR) | 0.02 | 0.13 | 0.50 | 0.86 |
| Mali (ML) | 0.59 | 1.51 | 6.40 | 11.88 |
| Niger (NE) | 0.08 | 0.62 | 2.98 | 6.11 |
| Nigeria (NG) | 2.16 | 11.44 | 45.30 | 79.39 |
| Senegal (SNG) | 0.30 | 0.59 | 2.35 | 4.12 |
| Sierra Leone (SL) | 0.01 | 0.19 | 0.71 | 1.17 |
| Togo (TG) | 0.10 | 0.22 | 0.84 | 1.46 |
| CAPP | 1.49 | 5.03 | 20.82 | 37.91 |
| Cameroon (CM) | 0.53 | 0.95 | 3.70 | 6.43 |
| Central African Republic (CF) | 0.01 | 0.12 | 0.43 | 0.72 |
| Chad (TD) | 0.02 | 0.55 | 2.31 | 4.23 |
| Congo (CG) | 0.07 | 0.37 | 1.49 | 2.63 |
| Democratic Republic of Congo (CD) | 0.62 | 2.32 | 10.03 | 18.87 |
| Equatorial Guinea (GQ) | 0.08 | 0.31 | 1.23 | 2.18 |
| Gabon (GA) | 0.16 | 0.41 | 1.62 | 2.85 |

Supplementary Table 12. Coal demand projections by country (Mtoe) – 1.5oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 0.13 | 0.44 | 0.08 | 0.00 |
| Algeria (DZ) | 0.10 | 0.24 | 0.07 | 0.00 |
| Libya (LY) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mauritania (MR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Morocco (MA) | 0.03 | 0.20 | 0.01 | 0.00 |
| Tunisia (TN) | 0.00 | 0.00 | 0.00 | 0.00 |
| SAPP | 17.57 | 19.32 | 0.22 | 0.01 |
| Angola (AO) | 0.00 | 0.00 | 0.00 | 0.00 |
| Botswana (BW) | 0.05 | 0.25 | 0.00 | 0.00 |
| Lesotho (LS) | 0.40 | 6.41 | 0.00 | 0.00 |
| Malawi (MW) | 0.05 | 0.37 | 0.00 | 0.00 |
| Mozambique (MZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Namibia (NM) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Africa (ZA) | 16.71 | 11.27 | 0.22 | 0.01 |
| Swaziland (SZ) | 0.01 | 0.07 | 0.00 | 0.00 |
| Zambia (ZM) | 0.10 | 0.55 | 0.00 | 0.00 |
| Zimbabwe (ZW) | 0.26 | 0.39 | 0.00 | 0.00 |
| EAPP | 0.94 | 4.96 | 0.02 | 0.00 |
| Egypt (EG) | 0.001 | 0.20 | 0.00 | 0.00 |
| Burundi (BI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Djibouti (DJ) | 0.18 | 0.23 | 0.02 | 0.00 |
| Eritrea (ER) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ethiopia (ET) | 0.25 | 2.05 | 0.00 | 0.00 |
| Kenya (KE) | 0.35 | 1.23 | 0.00 | 0.00 |
| Rwanda (RW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Somalia (SO) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Sudan (SS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sudan (SD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Tanzania (TZ) | 0.16 | 1.25 | 0.00 | 0.00 |
| Uganda (UG) | 0.00 | 0.00 | 0.00 | 0.00 |
| WAPP | 0.36 | 1.76 | 0.00 | 0.00 |
| Benin (BJ) | 0.03 | 0.28 | 0.00 | 0.00 |
| Burkina Faso (BF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Cote d Ivoire (CI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gambia (GM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ghana (GH) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea (GN) | 0.00 | 0.00 | 0.00 | 0.00 |
| Guinea Bissau (GW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Liberia (LR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mali (ML) | 0.00 | 0.00 | 0.00 | 0.00 |
| Niger (NE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Nigeria (NG) | 0.03 | 1.02 | 0.00 | 0.00 |
| Senegal (SNG) | 0.30 | 0.45 | 0.00 | 0.00 |
| Sierra Leone (SL) | 0.00 | 0.00 | 0.00 | 0.00 |
| Togo (TG) | 0.00 | 0.00 | 0.00 | 0.00 |
| CAPP | 0.00 | 0.00 | 0.00 | 0.00 |
| Cameroon (CM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Central African Republic (CF) | 0.00 | 0.00 | 0.00 | 0.00 |
| Chad (TD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Congo (CG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Democratic Republic of Congo (CD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Equatorial Guinea (GQ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gabon (GA) | 0.00 | 0.00 | 0.00 | 0.00 |

Supplementary Table 13. Oil demand projections by country (Mtoe) – 1.5oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 43.37 | 46.10 | 7.90 | 0.89 |
| Algeria (DZ) | 16.73 | 17.60 | 2.91 | 0.31 |
| Libya (LY) | 10.74 | 11.30 | 1.87 | 0.20 |
| Mauritania (MR) | 0.64 | 0.45 | 0.14 | 0.04 |
| Morocco (MA) | 10.89 | 11.95 | 2.12 | 0.23 |
| Tunisia (TN) | 4.37 | 4.80 | 0.85 | 0.09 |
| SAPP | 36.04 | 39.92 | 9.06 | 1.85 |
| Angola (AO) | 4.92 | 6.36 | 2.14 | 0.68 |
| Botswana (BW) | 1.04 | 0.85 | 0.27 | 0.08 |
| Lesotho (LS) | 0.21 | 0.22 | 0.06 | 0.02 |
| Malawi (MW) | 0.22 | 1.25 | 0.41 | 0.13 |
| Mozambique (MZ) | 1.12 | 2.16 | 0.72 | 0.23 |
| Namibia (NM) | 1.27 | 0.73 | 0.23 | 0.07 |
| South Africa (ZA) | 24.66 | 24.94 | 4.15 | 0.33 |
| Swaziland (SZ) | 0.47 | 0.25 | 0.08 | 0.02 |
| Zambia (ZM) | 0.93 | 1.84 | 0.61 | 0.19 |
| Zimbabwe (ZW) | 1.21 | 1.31 | 0.40 | 0.12 |
| EAPP | 43.96 | 55.39 | 14.22 | 3.59 |
| Egypt (EG) | 0.12 | 0.68 | 0.23 | 0.07 |
| Burundi (BI) | 0.08 | 0.12 | 0.04 | 0.01 |
| Djibouti (DJ) | 27.90 | 27.90 | 5.32 | 0.85 |
| Eritrea (ER) | 0.09 | 0.33 | 0.10 | 0.03 |
| Ethiopia (ET) | 3.04 | 6.89 | 2.16 | 0.64 |
| Kenya (KE) | 3.95 | 4.14 | 1.29 | 0.38 |
| Rwanda (RW) | 0.25 | 0.92 | 0.28 | 0.08 |
| Somalia (SO) | 0.11 | 1.08 | 0.37 | 0.12 |
| South Sudan (SS) | 16.32 | 26.50 | 8.06 | 2.37 |
| Sudan (SD) | 4.00 | 5.35 | 1.71 | 0.52 |
| Tanzania (TZ) | 2.64 | 4.21 | 1.43 | 0.46 |
| Uganda (UG) | 1.40 | 3.15 | 1.12 | 0.37 |
| WAPP | 30.74 | 45.05 | 14.64 | 4.53 |
| Benin (BJ) | 1.65 | 0.94 | 0.30 | 0.09 |
| Burkina Faso (BF) | 0.88 | 1.51 | 0.51 | 0.16 |
| Cote d Ivoire (CI) | 1.74 | 2.39 | 0.75 | 0.23 |
| Gambia (GM) | 0.10 | 0.16 | 0.05 | 0.02 |
| Ghana (GH) | 3.76 | 3.30 | 1.01 | 0.30 |
| Guinea (GN) | 0.81 | 0.90 | 0.29 | 0.09 |
| Guinea Bissau (GW) | 0.11 | 0.14 | 0.04 | 0.01 |
| Liberia (LR) | 0.30 | 0.34 | 0.11 | 0.03 |
| Mali (ML) | 0.78 | 1.55 | 0.54 | 0.17 |
| Niger (NE) | 0.49 | 1.60 | 0.63 | 0.23 |
| Nigeria (NG) | 18.06 | 29.64 | 9.58 | 2.95 |
| Senegal (SNG) | 1.17 | 1.53 | 0.50 | 0.15 |
| Sierra Leone (SL) | 0.28 | 0.50 | 0.15 | 0.04 |
| Togo (TG) | 0.61 | 0.57 | 0.18 | 0.05 |
| CAPP | 4.95 | 13.03 | 4.41 | 1.41 |
| Cameroon (CM) | 1.38 | 2.46 | 0.78 | 0.24 |
| Central African Republic (CF) | 0.10 | 0.30 | 0.09 | 0.03 |
| Chad (TD) | 0.23 | 1.44 | 0.49 | 0.16 |
| Congo (CG) | 0.77 | 0.97 | 0.32 | 0.10 |
| Democratic Republic of Congo (CD) | 0.90 | 6.00 | 2.12 | 0.70 |
| Equatorial Guinea (GQ) | 0.81 | 0.80 | 0.26 | 0.08 |
| Gabon (GA) | 0.77 | 1.06 | 0.34 | 0.11 |

Supplementary Table 14. Natural gas demand projections by country (Mtoe) – 1.5oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 15.79 | 16.59 | 9.31 | 0.89 |
| Algeria (DZ) | 11.18 | 10.12 | 4.93 | 0.47 |
| Libya (LY) | 2.72 | 2.46 | 1.20 | 0.11 |
| Mauritania (MR) | 0.00 | 0.00 | 0.00 | 0.00 |
| Morocco (MA) | 0.06 | 0.12 | 0.09 | 0.01 |
| Tunisia (TN) | 1.84 | 3.89 | 3.08 | 0.30 |
| SAPP | 3.98 | 9.82 | 5.95 | 3.75 |
| Angola (AO) | 0.63 | 2.20 | 2.79 | 2.66 |
| Botswana (BW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Lesotho (LS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Malawi (MW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Mozambique (MZ) | 0.14 | 0.75 | 0.93 | 0.88 |
| Namibia (NM) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Africa (ZA) | 3.21 | 6.86 | 2.22 | 0.21 |
| Swaziland (SZ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Zambia (ZM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Zimbabwe (ZW) | 0.00 | 0.00 | 0.00 | 0.00 |
| EAPP | 12.75 | 14.94 | 7.51 | 2.32 |
| Egypt (EG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Burundi (BI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Djibouti (DJ) | 12.62 | 13.48 | 5.65 | 0.54 |
| Eritrea (ER) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ethiopia (ET) | 0.00 | 0.00 | 0.00 | 0.00 |
| Kenya (KE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Rwanda (RW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Somalia (SO) | 0.00 | 0.00 | 0.00 | 0.00 |
| South Sudan (SS) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sudan (SD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Tanzania (TZ) | 0.14 | 1.46 | 1.86 | 1.78 |
| Uganda (UG) | 0.00 | 0.00 | 0.00 | 0.00 |
| WAPP | 0.14 | 1.46 | 1.86 | 1.78 |
| Benin (BJ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Burkina Faso (BF) | 4.02 | 11.46 | 13.91 | 12.80 |
| Cote d Ivoire (CI) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gambia (GM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Ghana (GH) | 0.25 | 0.83 | 0.98 | 0.89 |
| Guinea (GN) | 0.00 | 0.05 | 0.07 | 0.07 |
| Guinea Bissau (GW) | 0.00 | 0.00 | 0.00 | 0.00 |
| Liberia (LR) | 0.00 | 0.31 | 0.37 | 0.35 |
| Mali (ML) | 0.00 | 0.00 | 0.00 | 0.00 |
| Niger (NE) | 0.00 | 0.00 | 0.00 | 0.00 |
| Nigeria (NG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Senegal (SNG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Sierra Leone (SL) | 3.76 | 10.27 | 12.49 | 11.50 |
| Togo (TG) | 0.00 | 0.00 | 0.00 | 0.00 |
| CAPP | 0.00 | 0.00 | 0.00 | 0.00 |
| Cameroon (CM) | 0.00 | 0.00 | 0.00 | 0.00 |
| Central African Republic (CF) | 0.00 | 0.64 | 0.79 | 0.73 |
| Chad (TD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Congo (CG) | 0.00 | 0.00 | 0.00 | 0.00 |
| Democratic Republic of Congo (CD) | 0.00 | 0.00 | 0.00 | 0.00 |
| Equatorial Guinea (GQ) | 0.00 | 0.00 | 0.00 | 0.00 |
| Gabon (GA) | 0.00 | 0.00 | 0.00 | 0.00 |
| NAPP | 0.00 | 0.28 | 0.34 | 0.32 |
| Algeria (DZ) | 0.00 | 0.37 | 0.45 | 0.41 |

Supplementary Table 15. Biofuels & waste demand projections by country (Mtoe) – 1.5oC scenario.

| Mtoe | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| NAPP | 2.63 | 6.84 | 18.57 | 24.98 |
| Algeria (DZ) | 0.06 | 0.68 | 2.84 | 4.33 |
| Libya (LY) | 0.18 | 2.04 | 8.53 | 13.00 |
| Mauritania (MR) | 0.17 | 1.44 | 1.31 | 1.13 |
| Morocco (MA) | 1.39 | 1.69 | 3.71 | 4.11 |
| Tunisia (TN) | 0.82 | 0.99 | 2.18 | 2.42 |
| SAPP | 41.60 | 70.58 | 71.82 | 63.88 |
| Angola (AO) | 6.05 | 20.40 | 19.61 | 17.67 |
| Botswana (BW) | 0.54 | 2.73 | 2.44 | 2.13 |
| Lesotho (LS) | 0.56 | 0.71 | 0.58 | 0.45 |
| Malawi (MW) | 0.70 | 4.01 | 3.76 | 3.29 |
| Mozambique (MZ) | 8.09 | 6.94 | 6.56 | 5.84 |
| Namibia (NM) | 0.17 | 2.35 | 2.10 | 1.75 |
| South Africa (ZA) | 10.91 | 13.14 | 18.49 | 17.24 |
| Swaziland (SZ) | 0.46 | 0.81 | 0.71 | 0.57 |
| Zambia (ZM) | 6.67 | 8.47 | 7.96 | 6.97 |
| Zimbabwe (ZW) | 7.44 | 11.03 | 9.61 | 7.99 |
| EAPP | 90.24 | 131.21 | 130.28 | 119.48 |
| Egypt (EG) | 0.98 | 2.17 | 2.08 | 1.85 |
| Burundi (BI) | 0.02 | 0.38 | 0.32 | 0.26 |
| Djibouti (DJ) | 1.73 | 1.96 | 10.70 | 15.14 |
| Eritrea (ER) | 0.46 | 1.04 | 0.91 | 0.76 |
| Ethiopia (ET) | 36.92 | 46.62 | 41.62 | 35.16 |
| Kenya (KE) | 10.92 | 13.28 | 11.77 | 9.91 |
| Rwanda (RW) | 1.19 | 2.94 | 2.52 | 2.04 |
| Somalia (SO) | 1.87 | 3.48 | 3.42 | 3.16 |
| South Sudan (SS) | 0.15 | 0.18 | 0.15 | 0.13 |
| Sudan (SD) | 6.82 | 7.97 | 7.80 | 6.73 |
| Tanzania (TZ) | 6.98 | 19.18 | 17.41 | 14.97 |
| Uganda (UG) | 18.15 | 25.41 | 24.57 | 22.22 |
| WAPP | 11.02 | 14.74 | 14.96 | 14.01 |
| Benin (BJ) | 125.81 | 181.16 | 167.70 | 146.90 |
| Burkina Faso (BF) | 1.82 | 3.02 | 2.76 | 2.41 |
| Cote d Ivoire (CI) | 2.30 | 4.85 | 4.62 | 4.10 |
| Gambia (GM) | 4.27 | 7.65 | 6.87 | 5.90 |
| Ghana (GH) | 0.10 | 0.50 | 0.48 | 0.44 |
| Guinea (GN) | 2.42 | 10.60 | 9.28 | 7.81 |
| Guinea Bissau (GW) | 2.28 | 2.88 | 2.63 | 2.31 |
| Liberia (LR) | 0.53 | 0.72 | 0.63 | 0.53 |
| Mali (ML) | 1.53 | 1.79 | 1.61 | 1.37 |
| Niger (NE) | 2.26 | 4.98 | 4.92 | 4.55 |
| Nigeria (NG) | 2.15 | 5.13 | 5.76 | 5.88 |
| Senegal (SNG) | 102.72 | 130.72 | 120.58 | 105.10 |
| Sierra Leone (SL) | 1.13 | 4.90 | 4.55 | 3.97 |
| Togo (TG) | 0.83 | 1.61 | 1.37 | 1.13 |
| CAPP | 1.50 | 1.83 | 1.63 | 1.41 |
| Cameroon (CM) | 31.78 | 49.80 | 48.41 | 44.09 |
| Central African Republic (CF) | 5.22 | 7.90 | 7.16 | 6.19 |
| Chad (TD) | 0.22 | 0.96 | 0.83 | 0.69 |
| Congo (CG) | 1.23 | 4.60 | 4.48 | 4.07 |
| Democratic Republic of Congo (CD) | 1.22 | 3.10 | 2.88 | 2.53 |
| Equatorial Guinea (GQ) | 19.98 | 27.63 | 27.87 | 26.07 |
| Gabon (GA) | 0.09 | 0.65 | 0.61 | 0.53 |
| NAPP | 3.81 | 4.95 | 4.57 | 3.99 |

In the following section, information regarding the socio-economic situation, the power system, the energy reserves and renewable potential on a power pool level in Africa are presented.

- Supplementary Table 16. Information on socio-economic situation, energy reserves, power system, energy reserves and renewable potential on a power pool level in Africa.

|  | NAPP | WAPP | CAPP | SAPP | EAPP |
| --- | --- | --- | --- | --- | --- |
| Population (mil.) (2018)1 | 1009 | 3762 | 1381 | 2323 | 4000 |
| GDP (2010$B) (2017)1 | 4205 | 6428 | 1306 | 6880 | 5179 |
| Electricity intensity (kWh/cap) (2016)2 | 1166 | 168 | 136 | 1121 | 513 |
| Electricity access (%) (2016)2 | 97 | 52 | 25 | 86 | 54 |
| Renewables (%) (2018)3 | 16 | 29 | 51 | 55 | 39 |
| Reserves |  |  |  |  |  |
| Coal(Mt) (2017)4 | 80 | 386 | 100 | 37886 | 18 |
| Crude (Mb) (2019)4 | 61 | 37 | 7 | 8 | 6 |
| NG(Tcf) (2019)4 | 230 | 201 | 14 | 117 | 72 |
| Uranium(Mt) (2019)5,6 | - | 191 | 57 | 969 | - |
| Potential |  |  |  |  |  |
| Solar(TWh/yr) (2018)7 | 189 | 127 | 92 | 304 | 395 |
| Wind(TWh/yr) (2018) 7 | 138 | 43 | 14 | 115 | 197 |
| Hydro(TWh/yr) (2018)8,9 | 59 | 101 | 571 | 415 | 335 |

In the following section, the techno-economic assumptions used to develop the model for Africa are presented.

- Supplementary Table 17: Economic parameters for generic power generation technologies10–13.

| Technology | Overnight Investment Cost (USD/kW) |  |  |  | Operational and Maintenance Cost (USD/kW) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015 | 2030 | 2050 | 2070 | 2015 | 2030 | 2050 | 2070 |
| Diesel (centralised) | 1200 | 1200 | 1200 | 1200 | 35 | 35 | 35 | 35 |
| Diesel 1 kW system (decentralised) | 752 | 752 | 752 | 752 | 23 | 23 | 23 | 23 |
| HFO | 1467 | 1467 | 1467 | 1467 | 44 | 44 | 44 | 44 |
| OCGT | 400 | 400 | 400 | 400 | 20 | 20 | 20 | 20 |
| CCGT | 700 | 700 | 700 | 700 | 25 | 25 | 25 | 25 |
| CCGT - CCS | 2450 | 2200 | 2000 | 1800 | 80 | 70 | 70 | 70 |
| Supercritical coal | 1600 | 1600 | 1600 | 1600 | 65 | 65 | 65 | 65 |
| Coal + CCS | 4500 | 4100 | 3700 | 3300 | 160 | 150 | 130 | 110 |
| Hydro (large scale) | 2100 | 2100 | 2100 | 2100 | 55 | 55 | 55 | 55 |
| Hydro (small scale) | 3300 | 3300 | 3300 | 3300 | 65 | 65 | 65 | 65 |
| Hydro (medium scale) | 2100 | 2100 | 2100 | 2100 | 55 | 55 | 55 | 55 |
| Biomass | 2150 | 2100 | 2000 | 1900 | 75 | 75 | 65 | 55 |
| Biomass (CHP small) | 4800 | 4600 | 4400 | 4200 | 180 | 170 | 170 | 170 |
| Biomass CCS | 4256 | 3864 | 3360 | 3100 | 91.28 | 91.28 | 91.28 | 89.89 |
| Nuclear | 4000 | 4000 | 4000 | 4000 | 170 | 170 | 170 | 170 |
| Geothermal | 3100 | 2900 | 2700 | 2500 | 60 | 60 | 50 | 40 |
| Wind onshore | 1880 | 1760 | 1680 | 1600 | 48 | 44 | 44 | 44 |
| Wind offshore | 4700 | 3200 | 2600 | 2000 | 165 | 125 | 105 | 85 |
| Solar PV (centralised) | 2400 | 1400 | 1080 | 760 | 24 | 22 | 22 | 22 |
| Solar PV (decentralised) | 2840 | 1640 | 1240 | 840 | 28 | 26 | 26 | 26 |
| Solar PV with battery | 4449 | 2373 | 1845 | 1364 | 48 | 46 | 46 | 46 |
| Solar CSP | 5050 | 3800 | 2900 | 2000 | 200 | 150 | 110 | 70 |
| Solar CSP with storage | 6789 | 4929 | 3997 | 3065 | 228 | 178 | 138 | 90 |

- Supplementary Table 18. Technical parameters for generic power generation technologies10–13.

| Technology | Lifetime (Years) | Construction time (Years) | Efficiency (%) |  |  |  | Load factor (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2015-2070 | 2015-2070 | 2015 | 2030 | 2050 | 2070 | 2015-2070 |
| Diesel (centralised) | 25 | 2 | 35 | 35 | 35 | 35 | 80 |
| Diesel 1 kW system (decentralised) | 10 | <1 | 16 | 16 | 16 | 16 | 30 |
| HFO | 25 | 2 | 35 | 35 | 35 | 35 | 80 |
| OCGT | 25 | 2 | 38 | 39 | 43 | 47 | 85 |
| CCGT | 30 | 3 | 58 | 59 | 61 | 63 | 85 |
| CCGT - CCS | 30 | 3 | 51 | 52 | 56 | 60 | 85 |
| Supercritical coal | 30 | 4 | 39 | 39 | 39 | 39 | 85 |
| Coal + CCS | 30 | 4 | 32 | 35 | 37 | 39 | 85 |
| Hydro (large scale) | 50* | 4 | 100 | 100 | 100 | 100 | Varies |
| Hydro (small scale) | 30* | 4 | 100 | 100 | 100 | 100 | Varies |
| Hydro (medium scale) | 50* | 4 | 100 | 100 | 100 | 100 | Varies |
| Biomass | 40 | 3 | 35 | 35 | 35 | 35 | 50 |
| Biomass (CHP small) | 40 | 2 | 65 | 65 | 65 | 65 | 50 |
| Biomass CCS | 40 | 3 | 27 | 29 | 30 | 31 | 50 |
| Nuclear | 60 | 7 | 33 | 33 | 33 | 33 | 85 |
| Geothermal | 25 | 4 | 80 | 80 | 80 | 80 | 85 |
| Wind onshore | 25 | 1.5 | 100 | 100 | 100 | 100 | Varies |
| Wind offshore | 25 | 3 | 100 | 100 | 100 | 100 | Varies |
| Solar PV (centralised) | 25 | 1 | 100 | 100 | 100 | 100 | Varies |
| Solar PV (decentralised) | 20 | 1 | 100 | 100 | 100 | 100 | Varies |
| Solar PV with battery | 20 | 1 | 100 | 100 | 100 | 100 | Varies |
| Solar CSP | 25 | 3 | 100 | 100 | 100 | 100 | Varies |
| Solar CSP with storage | 25 | 3 | 100 | 100 | 100 | 100 | Varies |

*Instead of this generic value, in our model it is assumed that the lifetime of the new hydropower plants is 80 years.

- Supplementary Table 19. Economic parameters for generic conversion technologies11.

| Technology | Capital & Fixed cost per unit of output (USD/PJ) |  | Variable cost per unit of output (USD/GJ) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2015-2070 | 2015 |  | 2030 | 2070 |  |
| LNG Liquefaction plant | 1520000 | 0.63 |  | 0.95 | 0.95 |  |
| Gas Regasification plant including LNG storage | 855000 | 0.63 |  | 0.63 | 0.63 |  |
| Crude oil refinery | 24.13 | 0.72 |  | 0.72 | 0.72 |  |

- Supplementary Table 20. Technical parameters for generic conversion technologies11.

| Technology | Construction Time (Years) | Lifetime (Years) | Efficiency (%) | Load factor (%) |
| --- | --- | --- | --- | --- |
|  | 2015-2070 | 2015-2070 | 2015-2070 | 2015-2070 |
| LNG Liquefaction plant | 4 | 25 | 0.85 | Varies |
| Gas Regasification plant including LNG storage | 4 | 25 | 0.98 | Varies |
| Crude oil refinery | 4 | 35 | Varies | Varies |

- Supplementary Table 21. Water factors per fuel-process14.

| Fuel | Processes | Withdrawal (Mm3 /MWhf*) | Consumption (Mm3 /MWhf*) | Output Water (Mm3 /MWhf*) |
| --- | --- | --- | --- | --- |
| Coal | Extraction and processing | 0.0529 | 0.0523 | 0.0006 |
| Crude oil | Extraction (first and secondary recovery) | 0.2011 | 0.2011 | 0.000 |
|  | Refining | 0.171 | 0.171 | 0.000 |
| Natural gas | Extraction and processing | 0.0019 | 0.0019 | 0.000 |
| Biomass | Extraction | - | - | - |
| Uranium | Surface mining and milling | 0.0059 | 0.0059 | 0 |
|  | Underground mining and milling | 0.0028 | 0.0028 | 0 |

*The units for fuel extraction and processing is Mm3 /MWhf.

- Supplementary Table 22. Water factors per technology type 14.

| Fuel | Processes | Technology Type | Withdrawal (Mm3 /MWh*) | Consumption (Mm3 /MWh*) | Output Water (Mm3 /MWh*) |
| --- | --- | --- | --- | --- | --- |
| Biomass-fired power plants | AIR |  | 0.000 | 0 | 0.000 |
| Coal-fired power plants | AIR | Dry cooling - SUBCR | 0.322 | 0.322 | 0.000 |
|  | MDT/NDT | Cooling tower - SUBCR | 2.5 | 2.008 | 0.492 |
|  | OTF/OTS | Once-through - SUBCR | 102.54 | 0.43 | 102.11 |
| Coal-fired power plants with CCS | MDT/NDT | Cooling tower - SUBCR | 4.92 | 3.56 | 1.36 |
| LFO & HFO-fired power plants | AIR | Dry cooling - CC | 0.015 | 0.015 | 0.000 |
|  | MDT/NDT | Cooling Tower - CC | 0.947 | 0.795 | 0.152 |
|  | OTF/OTS | Once-through - CC | 34.091 | 0.379 | 33.712 |
|  | AIR | Dry cooling - SUBCR | 0.367 | 0.367 | 0.000 |
|  | MDT/NDT | Cooling tower - SUBCR | 4.545 | 2.765 | 1.780 |
|  | OTF/OTS | Once through - SUBCR | 136.364 | 1.098 | 135.266 |
| Natural gas-fired power plants | AIR | Dry cooling - CC | 0.015 | 0.015 | 0.000 |
|  | MDT/NDT | Cooling tower - CC | 0.947 | 0.795 | 0.152 |
|  | OTF/OTS | Once-through - CC | 34.091 | 0.379 | 33.712 |
|  | AIR | Dry cooling - SUBCR | 0.367 | 0.367 | 0.000 |
|  | MDT/NDT | Cooling tower - SUBCR | 4.545 | 2.765 | 1.780 |
|  | OTF/OTS | Once through - SUBCR | 136.364 | 1.098 | 135.266 |
| Natural gas-fired power plants with CCS | MDT/NDT | Cooling tower - CC | 1.93 | 1.44 | 0.49 |
| Wind power plant |  | Wind turbine | 0.004 | 0.001 | 0.003 |
| Nuclear power plants |  | Once through - SUBCR | 178.03 | 1.515 | 176.515 |
| Geothermal | AIR | Dry cooling - SUBCR | 0 | 0.000 | 0.000 |
|  | MDT/NDT | Cooling tower - SUBCR | 0.068 | 0.042 | 0.026 |
|  | OTF/OTS |  | 1.134 | 1.134 | 0.000 |
| Biofuel-fired power plants |  |  | 9 | 7.200 | 1.800 |
| Solar power plants | N/A | PV | 0.023 | 0.023 | 0.000 |
|  | AIR | CSP | 0.098 | 0.098 | 0.000 |
|  | N/A | PV (Utility scale) | 0.023 | 0.023 | 0.000 |

*The units for plant operation is Mm3 /MWhe.

- Supplementary Table 23. Water factors per country for hydropower due to water loss through evaporation14.

| Country | Evapotranspiration / Electricity (Mcm/GWh) |
| --- | --- |
| Algeria (DZ) | 0.16 |
| Angola (AO) | 0.07 |
| Benin (BJ) | 1.43 |
| Botswana (BW) | 0.29 |
| Burkina Faso (BF) | 2.88 |
| Burundi (BI) | 0.02 |
| Cameroon (CM) | 0.24 |
| Central African Republic (CF) | 0.0001 |
| Chad (TD) | 0.24 |
| Democratic Republic of Congo (CD) | 0.13 |
| Djibouti (DJ) | 0.09 |
| Egypt (EG) | 0.45 |
| Equatorial Guinea (GQ) | 0.01 |
| Eritrea (ER) | 0.21 |
| Ethiopia (ET) | 0.09 |
| Gabon (GA) | 0.03 |
| Gambia (GM) | 0.18 |
| Ghana (GH) | 1.69 |
| Guinea (GN) | 0.15 |
| Guinea-Bissau (GW) | 0.18 |
| Cote d Ivoire (CI) | 1.11 |
| Kenya (KE) | 0.06 |
| Lesotho (LS) | 0.0004 |
| Liberia (LR) | 0.44 |
| Libya (LY) | 0.30 |
| Malawi (MW) | 0.001 |
| Mali (ML) | 0.33 |
| Mauritania (MR) | 0.33 |
| Morocco (MA) | 0.27 |
| Mozambique (MZ) | 0.28 |
| Namibia (NA) | 0.01 |
| Niger (NE) | 0.94 |
| Nigeria (NG) | 0.38 |
| Republic of the Congo (CG) | 0.07 |
| Rwanda (RW) | 0.04 |
| Senegal (SN) | 0.24 |
| Sierra Leone (SL) | 0.06 |
| Somalia (SO) | 0.07 |
| South Africa (ZA) | 0.08 |
| South Sudan (SS) | 0.11 |
| Sudan (SD) | 0.34 |
| Swaziland (SZ) | 0.02 |
| Tanzania (TZ) | 0.43 |
| Togo (TG) | 1.02 |
| Tunisia (TN) | 0.27 |
| Uganda (UG) | 0.001 |
| Zambia (ZM) | 0.55 |
| Zimbabwe (ZW) | 1.35 |

- Supplementary Table 24. Identified fossil fuel reserves in Africa.

| Reserves | Total Recoverable Coal (mil. short tons) 4 | Crude Oil proved reserves (bil. barrels) 4 | Natural Gas proved reserves (tril. cubic feet)4 | Uranium (tonnes)5,6 |
| --- | --- | --- | --- | --- |
|  | 2017 | 2019 | 2019 | 2019 |
| Algeria | 65.04 | 12.2 | 159.05 | Exploratory |
| Angola | - | 8.38 | 14.91 | - |
| Benin | - | 0.01 | - | - |
| Botswana | 127.72 | - | - | 193500 |
| Cameroon | - | 0.2 | 4.77 | - |
| Central African Republic | 3.31 | - | - | 5700 |
| Chad | - | 1.5 | NA | - |
| Congo (Brazzaville) | - | 1.6 | 3.2 | - |
| Congo (Kinshasa) | 97.00 | 0.18 | 0.04 | 51776 |
| Cote d´Ivoire | - | 0.1 | 1 | - |
| Egypt | 17.67 | 3.3 | 63 | - |
| Equatorial Guinea | - | 1.1 | 5.12 | - |
| Ethiopia | - | 0.00043 | 0.88 | - |
| Gabon | - | 2 | - | Exploratory |
| Ghana | - | 0.66 | 0.8 | - |
| Guinea | - | - | - | 179000 |
| Libya | - | 48.36 | 53.14 | - |
| Malawi | 2.20 | - | - | Exploratory |
| Mali | - | - | - | Exploratory |
| Mauritania | - | 0.02 | 15 | Exploratory |
| Morocco | 15.43 | 0.00068 | 0.05 | Exploratory |
| Mozambique | 1975.34 | - | 100 | - |
| Namibia | - | - | 2.2 | 199068 |
| Niger | 6.61 | 0.15 | NA | 11661 |
| Nigeria | 379.19 | 36.18 | 198.71 | Exploratory |
| Rwanda | - | - | 2 | - |
| South Africa | 34722.77 | 0.02 | 0.03 | 514421 |
| Sudan | - | 5 | 3 | - |
| Swaziland | 158.73 | - | - |  |
| Tanzania | 296.52 | - | 0.23 | 23200 |
| Tunisia | - | 0.43 | 2.3 | - |
| Uganda | - | 2.5 | 0.5 | - |
| Zambia | 49.60 | - | - | 37296 |
| Zimbabwe | 553.36 | - | - | 1720 |

- Supplementary Table 25. Renewable energy potential per country in Africa.

| Estimates of Renewable Energy Potential | Hydro (MW)8 | Small hydro (MW) 9 | Solar Thermal (TWh/y)7 | Solar PV (TWh/y)7 | Biomass (MW)10 |
| --- | --- | --- | --- | --- | --- |
| Algeria | n/a | n/a | 26530 | 27904 | n/a |
| Angola | n/a | 861 | 9786 | 13319 | 500 |
| Benin | n/a | 187 | - | 3898 | 761 |
| Botswana | n/a | 1 | 13070 | 13764 | 10 |
| Burkina Faso | - | 38 | - | 7742 | 1075 |
| Burundi | 1700 | 61 | 786 | 888 | n/a |
| Cameroon | 23000 | 615 | 3706 | 10105 | n/a |
| Central African Republic | n/a | 41 | 3471 | 5284 | n/a |
| Chad | - | - | 10284 | 10506 | n/a |
| Congo (Brazzaville) | n/a | 50 | 2 | 6778 | n/a |
| Congo (DRC,Kinshasa) | 100000 | 101 | 12439 | 22862 | 500 |
| Cote d'Ivoire | 1900 | 40.7 | 221 | 10325 | 3260 |
| Djibouti | n/a | - | 852 | 947 | n/a |
| Egypt | 3664 | 52 | 26605 | 32218 | n/a |
| Equatorial Guinea | n/a | 7 | - | 706 | n/a |
| Eritrea | n/a | - | 4349 | 4775 | n/a |
| Ethiopia | 45000 | 1500 | 22959 | 27154 | n/a |
| Gabon | n/a | 6 | 6 | 5402 | n/a |
| Gambia | n/a | 12 | 316 | 474 | 60 |
| Ghana | 2480 | 1245 | 229 | 7644 | 4449 |
| Guinea | n/a | 198 | 467 | 5204 | 1732 |
| Guinea-Bissau | 184 | - | 906 | 1493 | 205 |
| Kenya | 6000 | 3000 | 15399 | 23046 | n/a |
| Lesotho | n/a | 20 | 1122 | 938 | 10 |
| Liberia | 2300 | 65.9 | - | 667 | 1375 |
| Libya | n/a | - | 11823 | 13979 | n/a |
| Malawi | n/a | 150 | 4474 | 5210 | 200 |
| Mali | 1150 | 117 | - | 7906 | 447 |
| Mauritania | n/a | - | 4988 | 7990 | n/a |
| Morocco | n/a | 54 | 15127 | 15155 | n/a |
| Mozambique | n/a | 1000 | 16851 | 22024 | 1000 |
| Namibia | n/a | 108 | 29716 | 26183 | 50 |
| Niger | n/a | - | 8829 | 15669 | 266 |
| Nigeria | >14120 | 735 | 10045 | 32456 | 7291 |
| Rwanda | 500 | 48 | 789 | 892 | n/a |
| Senegal | 1,200 | n/a | 1537 | 7519 | 466 |
| Sierra Leone | - | 1513 | 197 | 1499 | 587 |
| Somalia | n/a | - | 13156 | 25687 | n/a |
| South Africa | n/a | 247 | 43275 | 42243 | 3000 |
| Sudan and South Sudan | 4920 | 27 | 77422 | 87817 | n/a |
| Swaziland | n/a | 16 | 559 | 572 | 200 |
| Tanzania | 3800 | 400 | 31482 | 38804 | 1000 |
| Togo | - | 144 | - | 1257 | 378 |
| Tunisia | n/a | 56 | 2045 | 4645 | n/a |
| Uganda | >4500 | 200 | 8582 | 9470 | n/a |
| Zambia | 6000 | 42 | 15691 | 17894 | 1000 |
| Zimbabwe | n/a | 120 | 11874 | 15864 | 1000 |

- Supplementary Table 26. Renewable energy potential per country in Africa.

| Estimates of Renewable Energy Potential | Wind (CF 20%) (TWh/y)7 |  | Wind (CF 30%) (TWh/y)7 | Wind (CF 40%) (TWh/y)7 | Geothermal (MW)10 |
| --- | --- | --- | --- | --- | --- |
| Algeria |  | 30155 | 2535.9 | 153.4 | n/a |
| Angola |  | 202 | - | - | - |
| Benin |  | 405 | - | - | - |
| Botswana |  | 9,793 | 303 | - | - |
| Burkina Faso |  | 4154 | 7.5 | - | - |
| Burundi |  | - | - | - | n/a |
| Cameroon |  | 979 | 15.9 | - | - |
| Central African Republic |  | 79 | - | - | - |
| Chad |  | 9165 | 1519.4 | 578.3 | - |
| Congo (Brazzaville) |  | - | - | - | - |
| Congo (DRC,Kinshasa) |  | 2173 | 41.4 | - | n/a |
| Cote d'Ivoire |  | 430 | - | - | - |
| Djibouti |  | 934 | 149.1 | 77.3 | 1000 |
| Egypt |  | 36601 | 6185 | 529.9 | n/a |
| Equatorial Guinea |  | - | - | - | - |
| Eritrea |  | 3154 | 412.4 | 129.1 | n/a |
| Ethiopia |  | 14838 | 3002.1 | 1981 | 5000 |
| Gabon |  | - | - | - | - |
| Gambia |  | 173 | 1.3 | - | - |
| Ghana |  | 606 | 2.4 | - | - |
| Guinea |  | 2 | - | - | - |
| Guinea-Bissau |  | 124 | - | - | - |
| Kenya |  | 22476 | 4446.4 | 1739.6 | 10000 |
| Lesotho |  | 599 | 40.1 | 3.7 | - |
| Liberia |  | - | - | - | - |
| Libya |  | 21649 | 5149.5 | 1079.5 | n/a |
| Malawi |  | 1986 | 262.1 | 42.4 | n/a |
| Mali |  | 1923 | - | - | - |
| Mauritania |  | 11822 | 2940.5 | 1337.8 | - |
| Morocco |  | 11297 | 1458.8 | 851 | n/a |
| Mozambique |  | 10805 | 395.9 | 5.2 | n/a |
| Namibia |  | 15196 | 497 | 4.9 | - |
| Niger |  | 14628 | 1262 | 55.8 | - |
| Nigeria |  | 12867 | 95.3 | - | n/a |
| Rwanda |  | - | - | - | 700 |
| Senegal |  | 5454 | 323.6 | 3 | - |
| Sierra Leone |  | - | - | - | - |
| Somalia |  | 43539 | 10616.4 | 8893.3 | - |
| South Africa |  | 41195 | 6076.3 | 1559.1 | n/a |
| Sudan and South Sudan |  | 61661 | 9837.8 | 2947.1 | 400 |
| Swaziland |  | 476 | 9.7 | - | - |
| Tanzania |  | 18456 | 2295.2 | 789.2 | 650 |
| Togo |  | 79 | - | - | - |
| Tunisia |  | 6842 | 1244 | 226.5 | n/a |
| Uganda |  | 815 | 100.7 | 23.8 | 450 |
| Zambia |  | 13229 | 1145 | 15.6 | n/a |
| Zimbabwe |  | 12137 | 1000.3 | 47.3 | - |

- Supplementary Table 27. Fuel price cost projections2,10,13,15.

GDP deflator 1 was used to adjust the values in 2015 USD.

| USD/GJ | 2015 | 2030 | 2050 | 2070 |
| --- | --- | --- | --- | --- |
| Crude Oil, imports | 8.33 | 14.26 | 16.89 | 17.40 |
| HFO, coastal | 6.60 | 9.83 | 11.64 | 11.99 |
| HFO, inland | 8.30 | 12.44 | 14.73 | 15.18 |
| Diesel, coastal | 11.20 | 16.70 | 19.78 | 20.38 |
| Diesel, inland | 12.90 | 19.24 | 22.79 | 23.48 |
| Gas, domestic | 7.10 | 10.48 | 11.43 | 12.46 |
| Gas, pipeline | 8.60 | 12.69 | 13.85 | 15.10 |
| Gas, imported (LNG) | 9.20 | 13.56 | 14.80 | 16.12 |
| Coal, domestic | 3.30 | 3.82 | 3.87 | 3.88 |
| Coal, imported | 4.90 | 5.84 | 5.91 | 5.92 |
| Biomass, moderate | 1.60 | 1.60 | 1.60 | 1.60 |
| Biomass, scarce | 3.90 | 3.90 | 3.90 | 3.90 |

- Supplementary Table 28. Losses in national transmission and distribution network per country in Africa17–21.

The energy conversion technologies are estimated to have 5% losses throughout the modelling period.

| Country | Transmission | Distribution |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2015 | 2030 | 2050 | 2070 |
| Algeria | 5% | 26.6% | 22.1% | 17.9% | 13.7% |
| Angola | 5% | 7.4% | 5.3% | 5.3% | 5.3% |
| Benin | 5% | 13.6% | 16.8% | 14.7% | 12.6% |
| Botswana | 4% | 16.6% | 13.5% | 11.5% | 9.4% |
| Burkina Faso | 5% | 46.1% | 46.3% | 42.1% | 37.9% |
| Burundi | 5% | 10.5% | 8.4% | 6.3% | 4.2% |
| Cameroon | 5% | 20.8% | 17.9% | 15.8% | 13.7% |
| Central African Republic | 5% | 20.8% | 17.9% | 15.8% | 13.7% |
| Chad | 5% | 20.8% | 17.9% | 15.8% | 13.7% |
| Congo (Brazzaville) | 5% | 46.3% | 43.2% | 38.9% | 34.7% |
| Congo (DRC,Kinshasa) | 5% | 10.5% | 8.4% | 6.3% | 5.3% |
| Cote d'Ivoire | 5% | 17.9% | 13.7% | 11.6% | 9.5% |
| Djibouti | 5% | 18.9% | 16.8% | 14.7% | 12.6% |
| Egypt | 3.6% | 8.7% | 7.7% | 5.6% | 3.5% |
| Equatorial Guinea | 5% | 27.5% | 20.0% | 15.8% | 11.6% |
| Eritrea | 5% | 19.1% | 13.7% | 11.6% | 9.5% |
| Ethiopia | 5% | 13.7% | 11.6% | 9.5% | 7.4% |
| Gabon | 5% | 40.4% | 35.8% | 31.6% | 27.4% |
| Gambia | 5% (2015), 4%( 2040) | 17.9% | 15.8% | 13.5% | 11.5% |
| Ghana | 5% | 17.9% | 17.9% | 15.8% | 13.7% |
| Guinea | 5% (2015), 4%( 2040) | 6.3% | 5.3% | 4.2% | 4.2% |
| Guinea-Bissau | 5% | 47.7% | 51.6% | 47.4% | 43.2% |
| Kenya | 5% | 14.7% | 11.6% | 7.4% | 5.3% |
| Lesotho | 5% | 11.6% | 7.4% | 6.3% | 5.3% |
| Liberia | 5% | 32.4% | 30.5% | 26.3% | 22.1% |
| Libya | 5% | 28.4% | 27.4% | 23.2% | 18.9% |
| Malawi | 5% | 17.9% | 15.8% | 13.7% | 11.6% |
| Mali | 5% | 17.2% | 15.8% | 13.7% | 11.6% |
| Mauritania | 5% | 33.2% | 34.7% | 30.5% | 26.3% |
| Morocco | 5% | 11.6% | 8.4% | 6.3% | 4.2% |
| Mozambique | 5% | 11.6% | 10.5% | 6.3% | 4.2% |
| Namibia | 3% | 5.2% | 6.2% | 4.1% | 2.1% |
| Niger | 5% | 22.1% | 20.0% | 15.8% | 11.6% |
| Nigeria | 5% | 11.6% | 8.4% | 6.3% | 4.2% |
| Rwanda | 5% | 30.4% | 21.1% | 16.8% | 12.6% |
| Senegal | 5% | 10.5% | 8.4% | 6.3% | 4.2% |
| Sierra Leone | 5% | 43.2% | 41.1% | 32.6% | 24.2% |
| Somalia | 5% | 47.4% | 42.1% | 33.7% | 25.3% |
| South Africa | 4% | 5.2% | 4.2% | 4.2% | 4.2% |
| South Sudan | 5% (2015), 4%( 2040) | 5.3% | 5.3% | 4.0% | 4.0% |
| Sudan | 5% (2015), 4%( 2040) | 10% | 5.3% | 4.0% | 4.0% |
| Swaziland | 5% | 5.0% | 5.0% | 5.0% | 5.0% |
| Tanzania | 5% | 13.7% | 10.5% | 8.4% | 6.3% |
| Togo | 5% | 2.6% | 8.4% | 6.3% | 4.2% |
| Tunisia | 5% | 12.6% | 10.5% | 8.4% | 6.3% |
| Uganda | 5% | 13.7% | 11.6% | 9.5% | 7.4% |
| Zambia | 4% | 6.2% | 9.4% | 7.3% | 5.2% |
| Zimbabwe | 4% | 15.6% | 11.5% | 9.4% | 7.3% |

- Supplementary Table 29. National grid-connected renewable energy generation targets22,23.

| Country | Power pool | 2020 | 2024 | 2030 |
| --- | --- | --- | --- | --- |
| Algeria | NAPP |  |  | 27% |
| Egypt | EAPP | 20% |  |  |
| Libya | NAPP | 10% |  |  |
| Morocco | NAPP | 42% |  | 52% |
| Mauritania | NAPP | 20% |  | 35% |
| Tunisia | NAPP |  |  | 30% |
| Botswana | SAPP |  |  | 35% |
| Namibia | SAPP |  |  | 70% |
| South Africa | SAPP |  |  | 10% |
| Rwanda | EAPP |  | 60% | 60% |
| Sudan | EAPP |  |  | 20% |
| Benin | WAPP | 20% |  | 44% |
| Burkina Faso | WAPP | 23% |  | 50% |
| Cote d` Ivoire | WAPP |  |  | 16% (excl. large hydro) |
| Gambia | WAPP | 35% |  | 48% |
| Ghana | WAPP | 10% |  | 20% (excl. large hydro) |
| Guinea | CAPP | 25% |  | 30% (excl. large hydro) |
| Guinea – Bissau | WAPP | 30% |  | 50% |
| Liberia | WAPP | 25% |  | 30% |
| Mali | WAPP |  |  | 30% (excl. large hydro) |
| Niger | WAPP | 40% |  | 57% |
| Nigeria | WAPP | 20% |  | 30% |
| Senegal | WAPP | 20% |  | 30% (excl. large hydro) |
| Sierra Leone | WAPP | 30% |  | 50% |
| Togo | WAPP | 17% |  | 30% |

- Supplementary Table 30. Emission factors per input fuel12.

| Fuel | Emission factor (kg CO2/MMtoe) |
| --- | --- |
| Crude Oil | 1.880 |
| Coal | 2.405 |
| Natural Gas | 1.338 |
| Heavy Fuel Oil | 1.987 |
| Diesel | 1.845 |

- Supplementary Table 31. List of refineries in Africa24.

| Country | Name of the plant | Distillation Capacity (tb/d) | Nameplate Capacity (tb/d) | Earliest Year | Status |
| --- | --- | --- | --- | --- | --- |
| Algeria | Adrar | 13 |  | 2007 | OPR |
| Algeria | Algiers | 58 |  | 1964 | OPR |
| Algeria | Arzew | 90 |  | 1973 | OPR |
| Algeria | Skikda | 462 |  | 1993 | OPR |
| Algeria | Hassi Messaoud | 27 |  | 1979 | OPR |
| Angola | Luanda | 65 |  | 1958 | OPR |
| Angola | Luanda expansion |  |  | 2021 | PLN |
| Cameroon | Cape Limboh Limbe | 70 |  | 1981 | OPR |
| Chad | Ndajamena | 20 |  | 2011 | Not operational |
| Congo | Pointe Noire | 21 |  | 1982 | OPR |
| Cote d Ivoire | Abidjan | 84 |  | 1962 | OPR |
| Cote d Ivoire | Société Multinationale des Bitumes (SMB) |  |  |  |  |
| Egypt | Ameriya (Alexandria) | 88 | 81 | 1972 | OPR |
| Egypt | El Mex (Alexandria) | 115 |  | 1958 | OPR |
| Egypt | MIDOR (Alexandria) | 115 |  | 1994 | OPR |
| Egypt | MIDOR (Alexandria) expansion | 45 |  | 2020 | CON |
| Egypt | Assiut | 60 | 47 | 1987 | OPR |
| Egypt | El Suez | 68 |  | 1963 | OPR |
| Egypt | Mostorod | 142 |  | 1982 | OPR |
| Egypt | Nasr El Suez | 146 | 107.55 | 1913 | OPR |
| Egypt | Nasr Wadi Feran | 9 |  | 1990 | OPR |
| Egypt | Tanta | 35 |  | 1982 | OPR |
| Eritrea | Assab oil refinery | 17.5 | 0 | 1960 | Non-operated since 1997 |
| Gabon | Port Gentil | 24 |  | 1967 | OPR |
| Ghana | TEMA | 45 |  | 1963 | OPR |
| Liberia | Monrovia | 15 |  |  | Non-operated |
| Libya | Ras Lanuf plant | 220 |  | 1984 | OPR |
| Libya | Azzawiya and Benghazi | 120 |  | 1974 | OPR |
| Libya | Brega | 10 |  | 1970 | OPR |
| Libya | Sarir | 10 |  | 1989 | OPR |
| Libya | Tobruk | 20 |  | 1985 | OPR |
| Morocco | Mohammedia | 200 | 0 | 1961 | Non operation since 2015 |
| Niger | Zinder, Ganaram | 20 | 12 to 16 | 2011 | OPR |
| Nigeria | Port Harcourt refinery I and II | 210 | 50% | 1965 (I), 1989 (II) | OPR |
| Nigeria | Kaduna | 110 | 50% | 1983 | OPR |
| Nigeria | Warri | 125 | 50% | 1978 | OPR |
| Senegal | M´ Bao (Dakar) | 25 |  | 1963 | OPR |
| Sierra Leone | Freetown | 5 |  | 1970 | Non operated since 1992 |
| South Africa | Caltex | 100 |  | 1966 | OPR |
| South Africa | Durban | 135 | 120 | 1997 | OPR |
| South Africa | Sapref | 180 |  | 1963 | OPR |
| South Africa | Sasolburg | 105 |  | 1971 | OPR |
| Sudan | Khartoum | 100 |  | 2006 | OPR |
| Sudan | Al-Obeid | 10 |  | 1979 | OPR |
| Sudan | Concorp (Alshajara) | 10 |  | 2000 | OPR |
| Sudan | Abu Gabra | 2 |  | 1992 | OPR |
| Sudan | Port Sudan | 21.7 |  | 1964 | Not operational |
| Tunisia | Bizerte | 34 |  | 1963 | OPR |
| Tunisia | La Skhira | 150 |  |  | PLN |
| Zambia | Indeni-Ndola | 24 |  | 1973 | OPR |

*CON: committed plants that are in the implementation phase and will come into operation in the designated year, PLN: planned plants that don’t have a start date but are in the design and planning phase, OPR: operational plants

- Supplementary Table 32. List of LNG terminals in Africa25.

| Country | Project name | Capacity | Year | Status* |
| --- | --- | --- | --- | --- |
| Algeria | Arzew - GL1Z T1-6 | 7.9 MTPA | 1978 | OPR |
| Algeria | Arzew - GL2Z T1-6 | 8.2 MTPA | 1981 | OPR |
| Algeria | Skikda - GL1K Rebuild | 4.5 MTPA | 2013 | OPR |
| Algeria | Arzew - GL3Z | 4.7 MTPA | 2014 | OPR |
| Angola | Angola LNG T1 | 5.2 MTPA | 2014 | OPR |
| Benin | Cotonou (connect to Ghana, Togo) | 1.6 bcm/y |  | PLN |
| Benin | Cotonou extension | 1.1 bcm/y |  | PLN |
| Cameroon | PFLNG Satu | 92 MTPA |  | CON as of March 2018 |
| Cameroon | Cove Point LNG | 92 MTPA |  | CON as of March 2018 |
| Cameroon | Kribi FLNG | 92 MTPA |  | CON as of March 2018 |
| Cameroon | LNG1 | 12 MTPA | 2018 | CON |
| Cameroon | LNG2 | 8 MTPA | 2021 | PLN |
| Cote d´Ivoire | Grand Bassam | 5 MTPA |  | PLN |
| Djibouti | Djibouti FLNG | 3 MTPA | 2021 | CON |
| Egypt | SEGAS LNG T1 | 5 MTPA | 2005 | OPR |
| Egypt | Egyptian LNG T1 | 3.6 MTPA | 2005 | OPR |
| Egypt | Egyptian LNG T2 | 3.6 MTPA | 2005 | OPR |
| Egypt | Ain Sokhna Hoegh (floating) | 4.2 MTPA | 2015 | OPR |
| Egypt | Sumed BW (floating) | 5.7 MTPA | 2017 | OPR |
| Equatorial Guinea | EG LNG T1 | 3.7 MTPA | 2007 | OPR |
| Equatorial Guinea | Fortuna FLNG 1-2 | 4.4 MTPA | 2020-2025 | PLN |
| Ghana | Tema (Golar Tundra) | 1.8 bcm/y | 2017 | CON |
| Ghana | Tema | 2.6 bcm/y |  | CON - OPR in 2020 |
| Ghana | Tema extension | 2 bcm/y |  | CON - OPR in 2020 |
| Ghana | Ghana 1000 | 3.7 bcm/y |  | CON - OPR in 2020 |
| Kenya | Mombasa |  |  | PLN |
| Libya |  |  |  | PLN |
| Mauritania | Greater Tortue FLNG (shared with Senegal) | 2.5 MTPA | 2021 | CON |
| Mozambique | Coral South FLNG | 3.4 MTPA | 2022 | CON - OPR 2022 |
| Mozambique | Mamba LNG | 10 MTPA | 2020-2021 | PLN |
| Mozambique | Coral FLNG (Area 4) | 3.4 MTPA | 2022 | PLN |
| Mozambique | Mozambique LNG (Area 1) | 12 MTPA | 2023-2024 | PLN |
| Morocco | Jorf Lasfar | 2 bcm/y | 2021 | PLN |
| Morocco | Jorf Lasfar extension | 3 bcm/y | 2025 | PLN |
| Namibia | Walvis Bay | 0.6 bcm/y | 2017 | OPR |
| Nigeria | Nigeria LNG T1 | 3.3 MTPA | 2000 | OPR |
| Nigeria | Nigeria LNG T2 | 3.3 MTPA | 2000 | OPR |
| Nigeria | Nigeria LNG T3 | 3 MTPA | 2003 | OPR |
| Nigeria | Nigeria LNG T4 | 4.1 MTPA | 2006 | OPR |
| Nigeria | Nigeria LNG T5 | 4.1 MTPA | 2006 | OPR |
| Nigeria | Nigeria LNG T6 | 4.1 MTPA | 2008 | OPR |
| Nigeria | NLNG T7-8 | 8.6 MTPA | N/A | PLN |
| Republic of Congo | Congo-Brazzaville FLNG | 1.2 MTPA | 2020 | PLN |
| Senegal | Greater Tortue FLNG (shared with Mauritania) | 2.5 MTPA | 2021 | CON |
| South Africa | Saldanha Bay/Richards Bay |  |  | PLN |
| Sudan |  |  |  | PLN |
| Tanzania | Tanzania LNG (T1-3) | 15 MTPA | 2026-2027 | PLN |
| Tanzania | Tanzania LNG (T4) | 5 MTPA | N/A | PLN |
| Togo |  |  |  | PLN |

- Supplementary Table 33. Cross-border electricity interconnector projects in Africa.

| From | To | Capacity (MW) | Investment Cost | Status | Year | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Angola | DRC | 600 | 191 mil. USD | PLN | 2019 |  |
| Algeria | Morocco | 240 |  | OPR |  | North Africa Power Transmission Corridor |
| Spain | Morocco | 1400 |  | OPR |  |  |
| Spain | Morocco | 700 | 150 mil. € | PLN | 2026 | Spain shouldering about half the cost. It is due to be commissioned before 2026. Planned addition to an existing interconnector project 1.4 GW. |
| Benin | Niger | 330 | 730 mil. USD | PLN | 2022 | Station: MALANVILLE-ZABORI. Part of the Northcore interconnector |
| Benin | Ghana | 654.6 | 117.6 mil. USD | CON | 2020 | Station: Kara/Bembereke/Parakou (TB)–Yendi (GH) |
| Botswana | South Africa | 800 | - | OPR |  |  |
| Botswana | Zambia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Botswana | Namibia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Botswana | Zimbabwe | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Burkina Faso | Niger | 330 | 730 mil. USD | PLN | 2022 | Stations: OUAGADOUGOU-NIYAMEY. Part of the Northcore interconnector |
| Burundi | Rwanda | 17 |  | OPR |  | There are plans to increase the capacity |
| Cameroon | Chad | 1900 | 1362 mi. USD | PLN | 2020 | As option to invest in the following years (2020: 0.475GW, 2021: 0.475GW, 2022: 0.475GW, 2023: 0.475GW) |
| Cote d´Ivoire | Sierra Leone | 110 | 500 mil. USD | PLN | 2020 |  |
| Cote d´Ivoire | Ghana | 655.2 | 90 mil. USD | OPR | 2017 | Station: Riviera - Presea |
| Cote d´Ivoire | Burkina Faso | 327 |  | OPR |  |  |
| Cote d´Ivoire | Liberia | 337.6 | 59.7 mil. USD | CON | 2018 | Station: Man (CI)–Yekepa (LI) |
| DRC | Burundi | 15.5 |  | OPR |  |  |
| DRC | Burundi | 49 |  | CON | 2018 |  |
| DRC | Congo | 60 |  | OPR |  |  |
| DRC | Rwanda | 100 |  | OPR |  |  |
| DRC | Rwanda | 300 |  | CON | 2015 |  |
| DRC | South Africa | 3000 |  | PLN | 2023 | Included partly-2500MW |
| DRC | Zambia | 260 |  | OPR |  |  |
| Egypt | Sudan | 6600 | 6000 mil. USD | CON (400 MW in 2019) |  | North–South Power Transmission Corridor |
| Egypt | Jordan | 300 |  | OPR |  |  |
| Egypt | Saudi Arabia | 3000 |  | CON | 2021 |  |
| Ethiopia | Kenya | 2000 | 1,260 mil. USD | CON | 2019 | Wolayta-Sodo (ETH) and Suswa (KEN) substations; Over 95% of the construction of the Ethio-Kenya interconnection transmission line is completed and work on the project is executed by the respective countries. |
| Ethiopia | Djibouti | 180 |  | OPR |  |  |
| Ethiopia | Sudan | 200 |  | OPR |  |  |
| Ghana | Burkina Faso | 332.2 | 67 mil. USD | CON | 2017; 2019 | The line is operational but only used at 50% capacity due to the unfinished Kumasi –Bolgatanga line. Stations: Han (GH) - Bobo Dioulase (BU). Part of the Hub Intrazonal project. |
| Ghana | Cote d´Ivoire | 327 |  | OPR |  |  |
| Ghana | Togo | 438 |  | OPR |  |  |
| Ghana | Togo | 655.2 | 90 mil. USD | CON | 2017 | Station: VOLTA-DAVIÉ(LOMÉ); Part of the WAPP Coastal Transmission Backbone. The total cost for this project is divided between Ghana, Togo and Benin |
| Ghana | Benin | 655.2 | 90 mil. USD | CON | 2017 | Station: VOLTA-DAVIÉ(LOMÉ); Part of the WAPP Coastal Transmission Backbone |
| Ghana | Mali | 100 | 230 mil. USD | PLN | 2021-2022 |  |
| Gambia | Guinea Bissau | 329.1 | 90.6 mil. USD | CON | 2019 | Station: Soma (GA)–Bissau (GB) .Part of the OMVG project |
| Gambia | Senegal | 340.7 | 36.2 mil. USD | CON | 2019 | Station: Birkelane (SE)–Soma (GA). Part of the OMVG project |
| Guinea | Mali | 321.3 | 117.6 mil. USD | CON | 2020 | Station: Fomi (GU)–Bamako (MA). Part of the Hub Intrazonal project. |
| Guinea | Senegal | 286.3 | 289.8 mil. USD | CON | 2019 | Station: Kaolack (SE)–Linsan (GU). Part of the OMVG project |
| Kenya | Uganda | 103 |  | OPR |  |  |
| Kenya | Uganda | 440 | 380 mil. USD | CON | 2019 |  |
| Kenya | Tanzania | 1 |  | OPR |  |  |
| Kenya | Tanzania | 1300 |  | PLN | 2018 |  |
| Libya | Egypt | 170 |  | OPR | 1998 |  |
| Lesotho | South Africa | 230 |  | OPR |  |  |
| Morocco | Algeria | 480 |  | OPR |  | North Africa Power Transmission Corridor |
| Mali | Cote d´Ivoire | 319.7 | 136.9 mil. USD | CON | 2017 | Station: Segou (MA)– Ferkessedougou (CI). Part of the Hub Intrazonal project. |
| Malawi | Mozambique | 2400 | 125 mil. USD | PLN | 2020 |  |
| Malawi | Tanzania | 180 | 829 mil. USD | PLN | 2022 |  |
| Mozambique | South Africa | 3850 |  | OPR |  | PLN-600 MW |
| Mozambique | Swaziland | 1450 |  | OPR |  |  |
| Mozambique | Zimbabwe | 500 |  | OPR |  | PLN-500 MW |
| Namibia | Botswana | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Namibia | South Africa | 750 |  | OPR |  |  |
| Namibia | Zambia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Namibia | Zimbabwe | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Niger | Burkina Faso | 358 | 250.4 mil. USD | PLN | 2023: 137MW, 2024: 11MW, 2026: 59MW, 2027: 21MW, 2028: 31MW, 2029: 53MW, 2030: 46 MW | Station: Niamey (NI)–Ouagadougou (BU). Part of the Corridor Nord interconnector |
| Niger | Benin | 30 | 166.6 mil. USD | PLN | 2030 | Station: Zabori (NI)–Bembereke (TB) |
| Nigeria | Benin | 686 |  | OPR |  |  |
| Nigeria | Benin | 494 | 164.6 mil. USD | PLN | 2027: 65MW, 2028: 126MW, 2029: 173MW, 2030: 130MW | Station: Kaindhji (NG)–Kara/Bembereke/Parakou (TB). Part of Dorsale Mediane project |
| Nigeria | Niger | 169 |  | OPR |  |  |
| Nigeria | Niger | 653.1 | 143.1 mil. USD | PLN | 2020 | Station: Birnin Kebbi (NG)–Niamey (NI) |
| Nigeria | Togo | 646.7 | 164.6 mil. USD | CON | 2020 | Station: Kaindhji (NG)–Kara/Bembereke/Parakou (TB) |
| Rwanda | Burundi | 12 |  | OPR |  |  |
| Rwanda | Tanzania | 400 | 33 mil. USD | CON | 2019 |  |
| Senegal | Mauritania | 250 | 207 mil. USD | PLN | 2021 |  |
| South Africa | Swaziland | 1450 |  | OPR |  |  |
| South Africa | Zimbabwe | 600 |  | OPR |  |  |
| Togo | Benin | 345 |  | OPR |  |  |
| Togo | Ghana | 654.6 | 117.6 mil. USD | CON | 2020 | Station: Kara/Bembereke/Parakou (TB)–Yendi (GH) |
| Tanzania | Burundi | 27 |  | CON | 2018 |  |
| Tanzania | Kenya | 2000 | 310 mil. USD | CON | 2020 | Isinia-Singida substation; Part of the ZTK Transmission Interconnector |
| Tanzania | Uganda | 58 |  | OPR |  |  |
| Tanzania | Zambia | 530 | 152.19 mil. USD | CON | 2020 | Part of the ZTK Transmission Interconnector. |
| Tunisia | Algeria | 480 |  | OPR |  |  |
| Uganda | Kenya | 145 |  | OPR |  |  |
| Uganda | Kenya | 300 |  | CON | 2015 |  |
| Uganda | Tanzania | 70 |  | OPR |  |  |
| Uganda | Tanzania | 200 | - | PLN | 2022 |  |
| Uganda | Rwanda | 145 | 58 mil.USD | OPR |  |  |
| Uganda | Rwanda | 300 |  | CON | 2015 |  |
| Zambia | Zimbabwe | 1400 |  | OPR |  |  |
| Zambia | Tanzania | 458 | 172.6 mil. USD | CON | 2020 | Part of the ZTK Transmission Interconnector. 386 MW in 2020, based on the Zambia´s projected electricity generation and in 2025 will be 458MW (max. power transfer). |
| Zambia | Botswana | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Zambia | Namibia | 300 | 223 mil. USD | PLN | 2021 | Part of the Zizabona project |
| Guinea Bissau | Guinea | 309.6 |  | CON | 2019 |  |
| Burkina Faso | Mali | 305.8 |  | CON | 2020 |  |
| Liberia | Guinea | 337.6 |  | CON | 2018 |  |
| Mali | Mauritania | 250 |  | PLN | 2021 |  |
| Mali | Sierra Leone | 327 |  | PLN | 2020 |  |
| Mali | Senegal | 329.1 |  | CON | 2020 |  |
| Niger | Togo | 649.7 |  | PLN | 2020 |  |
| Sierra Leone | Guinea | 333.7 |  | CON | 2018 |  |
| Senegal | Gambia | 340.7 |  | CON | 2019 |  |
| Senegal | Guinea | 286.3 |  | CON | 2019 |  |
| Senegal | Mali | 100 |  | OPR |  |  |

- Supplementary Table 34. Gas pipeline projects in Africa26,27.

| From | To | Capacity (MW) | Investment Cost | Status | Year | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Algeria | Italy | 8 bcm/year | 2.5 | OPR | 2014 | GALSI project |
| Algeria | Italy | 30.2 bcm/year increased to 33.5 bcm/year by 2012 | 6.25 | OPR | 1983 | Trans-Mediterranean project |
| Algeria | Portugal | 12 bcm/year | 2.3 | OPR | 1996 | Maghreb-Europé project (Pedro Duran Farell) (through Morocco, Spain) |
| Algeria | Spain | 8 bcm/year | 0.90 € | OPR | 2010 | Medgaz project (offshore) |
| Algeria | Spain | 30 bil.cub.litres | 12 | 2015 Com. | 2021 | Trans-Saharan project |
| Algeria | Tunisia | 30.2 bcm/year increased to 33.5 bcm/year by 2012 | 6.25 | OPR | 1983 | Trans-Mediterranean project |
| Egypt | Turkey | 10.3 bcm/year | 1.2 | Out of order since 2012. To re-open in 2019 | 2003 | Arab project (through Israel-Jordan-Syria-Lebanon) |
| Israel | Egypt | 6 bcm/year |  | CON | 2020 | EMG pipeline |
| Cyprus | Egypt |  |  | PLN |  |  |
| Ethiopia | Djibouti | 3 bil. cub. feet/y | 3.2 | CON (Signed) | 2021 | Ethiopia - Djibouti project |
| Libya | Italy | 11 bcm/year |  | OPR | 2004 | Greenstream project |
| Mozambique | South Africa |  | 6 | PLN (not signed) | 2021 | African Renaissance project |
| Nigeria | Algeria | 30 bil.cub.litres | 12 | 2015 Commissioned | 2021 | Trans-Saharan project |
| Nigeria | Benin | 0.2-0.6 mil.cub.feet a day | 0.9 | OPR | 2011 | West African project |
| Nigeria | Cote d´Ivoire | 0.2-0.6 mil.cub.feet a day |  | PLN | 2025 (Estimated) | West African project |
| Nigeria | Ghana | 0.2-0.6 mil.cub.feet a day | 0.9 | OPR | 2011 | West African project |
| Nigeria | Niger | 30 bil.cub.litres | 12 | 2015 Commissioned | 2021 | Trans-Saharan project |
| Nigeria | Togo | 0.2-0.6 mil.cub.feet a day | 0.9 | OPR | 2011 | West African project |
| Tanzania | Uganda | 3 bil.cub.feet/ | 3.55 | CON (2018 the agreement signed) | 2026 | Tanzania – Uganda project (LNG transport) |

- Supplementary Table 35. Units

| Energy |  |  |
| --- | --- | --- |
| PJ | Petajoule | 1015 J |
| GJ | Gigajoule | 109 J |
| toe | Tonne of oil equivalent |  |
| ktoe | Thousand tonnes of oil eq. | 103 toe |
| Mtoe | Million tonnes of oil eq. | 106 toe |
| Gtoe | Giga tonnes of oil eq. | 109 toe |
| Mbl/d | Million barrels per day | 106 bl/d |
| Mt | Million metric tonnes | 106 t |
| MTPA | Million tonnes per annum |  |
| tb/d | Thousand barrels per day |  |
| bcm/y | billion cubic meter per year |  |
| Electricity |  |  |
| GW | Gigawatts | 109 W |
| TWh | terawatt-hours | 1012 W |
| Water |  |  |
| Mm3 | Million cubic meters |  |
| MCM | Million cubic meters |  |
| Prices |  |  |
| $/bbl | $ per barrel of oil |  |
| $/boe | $ per barrel of oil eq. |  |
| Emissions and related |  |  |
| tCO2 | Tonne CO2 |  |
| MtCO2 | Million tonne CO2 |  |
| Monetary units |  |  |
| M$ | Million USD |  |

- Supplementary tables 36-38: Energy balances, GHG emissions and costs in Africa among the scenarios.

Supplementary Table 36. Energy balance, GHG emissions and costs in the Reference scenario.

| African continent | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| Energy Balance, by fuel (Mtoe) |  |  |  |  |
| Primary Production | 1308.32 | 1587.13 | 1076.09 | 952.09 |
| Coal | 122.87 | 154.77 | 152.57 | 124.89 |
| Crude Oil | 445.41 | 398.88 | 210.29 | 60.34 |
| Oil products | 155.72 | 130.70 | 110.05 | 80.19 |
| Natural gas | 262.34 | 406.30 | 8.71 | 10.75 |
| Nuclear | 3.34 | 3.34 | 0.00 | 0.00 |
| Hydro | 9.39 | 28.09 | 51.77 | 61.77 |
| Other renewables (Solar, Wind, Geoth.) | 1.26 | 20.35 | 124.35 | 228.97 |
| Biofuels/Waste | 308.00 | 444.71 | 418.35 | 385.18 |
| Electricity | 0.00 | 0.00 | 0.00 | 0.00 |
| Imports | 123.14 | 244.53 | 679.52 | 1032.34 |
| Coal | 3.74 | 37.19 | 235.92 | 405.05 |
| Crude Oil | 32.46 | 39.04 | 46.11 | 57.22 |
| Oil products | 80.16 | 138.71 | 232.92 | 319.29 |
| Natural gas | 3.55 | 18.59 | 148.97 | 230.10 |
| Nuclear | 0.00 | 0.00 | 0.00 | 0.00 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Other renewables (Solar, Wind, Geoth.) | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | 0.00 | 0.00 | 0.00 | 0.00 |
| Electricity | 3.24 | 11.00 | 15.60 | 20.68 |
| Exports | -588.68 | -682.56 | -235.08 | -131.14 |
| Coal | -54.47 | -51.72 | -51.72 | -51.55 |
| Crude Oil | -320.58 | -305.90 | -145.23 | -36.55 |
| Oil products | -40.35 | -28.44 | -15.98 | -10.52 |
| Natural gas | -169.30 | -284.80 | -5.84 | -11.14 |
| Nuclear | 0.00 | 0.00 | 0.00 | 0.00 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Other renewables (Solar, Wind, Geoth.) | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | -0.75 | -0.70 | -0.70 | -0.70 |
| Electricity | -3.24 | -11.00 | -15.60 | -20.68 |
| Total Primary Energy Supply | 842.77 | 1149.10 | 1520.54 | 1853.29 |
| Power generation (Mtoe) |  |  |  |  |
| Fuel Inputs Therm. Power | -125.54 | -160.08 | -342.61 | -513.73 |
| Coal | -52.28 | -102.85 | -292.91 | -432.32 |
| Oil products | -25.08 | -5.18 | 0.00 | 0.00 |
| Natural gas | -43.82 | -46.92 | -48.10 | -80.76 |
| Nuclear | -3.34 | -3.34 | 0.00 | 0.00 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | -1.01 | -1.79 | -1.61 | -0.64 |
| Gross Elec. Generation | 64.09 | 118.76 | 320.49 | 509.58 |
| Coal | 20.39 | 40.11 | 114.24 | 168.61 |
| of which CCS | 0.00 | 0.00 | 0.00 | 0.00 |
| Oil | 9.28 | 1.84 | 0.02 | 0.01 |
| Natural gas | 22.01 | 26.10 | 29.06 | 49.81 |
| of which CCS | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | 0.66 | 1.16 | 1.04 | 0.42 |
| of which CCS | 0.00 | 0.00 | 0.00 | 0.00 |
| Nuclear | 1.10 | 1.10 | 0.00 | 0.00 |
| Hydro | 9.39 | 28.09 | 51.77 | 61.77 |
| Wind | 0.77 | 5.50 | 24.25 | 39.29 |
| Solar | 0.37 | 11.98 | 95.77 | 180.57 |
| Other | 0.12 | 2.86 | 4.34 | 9.11 |
| Oil refineries (Mtoe) |  |  |  |  |
| Crude Oil | -157.29 | -132.02 | -111.16 | -81.00 |
| Oil products | 155.72 | 130.70 | 110.05 | 80.19 |
| Losses (Mtoe) |  |  |  |  |
| Power generation | -9.81 | -15.69 | -30.61 | -43.64 |
| Total Final Consumption (Mtoe) |  |  |  |  |
| Total | 559.04 | 838.21 | 1137.55 | 1386.85 |
| Coal | 18.87 | 35.60 | 41.81 | 43.90 |
| Oil products | 159.10 | 224.09 | 310.93 | 370.01 |
| Natural gas | 36.41 | 58.61 | 103.31 | 146.32 |
| Biofuels/Waste | 291.83 | 421.37 | 396.47 | 365.80 |
| Electricity | 52.83 | 98.54 | 285.03 | 460.82 |
| GHG emissions (Mt CO2 eq.) |  |  |  |  |
| Total CO2 | 1221.31 | 1900.95 | 2583.64 | 3465.77 |
| Total System Costs (Mil. USD) |  |  |  |  |
| Investment | 26798.72 | 66298.83 | 172152.25 | 198680.63 |
| Fixed | 7996.16 | 16309.19 | 44834.87 | 72433.82 |
| Variable | 117380.72 | 222742.70 | 462235.17 | 629202.93 |

Supplementary Table 37. Energy balance, GHG emissions and costs in the 2.0oC scenario.

| African continent | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| Energy Balance, by fuel (Mtoe) |  |  |  |  |
| Primary Production | 1314.21 | 1412.29 | 1041.70 | 1006.38 |
| Coal | 124.49 | 108.38 | 27.94 | 6.68 |
| Crude Oil | 445.53 | 381.55 | 121.21 | 34.69 |
| Oil products | 155.73 | 129.31 | 69.32 | 34.96 |
| Natural gas | 266.47 | 284.79 | 189.81 | 111.80 |
| Nuclear | 3.34 | 3.34 | 0.00 | 6.79 |
| Hydro | 9.39 | 30.97 | 55.30 | 62.41 |
| Other renewables (Solar, Wind, Geoth.) | 1.26 | 19.28 | 135.19 | 255.27 |
| Biofuels/Waste | 308.00 | 454.67 | 442.93 | 493.78 |
| Electricity | 0.00 | 0.00 | 0.00 | 0.00 |
| Imports | 119.30 | 212.59 | 254.02 | 314.47 |
| Coal | 3.79 | 21.05 | 9.36 | 1.03 |
| Crude Oil | 32.46 | 35.05 | 27.23 | 8.95 |
| Oil products | 76.21 | 115.45 | 107.83 | 81.33 |
| Natural gas | 3.58 | 30.64 | 73.17 | 97.62 |
| Nuclear | 0.00 | 0.00 | 18.08 | 103.20 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Other renewables (Solar, Wind, Geoth.) | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | 0.00 | 0.00 | 0.00 | 0.00 |
| Electricity | 3.25 | 10.41 | 18.36 | 22.36 |
| Exports | -589.98 | -536.72 | -182.15 | -51.80 |
| Coal | -55.64 | -43.52 | -6.81 | -1.58 |
| Crude Oil | -320.69 | -285.98 | -78.42 | -8.33 |
| Oil products | -40.35 | -19.00 | -4.86 | -1.87 |
| Natural gas | -169.30 | -177.11 | -73.02 | -16.96 |
| Nuclear | 0.00 | 0.00 | 0.00 | 0.00 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Other renewables (Solar, Wind, Geoth.) | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | -0.75 | -0.70 | -0.70 | -0.70 |
| Electricity | -3.25 | -10.41 | -18.36 | -22.36 |
| Total Primary Energy Supply | 843.53 | 1088.17 | 1113.56 | 1269.05 |
| Power generation (Mtoe) |  |  |  |  |
| Fuel Inputs Therm. Power | -124.64 | -125.59 | -172.40 | -290.99 |
| Coal | -52.39 | -54.21 | -26.92 | -5.60 |
| Oil products | -20.23 | -4.74 | -0.85 | -0.90 |
| Natural gas | -47.67 | -56.82 | -109.63 | -118.26 |
| Nuclear | -3.34 | -3.34 | -18.08 | -109.99 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | -1.01 | -6.46 | -16.93 | -56.25 |
| Gross Elec. Generation | 63.90 | 108.57 | 278.72 | 448.67 |
| Coal | 20.43 | 21.14 | 10.50 | 2.18 |
| of which CCS | 0.00 | 0.00 | 0.00 | 0.00 |
| Oil | 7.58 | 1.66 | 0.31 | 0.32 |
| Natural gas | 23.47 | 32.07 | 65.86 | 72.43 |
| of which CCS | 0.00 | 0.00 | 10.65 | 23.69 |
| Biofuels/Waste | 0.66 | 2.34 | 5.60 | 19.76 |
| of which CCS | 0.00 | 1.81 | 5.24 | 16.29 |
| Nuclear | 1.10 | 1.10 | 5.97 | 36.30 |
| Hydro | 9.39 | 30.97 | 55.30 | 62.41 |
| Wind | 0.77 | 5.32 | 31.44 | 57.27 |
| Solar | 0.37 | 10.97 | 98.78 | 188.68 |
| Other | 0.12 | 3.00 | 4.97 | 9.31 |
| Oil refineries (Mtoe) |  |  |  |  |
| Crude Oil | -157.30 | -130.62 | -70.02 | -35.31 |
| Oil products | 155.73 | 129.31 | 69.32 | 34.96 |
| Losses (Mtoe) |  |  |  |  |
| Power generation | -9.75 | -14.04 | -25.62 | -37.22 |
| Total Final Consumption (Mtoe) |  |  |  |  |
| Total | 560.44 | 810.28 | 891.23 | 1006.66 |
| Coal | 19.25 | 30.17 | 3.40 | 0.50 |
| Oil products | 159.96 | 210.18 | 163.05 | 108.01 |
| Natural gas | 36.70 | 53.59 | 71.43 | 75.71 |
| Biofuels/Waste | 291.84 | 426.40 | 405.24 | 416.20 |
| Electricity | 52.70 | 89.94 | 248.12 | 406.24 |
| GHG emissions (Mt CO2 eq.) |  |  |  |  |
| Total CO2 | 1219.53 | 1550.83 | 1041.22 | 432.84 |
| Total System Costs (Mil. USD) |  |  |  |  |
| Investment | 26439.16 | 63733.52 | 171803.12 | 244331.64 |
| Fixed | 7997.83 | 14930.62 | 41149.80 | 79230.15 |
| Variable | 117682.02 | 219889.56 | 263958.05 | 253435.96 |

Supplementary Table 38. Energy balance, GHG emissions and costs in the 1.5oC scenario.

| African continent | 2015 | 2030 | 2050 | 2065 |
| --- | --- | --- | --- | --- |
| Energy Balance, by fuel (Mtoe) |  |  |  |  |
| Primary Production | 1312.93 | 1353.17 | 847.45 | 876.62 |
| Coal | 123.54 | 99.59 | 15.20 | 4.84 |
| Crude Oil | 445.47 | 364.27 | 41.11 | 2.90 |
| Oil products | 155.70 | 112.46 | 22.06 | 3.09 |
| Natural gas | 266.01 | 268.93 | 109.79 | 43.08 |
| Nuclear | 3.34 | 3.34 | 0.61 | -0.44 |
| Hydro | 9.39 | 29.42 | 56.72 | 65.20 |
| Other renewables (Solar, Wind, Geoth.) | 1.26 | 18.63 | 130.91 | 245.94 |
| Biofuels/Waste | 308.23 | 456.51 | 471.05 | 512.02 |
| Electricity | 0.00 | 0.00 | 0.00 | 0.00 |
| Imports | 119.24 | 211.58 | 134.47 | 294.70 |
| Coal | 3.78 | 18.12 | 3.18 | 0.91 |
| Crude Oil | 32.46 | 30.29 | 5.35 | 0.85 |
| Oil products | 76.18 | 118.72 | 33.92 | 10.29 |
| Natural gas | 3.56 | 33.02 | 28.20 | 37.73 |
| Nuclear | 0.00 | 0.00 | 45.33 | 222.29 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Other renewables (Solar, Wind, Geoth.) | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | 0.00 | 0.00 | 0.00 | 0.00 |
| Electricity | 3.25 | 11.44 | 18.48 | 22.64 |
| Exports | -589.33 | -507.09 | -60.27 | -34.87 |
| Coal | -55.04 | -37.07 | -1.71 | -0.24 |
| Crude Oil | -320.65 | -280.95 | -24.19 | -0.62 |
| Oil products | -40.35 | -9.52 | -0.73 | -0.16 |
| Natural gas | -169.30 | -167.40 | -14.46 | -10.52 |
| Nuclear | 0.00 | 0.00 | 0.00 | 0.00 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Other renewables (Solar, Wind, Geoth.) | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | -0.75 | -0.70 | -0.70 | -0.70 |
| Electricity | -3.25 | -11.44 | -18.48 | -22.64 |
| Total Primary Energy Supply | 842.83 | 1057.66 | 921.65 | 1136.45 |
| Power generation (Mtoe) |  |  |  |  |
| Fuel Inputs Therm. Power | -124.19 | -122.92 | -174.38 | -393.02 |
| Coal | -52.30 | -52.83 | -16.33 | -5.50 |
| Oil products | -20.17 | -4.99 | -0.33 | -0.19 |
| Natural gas | -47.36 | -55.73 | -89.95 | -54.56 |
| Nuclear | -3.34 | -3.34 | -45.95 | -232.09 |
| Hydro | 0.00 | 0.00 | 0.00 | 0.00 |
| Biofuels/Waste | -1.01 | -6.03 | -21.83 | -100.68 |
| Gross Elec. Generation | 63.72 | 105.10 | 271.20 | 457.91 |
| Coal | 20.40 | 20.60 | 6.37 | 2.14 |
| of which CCS | 0.00 | 0.00 | 0.00 | 0.31 |
| Oil | 7.56 | 1.75 | 0.13 | 0.07 |
| Natural gas | 23.36 | 31.41 | 54.14 | 33.57 |
| of which CCS | 0.00 | 0.00 | 7.84 | 7.84 |
| Biofuels/Waste | 0.66 | 2.19 | 7.76 | 34.40 |
| of which CCS | 0.00 | 1.68 | 6.23 | 30.10 |
| Nuclear | 1.10 | 1.10 | 15.16 | 76.59 |
| Hydro | 9.39 | 29.42 | 56.72 | 65.20 |
| Wind | 0.77 | 5.29 | 28.18 | 50.12 |
| Solar | 0.37 | 10.46 | 96.87 | 185.40 |
| Other | 0.12 | 2.89 | 5.86 | 10.41 |
| Oil refineries (Mtoe) |  |  |  |  |
| Crude Oil | -157.27 | -113.60 | -22.28 | -3.13 |
| Oil products | 155.70 | 112.46 | 22.06 | 3.09 |
| Losses (Mtoe) |  |  |  |  |
| Power generation | -9.71 | -13.62 | -25.21 | -39.15 |
| Total Final Consumption (Mtoe) |  |  |  |  |
| Total | 560.13 | 801.02 | 757.84 | 837.24 |
| Coal | 18.99 | 26.48 | 0.32 | 0.01 |
| Oil products | 159.97 | 206.05 | 52.18 | 12.41 |
| Natural gas | 36.54 | 53.09 | 37.01 | 20.08 |
| Biofuels/Waste | 292.06 | 428.57 | 427.33 | 391.22 |
| Electricity | 52.57 | 86.84 | 240.99 | 413.53 |
| GHG emissions (Mt CO2 eq.) |  |  |  |  |
| Total CO2 | 1216.98 | 1485.98 | 367.93 | -308.12 |
| Total System Costs (Mil. USD) |  |  |  |  |
| Investment | 26205.23 | 51490.33 | 253916.96 | 242279.84 |
| Fixed | 7996.33 | 14472.32 | 42279.16 | 86399.00 |
| Variable | 117516.86 | 219203.33 | 146398.76 | 100217.25 |

- Supplementary Table 39. Available power generation and conversion technologies.

|  | Fossil and nuclear | Renewable | CCS |
| --- | --- | --- | --- |
| Centralized | Diesel | Geothermal | Biomass |
|  | Heavy Fuel Oil | Biomass & Waste CHP | Coal |
|  | Natural Gas: OCGT, CCGT | Hydro: Small, Medium, Large* | Natural Gas |
|  | Supercritical Coal | Wind: onshore, offshore |  |
|  | Nuclear | Solar PV (utility scale) |  |
|  |  | CSP with storage |  |
|  |  | CSP without storage |  |
| Decentralized | Diesel 1kW | Solar PV (roof top) |  |
|  |  | Solar PV with battery |  |

*OCGT: Open Cycle Gas Turbine, CCGT: Combined Cycle Gas Turbine, Small Hydro (<10MW), Medium Hydro (10-100MW), Large Hydro (>100MW)

Supplementary References

1.	The World Bank. World Bank Open Data | Data. https://data.worldbank.org/ (2018).

2.	Agency, I. E. World Energy Outlook 2017. (2017).

3.	S&P Global Platts. “World Electric Power Plants Database", December 2015. (2015).

4.	U.S. Energy Information Administration (EIA). https://www.eia.gov/.

5.	Welcome - ENERGYDATA.INFO. https://energydata.info/en/.

6.	World Nuclear Association. Uranium in Africa - World Nuclear Association. https://www.world-nuclear.org/information-library/country-profiles/others/uranium-in-africa.aspx.

7.	International Renewable Energy Agency, Abhu Dhabi. Estimating the Renewable Energy Potential in Africa: A GIS-based approach. 73 (2014).

8.	IRENA. Analysis of Infrastructure for Renewable Power in Eastern and Southern Africa. (2015).

9.	United Nations Industrial Development Organization. World Small Hydropower Development Report 2016. (2016).

10.	International Renewable Energy Agency, Abu Dhabi. Planning and prospects for renewable power: West Africa. (2018).

11.	Energy-Economics-Environment Modeling Laboratory. PRIMES model. (2018).

12.	IEA-ETSAP | Energy Supply Technologies Data. https://iea-etsap.org/index.php/energy-technology-data/energy-supply-technologies-data.

13.	International Energy Agency. World Energy Outlook ‘New policies scenario’. (2016).

14.	Gonzalez Sanchez, R. et al. Freshwater use of the energy sector in Africa. Appl. Energy270, 115171 (2020).

15.	Agency, I. E. World Energy Outlook 2016. (2016).

16.	Internationa Energy Agency. World Energy Outlook - ‘New policies scenario’. (2018).

17.	IEA. World Energy Balances 2017. (OECD Publishing, 2017).

18.	Miketa, A. & Merven, B. Southern African Power Pool: Planning and Prospects for Renewable Energy. https://www.irena.org/publications/2013/Jun/Southern-African-Power-Pool-Planning-andProspects-for-Renewable-Energy (2013).

19.	African Union-African Energy Commission. ´Key Africa Energy Statistics. (2018).

20.	OECD. World energy balances. https://www.oecd-ilibrary.org/energy/data/iea-world-energy-statistics-and-balances/world-energy-balances_data-00512-en.

21.	United Nations Department for Economic and Social Affairs Statistics Division. UNSD — Energy Balances 2016. https://unstats.un.org/unsd/energystats/pubs/balance/ (2018).

22.	REN21. REN21 Building the sustainable energy future with renewable energy. REN21 https://www.ren21.net/.

23.	IEA International Energy Agency. IEA - Renewable Energy. https://www.iea.org/policiesandmeasures/renewableenergy/.

24.	McKinsey&Company. African refineries. http://www.mckinseyenergyinsights.com/resources/refinery-reference-desk/african-refineries/.

25.	International Gas Union. 2018 World LNG Report. (2018).

26.	Hydrocarbons Technology. Hydrocarbons Technology - The leading site for news and procurement in the hydrocarbons industry. Hydrocarbons Technology https://www.hydrocarbons-technology.com/.

27.	NEPAD Transforming Africa. | AUDA-NEPAD. https://www.nepad.org/.