Annex 1. Equipment Selection for the Overburden Process

1. Calculating the Number of Loader Required

H = 10 + 0.57 (Cc - 6) (Jimeno et al., 1995). In this formula, Cc: The bucket size of the loader (m3) (bucket capacity), H: Bench height, so if a bench height of 12 meters is chosen, Cc = 9.51 m3 is calculated. Thus, a bucket capacity of up to 9.51 m3 can be determined. If the loader has a bucket capacity of 8.1 m3 for an average excavation density of 2.30 t/m3, and a cycle time of 25 seconds, the hourly capacity of the loader can be calculated according to the following formula: Vlc = [a x Kf x Rw ] / [(1+k) x T]. In this formula, the symbols and their meanings are as follows: Vlc: Loader's hourly capacity, a: Selected bucket volume (8.1 m3), Moh: Amount of overburden excavation to be made per hour (m3/h), Kf: Bucket filling factor (0.90), Rw: Workplace efficiency (85%), k: Swelling factor (0.3), T: Bucket cycle time (25 sec). Vlc = [8.1 m3 x 0.90 x 0.85] / [(1+0.3) x 25 sec] = 0.190 m3/sec = 686.38 tons/h. Moh/Vlc = [0.4330 / 0.275] m3/sec = 2.10. Therefore, an average of 2 active loaders are required for the stripping of the entire site. Since there are 25 different matches in this study, not all calculations are shown here; only an example calculation is presented.

2. Loader Net Uptime & Productivity Analysis

The meanings and units of the terms to be used in the calculations are as follows:

• Possible hours (Typ): The company's scheduled annual operating time (hours/year),

• Unused hours (by+a): Faults and annual periodic maintenance (hours/year),

• Net mechanical time available for use (Tm) = Available hours (Tp) – Unused hours (Typ)

• Mechanical availability (Ktt) = [Possible hours (Tp) – Unused hours (Typ)] / Possible hours (Tp)

• Actual net working time (Tn) (Time losses due to weather conditions, compatibility factors, etc., Tk) = Tm – Tk

• With mechanical efficiency = (Tn / Tm), the loader used for excavation will be active for an average of 2.10/3 = 70% of its net operating time (excluding downtime and breakdowns) throughout its entire service life. Therefore, considering its backups, the required net operating time is approximately Tn = 70% x 24 (h/day) x 300 (days/year) = 5028.54 hours/year.

3. Calculation of Required Number of Trucks

3.1. Selection of a truck compatible with the loader

Here, a CAT 775G (56-ton) loader has been selected. The loader will fill the truck at an average speed of 0.19 m³/s (= 0.19 m³/sec x 2.30 tons/m³ (tons/sec) x 25 seconds = 10.96 tons/cycle) with a bucket cycle time of 25 seconds. Considering different truck types, the loader selection is as follows: If the 56-ton 775G truck is selected, [56 tons / (10.96 tons / loader cycle time)] = 5.11. This means that with an average of 5 passes (5 x 10.96 tons = 54.80 tons), 97.88% of the truck's capacity can be loaded. Therefore, this truck was chosen. Calculating the required number of loaders also requires calculating the truck's cycle time.

• T: Truck cycle time = tm  + tb + tg   + td

• tm  (Maneuver time) + tb  (Uploaded time) = (20) + (34.50) = 54.5 sec,

• tg: Time spent on the road during a loaded trip,

• td: Time spent on the road during an empty return trip,

5 pass x 25 sec (Loader bucket cycle time) = 125 sec. T= (54.5 + 125 + 289 + 199.71) sec = 668.21 sec. This value (668.215 sec) is the maximum cycle time for a truck in the mine site.

3.2. The truck's minimum hourly capacity

[56 tons  x 90% (efficiency)] / 668.21 sec = 0.075 tons/sec = 271.53 tons/h => 686.58 (tons/h) / 271.53 (tons/h) = 2.53. So, the capacity of one loader is equivalent to ~3 trucks.

3.3. The average number of active trucks required for overburden removal at the mine site

The average number of active trucks required for overburden removal at the mine site = (Average hourly capacity of a loader / Average hourly capacity of a truck) => (686.58 tons/h) / (271.53 tons/h) = 2.45 (trucks/loader). This means that for 3 actively working loaders (2.45 trucks/loader), 3 loaders equal 7.36 trucks, or 8 trucks in terms of capacity for overburden.