Supplementary Information

Enhanced 2,3-Butanediol Purification Using a Hybrid Extraction Process

Pramod M Gawal*, and Sweta Lataye

Department of Chemical Engineering, Indian Institute of Technology Guwahati, Assam-781039, INDIA

Table S1. Correlation coefficient for the proposed model

| Std.Dev. | 0.4637 | R2 | 0.9997 |
| --- | --- | --- | --- |
| Mean | 76.54 | Adjusted R2 | 0.9994 |
| C.V.% | 0.6058 | Predicted R2 | 0.9985 |
|  |  | Adeq Precision | 166.5199 |

Table S2. Calculation of the recovery of 2,3-butanediol.

2,3-butanediol: Feed volume: 1000 ml 2,3-BD concentration in broth:76.5g/L Density of 2,3-BD=0.987 g/mL 2,3-BD in feed: (76.5×1)/0.987 = 77.5 mL Water in feed:1000-77.5=922.5 mL Below is a calculation based on the volume fraction of 2,3-BD, water, and isobutanol 2,3-BD in extract phase: 0.20× 380=76.20 mL 2,3-BD in aqueous phase: 0.0013×910=1.26 mL 2,3-BD in DC (bottom): 0.9996×76 = 75.97 mL Extraction efficiency (Y)= (75.97/77.5) ×100 =98.31% Distribution coefficient (KD) = (76.20/1.26) =60.47 2,3-BD recovery (%) = (75.97/ 77.5) ×100 = 98.02 % Isobutanol: Isobutanol initially used: 300 mL Isobutanol in (Distillate): 0.9379×300=281.37 mL The solvent recovered = (281.37/300) × 100=94% The solvent loss = (300-281.37)/300= 6%

Table S3. Comparison of the 2,3-butanediol recovery rate using aqueous two-phase extraction with the earlier report.

| ATPS with composition | 2,3-BD Conc(g/L) | Extraction time (hr) | Partition coefficient | Extraction efficiency (%) | Ref. |
| --- | --- | --- | --- | --- | --- |
| 30% v/v isobutanol and 25% w/v K2HPO4 | 76.5 | 0.25 | 61.63 | 98.31 | Present work |
| 32 %(w/w) ethanol and 16% (w/w) ammonium sulfate | 42.24 | 10 | 7.1 | 91.7 | [1] |
| 30% w/v (NH4)2SO4 and 50% v/v isopropanol | 68.2 | 2 | 45 | 97 | [2] |
| 34% (w/w) 2-propanol and 20% (w/w) ammonium sulfate | 63.84 | - | 9.9 | 93.7 | [3] |
| 35%butanol and 30%NaCl | 75.25 | 0.25 | 5.9 | 97.08 | [4] |

Figure S1. Chromatogram of glucose-based fermentation broth.

Figure S2. Calibration graph of the standard of 2,3-BD.

Figure S3. Chromatogram of recovered 2,3-butanediol.

Figure S4. NMR spectra of (a) 13C and (b) 1H of purified 2,3-BD.

References

1. 	Li, Z., Teng, H., & Xiu, Z. (2010). Aqueous two-phase extraction of 2 , 3-butanediol from fermentation broths using an ethanol / ammonium sulfate system. Process Biochemistry, 45(5), 731–737. https://doi.org/10.1016/j.procbio.2010.01.011

2. 	Narisetty, V., Amraoui, Y., Abdullah, A., Ahmad, E., Agrawal, D., Parameswaran, B., … Kumar, V. (2021). High yield recovery of 2,3-butanediol from fermented broth accumulated on xylose rich sugarcane bagasse hydrolysate using aqueous two-phase extraction system. Bioresource Technology, 337(June), 125463. https://doi.org/10.1016/j.biortech.2021.125463

3. 	li hui Sun, & Xiu, J. Æ. Z. (2009). Aqueous two-phase extraction of 2 , 3-butanediol from fermentation broths by isopropanol / ammonium sulfate system. Biotechnol Lett, 371–376. https://doi.org/10.1007/s10529-008-9874-3

4. 	Gawal, P. M. (2025). Cost-effective 2,3-BD separation: insights from aqueous two-phase extraction-assisted distillation. Biomass Conversion and Biorefinery, 15(5), 6757–6765. https://doi.org/10.1007/s13399-024-05392-w