Particulate Oxidative Potential (OP) Associated with Fireworks Activity during Diwali at a site in the Indo-Gangetic Plain

Isha Goyal, Puneet Kumar Verma, K. Maharaj Kumari and Anita Lakhani*

Department of Chemistry, Dayalbagh Educational Institute, Dayalbagh, Agra (UP), 282005

E-mail address: anita.lakhani01@gmail.com, anitalakhani@dei.ac.in

2.3. Backward trajectory analysis

The origin and transport pathways of the air masses arriving at the sampling site (27.16°N, 78.08°E) in Agra were characterized by 72-hr backward trajectory modelling with the HYSPLIT model version 4.0 provided by the National Oceanic and Atmospheric Administration (NOAA) Air Resources Laboratory 1. The input meteorological data with a horizontal resolution of 1°× 1° latitude-longitude and a temporal resolution of 6 hr were acquired from the Global Data Assimilation System (ftp://arlftp.arlhq.noaa.gov/pub/archives/ gdas1). The air mass backward trajectories begin at 12:00 UTC each day during Diwali were calculated with a starting height of 500 AGL. The backward trajectories were plotted a day before Diwali (26/10/2019), Diwali day (27/10/2019) and a day after Diwali (28/10/2019).

List of Figures

Fig. S1. Air mass backward trajectory during Diwali Period

Fig. S1. Air mass backward trajectory during Diwali Period

List of Tables

Table S1. Working parameters of ICP-OES for analysis

TableS2. Detection limits of metals by ICP-OES

Table S3: PM2.5 mass concentrations and meteorological parameters during pre-Diwali, Diwali and post-Diwali

Table S4. Pearson correlation coefficients (r) of volume-based DTT activity (DTTv) with mass concentration of PM2.5 components

Table S1. Working parameters of ICP-OES for analysis

| Parameters |  |  | Value |
| --- | --- | --- | --- |
| Plasma flow |  |  | 12.0 l min-1 |
| Nebulizer flow |  |  | 0.70 l min-1 |
| Make up flow |  |  | 0.00 l min-1 |
| Auxiliary flow |  |  | 1.00 l min-1 |
| Pump rate |  |  | 12 rpm |
| Power |  |  | 1.2 kW |
| Viewing mode |  |  | Synchronous Vertical Dual View (SVDV) |
| Viewing height |  |  | 8.0 mm |
| Stabilization time |  |  | 15 sec |
| Uptake delay |  |  | 25 sec |
| Replicate per scan |  |  | 3 |

TableS2. Detection limits of metals by ICP-OES

| Metal Detection limits |  | ng m-3 |
| --- | --- | --- |
| Al |  | 0.7 |
| Ba |  | 0.2 |
| Be |  | 0.2 |
| Cd |  | 1.5 |
| Cr |  | 1.4 |
| Cu |  | 0.3 |
| Mn |  | 1.2 |
| Ni |  | 0.6 |
| Pb |  | 1.3 |
| Se |  | 0.7 |
| V |  | 0.4 |
| Zn |  | 1.9 |
| Ca |  | 0.4 |
| Fe |  | 3.3 |
| K |  | 0.1 |
| Mg |  | 1.7 |
| Na |  | 0.9 |

Table S3. PM2.5 mass concentrations and meteorological parameters during pre-Diwali, Diwali and post-Diwali

| Parameters | pre-Diwali |  | Diwali |  | post-Diwali |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Day | Night | Day | Night | Day | Night |
| PM2.5 | 115.2 ± 9.6 | 152.1 ± 1.6 | 278.8 ± 221.3 | 460.3 ± 68.1 | 317.9 | 279.9 |
| Pressure (mm Hg) | 747.1 ± 0.1 | 746 ± 0.0 | 746.8 ± 0.1 | 746.0 ± 0.0 | 746.7 ± 0.2 | 746.0 ± 0.0 |
| RH (%) | 41.9 ± 4.0 | 61.9 ± 5.7 | 43.4 ± 1.2 | 59.6 ± 0.8 | 47.4 ± 5.5 | 62.5 ± 3.5 |
| WS (m/s) | 1.2 ± 0.1 | 0.8 ± 0.1 | 1.1 ± 0.6 | 0.7 ± 0.3 | 0.9 ± 0.4 | 0.4 ± 0.1 |
| SR (W/m2) | 353.7 ± 8.7 | 24.9 ± 0.4 | 298.8 ± 58.1 | 23.2 ± 2.5 | 240.6 ± 75.2 | 22.3 ± 1.8 |
| Rainfall (mm) | 0.0 ± 0.0 | 0.0 ± 0.0 | 0.0 ± 0.0 | 0.0 ± 0.0 | 0.0 ± 0.0 | 0.0± 0.0 |
| WD (degree) | S-SW |  | S-SW |  | S-SW |  |

Table S4. Pearson correlation coefficients (r) of volume-based DTT activity (DTTv) with mass concentration of PM2.5 components

|  | Al | Ba | Be | Cd | Cr | Cu | Mn | Ni | Pb | Se | V | Zn | Ca | Fe | K | Mg | Na | PM2.5 | DTTv | DTTm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Al | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ba | -0.06 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Be | 0.13 | 0.92* | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cd | 0.55 | 0.63 | 0.86* | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cr | -0.02 | 0.83* | 0.93* | 0.78* | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cu | 0.31 | 0.74 | 0.79* | 0.75* | 0.81* | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mn | 0.25 | 0.64 | 0.59 | 0.47 | 0.25 | 0.27 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ni | -0.29 | 0.82* | 0.81* | 0.53 | 0.89* | 0.72* | 0.22 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Pb | 0.99* | -0.07 | 0.09 | 0.49 | -0.09 | 0.25 | 0.31 | -0.34 | 1 |  |  |  |  |  |  |  |  |  |  |  |
| Se | 0.36 | 0.90* | 0.89* | 0.78* | 0.74* | 0.80* | 0.7 | 0.6 | 0.36 | 1 |  |  |  |  |  |  |  |  |  |  |
| V | 0.22 | 0.77* | 0.96* | 0.93* | 0.93* | 0.76* | 0.43 | 0.76* | 0.15 | 0.77 | 1 |  |  |  |  |  |  |  |  |  |
| Zn | 0.76 | 0.48 | 0.61 | 0.78* | 0.41 | 0.55 | 0.61 | 0.17 | 0.76* | 0.78 | 0.59 | 1 |  |  |  |  |  |  |  |  |
| Ca | 0.19 | 0.68 | 0.65 | 0.56 | 0.74* | 0.72* | 0.09 | 0.66 | 0.14 | 0.72 | 0.59 | 0.45 | 1 |  |  |  |  |  |  |  |
| Fe | -0.45 | 0.07 | 0.04 | -0.14 | -0.06 | -0.08 | 0.29 | 0.13 | -0.42 | -0.16 | 0.01 | -0.29 | -0.62 | 1 |  |  |  |  |  |  |
| K | 0.56 | 0.17 | 0.07 | 0.17 | 0.07 | 0.53 | -0.02 | -0.01 | 0.56 | 0.43 | -0.02 | 0.44 | 0.56 | -0.58 | 1 |  |  |  |  |  |
| Mg | -0.15 | 0.25 | 0.08 | -0.07 | 0.3 | 0.4 | -0.39 | 0.43 | -0.18 | 0.19 | 0.01 | -0.13 | 0.74* | -0.56 | 0.6 | 1 |  |  |  |  |
| Na | 0.79 | -0.34 | -0.1 | 0.33 | -0.21 | 0.13 | 0.07 | -0.51 | 0.76* | 0.02 | 0.04 | 0.44 | -0.29 | -0.02 | 0.27 | -0.49 | 1 |  |  |  |
| PM2.5 | -0.13 | 0.44 | 0.47 | 0.36 | 0.59 | 0.32 | -0.04 | 0.7 | -0.18 | 0.33 | 0.5 | 0.15 | .716* | -0.38 | -0.01 | 0.54 | -0.6 | 1 |  |  |
| DTTv | 0.11 | 0.92* | 0.94* | .79* | .89* | .77* | 0.53 | .85* | 0.08 | 0.87* | 0.88* | 0.53 | 0.76* | -0.12 | 0.15 | 0.3 | -0.27 | 0.66 | 1 |  |
| DTTm | 0.07 | 0.7 | 0.71* | 0.6 | 0.6 | 0.54 | 0.57 | 0.7 | 0.07 | 0.63 | 0.66 | 0.29 | 0.49 | 0.09 | -0.02 | 0.17 | -0.26 | 0.57 | 0.85* | 1 |

* Correlation is significant at the 0.01 level (2-tailed).