APPENDIX A.1 : Proof of dose minimization/maximization localization

We wish to prove that the dose for the case where the fibroglandular tissue is wholly concentrated near the top of the breast is greater than or equal to the dose for the case where the fibroglandular tissue is split into two segments: one at the top of the breast and one separated from the first by an area of adipose tissue. The net length and the fibroglandular length remain constant.

Dose is given as:

D=kSEμtρe-μl4πr2

k, S, E, and 4π are constant. µt/ρ is dependent on the target material, which, in this case is always fibroglandular tissue, so it is constant. The dose then, is proportional to the remaining variables.

D∝e-μlr2

Ḋ is the dose rate. We can substitute this for dose, since the photons are delivered at a constant rate.

D∝e-μlr2

r is the distance from the source to the dose point, which is the distance from the source to the surface, s, added to the distance from the surface to the point of interest, l.

D∝e-μl(l+s)2

Let μf and μa indicate the fibroglandular and adipose attenuation coefficients. The shield can be adipose tissue, fibroglandular tissue, or both, the equation has to be re-written specific to the voxel. Figure A1(a) and (b) shows two scenarios.  In Figure A.1(a) there is fibroglandular tissue all clumped at the top (left in figure), near the source. In Figure A.1(b) there is some adipose tissue in between.

Lf

0

Lf

L

F

Lf - a

Lf –a + b

Lf+b

0

b

F

a

(a)

(b)

L

src

src

l

l

Figure A.1. (a) Fibroglandular (red) is at the top, near the source. The length variable ls small “l”. (b) Fibroglandular tissue is fragmented with a layer of adipose of length b in between. The length variable is small “l”.

When the fibroglandular tissue is all concentrated at the top of the breast, as depicted in Figure A.1(a), we can calculate the dose given to any one voxel at length l from top (left) as

D1∝e-μfl(l+s)2

Then across all voxels

D1∝0Lfe-μfl(l+s)2dl

Where Lf is the length of fibroglandular tissue in Figure A1(a).

When the fibroglandular tissue is split into two segments, as depicted in Figure A.1(B), one at the top of the breast and one separated from the first by a layer of adipose tissue of length “b”, the dose is proportional to

D2∝0Lf-ae-μfll+s2dl+ Lf-a+bLf+be-[μfLf-a+μab+μfl-Lf-a+b](l+s)2dl

Where a is a number between 0 and Lf, and b is a number greater than 0.

This simplifies to

D2∝0Lf-ae-μfll+s2dl+ e-μabLf-a+bLf+be-μf(l-b)(l+s)2dl

Note we deliberately kept the eμfb term inside the integral. Changing variable, l'=l-b, we get

D2∝0Lf-ae-μfll+s2dl+ e-μabLf-aLfe-μfl'(l'+b+s)2dl'

Since e-μab≤1   and 1/(l'+s+b)≤  1/(l'+s)  for b ≥0, we will get

D2≤0Lf-ae-μfll+s2dl+ Lf-aLfe-μfl'(l'+s)2dl'

We see RHS = 0Lf-ae-μfll+s2dl+ Lf-aLfe-μfl(l+s)2dl  =0Lfe-μfl(l+s)2dl= D1

Therefore, D2≤D1.

Corollary 1:  If there are multiple adipose segments for the same net length of fibroglandular tissue, we can apply this process multiple times across appropriate sub-lengths and we will obtain the same results.

Corollary 2: By very similar mathematics or logic we can prove that when the fibroglandular tissue is concentrated at the bottom (right in our diagram), the dose to the fibroglandular tissue will be lower than any other configuration for the given net length of fibroglandular and adipose tissue.

APPENDIX 2.2: Table of DgN from Reconstructed volumes

Table A.2: Comparison of actual DgN values for simulated breasts from Monte Carlo simulations with DgN values obtained by reconstruction. Original simulated objects are shaded and their DgN values bolded. Reconstructed objects are not shaded, and the bolded reconstruction DgN values are those which matched the fibroglandular location of the original object. The DgN calculations for 20% is shown for 4 breast sizes. The GF 30-50% is shown for one size. For readability the 4 sizes are separated into sub-tables. The bolded values are compared separately in Figure 8 in main text.

| Object Size and glandular fraction | Location of fibroglandular tissue (original object) | Dose minimized (lower), central, or maximized (upper) for corresponding reconstructed object | DgN |
| --- | --- | --- | --- |
| 11cm x 3cm, 20% | Upper |  | 0.3731 |
|  |  | Minimized | 0.2054 |
|  |  | Central | 0.2823 |
|  |  | Maximized | 0.3740 |
| 11cm x 3cm, 20% | Central |  | 0.2771 |
|  |  | Minimized | 0.2055 |
|  |  | Central | 0.2825 |
|  |  | Maximized | 0.3744 |
| 11cm x 3cm, 20% | Lower |  | 0.2062 |
|  |  | Minimized | 0.2053 |
|  |  | Central | 0.2824 |
|  |  | Maximized | 0.3746 |

| Object Size and glandular fraction | Location of fibroglandular tissue (original object) | Dose minimized (lower), central, or maximized (upper) for corresponding reconstructed object | DgN |
| --- | --- | --- | --- |
| 12cm x 4cm, 20% | Upper |  | 0.3190 |
|  |  | Minimized | 0.1473 |
|  |  | Central | 0.2179 |
|  |  | Maximized | 0.3157 |
| 12cm x 4cm, 20% | Central |  | 0.2168 |
|  |  | Minimized | 0.1475 |
|  |  | Central | 0.2181 |
|  |  | Maximized | 0.3161 |
| 12cm x 4cm, 20% | Lower |  | 0.1484 |
|  |  | Minimized | 0.1475 |
|  |  | Central | 0.2184 |
|  |  | Maximized | 0.3166 |
| 12cm x 4cm, 30% | Upper |  | 0.2938 |
|  |  | Minimized | 0.1557 |
|  |  | Central | 0.2155 |
|  |  | Maximized | 0.2914 |
| 12cm x 4cm, 30% | Central |  | 0.2144 |
|  |  | Minimized | 0.1559 |
|  |  | Central | 0.2157 |
|  |  | Maximized | 0.2917 |
| 12cm x 4cm, 30% | Lower |  | 0.1572 |
|  |  | Minimized | 0.1559 |
|  |  | Central | 0.2157 |
|  |  | Maximized | 0.2917 |
| 12cm x 4cm, 50% | Upper |  | 0.2577 |
|  |  | Minimized | 0.1756 |
|  |  | Central | 0.2118 |
|  |  | Maximized | 0.2482 |
| 12cm x 4cm, 50% | Central |  | 0.2119 |
|  |  | Minimized | 0.1758 |
|  |  | Central | 0.2119 |
|  |  | Maximized | 0.2483 |
| 12cm x 4cm, 50% | Lower |  | 0.1746 |
|  |  | Minimized | 0.1759 |
|  |  | Central | 0.2121 |
|  |  | Maximized | 0.2485 |

| Object Size and glandular fraction | Location of fibroglandular tissue (original object) | Dose minimized (lower), central, or maximized (upper) for corresponding reconstructed object | DgN |
| --- | --- | --- | --- |
| 13cm x 5cm, 20% | Upper |  | 0.2527 |
|  |  | Minimized | 0.1072 |
|  |  | Central | 0.1717 |
|  |  | Maximized | 0.2738 |
| 13cm x 5cm, 20% | Central |  | 0.1724 |
|  |  | Minimized | 0.1075 |
|  |  | Central | 0.1721 |
|  |  | Maximized | 0.2742 |
| 13cm x 5cm, 20% | Lower |  | 0.1088 |
|  |  | Minimized | 0.1076 |
|  |  | Central | 0.1720 |
|  |  | Maximized | 0.2743 |

| Object Size and glandular fraction | Location of fibroglandular tissue (original object) | Dose minimized (lower), central, or maximized (upper) for corresponding reconstructed object | DgN |
| --- | --- | --- | --- |
| 14cm x 6cm, 20% | Upper |  | 0.2433 |
|  |  | Minimized | 0.0796 |
|  |  | Central | 0.1383 |
|  |  | Maximized | 0.2386 |
| 14cm x 6cm, 20% | Central |  | 0.1392 |
|  |  | Minimized | 0.0798 |
|  |  | Central | 0.1387 |
|  |  | Maximized | 0.2390 |
| 14cm x 6cm, 20% | Lower |  | 0.0812 |
|  |  | Minimized | 0.0800 |
|  |  | Central | 0.1389 |
|  |  | Maximized | 0.2394 |