Supplementary Material

# Implementation of an integrated health risk assessment coupled with spatial interpolation and source apportionment: A case study of soil heavy metals, China

3 Tables

Table S1

Parameters detailed description of RfD.

| Item |  | RfD (mg/(kg).d) |  |  |
| --- | --- | --- | --- | --- |
| Instructions |  | Estimate of maximum permissible risk to a human population through daily exposure during a lifetime from individual element |  |  |
| Reference Value | element | ingestion | inhalation | dermal |
|  | As | 3.00E-04 | 1.23E-04 | 3.01E-04 |
|  | Cd | 1.00E-03 | 1.00E-05 | 1.00E-03 |
|  | Cr | 3.00E-03 | 6.00E-05 | 2.86E-05 |
|  | Hg | 3.00E-04 | 8.75E-05 | 2.10E-05 |
|  | Pb Zn | 3.50E-03 3.00E-01 | 5.25E-04 6.00E-02 | 3.52E-03 3.00E-01 |

Table S2

The other parameters of human health risk model.

| Symbol | Instructions | Reference value |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | children | adults |  |
| IngR | ingestion rate | 200 | 100 | mg/d |
| InhR | inhalation rate | 0.012 | 0.03 | kg/ d |
| EF | exposure frequency | 350 |  | d/ y |
| ED | exposure duration | 6 | 30 | y |
| AT | averaging time | 3650 | 25550 | d |
| PEF | emission factor | 1.36×109 |  | m3/kg |
| SL | skin adherence factor | 0.2 | 0.7 | mg·cm2/ kg |
| SA | exposed skin surface area | 2800 | 5700 | cm2 |
| ABS | dermal absorption factor | 10-4 |  | - |

Note: - represents non-dimensional parameter

Table S3

The categorization and the fitted results of the concentrations of soil heavy metals by PMF model.

| Species | Category | r2 | Intercept | Intercept SE | Slope | Slope SE | SE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| As | Strong | 0.35 | 3.42 | 0.085 | 0.59 | 0.0092 | 1.24 |
| Cd | Strong | 0.99 | 0.0053 | 0.00021 | 0.97 | 0.00072 | 0.0084 |
| Cr | Strong | 0.97 | 1.49 | 0.42 | 0.96 | 0.0019 | 13.49 |
| Ni | Strong | 0.99 | 5.51 | 0.17 | 0.95 | 0.0012 | 4.64 |
| Pb | Strong | 0.68 | 24.81 | 0.34 | 0.56 | 0.0047 | 12.41 |
| Zn | Strong | 0.49 | 51.30 | 0.64 | 0.45 | 0.0056 | 16.69 |