Appendix A: Methods for Puget Sound temperature, salinity, and dissolved oxygen profile data, 1932-2024

| Data Source | Time Period | Profile Type | Methods |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Temperature | Salinity | DO |
| University of Washington Oceano- graphic Lab- oratories*a,b | 1932 - 1942 | Bottle (likely silver-lined Nansen from [1]) | Reversing thermo- meters on bottle sampling devicesa | Knudson method following [2]c | Winkler titration (inferred) following [1]c,d |
|  | 1947- Sep 1952 | Bottle (infer similar to preceding era) |  |  |  |
|  | Oct 1952 - Jun 1957 |  | Deep-sea reversing thermometers corrected according to equation of state follow- ing [3]a,c |  |  |
|  | Jun 1957 - 1966 |  |  | Modified Wenner-Smith- Soule salinity bridge following [4]c |  |
|  | 1967 - 1972 |  | Reversing thermometers (inferred) | Electrical conductivity methods (inferred) | Winkler titration (inferred) |
|  | 1974 - 1975 | Bottle; non- metallic Scott- Richards typeb | Deep-sea reversing thermometersb | Electrical conductivity using University of Washington salinity bridge following [4]b | Modified Winkler method following [1] with error of less than 5%b |
| WA Dept. of Ecologye** | 1973 - Nov 1983 | Bottle + in-situ profiler (inferred, unknown types)*** | Unknown | Unknown | Unknown |
|  | Dec 1983 - Jun 1988 | Bottle (inferred) + in-situ profiler |  |  | Unknown (ex situ) + YSI, Inc. sonde (in situ, inferred) |
|  | Jul 1988 - Oct 1989 | Bottle + in situ profiler (inferred) |  |  | Titration (ex situ, unknown method) |
|  | Nov 1989 - Aug 16 1998 | Bottle + CTD | SBE 19 (in situ only) | Unknown (ex situ) + SBE 19 | Modified Winkler method following [6] + SBE 22 |
|  | Aug 17 1998 - Dec 1998 |  |  | Autosal salinometer following [7] + SBE 19 | Standard Methods 17th Ed., Modified Winkler Method + SBE 22 |
|  | Jan 1999 - Aug 30 1999 | Bottle (inferred) + CTD |  |  | Unknown |
|  | Aug 31 1999 - Jul 4 2000 | Bottle + CTD |  |  | Unknown (ex situ) + SBE 22, 23 |
|  | Jul 5 2000 - Oct 8 2001 |  |  |  | Winkler titration following [5] + SBE 22, 23 |
|  | Oct 9 2001 - Nov 2004 |  | SBE 3, 19 (in situ only) | Autosal salinometer following [7] + SBE 4, 19 |  |
|  | Dec 2004 - Jun 29 2007 |  | SBE 3, 19, 19plus (in situ only) | Autosal salinometer following [7] + SBE 4, 19, 19plus | Winkler titration following [5] + SBE 23, 43 |
|  | Jun 30 2007 - Jul 25 2012 |  |  |  | Winkler titration following [5] + SBE 43 |
|  | Jul 26 2012 - Dec 6 2012 |  | SBE 3, 19plus (in situ only) | Autosal salinometer following [7] + SBE 4, 19plus |  |
|  | Dec 7 2012 - Oct 15 2019 |  | SBE 3 (in situ only) | Autosal salinometer following [7] + SBE 4 |  |
|  | Oct 16 2019 - Apr 2021 |  |  |  | Unknown (ex situ) + SBE 43 |
|  | May 2021 - Aug 3 2021 |  |  | Autosal salinometer following [7] & Orion Star Dura Probe + SBE 4 |  |
|  | Aug 4 2021 - 2024 |  |  | Orion Star Dura Probe + SBE 4 |  |
| King Countyf,g, h** | 1965 - 1969 | Bottle (inferred) | Alcohol or mercury thermometer (ex situ only) | Unknown | Winkler titration |
|  | 1970 - 1997 | Bottle + handheld sonde (inferred) |  |  | Winkler titration + YSI handheld sonde |
|  | 1998 - Jun 2004 | Bottle + CTD | SBE sensor (in situ only, unknown type) | Unknown (ex situ) + SBE sensor (unknown type, inferred) | Winkler titration + SBE sensor (unknown type, factory calibration used) |
|  | Jul 2004 - Jan 2005 |  |  |  | Winkler titration + SBE 43 (factory calibration used) |
|  | Jan/Feb 2005 - Dec 2009 | Bottle (Niskin) + CTD | SBE 25 (in situ only) | Unknown (ex situ) + SBE 25 | Winkler titration + SBE 43 (slope of curve and voltage offset from Winkler titrations) |
|  | Jan 2010 - Aug 6 2017 |  |  |  | Winkler titration + SBE 43 (correction factor calculated daily based on difference between Winkler DO and CTD, then multiplied by slope of curve of original factory calibration) |
|  | Aug 7 2017 - Apr 2020 |  | SBE 25plus (in situ only, with manufacturer annual calibration) | Unknown (ex situ) + SBE 25 (with manufacturer annual calibration) |  |
|  | May 2020 - 2024 |  | SBE 3 (in situ only, with manufacturer annual calibration) | SBE 4 (with manufacturer annual calibration) |  |
| NOAA NCEIi** | Feb 2008 - Apr 2017 | Bottle (Niskin)**** | SBE 9plus | SBE 4 | Winkler titration following [5] |
|  | May 2017 - Jun 2017 |  | SBE 19plus with SBE55 Eco CTD |  |  |
|  | Jul 2017 - Sep 2017 |  | SBE 9plus |  |  |
|  | Oct 2017 - Mar 2018 |  | SBE 19plus with SBE55 Eco CTD |  |  |
|  | Apr 2018 - Sep 2018 |  | SBE 9plus |  |  |
|  | Oct 2018 |  | SBE 19plus with SBE55 Eco CTD |  |  |

Table 4 Data sampling details for datasets used in this work; information is the best available from available sources and correspondence with sampling agencies. Lack of qualifying information indicates a record gap as opposed to methodological concern. “Profile Type” refers to bottle and CTDs casts, which are analyzed ex situ and in situ, respectively. Different types of profiling are called out explicitly. Note the abbreviations CTD (conductivity, temperature, and depth sensor) and SBE (Sea-Bird Electronics, Inc. sensor). Cited methods are as follows: [1] Thompson & Robinson (1939), [2] Oxner (1920), [3] Sverdrup (1947), [4] Paquette (1958), [5] Carpenter (1965), [6] Standard Methods for the Examination of Water and Wastewater (17th Ed.) (1989), [7] JGOFS (Chapter 5 –

Salinity by Autosal Salinometer) (Protocols for the Joint Global Ocean Flux Study (JGOFS) Core Measurements, 1994).

*In 1951, changed to Department of Oceanographyb; later became School of Oceanography.

**Information about sampling is included in cited data sources’ metadata.

***Sonde profiles included in “CTD” data type given lack of demarcation in raw data.

****Raw CTD data is not included in this data analysis; however, included bottle data uses discrete, concurrent, in situ CTD measurements for temperature and salinity as opposed to ex situ measurements from bottle samples. aCollias (1970)

bCollias & Lincoln (1977)

cCollias et al. (1974)

dBarnes & Collias (1958)

eWA Dept. of Ecology (2024)

fKing County (2024a)

gKing County (2024b)

hKing County (2024c)

iAlin et al. (2021)