Assessment of human exposure to terpenes from fragrant plants in homes using passive sampling

Wonsik Shina, Arum Leeb, Jaeho Shinb, Eugene Songb, Stefana Sochichiua, Yerim Kooa, Yongseok Hongc, Ji Hoon Seod, e, Thanh-Dat Phama, f, and Pil-Gon Kimg, h*

aDivision of Environmental Science and Ecological Engineering, Korea University, 145 Anam-ro, Seongbuk-gu, Seoul 02841, Republic of Korea

bDepartment of Consumer Science, Chungbuk National University, Cheongju 28644, Republic of Korea

cDepartment of Environmental Engineering, College of Science and Technology, Korea University Sejong Campus, Sejong City 30019, Republic of Korea

dHarvard Medical School, Brigham and Women's Hospital, 75 Francis Street, Boston, MA 02115, USA

eSchool of Health and Environmental Science & Department of Health and Safety Convergence Science, Korea University, 145 Anam-Ro, Seoul 02841, Republic of Korea

fBK21 FOUR R&E Center for Environmental Science and Ecological Engineering, Korea University, Seoul 02841, Republic of Korea

gDepartment of Environmental Education, Mokpo National University, Jeonnam 58554, Republic of Korea

hConvergence center for Green anti-Aging Research, Muan 58554, Republic of Korea

*Corresponding author: Pil-Gon Kim

# Phone: +82 61 450 2784

# E-mail: pilgonkim@mnu.ac.kr

# ORCID: 0000-0001-7219-1529

Contents of the supplementary materials

Fig. S1: The mass accumulation rates (µg h-1) of the ePTFE PS for BTEX and styrene (STR). The targeted VOCs were spiked into the 125 L test chamber at 0.5, 1, and 1.5 μL, respectively.

Fig. S2: The mass accumulation rates (µg h-1) of the ePTFE PS for targeted terpenes. A spiking solution composed of targeted terpenes with the same concentration was prepared and injected into the 125 L test chamber at 2, 4, and 6 μL respectively.

Fig. S3: The linear relationship between signals in arbitrary units measured by PTR-TOF-MS and mass of limonene (mg) injected into the 125 L test chamber.

Table S1: Concentrations (µg m-3) of VOCs in homes with and without deploying plants grown in a pot.

Table S2: Concentrations (µg m-3) of VOCs in homes with and without deploying plants grown using hydroculture.

Table S3: Volume (m3) of investigated homes.

Table S4: The sampling rates (L hr-1) of the ePTFE PS for targeted VOCs.

Table S5: Operation parameters for quantifying targeted VOCs.

Table S6: Selected m/z values for quantifying targeted VOCs using GC-MS.

Table S7: Reference concentration (μg m-3) of the targeted VOCs for health risk assessment.

Table S8: HQ values for non-carcinogenic risks about targeted VOCs according to plant growing conditions at different age groups.

Fig. S1. The mass accumulation rates (µg h-1) of the ePTFE PS for BTEX and styrene (STR). The targeted VOCs were spiked into the 125 L test chamber at 0.5, 1, and 1.5 μL, respectively.

Fig. S2. The mass accumulation rates (µg h-1) of the ePTFE PS for targeted terpenes. A spiking solution composed of targeted terpenes with the same concentration was prepared and injected into the 125 L test chamber at 2, 4, and 6 μL respectively.

Fig. S3. The linear relationship between signals in arbitrary units measured by PTR-TOF-MS and mass of limonene (mg) injected into the 125 L test chamber.

Table S1. Concentrations (µg m-3) of VOCs in homes with and without deploying plants grown in a pot.

|  | benzene |  | toluene |  | ethylbenzene |  | m, p-xylene |  | o-xylene |  | styrene |  | limonene |  | α-pinene |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Volunteers ID | *Without | **With | Without | With | Without | With | Without | With | Without | With | Without | With | Without | With | Without | With |
| 1 | 1.3 | 1.7 | 9.5 | 16.8 | 2.3 | 2.4 | 3.9 | 4.2 | 2.5 | 2.2 | 1.0 | 0.9 | 52.1 | 49.4 | 4.7 | 6.0 |
| 2 | <MDL | <MDL | 39.6 | 30.1 | 14.9 | 11.8 | 24.3 | 19.6 | 5.8 | 5.4 | 3.8 | 3.0 | 42.7 | 41.0 | 11.8 | 18.3 |
| 3 | <MDL | <MDL | 13.1 | 7.4 | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | 13.4 | <MDL | <MDL |
| 4 | 1.0 | 1.1 | 20.3 | 19.5 | <MDL | 1.1 | 1.6 | 1.9 | <MDL | 0.9 | <MDL | 0.8 | 122.9 | 185.1 | 4.5 | 7.6 |
| 5 | 0.9 | 0.9 | 19.1 | 23.0 | 30.6 | 30.0 | 22.8 | 22.4 | 7.1 | 7.4 | 0.9 | 2.9 | 33.9 | 72.0 | 6.7 | 5.7 |
| 6 | <MDL | <MDL | 178.7 | 130.7 | 4.9 | 3.7 | 18.2 | 13.2 | 11.1 | 8.4 | 1.9 | 1.4 | 25.7 | 26.9 | 4.7 | 4.3 |
| 7 | 0.6 | 0.6 | 12.6 | 19.7 | 1.5 | 1.9 | 2.2 | 2.3 | 0.9 | 1.0 | 1.0 | 1.7 | 61.9 | 95.0 | 2.1 | 4.9 |
| 8 | 0.9 | 1.2 | 7.9 | 13.8 | <MDL | 1.5 | 1.8 | 3.1 | 0.8 | 1.2 | 0.8 | 2.3 | 63.1 | 185.0 | 4.0 | 9.9 |
| 9 | 1.5 | 1.9 | 9.2 | 12.5 | 3.9 | 6.1 | 4.3 | 6.3 | 1.6 | 2.1 | 0.8 | 1.5 | 12.4 | 45.9 | 5.1 | 5.0 |
| 10 | <MDL | <MDL | 33.3 | 22.0 | 5.7 | 3.1 | 5.3 | 2.1 | 2.7 | 1.2 | 5.8 | 3.0 | 45.6 | 29.3 | 8.0 | 4.7 |
| 11 | <MDL | <MDL | 15.6 | 8.0 | 14.5 | 7.4 | 2.6 | 1.3 | 1.3 | 1.8 | 5.6 | 3.6 | 16.8 | 40.1 | 4.2 | 6.6 |
| 12 | 0.9 | 1.1 | 5.7 | 8.1 | 1.3 | 1.6 | 2.1 | 3.0 | 0.7 | 1.6 | 0.6 | 1.0 | 33.9 | 75.8 | 3.7 | 8.8 |
| 13 | 1.4 | 1.3 | 9.8 | 10.1 | 4.5 | 4.3 | 5.1 | 5.4 | 1.9 | 1.8 | 1.2 | 1.1 | 40.3 | 49.6 | 14.6 | 17.2 |
| 14 | 0.8 | 0.8 | 5.3 | 5.8 | 1.1 | 1.3 | 1.4 | 2.1 | <MDL | <MDL | 1.2 | 1.3 | 6.8 | 42.5 | 3.0 | 5.8 |
| 15 | <MDL | <MDL | 8.1 | 7.3 | 1.8 | 1.3 | 2.6 | 1.7 | 2.0 | 1.5 | 0.7 | <MDL | 28.8 | 24.0 | 4.4 | 4.3 |
| 16 | 2.5 | 2.8 | 56.6 | 58.5 | 100.8 | 110.6 | 72.9 | 80.7 | 23.4 | 25.4 | 0.6 | 1.3 | 24.9 | 27.2 | <MDL | 2.3 |
| 17 | <MDL | <MDL | 43.2 | 39.9 | 14.3 | 14.3 | 10.3 | 10.8 | 3.2 | 3.1 | 5.2 | 5.9 | 107.3 | 111.2 | 9.1 | 7.3 |
| 18 | <MDL | <MDL | 10.8 | 6.2 | 3.8 | <MDL | 5.4 | <MDL | 2.1 | <MDL | <MDL | <MDL | <MDL | 19.2 | 2.3 | 4.0 |
| 19 | <MDL | <MDL | 8.9 | 17.7 | 6.1 | 12.3 | 3.0 | 6.2 | 1.2 | 2.4 | 2.2 | 3.8 | 18.5 | 43.1 | <MDL | 2.6 |
| 20 | 1.8 | 1.0 | 9.5 | 6.4 | 3.6 | 3.0 | 4.1 | 3.6 | 1.5 | 1.2 | 0.7 | 0.8 | 48.1 | 64.9 | 8.8 | 6.5 |
| 21 | <MDL | <MDL | 9.5 | 14.5 | 1.4 | 2.1 | 1.3 | 2.3 | <MDL | 0.7 | 1.5 | 2.2 | 6.1 | 15.3 | <MDL | <MDL |
| 22 | <MDL | <MDL | 29.0 | 31.1 | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | 8.1 | <MDL | <MDL | <MDL |
| 23 | <MDL | 0.7 | 18.1 | 37.7 | <MDL | <MDL | <MDL | 0.8 | <MDL | <MDL | <MDL | <MDL | <MDL | 45.8 | <MDL | 9.0 |
| 24 | <MDL | <MDL | 9.1 | 12.1 | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | 0.6 | 162.7 | 101.8 | 16.9 | 11.5 |
| 25 | 0.7 | 1.3 | 8.8 | 18.1 | <MDL | <MDL | <MDL | 1.3 | <MDL | <MDL | <MDL | 1.4 | 18.4 | 54.2 | <MDL | 6.2 |
| 26 | 0.8 | 1.2 | 15.7 | 19.7 | 4.5 | 4.5 | 4.7 | 6.1 | 2.9 | 3.5 | 1.7 | 2.4 | 31.5 | 43.1 | 2.5 | 3.7 |
| 27 | 0.9 | 1.2 | 15.7 | 13.9 | 4.2 | 4.2 | 4.8 | 4.0 | 2.0 | 1.5 | 1.4 | 0.8 | 68.1 | 54.1 | 45.4 | 92.8 |
| 28 | <MDL | <MDL | 25.5 | 60.2 | <MDL | 1.4 | <MDL | 1.6 | <MDL | 1.1 | 0.9 | 3.0 | 27.1 | 122.4 | 2.1 | 8.9 |
| 29 | 0.6 | 0.7 | 9.1 | 7.8 | 1.1 | 1.7 | 1.6 | 2.4 | 0.8 | 2.3 | <MDL | <MDL | 9.7 | 36.2 | <MDL | 3.9 |
| 30 | 0.8 | 1.3 | 17.8 | 31.7 | 30.2 | 53.3 | 21.6 | 38.8 | 7.9 | 12.8 | 1.4 | 1.9 | 62.4 | 115.6 | <MDL | 2.6 |
| Mean ± SD*** | 0.6 | 0.7 | 22.5 | 23.7 | 8.7 | 9.6 | 7.7 | 8.3 | 2.9 | 3.1 | 1.4 | 1.7 | 39.7 | 61.0 | 5.9 | 9.1 |
|  | ± 0.6 | ± 0.7 | ± 31.9 | ± 24.7 | ± 19.1 | ± 21.9 | ± 14.1 | ± 16.0 | ± 4.7 | ± 5.1 | ± 1.6 | ± 1.3 | ± 37.3 | ± 45.8 | ± 8.6 | ± 16.3 |
| *Without: Sample collected without deploying plants in indoors. **With: Sample collected by deploying plants in indoors. ***SD: standard deviation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table S2. Concentrations (µg m-3) of VOCs in homes with and without deploying plants grown using hydroculture.

|  | benzene |  | toluene |  | ethylbenzene |  | m, p-xylene |  | o-xylene |  | styrene |  | limonene |  | α-pinene |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Volunteers ID | *Without | **With | Without | With | Without | With | Without | With | Without | With | Without | With | Without | With | Without | With |
| 1 | 2.0 | 1.8 | 15.8 | 21.0 | 2.5 | 2.9 | 5.0 | 4.6 | 2.4 | 2.1 | 1.7 | 1.3 | 91.2 | 125.8 | 11.4 | 13.0 |
| 2 | <MDL | <MDL | 12.8 | 5.6 | 4.7 | 2.4 | 7.6 | 3.4 | 2.2 | 1.2 | 1.3 | 0.7 | 24.6 | 40.0 | 4.2 | 2.5 |
| 3 | <MDL | <MDL | 7.1 | 8.5 | <MDL | <MDL | 0.7 | <MDL | <MDL | <MDL | <MDL | 0.7 | <MDL | 26.8 | <MDL | <MDL |
| 4 | 1.2 | 0.8 | 9.4 | 9.1 | 1.1 | <MDL | 1.9 | 1.7 | 0.8 | <MDL | 0.7 | 0.6 | 120.2 | 105.0 | 5.5 | 5.1 |
| 5 | 1.3 | 1.4 | 24.6 | 26.0 | 35.4 | 37.9 | 27.1 | 29.0 | 9.0 | 9.5 | 1.2 | 1.3 | 54.9 | 80.0 | 6.6 | 7.3 |
| 6 | <MDL | <MDL | 113.9 | 136.5 | 4.8 | 5.6 | 15.6 | 20.1 | 9.3 | 12.0 | 1.7 | 1.9 | 22.4 | 47.7 | 5.1 | 6.2 |
| 7 | <MDL | 0.8 | 7.1 | 21.8 | 1.2 | 2.4 | <MDL | 2.7 | <MDL | 1.2 | 4.6 | 2.4 | 18.9 | 115.8 | <MDL | 8.4 |
| 8 | 1.3 | 1.0 | 5.5 | 7.7 | <MDL | 1.2 | 1.7 | 2.3 | <MDL | 0.8 | <MDL | 1.1 | 19.0 | 130.7 | 3.5 | 11.8 |
| 9 | 1.6 | 1.3 | 10.3 | 9.0 | 3.2 | 3.5 | 4.3 | 4.2 | 1.5 | 1.3 | 0.6 | 1.0 | 13.4 | 30.8 | 4.0 | 3.9 |
| 10 | <MDL | <MDL | 29.3 | 26.4 | 5.3 | 4.4 | 4.4 | 3.3 | 1.7 | 1.6 | 4.9 | 4.5 | 42.1 | 38.1 | 5.2 | 4.7 |
| 11 | 1.6 | <MDL | 41.0 | 7.9 | 25.8 | 5.3 | 4.9 | <MDL | 1.8 | <MDL | 7.5 | 1.1 | 18.6 | 11.0 | 7.8 | <MDL |
| 12 | 0.9 | 1.9 | 7.0 | 10.1 | 1.4 | 1.6 | 1.8 | 2.1 | 0.7 | 0.9 | <MDL | 1.1 | 30.3 | 146.7 | 4.6 | 13.2 |
| 13 | 1.5 | 1.9 | 8.7 | 11.9 | 4.5 | 6.6 | 5.0 | 6.8 | 1.6 | 2.4 | 1.1 | 1.5 | 29.3 | 69.6 | 12.5 | 24.9 |
| 14 | 0.9 | 1.3 | 3.9 | 4.8 | 2.1 | 2.6 | 2.0 | 2.5 | 0.8 | 1.0 | 0.8 | 1.1 | 6.7 | 63.4 | 3.1 | 5.9 |
| 15 | <MDL | <MDL | 5.4 | 10.1 | 1.0 | 1.5 | 1.2 | 2.3 | 1.6 | 1.3 | <MDL | <MDL | 13.9 | 42.5 | <MDL | 7.1 |
| 16 | 2.9 | 2.5 | 60.6 | 55.6 | 105.5 | 93.4 | 76.6 | 68.4 | 24.7 | 21.5 | 1.5 | 1.4 | 28.4 | 75.4 | 2.4 | 4.7 |
| 17 | <MDL | <MDL | 12.9 | 15.6 | 4.0 | 5.8 | 2.9 | 4.2 | 1.2 | 1.5 | 1.3 | 1.9 | 28.0 | 58.4 | 2.0 | 5.0 |
| 18 | <MDL | <MDL | 6.0 | 6.7 | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | 10.0 | 20.4 | 3.4 | 8.1 |
| 19 | <MDL | <MDL | 20.8 | 24.0 | 12.9 | 16.1 | 7.0 | 7.3 | 2.2 | 2.7 | 4.5 | 5.9 | 28.7 | 80.3 | 2.5 | 3.7 |
| 20 | 1.3 | 3.3 | 6.4 | 19.7 | 3.0 | 7.9 | 3.3 | 8.2 | 1.2 | 2.8 | <MDL | 1.6 | 28.6 | 138.4 | 3.9 | 17.6 |
| 21 | <MDL | <MDL | 4.4 | 4.1 | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | 23.1 | <MDL | 2.0 |
| 22 | <MDL | <MDL | 15.7 | 34.6 | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | <MDL | 0.6 | 11.6 | 218.2 | <MDL | 14.2 |
| 23 | 1.1 | <MDL | 108.1 | 7.9 | 2.5 | <MDL | 3.2 | <MDL | 1.3 | <MDL | 0.9 | <MDL | <MDL | 41.8 | <MDL | 17.1 |
| 24 | 0.7 | <MDL | 13.9 | 13.1 | 1.0 | 1.0 | 1.3 | 0.8 | <MDL | <MDL | <MDL | 0.6 | 252.6 | 108.9 | 21.5 | 11.4 |
| 25 | 1.4 | 1.2 | 20.0 | 20.2 | 1.4 | 1.1 | 2.2 | 1.5 | 1.2 | 0.8 | 2.0 | 1.7 | 63.6 | 73.9 | 7.5 | 8.0 |
| 26 | 1.5 | 1.3 | 26.0 | 24.6 | 5.4 | 5.7 | 6.8 | 7.2 | 3.2 | 3.0 | 2.4 | 2.4 | 41.4 | 59.3 | 2.1 | 5.8 |
| 27 | 0.8 | 1.1 | 8.1 | 10.6 | 2.4 | 3.7 | 2.4 | 3.6 | 0.8 | 1.2 | <MDL | 1.1 | 17.8 | 48.8 | 9.6 | 16.5 |
| 28 | <MDL | <MDL | 42.8 | 59.0 | <MDL | 1.7 | <MDL | 2.6 | 1.3 | 1.5 | 1.6 | 2.4 | 56.2 | 157.6 | 4.2 | 9.9 |
| 29 | 1.3 | 1.2 | 12.3 | 14.3 | 1.6 | 1.8 | 2.8 | 3.5 | 1.0 | 3.3 | <MDL | <MDL | 17.1 | 321.0 | <MDL | 93.0 |
| 30 | 0.5 | <MDL | 14.5 | 12.9 | 23.0 | 20.2 | 16.4 | 13.9 | 5.9 | 4.8 | 0.6 | 0.6 | 37.5 | 46.6 | <MDL | <MDL |
| Mean ± SD*** | 0.8 | 0.8 | 22.5 | 21.3 | 8.5 | 7.9 | 6.9 | 6.9 | 2.6 | 2.6 | 1.4 | 1.4 | 37.6 | 84.9 | 4.4 | 11.0 |
|  | ± 0.7 | ± 0.9 | ± 27.4 | ± 25.5 | ± 20.1 | ± 17.8 | ± 14.4 | ± 13.2 | ± 4.7 | ± 4.4 | ± 1.7 | ± 1.2 | ± 48.2 | ± 65.7 | ± 4.5 | ± 16.5 |
| *Without: Sample collected without deploying plants in indoor. **With: Sample collected by deploying plants in indoor. ***SD: standard deviation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table S3. Volume (m3) of investigated homes.

| Volunteers ID | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| *Volume (m3) | 23 | 25 | 25 | 23 | 23 | 23 | 23 | 24 | 23 | 20 | 22 | 23 | 20 | 23 | 17 | 23 | 15 | 23 | 39 | 21 | 11 | 23 | 24 | 20 | 25 | 25 | 20 | 29 | 13 | 22 |
| *Volume of the residential spaces were investigated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table S4. The sampling rates (L hr-1) of the ePTFE PS for targeted VOCs.

| Compound | Sampling rates (L hr-1) |
| --- | --- |
| benzene | 28.33 |
| toluene | 26.99 |
| ethylbenzene | 28.16 |
| m, p-xylene | 26.16 |
| o-xylene | 24.52 |
| styrene | 23.31 |
| limonene | 8.30 |
| α-pinene | 11.00 |

Table S5. Operation parameters for quantifying targeted VOCs.

| GC | Agilent 6890A gas chromatography |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MS | Agilent 5973 N mass selective detector |  |  |  |  |  |  |
|  | Agilent 7683 auto-injector |  |  |  |  |  |  |
| column | Agilent DB-5 column (30 m × 0.25 mm × 0.25 μm) |  |  |  |  |  |  |
| SCAN and SIM mode using Electron Ionization (EI) |  |  |  |  |  |  |  |
| Inlet Temperature | 250 °C |  |  |  |  |  |  |
| column oven gradient | Hold time (min) | 2 |  | 0 |  |  | 0 |
|  | oven temp. (°C) | 40 |  | 100 |  |  | 270 |
| Injection volume | 2 µL |  |  |  |  |  |  |
| Injection mode | split ratio 5:1 |  |  |  |  |  |  |
| Carrier Gas | Helium |  |  |  |  |  |  |
| Column Flow | 1.2 mL min-1 |  |  |  |  |  |  |
| Transfer Line Temperature | 250 °C |  |  |  |  |  |  |
| Ion Source Temperature | 200 °C |  |  |  |  |  |  |
| Quadrupole Temperature | 150 °C |  |  |  |  |  |  |

Table S6. Selected m/z values for quantifying targeted VOCs using GC-MS.

| Compound | m/z |
| --- | --- |
| benzene | 78 |
| toluene | 91 |
| ethylbenzene | 91 |
| m, p-xylene | 91 |
| o-xylene | 91 |
| styrene | 104 |
| limonene | 68 |
| α-pinene | 93 |

Table S7. Reference concentration (μg m-3) of the targeted VOCs for health risk assessment.

| VOCs | RfC (μg m-3) | Basis |
| --- | --- | --- |
| benzene | 30 | EPA IRIS, Seo et al. (2025) |
| toluene | 5000 |  |
| ethylbenzene | 1000 |  |
| m, p-xylene | 100 |  |
| o-xylene | 100 |  |
| styrene | 1000 |  |
| limonene | 9000 |  |
| α-pinene | 4500 |  |

Table S8. HQ values for non-carcinogenic risks about targeted VOCs according to plant growing conditions at different age groups.

|  |  | Potted culture |  |  |  |  |  |  |  |  | hydroculture |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Age group | Mean ±SD | Min | 95th |  | Max |  |  |  | Mean ±SD |  | Min |  | 95th |  | Max |  |
| benzene | Infants | 2.18E-02 ± 2.00E-02 | 3.37.E-03 | 5.31.E-02 |  | 8.21.E-02 |  |  |  | 2.40E-02 ± 2.51E-02 |  | 3.37.E-03 |  | 6.54.E-02 |  | 9.68.E-02 |  |
|  | Toddlers | 1.96E-02 ± 1.80E-02 | 3.03.E-03 | 4.77.E-02 |  | 7.37.E-02 |  |  |  | 2.15E-02 ± 2.25E-02 |  | 3.03.E-03 |  | 5.87.E-02 |  | 8.69.E-02 |  |
|  | Children | 1.96E-02 ± 1.80E-02 | 3.03.E-03 | 4.77.E-02 |  | 7.37.E-02 |  |  |  | 2.15E-02 ± 2.25E-02 |  | 3.03.E-03 |  | 5.87.E-02 |  | 8.69.E-02 |  |
|  | Teenagers | 2.18E-02 ± 2.00E-02 | 3.37.E-03 | 5.31.E-02 |  | 8.21.E-02 |  |  |  | 2.40E-02 ± 2.51E-02 |  | 3.37.E-03 |  | 6.54.E-02 |  | 9.68.E-02 |  |
|  | Adults | 2.18E-02 ± 2.00E-02 | 3.37.E-03 | 5.31.E-02 |  | 8.21.E-02 |  |  |  | 2.40E-02 ± 2.51E-02 |  | 3.37.E-03 |  | 6.54.E-02 |  | 9.68.E-02 |  |
| toluene | Infants | 4.17E-03 ± 4.35E-03 | 1.02.E-03 | 1.05.E-02 |  | 2.30.E-02 |  |  |  | 3.75E-03 ± 4.48E-03 |  | 7.22.E-04 |  | 1.01.E-02 |  | 2.40.E-02 |  |
|  | Toddlers | 3.74E-03 ± 3.90E-03 | 9.16.E-04 | 9.39.E-03 |  | 2.07.E-02 |  |  |  | 3.37E-03 ± 4.03E-03 |  | 6.48.E-04 |  | 9.08.E-03 |  | 2.16.E-02 |  |
|  | Children | 3.74E-03 ± 3.90E-03 | 9.16.E-04 | 9.39.E-03 |  | 2.07.E-02 |  |  |  | 3.37E-03 ± 4.03E-03 |  | 6.48.E-04 |  | 9.08.E-03 |  | 2.16.E-02 |  |
|  | Teenagers | 4.17E-03 ± 4.35E-03 | 1.02.E-03 | 1.05.E-02 |  | 2.30.E-02 |  |  |  | 3.75E-03 ± 4.48E-03 |  | 7.22.E-04 |  | 1.01.E-02 |  | 2.40.E-02 |  |
|  | Adults | 4.17E-03 ± 4.35E-03 | 1.02.E-03 | 1.05.E-02 |  | 2.30.E-02 |  |  |  | 3.75E-03 ± 4.48E-03 |  | 7.22.E-04 |  | 1.01.E-02 |  | 2.40.E-02 |  |
| ethylbenzene | Infants | 8.44E-03 ± 1.93E-02 | 3.96.E-04 | 3.77.E-02 |  | 9.73.E-02 |  |  |  | 7.01E-03 ± 1.57E-02 |  | 3.96.E-04 |  | 2.63.E-02 |  | 8.22.E-02 |  |
|  | Toddlers | 7.57E-03 ± 1.73E-02 | 3.56.E-04 | 3.38.E-02 |  | 8.74.E-02 |  |  |  | 6.29E-03 ± 1.41E-02 |  | 3.56.E-04 |  | 2.36.E-02 |  | 7.38.E-02 |  |
|  | Children | 7.57E-03 ± 1.73E-02 | 3.56.E-04 | 3.38.E-02 |  | 8.74.E-02 |  |  |  | 6.29E-03 ± 1.41E-02 |  | 3.56.E-04 |  | 2.36.E-02 |  | 7.38.E-02 |  |
|  | Teenagers | 8.44E-03 ± 1.93E-02 | 3.96.E-04 | 3.77.E-02 |  | 9.73.E-02 |  |  |  | 7.01E-03 ± 1.57E-02 |  | 3.96.E-04 |  | 2.63.E-02 |  | 8.22.E-02 |  |
|  | Adults | 8.44E-03 ± 1.93E-02 | 3.96.E-04 | 3.77.E-02 |  | 9.73.E-02 |  |  |  | 7.01E-03 ± 1.57E-02 |  | 3.96.E-04 |  | 2.63.E-02 |  | 8.22.E-02 |  |
| m, p-xylene | Infants | 7.29E-02 ± 1.40E-01 | 2.90.E-03 | 2.76.E-01 |  | 7.10.E-01 |  |  |  | 7.06E-02 ± 1.57E-01 |  | 3.96.E-03 |  | 2.63.E-01 |  | 8.22.E-01 |  |
|  | Toddlers | 6.54E-02 ± 1.26E-01 | 2.61.E-03 | 2.48.E-01 |  | 6.38.E-01 |  |  |  | 6.34E-02 ± 1.41E-01 |  | 3.56.E-03 |  | 2.36.E-01 |  | 7.38.E-01 |  |
|  | Children | 6.54E-02 ± 1.26E-01 | 2.61.E-03 | 2.48.E-01 |  | 6.38.E-01 |  |  |  | 6.34E-02 ± 1.41E-01 |  | 3.56.E-03 |  | 2.36.E-01 |  | 7.38.E-01 |  |
|  | Teenagers | 7.29E-02 ± 1.40E-01 | 2.90.E-03 | 2.76.E-01 |  | 7.10.E-01 |  |  |  | 7.06E-02 ± 1.57E-01 |  | 3.96.E-03 |  | 2.63.E-01 |  | 8.22.E-01 |  |
|  | Adults | 7.29E-02 ± 1.40E-01 | 2.90.E-03 | 2.76.E-01 |  | 7.10.E-01 |  |  |  | 7.06E-02 ± 1.57E-01 |  | 3.96.E-03 |  | 2.63.E-01 |  | 8.22.E-01 |  |
| o-xylene | Infants | 2.72E-02 ± 4.45E-02 | 2.90.E-03 | 9.52.E-02 |  | 2.24.E-01 |  |  |  | 2.38E-02 ± 3.88E-02 |  | 2.90.E-03 |  | 9.57.E-02 |  | 1.89.E-01 |  |
|  | Toddlers | 2.44E-02 ± 3.99E-02 | 2.61.E-03 | 8.55.E-02 |  | 2.01.E-01 |  |  |  | 2.13E-02 ± 3.49E-02 |  | 2.61.E-03 |  | 8.59.E-02 |  | 1.70.E-01 |  |
|  | Children | 2.44E-02 ± 3.99E-02 | 2.61.E-03 | 8.55.E-02 |  | 2.01.E-01 |  |  |  | 2.13E-02 ± 3.49E-02 |  | 2.61.E-03 |  | 8.59.E-02 |  | 1.70.E-01 |  |
|  | Teenagers | 2.72E-02 ± 4.45E-02 | 2.90.E-03 | 9.52.E-02 |  | 2.24.E-01 |  |  |  | 2.38E-02 ± 3.88E-02 |  | 2.90.E-03 |  | 9.57.E-02 |  | 1.89.E-01 |  |
|  | Adults | 2.72E-02 ± 4.45E-02 | 2.90.E-03 | 9.52.E-02 |  | 2.24.E-01 |  |  |  | 2.38E-02 ± 3.88E-02 |  | 2.90.E-03 |  | 9.57.E-02 |  | 1.89.E-01 |  |
| styrene | Infants | 1.47E-03 ± 1.17E-03 | 2.46.E-04 | 3.26.E-03 |  | 5.19.E-03 |  |  |  | 1.23E-03 ± 1.08E-03 |  | 2.46.E-04 |  | 3.13.E-03 |  | 5.19.E-03 |  |
|  | Toddlers | 1.32E-03 ± 1.05E-03 | 2.21.E-04 | 2.93.E-03 |  | 4.66.E-03 |  |  |  | 1.10E-03 ± 9.72E-04 |  | 2.21.E-04 |  | 2.81.E-03 |  | 4.66.E-03 |  |
|  | Children | 1.32E-03 ± 1.05E-03 | 2.21.E-04 | 2.93.E-03 |  | 4.66.E-03 |  |  |  | 1.10E-03 ± 9.72E-04 |  | 2.21.E-04 |  | 2.81.E-03 |  | 4.66.E-03 |  |
|  | Teenagers | 1.47E-03 ± 1.17E-03 | 2.46.E-04 | 3.26.E-03 |  | 5.19.E-03 |  |  |  | 1.23E-03 ± 1.08E-03 |  | 2.46.E-04 |  | 3.13.E-03 |  | 5.19.E-03 |  |
|  | Adults | 1.47E-03 ± 1.17E-03 | 2.46.E-04 | 3.26.E-03 |  | 5.19.E-03 |  |  |  | 1.23E-03 ± 1.08E-03 |  | 2.46.E-04 |  | 3.13.E-03 |  | 5.19.E-03 |  |
| limonene | Infants | 5.97E-03 ± 4.47E-03 | 2.33.E-04 | 1.53.E-02 |  | 1.81.E-02 |  |  |  | 8.30E-03 ± 6.43E-03 |  | 1.08.E-03 |  | 1.87.E-02 |  | 3.14.E-02 |  |
|  | Toddlers | 5.36E-03 ± 4.02E-03 | 2.09.E-04 | 1.38.E-02 |  | 1.62.E-02 |  |  |  | 7.45E-03 ± 5.77E-03 |  | 9.66.E-04 |  | 1.68.E-02 |  | 2.82.E-02 |  |
|  | Children | 5.36E-03 ± 4.02E-03 | 2.09.E-04 | 1.38.E-02 |  | 1.62.E-02 |  |  |  | 7.45E-03 ± 5.77E-03 |  | 9.66.E-04 |  | 1.68.E-02 |  | 2.82.E-02 |  |
|  | Teenagers | 5.97E-03 ± 4.47E-03 | 2.33.E-04 | 1.53.E-02 |  | 1.81.E-02 |  |  |  | 8.30E-03 ± 6.43E-03 |  | 1.08.E-03 |  | 1.87.E-02 |  | 3.14.E-02 |  |
|  | Adults | 5.97E-03 ± 4.47E-03 | 2.33.E-04 | 1.53.E-02 |  | 1.81.E-02 |  |  |  | 8.30E-03 ± 6.43E-03 |  | 1.08.E-03 |  | 1.87.E-02 |  | 3.14.E-02 |  |
| α-pinene | Infants | 1.78E-03 ± 3.19E-03 | 1.89.E-04 | 3.48.E-03 |  | 1.81.E-02 |  |  |  | 2.18E-03 ± 3.23E-03 |  | 1.89.E-04 |  | 4.23.E-03 |  | 1.82.E-02 |  |
|  | Toddlers | 1.60E-03 ± 2.87E-03 | 1.69.E-04 | 3.13.E-03 |  | 1.63.E-02 |  |  |  | 1.95E-03 ± 2.90E-03 |  | 1.69.E-04 |  | 3.79.E-03 |  | 1.63.E-02 |  |
|  | Children | 1.60E-03 ± 2.87E-03 | 1.69.E-04 | 3.13.E-03 |  | 1.63.E-02 |  |  |  | 1.95E-03 ± 2.90E-03 |  | 1.69.E-04 |  | 3.79.E-03 |  | 1.63.E-02 |  |
|  | Teenagers | 1.78E-03 ± 3.19E-03 | 1.89.E-04 | 3.48.E-03 |  | 1.81.E-02 |  |  |  | 2.18E-03 ± 3.23E-03 |  | 1.89.E-04 |  | 4.23.E-03 |  | 1.82.E-02 |  |
|  | Adults | 1.78E-03 ± 3.19E-03 | 1.89.E-04 | 3.48.E-03 |  | 1.81.E-02 |  |  |  | 2.18E-03 ± 3.23E-03 |  | 1.89.E-04 |  | 4.23.E-03 |  | 1.82.E-02 |  |

References

U.S. Environmental Protection Agency. (n.d.). Integrated Risk Information System (IRIS). Retrieved September 18, 2025, from https://www.epa.gov/iris

Seo, J. H., Kim, P. G., Choi, Y. H., Shin, W., Sochichiu, S., Khoshakhlagh, A. H., & Kwon, J. H. (2025). Evaluation of personal exposure to volatile organic chemicals (VOCs) in small-scale dry-cleaning facilities using passive sampling. Atmospheric Environment, 353, 121235. https://doi.org/10.1016/j.atmosenv.2025.121235