Table S1

|  | Training and testing cohort |  |  | Validation cohort |  |
| --- | --- | --- | --- | --- | --- |
| PET/CT scanner | Biography 16 PET/CT |  |  | uEXPLORER |  |
| PET scan | Myocardial perfusion imaging | Myocardial metabolism imaging | Myocardial perfusion imaging |  | Myocardial metabolism imaging |
| Imaging agents | 13N-NH3 | 18F-FDG | 13N-NH3 |  | 18F-FDG |
| Injected dose | 370-555 Mbq | 3.7 MBq/kg | 370-555 Mbq |  | 3.7 MBq/kg |
| Fasting period | - | ≥ 6 h | - |  | ≥ 6 h |
| Blood glucose regulation | - | Oral glucose load+intravenous insulin injection | - |  | Oral glucose load+intravenous insulin injection |
| Imaging mode | List mode: gated/dynamic | List mode: static | List mode: gated/dynamic |  | List mode: static |
| Imaging duration | 15 min | 15 min | 10 min |  | 10 min |
| Bed units | 1 | 1 | 3 |  | 3 |
| Reconstruction method | OSEM (2 iterations, 24 subsets) | OSEM (2 iterations, 24 subsets) | OSEM (3 iterations, 20 subsets) |  | OSEM (3 iterations, 20 subsets) |
| Full-Width at Half-Maximum | 5.0 mm | 5.0 mm | 3.0 mm |  | 3.0 mm |
| Image matrix | 168×168 | 168×168 | 192×192 |  | 192×192 |
| Voxel size | 2.0×2.0×2.5 | 2.0×2.0×2.5 | 2.3×2.3×2.9 |  | 2.3×2.3×2.9 |
| Reconstruction sequences | Static and dynamic sequence | Static sequence | Static and dynamic sequence |  | Static sequence |
| Rebinned method for dynamic sequence | 21 frames (12×10 s, 6×30 s, 2×60 s, 1×180 s) | - | 21 frames (12×10 s, 6×30 s, 2×60 s, 1×180 s) |  | - |
| CT scan | Attenuation corrected CT | Attenuation corrected CT | Attenuation corrected CT |  | Attenuation corrected CT |
| Tube voltage | 140 kV | 140 kV | 120 kV |  | 120 kV |
| Tbe current | 80 mA | 80 mA | 100 mA |  | 100 mA |
| Pitch | 1.0 | 1.0 | 1.0 |  | 1.0 |
| Collimation | - | - | 80×0.5 mm |  | 80×0.5 mm |
| Rotation time | 0.5 s | 0.5 s | 0.5 s |  | 0.5 s |
| Image matrix | 512×512 | 512×512 | 512×512 |  | 512×512 |
| Slice thickness | 3 mm | 3 mm | 3 mm |  | 3 mm |