“Microbial Synergy: Fungal Laccase-Mediated Optimized Bio-detoxification and Extremophile-Driven Bio-devulcanization of Ground Tire Rubber for Sustainable Recycling”

Journal name: Biodegradation

Harmen Shaha, d*, FaridahYusofa,b, Md Zahangir Alama,c

a Department of Chemical Engineering and Sustainability, International Islamic University  Malaysia (IIUM), P.O. Box 10, 50728, Kuala Lumpur, Malaysia.

b Bioprocess and Molecular Engineering Research Unit (BPMERU), IIUM, P.O. Box 10, 50728, Kuala Lumpur, Malaysia.

c Bioenvironmental Engineering Research Center (BERC), IIUM, P.O. Box 10, 50728, Kuala Lumpur, Malaysia.

d Vidya Vihar Institute of Technology, BIADA, Maranga, Purnea, PIN – 854301, India.

* Corresponding author

Email address: harmenshah1@gmail.com

### SUPPLEMENTARY MATERIAL

### Equation S1:	Laccase assay

It was performed by adopting the method of Ride (1980) as explained in section 2.6.

Laccase activity was calculated as follows:

Units / ml enzyme =  (∆A530 /min Test  -   r A530  /min Blank) (DF)0.001  (0.5)	(1)

Where;

DF = Dilution factor

0.001 = the change in the ∆A530 / min per unit of laccase at pH 6.6 at 30ºC in a 3 ml of

Reaction mixture

0.5 = Volume (mL, milliliter) of enzyme sample

Units / mg solid = units/mlenzymemg solid/mlenzyme	(2)