Supplementary Table S1. Sampling design and measurement protocol.

| Measurement | Analyte | Standard / method | Instrument/analysis | Sampling media | Flow rate (L/min) | Duration (min) | Inlet height (cm) | n | Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fibrous dust (air) a | Fibers counted by PCM | HSG 248+A1 | Phase-contrast microscopy (PCM) | Membrane filter | 4.0 | 120 | 120 | 12 | 9-10 May 2024 |
| Asbestos identification b | Asbestos fibers | VDI 3866 Part 5 | SEM with EDXA |  |  |  |  |  |  |
| Total dust (air) c | Gravimetric total dust | MDHS 14/3 | Gravimetric analysis (sampler + laboratory balance) | Filter cassette | 2.2 | 120 | 120 | 6 | 9 May 2024 |
| Respirable dust (air) c | Gravimetric respirable dust | MDHS 14/3 | Respirable cyclone + gravimetric analysis | Respirable cyclone + filter | 1.7 | 120 | 120 | 6 |  |

Note. PCM = phase-contrast microscopy; SEM/EDXA = scanning electron microscopy with energy-dispersive X-ray analysis; HSG = Health and Safety Guidance; VDI = Verein Deutscher Ingenieure; MDHS = Methods for the Determination of Hazardous Substances. n indicates the number of samples collected for the specified analyte/outcome. Blank cells indicate that the parameter was not applicable or not recorded.

a Fibers were counted by PCM when > 5 μm in length, < 3 μm in width, and with an aspect ratio ≥ 3:1.

b SEM/EDXA (VDI 3866 Part 5) was used to differentiate asbestos from non-asbestos fibers; results were reported as detected/not detected.

c For gravimetric sampling (MDHS 14/3), sampling trains were calibrated with a TSI primary flow calibrator immediately before sampling.

Supplementary Table S2. Demolition vs control effect sizes using bootstrap CI and uncertainty-propagated (Monte Carlo) intervals

| Outcome | Point difference (Δ=demolition-control) | Bootstrap 95% CI | Uncertainty-propagated 95% interval (Monte Carlo) |
| --- | --- | --- | --- |
| Total dust (mg/m³) | 0.245 | 0.180-0.310 | -0.003 to 0.494 |
| Respirable dust (mg/m³) | 0.395 | 0.300-0.490 | 0.262 to 0.527 |

Note. Point difference (Δ) is mean (demolition - control). Bootstrap 95% CI obtained by resampling demolition sites with replacement (control fixed). Uncertainty-propagated 95% interval obtained via Monte Carlo simulation. For total dust, the uncertainty-propagated interval crosses zero (-0.003), indicating the difference may not be significant when measurement uncertainty is fully accounted for.

Supplementary Table S3. Representative particle characterization by SEM-EDX

| Sample ID | Particle Type | Morphology | Major Elements (EDX) |
| --- | --- | --- | --- |
| H79 | Fibrous | Fiber | Cr, Oa |
| H76 | Particulate | Non-fibrous | Fe, Ob |
| J124 | Particulate | Non-fibrous | C, Ti, Sc |
| H72 / H74 | Particulate | Non-fibrous | Ca, Si, Od |
| H73 / J123 | Particulate | Non-fibrous | Ca, Od |
| H71 / H78 | Organic Fiber | Fiber | C, Cl, Ke |

a Cr, O: Spectra consistent with chromium(III) oxide (Cr2O3), likely derived from pigments (e.g., chrome green) or passivated metal shavings. b Fe, O: Indicative of iron oxides (FexOy) or oxidized steel particles generated from reinforcement cutting. c C, Ti, S: The presence of Titanium (Ti) alongside Carbon (C) and Sulfur (S) identifies this as a paint fragment containing titanium dioxide (TiO2) pigment. d Ca, Si, O / Ca, O: Calcium-silicate and calcareous signatures represent the dominant mineral dust load, consistent with concrete, cement, and plaster materials. e C, Cl, K: High carbon content distinguishes these as organic fibers (e.g., cellulose, synthetic polymers) rather than hazardous inorganic fibers.