Integrating geodetic constraints from major faults and the subduction zone into probabilistic seismic hazard assessments (PSHA) for Guatemala

Mario Arroyo-Solórzano1,2, Carlos Gamboa-Canté3,4, Jeremy Maurer5, Omar Flores4, María Belén Benito3 and Fabrice Cotton1,2

1 GFZ Helmholtz Centre for Geosciences, Telegrafenberg, 14473 Potsdam, Germany.

2 Institute of Geosciences, University of Potsdam, 14476 Potsdam, Germany.

3 Departamento de Ingeniería y Morfología del Terreno, Escuela Técnica Superior de Ingenieros de Caminos, Canales y Puertos, Universidad Politécnica de Madrid, 28040 Madrid, Spain.

4 Centro de Estudios Superiores de Energía y Minas, Facultad de Ingeniería, Universidad de San Carlos de Guatemala, Guatemala City 01012, Guatemala.

5 Missouri University of Science and Technology, Geological Sciences and Geological and Petroleum Engineering, 1400 N Bishop, Rolla, MO 65401, USA.

Mario Arroyo-Solórzano: marroyo@gfz.de, corresponding author ORCID: 0000-0002-1653-2680

Carlos Gamboa-Canté: carlosjose.gamboa@upm.es ORCID: 0000-0002-9921-5911

Jeremy Maurer: jmaurer@mst.edu ORCID: 0000-0002-3624-5961

Omar Flores: 1798801280101@ingenieria.usac.edu.gt ORCID: 0009-0009-2977-1638

María Belén Benito: mariabelen.benito@upm.es ORCID: 0000-0003-3582-6438

Fabrice Cotton: fcotton@gfz.de ORCID: 0000-0002-9242-3996

TABLES

Table 1. MFD characterization based on geodetic information of the subduction interplate sources considering coupling information from Garnier et al. (2022) and using Mw 4.5 as Mmin for the seismicity parameters calculation.

| Interplate segment | Geometric characteristics |  |  |  |  |  | 1b. Earthquake Rates (Nmin) variation |  |  |  | 2b. Mmax variation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Depth (km) | L (km) | W (km) |  | Dip (°) | Rake (°) | a-val | b-val | Nmin | Mmax | a-val | b-val | Nmin | Mmax |
| Gsi9a | 10-40 | 235 | 60 |  | 30 | 90 | 5.86 | 1.09 | 8.97 | 7.8 | 5.76 | 1.09 | 7.16 | 8.0 |
| Gsi9b | 10-40 | 280 | 55 | 35 |  | 90 | 5.95 | 1.04 | 18.42 | 8.2 | 5.80 | 1.04 | 13.18 | 8.2 |
| Ssi5 | 10-40 | 260 | 58 | 30 |  | 90 | 6.62 | 1.25 | 9.94 | 7.7 | 6.56 | 1.25 | 8.61 | 8.0 |

L, length; W, width; a-val, a-value; b-val, b-value; Nmin, cumulative events from Mmin (Mw 4.5); Mmax, maximum magnitude inferred.

Table 2. MFD characterization based on the partitioning of seismic moment between background seismicity and main faults, using geodetic information from Garnier et al. (2022), Mw 4.5 as Mmin, and two approaches to constrain Mmax.

| - Mmax from scaling relations for Central America (Arroyo-Solórzano et al., 2024) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seismic Zone | Fault | Seismic Source | Slip rate (mm/yr) | Recurrence parameters and Mmax using RSB85 |  |  |  | Recurrence parameters and Mmax using UHR86 |  |  |  |
|  |  |  |  | a-val | b-val | Nmin | Mmax | a-val | b-val | Nmin | Mmax |
| G6 | Polochic | P01 | 3.7 | 1.39 | 0.56 | 0.07 | 7.6 | 1.39 | 0.56 | 0.07 | 7.6 |
|  |  | P02 | 3.7 | 1.37 | 0.56 | 0.07 | 7.5 | 1.37 | 0.56 | 0.07 | 7.5 |
|  | Motagua | M01 | 5.1 | 1.55 | 0.56 | 0.11 | 7.4 | 1.55 | 0.56 | 0.11 | 7.4 |
|  |  | M02 | 11.9 | 1.93 | 0.56 | 0.26 | 7.3 | 1.93 | 0.56 | 0.26 | 7.3 |
|  |  | M03 | 13.9 | 1.98 | 0.56 | 0.29 | 7.1 | 1.98 | 0.56 | 0.29 | 7.1 |
|  |  | M04 | 16.2 | 2.03 | 0.56 | 0.32 | 7.2 | 2.03 | 0.56 | 0.32 | 7.2 |
|  | Background |  | - | 4.60 | 0.94 | 6.75 | 4.5 | 4.01 | 0.84 | 4.57 | 4.5 |
| G4 | Jalpatagua | J01 | 3.0 | 3.23 | 0.88 | 0.19 | 6.7 | 2.83 | 0.81 | 0.15 | 6.7 |
|  |  | J02 | 5.5 | 3.48 | 0.88 | 0.33 | 6.8 | 3.08 | 0.81 | 0.27 | 6.8 |
|  |  | J03 | 7.1 | 3.64 | 0.88 | 0.48 | 7.0 | 3.22 | 0.81 | 0.38 | 7.0 |
|  | Background |  | - | 5.11 | 1.03 | 31.02 | 5.8 | 4.16 | 1.00 | 4.58 | 5.8 |
| - Mmax using probabilities distribution based on both empirical and observed magnitudes (Tavakolizadeh et al., 2025) |  |  |  |  |  |  |  |  |  |  |  |
| Seismic Zone | Fault | Seismic Source | Slip rate (mm/yr) | Recurrence parameters and Mmax using RSB85 |  |  |  | Recurrence parameters and Mmax using UHR86 |  |  |  |
|  |  |  |  | a-val | b-val | Nmin | Mmax | a-val | b-val | Nmin | Mmax |
| G6 | Polochic | P01 | 3.7 | 1.71 | 0.62 | 0.08 | 7.7 | 1.19 | 0.54 | 0.06 | 7.7 |
|  |  | P02 | 3.7 | 1.83 | 0.62 | 0.11 | 7.4 | 1.32 | 0.54 | 0.08 | 7.4 |
|  | Motagua | M01 | 5.1 | 1.93 | 0.62 | 0.14 | 7.4 | 1.42 | 0.54 | 0.10 | 7.4 |
|  |  | M02 | 11.9 | 2.30 | 0.62 | 0.33 | 7.3 | 1.80 | 0.54 | 0.24 | 7.3 |
|  |  | M03 | 13.9 | 2.17 | 0.62 | 0.24 | 7.4 | 1.67 | 0.54 | 0.17 | 7.4 |
|  |  | M04 | 16.2 | 2.46 | 0.62 | 0.47 | 7.1 | 1.98 | 0.54 | 0.35 | 7.1 |
|  | Background |  | - | 4.44 | 0.92 | 5.92 | 4.5 | 4.93 | 1.06 | 4.87 | 4.6 |
| G4 | Jalpatagua | J01 | 3.0 | 1.97 | 0.66 | 0.10 | 6.7 | 3.11 | 0.86 | 0.18 | 6.7 |
|  |  | J02 | 5.5 | 2.09 | 0.66 | 0.13 | 7.0 | 3.29 | 0.86 | 0.26 | 7.0 |
|  |  | J03 | 7.1 | 2.32 | 0.66 | 0.22 | 7.0 | 3.52 | 0.86 | 0.45 | 7.0 |
|  | Background |  | - | 4.58 | 0.85 | 39.73 | 5.6 | 3.68 | 0.92 | 2.84 | 5.8 |

RSB85 and UHR86, Reasenberg (1985) and Uhrhammer (1986) declustering methods; a-val, a-value; b-val, b-value; Nmin, cumulative events from Mmin (Mw 4.5); Mmax, maximum magnitude inferred according to each approach.

Table 3. MFD characterization based on the partitioning of seismic moment between background seismicity and main faults, using geodetic information from Maurer et al. (2025), Mw 4.5 as Mmin, and two approaches to constrain Mmax.

| - Mmax from scaling relations for Central America (Arroyo-Solórzano et al., 2024) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Seismic Zone | Fault | Seismic Source | Slip rate (mm/yr) | Recurrence parameters and Mmax using RSB85 |  |  |  | Recurrence parameters and Mmax using UHR86 |  |  |  |
|  |  |  |  | a-val | b-val | Nmin | Mmax | a-val | b-val | Nmin | Mmax |
| G6 | Polochic | P01 | 6.6 | 2.03 | 0.62 | 0.17 | 7.6 | 2.10 | 0.63 | 0.18 | 7.6 |
|  |  | P02 | 3.0 | 1.67 | 0.62 | 0.08 | 7.5 | 1.73 | 0.63 | 0.08 | 7.5 |
|  | Motagua | M01 | 3.0 | 1.70 | 0.62 | 0.08 | 7.4 | 1.76 | 0.63 | 0.08 | 7.4 |
|  |  | M02 | 8.8 | 2.17 | 0.62 | 0.24 | 7.3 | 2.23 | 0.63 | 0.25 | 7.3 |
|  |  | M03 | 9.7 | 2.19 | 0.62 | 0.25 | 7.1 | 2.25 | 0.63 | 0.26 | 7.1 |
|  |  | M04 | 9.0 | 2.14 | 0.62 | 0.23 | 7.2 | 2.21 | 0.63 | 0.23 | 7.2 |
|  | Background |  | - | 3.71 | 0.72 | 6.63 | 4.5 | 4.33 | 0.93 | 4.30 | 4.5 |
| G4 | Jalpatagua | J01 | 4.4 | 2.59 | 0.74 | 0.18 | 6.7 | 1.11 | 0.48 | 0.09 | 6.7 |
|  |  | J02 | 7.3 | 2.79 | 0.74 | 0.29 | 6.8 | 1.29 | 0.48 | 0.13 | 6.8 |
|  |  | J03 | 16.5 | 3.17 | 0.74 | 0.69 | 7.0 | 1.61 | 0.48 | 0.28 | 7.0 |
|  | Background |  | - | 5.45 | 1.13 | 31.39 | 5.2 | 4.21 | 0.91 | 10.41 | 5.8 |
| - Mmax using probabilities distribution based on both empirical and observed magnitudes (Tavakolizadeh et al., 2025) |  |  |  |  |  |  |  |  |  |  |  |
| Seismic Zone | Fault | Seismic Source | Slip rate (mm/yr) | Recurrence parameters and Mmax using RSB85 |  |  |  | Recurrence parameters and Mmax using UHR86 |  |  |  |
|  |  |  |  | a-val | b-val | Nmin | Mmax | a-val | b-val | Nmin | Mmax |
| G6 | Polochic | P01 | 6.6 | 2.43 | 0.69 | 0.21 | 7.7 | 2.10 | 0.64 | 0.17 | 7.7 |
|  |  | P02 | 3.0 | 2.18 | 0.69 | 0.12 | 7.4 | 1.86 | 0.64 | 0.10 | 7.4 |
|  | Motagua | M01 | 3.0 | 2.15 | 0.69 | 0.11 | 7.4 | 1.83 | 0.64 | 0.09 | 7.4 |
|  |  | M02 | 8.8 | 2.61 | 0.69 | 0.32 | 7.3 | 2.30 | 0.64 | 0.26 | 7.3 |
|  |  | M03 | 9.7 | 2.46 | 0.69 | 0.23 | 7.4 | 2.14 | 0.64 | 0.18 | 7.4 |
|  |  | M04 | 9.0 | 2.63 | 0.69 | 0.34 | 7.1 | 2.33 | 0.64 | 0.28 | 7.1 |
|  | Background |  | - | 5.11 | 1.09 | 5.43 | 4.5 | 4.43 | 0.95 | 4.22 | 4.5 |
| G4 | Jalpatagua | J01 | 4.4 | 3.22 | 0.85 | 0.25 | 6.7 | 1.68 | 0.58 | 0.12 | 6.7 |
|  |  | J02 | 7.3 | 3.35 | 0.85 | 0.34 | 7.0 | 1.74 | 0.58 | 0.14 | 7.0 |
|  |  | J03 | 16.5 | 3.83 | 0.85 | 1.00 | 7.0 | 2.21 | 0.58 | 0.40 | 7.0 |
|  | Background |  | - | 4.62 | 0.92 | 25.22 | 5.0 | 4.51 | 1.02 | 8.97 | 5.8 |

RSB85 and UHR86, Reasenberg (1985) and Uhrhammer (1986) declustering methods; a-val, a-value; b-val, b-value; Nmin, cumulative events from Mmin (Mw 4.5); Mmax, maximum magnitude inferred according to each approach.

Table 4. Summary of the seismic hazard components from the model of Gamboa-Canté et al. (2025) and their variations for this study.

| Seismic hazard components |  | KASHM 2025 (Gamboa-Canté et al., 2025) | This study |
| --- | --- | --- | --- |
| Seismic Source Model | Source type and geometry* | - Upper-plate sources modelled as “area source” for zones and “simple fault” for the major faults modeled. - Interplate and intraslab modeled as “complex fault”. - Seismic zoning is based on Alvarado et al. (2017). | The same as in the KASHM 2025, except that we consider the possibility of multi-ruptures (non-segmentation) of the interplate sources. Also, we consider another interplate seismic zone as Gsi9b. |
|  | Scaling relations and aspect ratio | - The magnitude-scaling used was according to the logic tree defined in the sensitivity analysis of Gamboa-Canté et al. (2025b). - Aspect ratio of 1.5 for all the zones. | The same as in the KASHM 2025. |
|  | Seismicity parameters* | - Taken from Gamboa-Canté et al. (2025b) based exclusively on the regional seismic catalog (Gamboa-Canté et al., 2024). | The same as in the KASHM 2025, except for the interplate sources and major faults. |
|  | Rupture characterization | -The strike, dip, and rake for the most likely rupture plane were defined primarily on recent fault maps and earthquake focal mechanisms of the regional seismic catalog (Gamboa-Canté et al., 2024). | The same as in the KASHM 2025. |
|  | Logic tree* | We used the logic tree defined in the sensitivity analysis of Gamboa-Canté et al. (2025b). | The same as in the KASHM 2025, but we add branches for slip rates and segmentation on the interplate and major faults. |
| Ground Motion Model (GMM) | Selection and logic tree | - Selected by local data calibration based on a review of the GMMs available for El Salvador and Costa Rica. - Logic trees used by tectonic domain and differentiating weights depending by Gamboa-Canté et al. (2025b). | The same as in the KASHM 2025. |
| Global parameters | Minimum magnitude, site effect and truncation levels | - 4.5 Mw, rock conditions with a constant time-averaged shear-wave velocity to a depth of 30 m (Vs30) of 760 m/s - Three truncation levels. | The same as in the KASHM 2025. |
|  | Intensity measures and return periods | - PGA and spectral accelerations (SA) from 0.1 to 1s and return periods of 475, 975 and 2475 yr. | Computations based on PGA and a return period of 475 yr. |

* Please note highlighted in bold the changes introduced on the seismic hazard components of the model.

Table 5. Fault aggravation factor (FAF) obtained from the evaluation of accelerations considering or not major faults in seismic hazard computations for Guatemala.

| Equation | Fault class |  | A0 | dc | Mean residual | RMSE | MAE | Std |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FAF(d)=1+A0*e-d/dc | VHSP | Slip rate > 8 mm/yr and Mmax > 7.0 | 2.89 | 18.77 | -0.013 | 0.283 | 0.237 | 0.283 |
|  | HSP | Slip rate ≤ 8 mm/yr or Mmax ≤ 7.0 | 1.13 | 15.30 | -0.008 | 0.267 | 0.222 | 0.267 |

VHSP is Very High Seismic Potential, HSP is High Seismic Potential, A0 represents the hazard increase on the fault, dc is the characteristic distance (km) controlling the decay of FAF with distance. The model metrics presented are the mean residual between observed and predicted values, the RMSE (root mean squared error), the MAE (mean absolute error), and the Std (standard deviation of residuals).