Table 1: Definitions used in assessed studies regarding axial measures

| AXIAL MEASUREMENTS The measurement was performed along the vertex normal and were centered on the corneal vertex (the cross point of the vertex normal and anterior corneal surface) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Preoperative anterior chamber depth | ACD (Anterior chamber depth) | AS-OCT 3D-OCT Interferometry | from epithelium | ACD epi | Distance the center of the corneal epithelium to the anterior lens surface.8,11,17–21 |
|  |  |  | from endothelium | ACD endo | Distance from the corneal endothelium to the anterior surface of the lens.8,9,13,19,22–36 |
|  |  |  |  | AQD/AD (aqueous depth) |  |
|  |  |  |  | ASD (anterior surface depth for the crystalline lens) | Distance between the anterior surface of the crystalline lens and the posterior surface of the cornea.37 |
|  |  |  | from endothelium | Lens-referenced ACD3 | distance from the endothelium at the center of the cornea to the anterior pole of the cataractous lens or IOL.5 |
|  |  | UBM | from epithelium | ACD | Measured from the anterior surface of the lens to the corneal epithelium.38 |
|  |  |  | from endothelium | ACD | Distance between the inner surface of the cornea and the anterior surface of the lens at the highest point of the anterior chamber in a direction parallel to the axis.39 |
|  |  |  |  | AD (aqueous depth) | AD measured from the anterior surface of the lens to the corneal endothelium.38 |
|  |  | Scheimpflug | The axial distance from the posterior corneal surface to the lens surface measured at the pupil centre.30,40 |  |  |
|  | ASD (Anterior segment depth) | Interferometry | ACDendo + central corneal thickness + lens thickness.41,42 |  |  |
| Postoperative anterior chamber depth | ALP (Anatomical lens position) | AS-OCT | Distance from the corneal epithelium to the IOL surface.17 |  |  |
|  | ALP (actual lens position) | Inteferometry | ACD postoperative (from the corneal epithelium) + ½ lens thickness postoperatively.42 |  |  |
|  |  | SS-OCT | Distance from the posterior surface of the cornea to the anterior surface of the intraocular lens, or “post-ACD” (postoperative anterior chamber depth).43 |  |  |
|  |  | AS-OCT | Distance from the central corneal epithelium to the midpoint of the anterior and posterior surfaces of the IOL on the visual axis.44 |  |  |
|  | Postoperative IOL position | AS-OCT | Distance between the corneal epithelium to the anterior surface of the IOL plus half of the thickness of the IOL.45 |  |  |
|  | ACDi (Postop. internal ACD) | AS-OCT | Distance between the posterior corneal surface and the anterior IOL surface.9,22 |  |  |
|  | AQDpost | Scheimpflug | Distance from the corneal posterior surface to the anterior surface of the implanted lens.40 |  |  |
|  | ACDe (postop.external ACD) | AS-OCT | Distance between the anterior corneal surface and the middle point of the IOL.9 |  |  |
|  | ELP (effective lens position) | AS-OCT | Distance from cornea vertex to IOL center.29 |  |  |
|  |  | Scheimpflug | Actual distance from corneal posterior surface to the anterior surface of implanted lens.40 |  |  |
|  | ELP/PP-ACD (pseudophakic anterior chamber depth) | AS-OCT | The distance from the central corneal endothelium to the anterior intraocular lens (IOL) surface.46 |  |  |
|  | PLP (predicted lens position) | Scheimpflug | the predicted postoperative distance from the corneal posterior surface to the anterior surface of implanted lens40, it can be calculated preoperatively. |  |  |
|  | FLP (Final lens position) | Interferometry | distance between the corneal epithelium and the anterior surface of the IOL.41 |  |  |

Table 2: Definitions used in assessed studies regarding horizontal measures

| HORIZONTAL MEASURES The horizontal measurements were obtained with the nasal and temporal angles at 0º and 180º, respectively |  |  |  |
| --- | --- | --- | --- |
| Angle to angle | ATA-W (Angle to angle width) | AS-OCT | The linear distance between 180º opposite points corresponding to the deepest point of the iridocorneal angle/angle recess.9,22,25,28,31 |
|  |  |  | AAD (Angle to angle distance): Horizontal width of the anterior chamber from the nasal to the temporal scleral spur.26 |
|  | AAD (Angle to angle diameter) | Scheimpflug | A line joining the scleral spurs. For this measurement, the point of intersection of the iris and posterior corneal surface was defined as scleral spur.30 |
|  | ACW (Anterior chamber width) | AS-OCT | The linear distance between 180º opposite points located at the peak of the scleral spur.13,24,25,27,32,33,35 |
|  | SSW (Scleral spur width) | AS-OCT | The horizontal scleral spur-to-spur distance.29 |

Table 3: Definitions used in assessed studies regarding combined axial and horizontal measures

| COMBINED AXIAL AND HORIZONTAL MEASUREMENTS |  |  |  |
| --- | --- | --- | --- |
| Lens rise | LV (Lens vault) | UBM | Perpendicular distance between the line joining the two scleral spurs and the highest point of the anterior surface of the lens.38,39 |
|  |  | AS-OCT | Perpendicular distance between the anterior lens surface to the horizontal line connecting the two scleral spurs.13,23,24,27,28,28,29,32–34,36,47–50 |
|  |  |  | Distance between the anterior lens surface and the lens equator quantifying the extent of the anterior surface’s bulging from the lens equator.10 |
|  | CLR (Crystalline lens rise) / | AS-OCT | Perpendicular anteroposterior distance between the anterior crystalline lens surface and the angle recess-to-angle recess line.5,8,25,26 |
|  | LR-ATA (Lens Rise from Angle to Angle) | AS-OCT | Perpendicular anteroposterior distance between the anterior crystalline lens surface and the angle recess-to-angle recess line.45 |
|  | LR-EQ (Lens rise from the equatorial plane (LR-EQ) | AS-OCT | distance between the point where the ray beam intersects with the anterior surface of the crystalline line and the equatorial plane.45 |
|  | aLT (anterior part of crystalline lens) | AS-OCT 3D-OCT | The anterior part of crystalline lens.18,19 |
|  | Anterior LT | AS-OCT | From the anterior surface to the central clear zone of the crystalline lens.22 |
|  | EPP (Equatorial Plane Position) | OCT-3D: | Distance between the plane of the lens equator and anterior capsule.11 |
| From cornea to equator | LMP (Lens meridian position/parameter) | AS-OCT | Distance from the corneal epithelium of the central cornea to the lens equator, which is reported to be the plane that connects the surfaces of the anterior and posterior capsule.17,18 |
|  |  | OCT-3D: | Distance from the center of the anterior cornea to the intersection of the anterior and posterior lens (equator or meridian of the lens).11,19 |
|  | LEP (Lens equatorial plane) | AS-OCT | distance from the corneal epithelium to the equator of the crystalline lens (the line crosses the intersections between the curvatures of the anterior and posterior surfaces of the crystalline lens).10,45 |
|  | preLP | AS-OCT | The preoperative lens position (preLP) was defined as the vertical distance from the central corneal epithelium to the equatorial plane of the lens.10 |
|  | LED (Lens equator depth) | AS-OCT | Distance from the posterior surface of the cornea to the equatorial crystalline lens width (ELW).22 |
|  | ESD (equatorial surface depth) | AS-OCT | Distance between the line (on the intersection point of the ACR and PCR) and the posterior surface of the cornea.37 |
| Angle to angle depth | SSD (Scleral spur distance) | AS-OCT | Perpendicular distance between the posterior corneal surface and a line drawn between the scleral spur on nasal and temporal sides of the horizontal AS-OCT scans.29 |
|  | AV (Anterior vault) | AS-OCT | The maximum perpendicular distance between the posterior corneal surface to the horizontal line connecting the two scleral spurs.13 |
|  |  |  | Is the sum of the lens vault and ACD in mm.32,36,38 |
|  | ATA-D (Angle to angle depth) | AS-OCT | Perpendicular distance between the posterior corneal surface and a line drawn between anterior chamber angle recess on nasal and temporal sides of the horizontal AS-OCT scans.9,22,31 |
|  | ACD virtual | AS-OCT | Distance between the corneal endothelium and a virtual line joining the horizontal angle-angle (calculated by adding the ACD from the endothelium and the CLR).8,26 |
|  | Angle-referenced ACD (ACD1) | AS-OCT | Distance from the endothelium to the center of the line joining the 2 iridocorneal angles.5 |

Table 4: Changes in measures pre to post operative (Cataract/Lensectomy surgery)

| Author, year |  | ACDpre Mean mm (SD) | ACD post Mean mm (SD) | Pre ATAd /ACDv LMP/SSD/EID | post ATAd/ACDv/SSD/EID | ELP / PostIOLP |
| --- | --- | --- | --- | --- | --- | --- |
| Romero45, 2026 |  | 3.17 ± 0.42 (2.19, 5.12) |  | 4.99 ± 0.27 (4.43, 5,73) CLEP |  | 5.11 ± 0.25 (4.39, 5.81) |
| Mita22, 2024 |  | 2.82 ± 0.34 | 4.06 ± 0.23 | 3.31 ± 0.21 ATAd |  |  |
| Gouvea17, 2023 |  | 3.33 ± 0.41 (NR) | 4.85 ± 0.37 (NR) ALP | 5.04 ± 0.35 (NR) LMP |  |  |
| Yoo51, 2022. | AL≤23mm <23-24.5] <24.5-26] AL>26mm | 2.86 ± 0.34 3.17 ± 0.33 3.43 ± 0.31 3.64 ± 0.30 |  |  |  | 4.75 ± 0.40 5.02 ± 0.39 5.38 ± 0.48 6.25 ± 0.76 |
| Allam38, 2022. |  | 3.27±0.37 (2.4–3.87) 3.28±0.34 (2.44-3.77) (UBM) | 4.02±0.31 (3.46-4.46) | 3.50±0.23 (2.92-3.75) AV |  |  |
| Wu29, 2021. |  | 1.75 ± 0.31 | 3.45 ± 0.27 | 2.82 ± 0.22 SSD | 2.88 ± 0.22 SSD | 4.43 ± 0.26 |
| Zhang40, 2021. |  | 2.69 ± 0.40 (1.75, 3.75) |  |  |  | 3.93 ± 0.29 (3.05, 4.72) PLP 3.96 ± 0.34 (3.19, 4.77) ELP |
| Yoo18, 2020 |  | 3.30 ± 0.40 (2.1, 4.3) |  | 4.82 ± 0.32 (4.1, 5.5) LMP |  |  |
| Castro-Alonso41, 2019 |  | 2.54 ± 0.42 (1.58, 3.90) | 4.01 ± 0.32 (3.25, 4.75) |  |  | FLP 4.55 ± 0.32 (3.77, 5.30) |
| Satou37, 2019 |  | 2.79 ± 0.38 (1.75–3.80) |  | 4.24 ± 0.32 (3.65–4.91) ESD |  | 4.04 ± 0.28 (3.42–4.87) aIOL |
| Tamaoki31, 2019 |  | 2.62 ± 0.43 (AQD) |  | 3.32 ± 0.20 ATA-d |  |  |
| Haddad11, 2019 |  | 3.46 ± 0.38 (2.40–4.60) |  | 5.11 ± 0.32 (3.80–6.10) LMP |  |  |
| Yoo et al.19, 2019 |  | 3.40 ± 0.49 (2.1 5.0) ACD 2.84 ± 0.48 (1.5,4.5) AD |  | 4.90 ± 0.39 (4.0, 6.7) LMP |  |  |
| Plat42, 2017 |  |  |  |  |  | 5.0 ± 0.31 |
| Hsia32, 2017 | Narrow A. Open A. | 2.49 ± 0.41 2.73 ± 0.43 |  | 3.14 ± 0.25 (AV) 3.17 ± 0.25 (AV) |  |  |
| Tsunehiro52, 2017 |  | 3.12 ± 0.37 | 3.42 ± 0.38 |  |  | 5.21 ± 0.23 |
| Goto9, 2016 |  | 2.64 ± 0.43 (1.65 3.58) | 4.15 ± 0.29 (3.41, 4.76) | 3.24 ± 0.25 (2.55, 3.79) ATAd | 3.28 ±0.23 (2.55, 3.78) ATAd | 5.05 (median) (4.99, 5.08) ACDe |
| Moghimi13, 2015 | All Narrow Open | 2.50±0.51 (1.53 to 3.58) 2.15±0.33 2.85± 0.40 |  | 3.05±0.20 (2.66 to 3.50) AV 2.96±0.17 3.14± 0.20 AV |  |  |
| Kucumen5, 2008 |  | 2.54 ± 0.46 | 3.91 ± 0.25 | 3.06 ± 0.25 arACD | 3.16 ± 0.19 arACD |  |

ELP = effective lens position

CLEP: Crystalline Lens Equatorial Plane	ATAd: Angle to angle depth		LMP: lens meridian position

AV: anterior vault		SSD: scleral spur distance	PLP: predicted lens position	ELP: Effective lens position

ESD: Equatorial surface depth	aIOL: Anterior IOL surface position		arACD: angle-referenced anterior chamber depth

Table 5: Standardization of biometric parameters in anterior segment

| TERM | STANDARDIZED TERM | OTHER TERMS | DEFINITION |
| --- | --- | --- | --- |
| AXIAL MEASUREMENTS |  |  |  |
| Preoperative anterior chamber depth | ACD | ACD endo, AQD, AD, ASD, Lens-referenced ACD | Distance from the corneal endothelium to the anterior surface of the lens |
| Postoperative anterior chamber depth | ALP (Actual lens position) | AnLP, ACDi, AQD post, ELP, PP-ACD, PLP | Distance between the posterior corneal surface and the anterior IOL surface |
| HORIZONTAL MEASUREMENTS |  |  |  |
| Spur to spur width | SS-W | ACW, AAD | The horizontal scleral spur-to-spur distance |
| Angle to angle width | ATA-W |  | The linear distance between 180º opposite points corresponding to the deepest point of the iridocorneal angle/angle recess |
| COMBINED AXIAL AND HORIZONTAL MEASUREMENTS |  |  |  |
| Lens Vault | LV |  | The perpendicular distance between the anterior lens surface to the horizontal line connecting the 2 scleral spurs. |
| Crystalline lens rise | CLR | LR-ATA | Perpendicular anteroposterior distance between the anterior crystalline lens surface and the angle recess–to–angle recess line |
| Lens rise from the equatorial plane | LR-EQ | aLT, EPP | distance between the point where the ray beam intersects with the anterior surface of the crystalline line and the equatorial plane |
| Lens equatorial plane | LEP | LMP, preLP | Distance from the corneal epithelium of the central cornea to the lens equator |
| Lens equator depth | LED |  | Distance from the corneal endothelium of the central cornea to the lens equator. |
| Anterior Vault | AV | SSD | Perpendicular distance between the posterior corneal surface to the horizontal line connecting the two scleral spurs. |
| Virtual ACD | ACDv | ATA-depth, Angle-referenced ACD | Distance from endothelium to angle/ angle recess. |

Table 6: Measures that estimate the ELP

| Author | Biometric variables | Predictive model |
| --- | --- | --- |
| Romero45, 2026 | LEP | AL + K + ACD + LT + WTW + LR-EQ |
| Chang43, 2024 | ACD, LT, AL | ALP = 0.084 + 0.041*AL + 0.705*ACD + 0.293LT |
| Allam38, 2022 |  | lens position (LP) = AC + 1/2 LT |
| Yoo51, 2022 | AL, ACD | ELP = -2.123 + 0.288*AL+0.107*ACD (Haigis formula) |
| Wu29, 2021 | AL, SSD, SSW | post-ACD = 0.192 + 0.867 × SSD + 0.163 × AL - 0.253 × SSW. |
| Zhang40 2021 | AQD, LT | ELP = 0.66 + 0.63*(AQD + 0.6LT) |
| Chui2, 2020 | ACD, LT | ELP = ACD + ½LT |
| Yoo18, 2020 |  | ELP = -1.143 + 0.148*AL + 0.428*LMP + 0.254*pLT |
| Yoo19, 2019 |  | LMP = 1.583 + 0.087*AL + 0.282*LMP |
| Norrby21, 2017 | AL, ACD | ALP = 3.074 + 0.06524*AL + 0.2957*ACD ALP = 4.415 + 0.3587*ACD |
| Tsunehiro52, 2017 | AL, Anterior Capsule Depth, ED, PCD | IOL central Depth (mm) = 0.018 × AL + 0.275 × ACD + 0.088 × equator + 0.208 × PCD (ACD+LT) + 1.776 (mm). |
| Goto9, 2016 | AL, ACD, ATA Depth | Postoperative ACDi = 0.456 + 0.586*ATA Depth + 0.202*preoperative ACD + 0.052*AL. |

pLT = posterior part of lens thickness 	LMP = lens meridian parameter

ASD: anterior surface depth			ESD: equatorial surface depth