Table S1. ISCEV Standard Ganzfeld ERG values for proband and normal reference range.

| Right Eye |  |  | Left Eye |  | Reference Range |  |
| --- | --- | --- | --- | --- | --- | --- |
| Amplitude (µV) | Latency (ms) | Step | Amplitude (µV) | Latency (ms) | Amplitude (µV, 5th – 95th %ile) | Latency (ms, 5th – 95th %ile) |
| 108.4 | 92.5 | DA 0.01 b wave | 47.4 | 96 | 116 – 268 | 76 – 106.5 |
| a-wave -145.7 b-wave 149.1 | 21.5 45 | DA 3.0 | a-wave -79.5 b-wave 84.7 | 16 44 | 100 – 229 168 – 381 | 14.5 – 17 46.5 – 59.5 |
| a-wave -192.2 b-wave 130.5 | 14.5 54 | DA 10.0 | a-wave -98.2 b-wave 88.3 | 14 56.5 | 125 - 279 192 – 389 | 11 – 15 47 – 59.5 |
| a-wave -33.5 b-wave 88.6 | 15 31 | LA 3.0 2Hz | a-wave -20.0 b-wave 59.2 | 15 30.5 | 12 – 39 64 – 175 | 13 – 15.5 27 – 31.5 |
| 65.1 | 28 | LA 3.0 30Hz | 46.1 | 27 | 43 – 143 | 24 – 30 |

DA = dark adapted. LA = light adapted.

NB: there is an interocular difference in amplitude for all stimulus conditions with the left eye having smaller responses, for DA 51 – 68% and LA 60 – 71%, respectively.

Table S2. b:a wave ratios.

| Right Eye | Stimulus | Left Eye | Reference Range Mean ±SD |
| --- | --- | --- | --- |
| 1.02 | DA 3.0 | 1.07 | 1.83 ±0.30 |
| 0.68 | DA 10.0 | 0.90 | 1.60 ±0.23 |
| 2.66 | LA 3.0 2Hz | 2.97 | 4.34 ±0.70 |

DA = dark adapted. LA = light adapted. SD = standard deviation. All stimuli are in cds/m2. Reference ranges taken from Bhatti et al. PMID: 28434717.

Table S3. Pattern ERG values for the proband and normal reference range.

| Right Eye |  | Waveform | Left Eye |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- |
| Amplitude (µV) | Latency (ms) |  | Amplitude (µV) | Latency (ms) | Amplitude (µV, 5th – 95th %ile) | Latency (ms, 5th – 95th %ile) |
| 1.35 | 45 | P50 | 1.12 | 48 | 1.92 – 5.36 | 45 – 54 |
| -2.52 | 91 | N95 | -2.08 | 90 | 3.59 – 8.03 | 80 - 110 |
| 1.87 |  | N95:P50 Ratio | 1.86 |  | 1.50 –1.87 |  |