##### Supplementary material

Datasets used in the study are available online in the repository archived in Zenodo.org (https://doi.org/10.5281/zenodo.5069750).

The repository contains a Jupyter notebook with the calculations and plots of the loss components. Furthermore, repository includes i) all the radargrams, ii) the processed sonar and GPR observations in text format and iii) the plots comparing bathymetry retrieved with the different instruments.

##### Appendix A

A total loss coefficient was computed for both the water-coupled GPR and drone-borne GPR. The values of the variables used for computing the different losses are shown in Table A1. Some of these variables are sensitive to temperature. For example, εwater varies significantly with water temperature from ≈80 at 20° to ≈87.7 at 0°  (Andryieuski et al., 2015; Malmberg and Maryott, 1956); similarly, a 2% increase of σwater per 1° C increase of water temperature typically occurs (Hayashi, 2004). The variables εwater and ε’’water also depend on the angular frequency of the GPR wave (Andryieuski et al., 2015); however,  for frequencies below 1 GHz (as for the GPRs used for bathymetry),  the frequency effect on εwater is not significant.

Table A1, Variables used for computing the different losses. Some of these variables depend on the variable ω, which refers to angular frequency of GPR waves, and T, which refers to temperature.

| Variable description | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Real component of relative permittivity of water | εwater (ω, Twater) | 81 | (-) |
| Real component of relative permittivity of air | εair (ω, Twater) | 1 | (-) |
| Permittivity of free space | ε0 | 8.8541· 10−12 | F⋅m−1 |
| Imaginary component of relative permittivity of water | ε’’water (ω,Twater) | 0.25 | (-) |
| Real component of relative permittivity of sediment (sand) | εsediment | 30 | (-) |
| Static conductivity of water | σwater (Twater) | 0.034 (e.g. Furesø) | Sm−1 |
| Absolute magnetic permeability (Water, air, nonmagnetic soil) | µ0≈µwater | ≈1.26·10-6 | H m−1 |
| Intrinsic impedance water | ηwater (ω,Twater) | 41.91 | Ohm |
| GPR frequency in air | fair | 80 | MHz |
| Flight altitude above water | h | 1 | m |

In the Supplementary material, an example of calculations of the different loss coefficients is documented as a Jupyter Notebook. For example, using the variable values from Table A1, the losses of Table A2 were computed.

Table A2, loss coefficients for each study area, for water-coupled or drone-borne GPR antenna. All units in Decibels.

| Variable description | Symbol | Computed values |
| --- | --- | --- |
| transmission loss | αtrans | 8.87 |
| propagation loss | αp | 12.70 |
| reflection loss | αr | 12.27 |
| spreading loss drone-borne | αspread, Drone-borne | 7.35 |
| spreading loss water coupled | αspread, Water-coupled | 3.87 |
| Total loss drone-borne | αtotal, Drone-borne | 41.2 |
| Total loss water coupled | αtotal, Water-coupled | 28.85 |