APPENDICES

Step-by-step counselling guide in using a pressurised metered-dose inhaler and facemask chamber17:

Improvised Counselling Check-list

MEDICATION DELIVERY DEVICES Inhaled medication using an inhaler device is the preferred route of delivery Direct and faster delivery of the medication into the airways The likelihood of systemic side effects is also reduced

DIFFERENTIATE THE RELIEVER AND CONTROLLER MEDICINES Reverse the airway bronchoconstriction RELIEVER MEDICINE To relieve symptoms of shortness of breath & wheezing To bring reliever medicine everywhere they go CONTROLLER MEDICINE Reduce inflammation in the airways Must be used regularly as directed by the doctor

AEROHAMBER WITH FACEMASK Visually check the chamber to ensure no foreign objects and all parts are intact Remove the cap of the pMDI and shake 3-5 times before use Attach the mouthpiece of the pMDI to the back of the facemask chamber Enclose the mask around the face and ensure a good seal Conventional method Hold tightly on to the pMDI and the back of the valved holding chamber Press the facemask gently against the child to ensure a good seal The other hand supports the child’s head to prevent it from moving Alternative method: Palm-seal method Place your arm around the back of the child’s neck Hold and grip the lower part of the facemask using the thumb, index and middle fingers while the palm, ring and little fingers support the chin and cheeks of the child to prevent the facemask from dislodging when the child’s head moves. On the other hand, be ready to press the pMDI. Press the pMDI once, allow the child to breathe normally for 8-10 breaths while holding the mask firmly. If additional puff is required, wait for 30-60 seconds before administering the second puff. Repeat the same steps as above. Wipe the mouth and offer some fluids to the child after using an inhaled corticosteroid.

CLEANING THE FACEMASK CHAMBER Clean the chamber and facemask every 1-2 weeks by submerging the parts in clean water. Shake out excess water and air-dry in a vertical position. Do not wipe to dry. Centre the alignment feature on the back piece with the flow indicator.