Supplementary Material

# Supplementary Table S1: Factors for calculating second-stage sensitivity in alternative emissions

- LNG combustion factors (1) EFLNG, mass CO2 per tonne LNG= CVLNG(MJkg)×EFgas (kg CO2/GJ)1000 = CVLNG×561000 With, CVLNG is the calorific value of LNG and EFgas is the IPCC Tier-1 gas combustion coefficient (56 kg CO₂ GJ⁻¹) (Environment Protection Authority, 2024; IPCC, 2006).

- LNG combustion (2) LNG=QWA,23/24LNG×EFLNG, mass With QWA,23/24LNG is the LNG conversion quantity and EFcombustionLNG is the LNG combustion factor.

- Iron-ore to steel conversion (WA) (3) Ore=QWA,23/24ore×AFore-to-steel With QWA,23/24ore is the iron ore to steel production and AFore-to-steel is the allocation factor (AF).

# Supplementary Table S2: Calculation of Scope 1,2, and 3 emissions of Western Australia (WA) in 2023-24; see methodology for accounting formulas

| Emission sources | Calculation |
| --- | --- |
| Resource industry | Scope1WA+ Scope2WA=39 MtCO₂-e |
| Iron ore to steel production QWA,23/24ore = 880 Mt (iron ore export) and AFore-to-steel = 1.14 tCO₂/t ore (allocation factor) | Iron-ore,downstreamWA= 880×1.14=1,003.2 MtCO2-e |
| LNG combustion of 110 Mt (QWA,23/24LNG) with a combustion factor of 2.688 tCO₂/t (EFLNG, mass) | LNG,downstreamWA=110×2.688=295.68 MtCO₂-e |
| Other minerals and processing Alumina: 11.4 at 1.33 tCO₂/t emission factor (EF) Gold: 0.203 Mt at 25.5 tCO₂/t EF Nickel: 0.16 Mt at 15 tCO₂/t EF Copper: 0.085 Mt at 4.0 tCO₂/t EF Zinc/Lead: 0.12 Mt combined at 3.5 tCO₂/t EF Mineral sands (Ilmenite, rutile, zircon): 3.5 Mt at 0.3 tCO₂/t EF Lithium (spodumene): 4 Mt at 0.25 tCO₂/t EF Rare earth oxides: 0.065 Mt at 6.0 tCO₂/t EF | ResidualWA=∑QWA,23/24c,residual×EFc=25.9 MtCO₂-e |
| Total value chain (Scope 3) | Scope3WA=1,003.2+295.68+25.9=1,324.78 MtCO₂-e |
| Combined mining footprint (Scope 1, 2, 3) | Scope1,2,3WA=39+1,324.78=1,363.78 MtCO₂-e |

# Supplementary Table S3: Calculation of Scope 1,2, and 3 emissions of Queensland (Qld) in 2023-24

| Emission sources | Calculation |
| --- | --- |
| Resource industry | Scope1Qld+ Scope2Qld=33 MtCO₂-e |
| Downstream coal Qcoal,23/24Qld = 270 (exportable coal production) and EFcoal= 2.86 tCO₂/t (conservative combustion emission factor) | Coal,downstreamQld=270×2.86=772.2 MtCO₂-e |
| LNG exports of 25 Mt (QLNG,23/24Qld) with a combustion factor of 2.688 tCO₂/t (EFLNG, mass) | LNG,downstreamQld=25×2.688=67.2 MtCO2-e |
| Other minerals and processing Alumina: 7 Mt at 1.33 tCO₂/t EF. Gold: 0.04 Mt at 25.5 tCO₂/t EF Nickel: 0.01 Mt at 15 tCO₂/t EF Copper: 0.2 Mt at 4 tCO₂/t EF Zinc/Lead: 0.3 Mt at 3.5 MtCO₂/t EF Mineral sands: 0.5 Mt at 0.3 MtCO₂/t EF Lithium (spodumene): 0.2 Mt at 0.25 MtCO₂/t EF Rare earths oxides: 0.01 Mt at 6 MtCO₂/t EF | ResidualQld=∑QQld,2324c,residual×EFc=12.59 MtCO₂-e |
| Total value chain (Scope 3) | Scope3Qld=772.2+67.2+12.59=851.99 MtCO₂-e |
| Combined mining footprint (Scope 1, 2, 3) | Scope1,2,3Qld=33+851.99 =884.99 MtCO₂-e |

# Supplementary Table S4: Alternative emissions of WA based on changes in emission factors, lower heating values, and allocation factors

| Ore AF (t CO₂/t) | Ore Scope 3 (Mt) | LNG LHV (MJ/kg) | LNG Scope 3 (Mt) | Other (Mt) | Scope 3 (Mt) (ore+ LNG+ other) | Scope 1+2+3 |
| --- | --- | --- | --- | --- | --- | --- |
| 0.60 | 528.00 | 45 (2.52 tCO₂/t) | 277.20 | 25.90 | 831.10 | 870.10 |
| 0.60 | 528.00 | 48 (2.688 tCO₂/t) | 295.68 |  | 849.58 | 888.58 |
| 0.60 | 528.00 | 50 (2.8 tCO₂/t) | 308.00 |  | 861.90 | 900.90 |
| 1.14 | 1003.20 | 45 (2.52 tCO₂/t) | 277.20 |  | 1,306.30 | 1,345.30 |
| 1.14 | 1003.20 | 48 (2.688 tCO₂/t) | 295.68 |  | 1,324.78 | 1,363.78 |
| 1.14 | 1003.20 | 50 (2.8 tCO₂/t) | 308.00 |  | 1,337.10 | 1,376.10 |

# Supplementary Table S5: Alternative emissions of Qld based on changes in emission factors, lower heating values, and allocation factors

| Coal EF (t CO₂/t) | Coal Scope 3 (Mt) | LNG LHV (MJ/kg) | LNG Scope 3 (Mt) | Other (Mt) | Scope 3 (Mt) (coal+ LNG+ other) | Scope 1+2+3 |
| --- | --- | --- | --- | --- | --- | --- |
| 2.40 | 648.00 | 45 (2.52 tCO₂/t) | 63.00 | 12.59 | 723.59 | 756.59 |
| 2.40 | 648.00 | 48 (2.688 tCO₂/t) | 67.20 |  | 727.79 | 760.79 |
| 2.40 | 648.00 | 50 (2.8 tCO₂/t) | 70.00 |  | 730.59 | 763.59 |
| 2.86 | 772.20 | 45 (2.52 tCO₂/t) | 63.00 |  | 847.79 | 880.79 |
| 2.86 | 772.20 | 48 (2.688 tCO₂/t) | 67.20 |  | 851.99 | 884.99 |
| 2.86 | 772.20 | 50 (2.8 tCO₂/t) | 70.00 |  | 854.79 | 887.79 |

# References

Environment Protection Authority (EPA) 2024, Greenhouse Gas Emissions, viewed 28 September 2025, <https://www.epa.wa.gov.au/sites/default/files/Policies_and_Guidance/Guideline%20%E2%80%93%20GHG%20Emissions%20-%20November%202024.pdf>.

Intergovernmental Panel on Climate Change (IPCC) 2006, Chapter 2: Stationary Combustion - IPCC Guidelines for National Greenhouse Gas Inventories, viewed 30 September 2025, <https://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/2_Volume2/V2_2_Ch2_Stationary_Combustion.pdf>.