Supporting Information for

Prediction and modeling of air quality factor concentrations based on machine learning in mining cities in China

Xiaolong Lia1, b, Zhihui Huanga1, Jinxiang Yanga, *, Yongsheng Chena, Yu Yuna, Jian Langa, Xiang Liua, Shiwen Zhanga, Huijun Wua, b

a School of Earth and Environment, Anhui University of Science and Technology, Huainan 232001, PR China.

b State Key Laboratory of Pollution Control and Resources Reuse, School of the Environment, Nanjing University, Nanjing 210023, PR China.

*Corresponding author:

Dr. Jinxiang Yang, Fax: +86-554-6668430; E-mail: yangjx2027@163.com

# 1. Study area

It is one of China’s 14 billion-ton coal production bases and six major coal power bases. Huainan has built 18 pairs of large mines and 6 large power plants. The smoke and pollutants released during coal mining, transportation and combustion are the main reasons that affect air quality. In addition, the volatilization and combustion of coal gangue will produce a large amount of particulate matter and atmospheric pollutants such as sulfur dioxide. It has a semi-humid climate (see Fig. S1), with a total area of 5571 km². Recoverable reserves of 15.3 billion tons, It is one of China’s important coal energy cities (Tong et al., 2020), 74% of the reserves in Anhui Province.

# 2. Air quality evaluation

According to the Technical Provisions on Ambient Air Quality Index (AQI) (Trial) (HJ 633-2012), the air quality sub-index for pollutant item P is calculated according to the formula (S1):

（S1）

Where: IAQIP—air quality index of pollutant P; CP—mass concentration value of pollutant P; BPHi—high value of the pollutant concentration limit in Tab. S2 close to CP; BPLo—close to CP in Tab. S2 IAQIHi—the air quality sub-index corresponding to BPHi in Tab. S2; IAQILo—the air quality sub-index corresponding to BPLo in Tab. S2. Then, the air quality index is calculated according to formula (S2):

（S2）

Where: IAQI—air quality sub-index, n—pollutant item.

# 3. Random Forest Model

Generally, mtry can default to the square root of the number of variables in the data set (classification model) or one-third (predictive model), as shown in equations (S3) and (S4):

（S3）

（S4）

Where M represents the number of features in the original data set. The initial value of ntree is usually defined as 1, and the default value is defined as 500. In this study, we tried to gradually increase from 1 to 500 to select the appropriate value, and determine the best parameters through the out-of-bag error rate (OOB) estimation method. The OOB error is usually regarded as an unbiased estimate of the generalization error, and its calculation method is as shown in equation (S5):

(S5)

where n is the number of OOB samples and  and  represent the observed and predicted values of sample i, respectively (Meng et al., 2018).

# 4. Support Vector Machine Model

Its basic ideas are summarized as follows (Aryafar et al., 2012; Liu et al., 2016):

First consider a training data set (xi, yi), xi∈Rn, i=1, 2...m, y∈{+1,-1}, where yi represents the known target attribute of the i-th case. In the learning phase, the goal is to find a linear function f(x) = wx+b, w, x∈Rn, b∈R, for which the difference between the actual measured value yi and the estimated value f(xi) of this linear function is at most equal to ε or [yi - f(xi)] < ε, where ε is the insensitive loss function, and an error below it means that it will not be punished. If the estimation error is considered by introducing the slack variables ζ and ζ* and the penalty factor C, the corresponding problem can be expressed as equation (S6):

(S6)

The constraint condition is formula (S7):

(S7)

The quadratic programming problem is transformed into its corresponding dual optimization problem, and the kernel function is introduced to realize nonlinearity, and the optimal regression function is obtained as formula (S8):

(S8)

Where C ≥ 𝛼𝑖, 𝛼𝑖∗ ≥ 0, i = 1,2….n. 𝛼𝑖 and 𝛼𝑖∗ (0 ≤ 𝛼𝑖, 𝛼𝑖∗ ≥ C) are Lagrangian multipliers, and K(xi,x) represents the kernel function. The generalization performance of SVM depends on C, ε, kernel type and corresponding kernel parameter settings. The kernel type is RBF (Equation (S9)):

(S9)

Among them γ is the kernel parameter, and xi and xj are two independent variables.

# 5. Index calculations

The specific formulas are shown in (S10)~(S13) ). R is used to characterize the degree of fit between the predicted value of the model and the actual value, MAE and RMSE are used to characterize the difference between the predicted value of the model and the actual value, and the consistency index IA is used to test the difference between the measured value and the simulated predicted value. Generally, the closer the R value is to 1, the closer the MAE value and the RMSE value are to 0, and the closer the IA index is to 1, the higher the similarity and the better the model simulation effect (Shepard, 1962; Kruskal, 1964).

(S10)

(S11)

(S12)

(S13)

Where n denotes the number of data,  denotes the predicted value of the air pollutant,  denotes the measured value of the air pollutant,  denotes the average of the predicted value of the air pollutant, and denotes the average of the measured values of air pollutants.

Fig. S1 Location of study area and meteorological monitoring sites.

Fig. S2 Characterization of meteorological factors as a function of month. (a), (b), (c), (d) and (e) are the variation of temperature, wind speed, humidity, air pressure and visibility with month, respectively.

Fig. S3 Characterization of the spatial distribution of air quality factor concentrations.

Fig. S4 Spatial and temporal distribution of air quality index (AQI).

Fig. S5 Training values simulation results of air quality factor concentrations by RF model. (a), (b), (c), (d), (e) and (f) are the simulation results of PM2.5, PM10, NO2, SO2, CO, O3 respectively.

Fig. S6 Test values simulation results of air quality factor concentrations by RF model. (a), (b), (c), (d), (e) and (f) are the simulation results of PM2.5, PM10, NO2, SO2, CO, O3 respectively.

Fig. S7 The residual error diagram of the simulation results of air quality factor concentrations by RF model.

Fig. S8 Training values simulation results of air quality factor concentrations by SVM model. (a), (b), (c), (d), (e) and (f) are the simulation results of PM2.5, PM10, NO2, SO2, CO, O3 respectively.

Fig. S9 Verification values simulation results of air quality factor concentrations by SVM model. (a), (b), (c), (d), (e) and (f) are the simulation results of PM2.5, PM10, NO2, SO2, CO, O3 respectively.

Fig. S10 The residual error diagram of the simulation results of air quality factor concentrations by SVM model.

Fig. S11 Line chart of the simulation accuracy of the RF model for air quality factor concentrations for different sample sizes.

Fig. S12 Line chart of the simulation accuracy of the SVM model for air quality factor concentrations for different sample sizes.

Tab. S1 Statistical table of the location of automatic monitoring points in Huainan City.

| Monitoring point | Name | North latitude | East longitude | Monitoring items | District (County) | Type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Panji district government | 116.8294° | 32.7737° | Air mass concentration | Panji District | National controlling point |
| 2 | Normal college | 117.0046° | 32.6448° | Air mass concentration | Tianjia 'an District | National controlling point |
| 3 | Xiejiaji district government | 116.8548° | 32.6014° | Air mass concentration | Xiejiaji District | National controlling point |
| 4 | Bagongshan government | 116.8278° | 32.6331° | Air mass concentration | Bagongshan District | National controlling point |
| 5 | Jiaogang Lake Scenic Area Management Office | 116.6328° | 32.6201° | Air mass concentration | Maoji experimental District | National controlling point |
| 6 | Yiyi Dairy Industrial Park | 117.0598° | 32.6469° | Air mass concentration | Datong District | National controlling point |
| 7 | Youth Activity Center of Fengtai County | 116.7019° | 32.7116° | Air mass concentration | Fengtai County | Provincial controlling point |
| 8 | Shou County old environmental protection building | 116.7821° | 32.5797° | Air mass concentration | Shou County | Provincial controlling point |

Tab. S2 Air quality sub-index and the concentration limit of the corresponding pollutant items.

| IAQI | Project concentration limit of pollutants |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PM2.5 | PM10 | NO2 | SO2 | CO | O3 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 50 | 35 | 50 | 40 | 50 | 2 | 100 |
| 100 | 75 | 150 | 80 | 150 | 4 | 160 |
| 150 | 115 | 250 | 180 | 475 | 14 | 215 |
| 200 | 150 | 350 | 280 | 800 | 24 | 265 |
| 300 | 250 | 420 | 565 | 1600 | 36 | 800 |
| 400 | 350 | 500 | 750 | 2100 | 48 | * |
| 500 | 500 | 600 | 940 | 2620 | 60 | * |

Tab. S3 Characterization of air quality factor concentrations (μg/m3) as a function of month.

| Month |  | PM2.5 | PM10 | NO2 | SO2 | CO | O3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Jan. | Range | 35.00-170.0 | 53.0-207.0 | 19.00-60.00 | 9.000-22.00 | 0.493-1.510 | 14.00-112.0 |
|  | Mean ± SD | 100.836.96 | 136.442.75 | 42.589.653 | 15.193.156 | 0.9400.278 | 51.0022.37 |
| Feb. | Range | 25.00-159.0 | 40.00-210.0 | 14.00-51.00 | 9.000-22.00 | 0.406-1.347 | 42.00-162.0 |
|  | Mean ± SD | 81.0035.38 | 107.348.61 | 25.508.946 | 12.392.699 | 0.8190.266 | 73.2926.52 |
| Mar. | Range | 32.00-133.0 | 60.00-189.0 | 22.00-56.00 | 9.000-26.00 | 0.333-1.184 | 69.00-142.0 |
|  | Mean ± SD | 69.0027.60 | 122.233.02 | 33.197.795 | 16.324.190 | 0.6550.193 | 106.019.22 |
| Apr. | Range | 27.00-106.0 | 46.00-221.0 | 15.00-37.00 | 6.000-18.00 | 0.447-1.093 | 42.00-183.0 |
|  | Mean ± SD | 50.6718.31 | 95.2736.15 | 24.675.671 | 11.872.751 | 0.6500.139 | 116.736.49 |
| May. | Range | 15.00-84.00 | 39.00-193.0 | 16.00-32.00 | 9.000-24.00 | 0.337-0.917 | 93.00-201.0 |
|  | Mean ± SD | 37.1314.79 | 87.6827.79 | 22.354.317 | 12.874.039 | 0.5640.151 | 142.929.72 |
| Jun. | Range | 18.00-50.00 | 36.00-106.0 | 11.00-28.00 | 9.000-15.00 | 0.447-0.873 | 79.00-203.0 |
|  | Mean ± SD | 32.138.131 | 68.7316.93 | 17.003.206 | 11.431.431 | 0.6390.099 | 159.832.99 |
| Jul. | Range | 13.00-45.00 | 37.00-97.00 | 10.00-23.00 | 9.000-15.00 | 0.397-0.838 | 62.00-205.0 |
|  | Mean ± SD | 25.358.393 | 53.7113.65 | 15.653.332 | 11.681.620 | 0.5780.118 | 138.332.54 |
| Aug. | Range | 7.000-45.00 | 22.00-87.00 | 12.00-23.00 | 9.000-17.00 | 0.391-1.004 | 36.00-222.0 |
|  | Mean ± SD | 22.239.312 | 49.9416.44 | 16.812.713 | 12.292.116 | 0.6030.141 | 133.542.01 |
| Sept. | Range | 16.00-65.00 | 37.00-119.0 | 16.00-40.00 | 11.00-22.00 | 0.501-0.990 | 80.00-205.0 |
|  | Mean ± SD | 33.4713.37 | 71.6321.92 | 23.375.750 | 14.802.398 | 0.7170.150 | 135.836.29 |
| Oct. | Range | 17.00-103.0 | 34.00-371.0 | 13.00-44.00 | 9.000-23.00 | 0.348-0.970 | 38.00-192.0 |
|  | Mean ± SD | 51.9721.22 | 111.767.34 | 29.717.963 | 14.483.414 | 0.6480.154 | 105.838.38 |
| Nov. | Range | 27.00-134.0 | 50.00-181.0 | 19.00-56.00 | 8.000-27.00 | 0.326-1.153 | 40.00-156.0 |
|  | Mean ± SD | 64.6727.10 | 123.741.74 | 36.778.460 | 15.404.643 | 0.6590.209 | 84.1033.74 |
| Dec. | Range | 32.00-157.0 | 53.00-219.0 | 32.00-65.00 | 10.00-28.00 | 0.502-1.245 | 9.000-99.00 |
|  | Mean ± SD | 83.5529.92 | 115.937.52 | 48.459.204 | 15.454.610 | 0.8590.195 | 52.8122.59 |

Tab.S4 Concentration (μg/m3) limits of basic items of ambient air pollutants.

| ID | Pollutant project | Average time | Concentration limit |  | Company |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Ⅰ | Ⅱ |  |
| 1 | SO2 | 24-hour average | 50 | 150 | μg/m3 |
| 2 | NO2 | 24-hour average | 80 | 80 | μg/m3 |
| 3 | CO | 24-hour average | 4 | 4 | mg/m3 |
| 4 | O3 | Maximum 8h average | 100 | 160 | μg/m3 |
| 5 | PM10 | 24-hour average | 50 | 150 | μg/m3 |
| 6 | PM2.5 | 24-hour average | 35 | 75 | μg/m3 |

Tab. S5 Spatial distribution table of air factor concentrations (μg/m3).

| Monitoring point |  | PM2.5 | PM10 | NO2 | SO2 | CO | O3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Range | 7.000-183.0 | 16.00-359.0 | 7.000-70.00 | 6.000-37.00 | 0.213-1.450 | 16.00-240.0 |
|  | Mean ± SD | 62.3838.48 | 96.0651.02 | 28.6713.35 | 13.875.529 | 0.6150.228 | 116.647.40 |
| 2 | Range | 8.000-170.0 | 26.00-419.0 | 7.000-74.00 | 4.000-24.00 | 0.261-2.323 | 14.00-261.0 |
|  | Mean ± SD | 49.4831.53 | 105.151.06 | 30.8314.11 | 11.303.994 | 0.7760.264 | 118.951.79 |
| 3 | Range | 5.000-166.0 | 12.00-347.0 | 6.000-72.00 | 2.000-55.00 | 0.292-1.463 | 5.000-243.0 |
|  | Mean ± SD | 51.7733.46 | 88.0547.30 | 27.8714.39 | 13.637.658 | 0.6330.207 | 121.550.23 |
| 4 | Range | 7.000-175.0 | 32.00-599.0 | 7.000-68.00 | 4.000-50.00 | 0.165-0.732 | 11.00-119.2 |
|  | Mean ± SD | 55.4032.56 | 97.6245.17 | 24.8411.37 | 14.885.438 | 0.7320.226 | 119.249.18 |
| 5 | Range | 5.000-157.0 | 11.00-317.0 | 7.000-104.0 | 3.000-30.00 | 0.179-1.532 | 6.000-229.0 |
|  | Mean ± SD | 46.9032.06 | 94.9049.83 | 32.5115.48 | 10.654.434 | 0.6140.211 | 116.147.24 |
| 6 | Range | 7.000-179.0 | 15.00-403.0 | 11.00-68.00 | 5.000-41.00 | 0.207-1.469 | 10.00-303.0 |
|  | Mean ± SD | 51.6432.95 | 87.7646.99 | 28.2613.00 | 14.364.123 | 0.7070.233 | 124.054.86 |
| 7 | Range | 12.00-151.0 | 18.00-276.0 | 3.000-143.0 | 3.000-41.00 | 0.107-4.894 | 13.00-223.0 |
|  | Mean ± SD | 51.7828.65 | 81.8739.63 | 29.8720.51 | 12.726.659 | 0.9000.467 | 98.7948.14 |
| 8 | Range | 5.000-165.0 | 10.00-292.0 | 2.000-95.00 | 1.000-30.00 | 0.030-1.600 | 15.00-231.0 |
|  | Mean ± SD | 47.6830.18 | 85.9945.40 | 28.9917.27 | 7.6304.349 | 0.5270.314 | 107.048.16 |

Tab. S6 Air quality index (AQI) classification standards and related information.

| AQI | Level | Status | Impact |
| --- | --- | --- | --- |
| 0～50 | Ⅰ | Excellent | Basically no air pollution |
| 51～100 | Ⅱ | Good | Some pollutants may have a weak impact on a very small number of highly sensitive people |
| 101～150 | III | Light pollution | The symptoms of susceptible population were mildly aggravated |
| 151～200 | IV | Moderate pollution | Further aggravate the symptoms of susceptible population |
| 201～300 | V | Heavy pollution | The symptoms of patients with heart and lung diseases were significantly increased |
| >300 | VI | Serious pollution | The exercise tolerance of healthy people decreased, and there were obvious strong symptoms |

Tab. S7 Correlation coefficient (R), means absolute error (MAE), root mean square error (RMSE) and index of agreement (IA) of models for simulating air mass factor concentrations.

| Model | Air quality factor | Training |  |  |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | R | MAE | RMSE | IA | R | MAE | RMSE | IA |
| RF | PM2.5 | 0.8334 | 12.7194 | 18.2286 | 0.8819 | 0.8306 | 14.5519 | 20.5328 | 0.8791 |
|  | PM10 | 0.7644 | 20.3378 | 30.1396 | 0.8152 | 0.7347 | 23.0081 | 31.3089 | 0.8135 |
|  | NO2 | 0.8146 | 5.0163 | 6.8218 | 0.8737 | 0.8055 | 5.858 | 8.0656 | 0.8589 |
|  | SO2 | 0.6024 | 2.2131 | 2.9605 | 0.6747 | 0.6148 | 2.0827 | 2.8904 | 0.6967 |
|  | CO | 0.6527 | 0.1189 | 0.1585 | 0.7333 | 0.6728 | 0.1237 | 0.1629 | 0.7635 |
|  | O3 | 0.8280 | 20.4189 | 25.4976 | 0.8892 | 0.8525 | 21.5885 | 26.8286 | 0.8957 |
| SVM | PM2.5 | 0.9370 | 7.3404 | 11.5629 | 0.9618 | 0.8985 | 10.7513 | 16.0926 | 0.9384 |
|  | PM10 | 0.8815 | 12.7067 | 22.1886 | 0.9207 | 0.8197 | 17.0364 | 26.8445 | 0.8886 |
|  | NO2 | 0.8916 | 3.5285 | 5.2669 | 0.9398 | 0.8213 | 5.6421 | 7.6125 | 0.8945 |
|  | SO2 | 0.6469 | 1.0283 | 1.2617 | 0.7313 | 0.6962 | 0.8519 | 1.0174 | 0.7331 |
|  | CO | 0.8192 | 0.0812 | 0.1212 | 0.8771 | 0.7934 | 0.1025 | 0.1366 | 0.8576 |
|  | O3 | 0.9143 | 16.1610 | 21.2095 | 0.9355 | 0.8884 | 18.9604 | 24.2976 | 0.9252 |

# References

Aryafar, A., Gholami, R., Rooki, R., et al., 2012. Heavy metal pollution assessment using support vector machine in the Shur River, Sarcheshmeh copper mine, Iran. Environ. Earth Sci. 67, 1191-1199.

Kruskal, J., 1964. Multidimensional scaling by optimizing goodness of fit to anonmetric hypothesis. Psychometrika. 29, 1-27.

Liu, Y., Wang, H., Zhang, H., et al., 2016. A comprehensive support vector machinebased classification model for soil quality assessment. Soil Till. Res. 155, 19-26.

Meng, X., Hand, J.L., Schichtel, B.A., et al., 2018. Space-time trends of PM2.5 constituents in the conterminous United States estimated by a machine learning approach, 2005-2015. Environ. Int. 121, 1137-1147.

MEP, 2012. Technical Regulation on Ambient Air Quality Index (on trial) (HJ-663-2012). Ministry of Environment protection of china, Beijing, China.

Shepard, R.N., 1962. The analysis of proximities: Multidimensional scaling with an unknown distance function. II. Psychometrika. 27, 219-246.

Tong, H., Geng, T., Ding, F., 2020. Lidar navigation observation and analysis of a heavy air pollution process in Huainan city. J. Hefei Univ. Tech. 43,1410-1416.