Assessment of an integrated coagulation–DAF and biological treatment system for olive mill wastewater at bench and pilot scales

Supplementary Material

Optimization of the Coagulation Step

The optimization of the coagulation process was conducted using a Central Composite Design (CCD) comprising. Thirty (30) experimental runs, with design coordinates detailed in Table S1.

Table S1. DOE matrix with four design factors with actual values of COD, phenol, and turbidity removal percentages.

| Run | Design Factors Investigated |  |  |  | Responses Removal % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | pH | Dose (mg/L) | Mixing rate (RPM) | Mixing time (Minutes) | COD (%) | Toxic phenol (%) | Turbidity (%) |
| 1 | 12 | 20000 | 50 | 1 | 4.43 | 57.80 | 50.00 |
| 2 | 10 | 20000 | 50 | 5 | 11.45 | 60.82 | 32.79 |
| 3 | 10 | 60000 | 50 | 1 | 12.20 | 29.93 | 51.23 |
| 4 | 10 | 20000 | 50 | 1 | 10.80 | 59.31 | 76.02 |
| 5 | 10 | 60000 | 50 | 5 | 11.66 | 69.11 | 58.20 |
| 6 | 12 | 20000 | 50 | 5 | 11.12 | 60.06 | 56.56 |
| 7 | 11 | 40000 | 50 | 3 | 10.15 | 78.15 | 62.50 |
| 8 | 12 | 60000 | 50 | 1 | 6.59 | 63.83 | 77.60 |
| 9 | 12 | 60000 | 50 | 5 | 16.63 | 66.85 | 64.41 |
| 10 | 11 | 20000 | 150 | 3 | 1.51 | 88.11 | 97.61 |
| 11 | 11 | 40000 | 150 | 3 | 15.87 | 75.13 | 98.02 |
| 12 | 11 | 40000 | 150 | 3 | 18.14 | 53.28 | 98.22 |
| 13 | 11 | 40000 | 150 | 5 | 16.63 | 80.41 | 97.88 |
| 14 | 11 | 60000 | 150 | 3 | 13.93 | 77.40 | 97.95 |
| 15 | 11 | 40000 | 150 | 1 | 13.17 | 84.93 | 96.72 |
| 16 | 12 | 40000 | 150 | 3 | 13.71 | 91.71 | 95.83 |
| 17 | 11 | 40000 | 150 | 3 | 16.63 | 32.94 | 98.43 |
| 18 | 11 | 40000 | 150 | 3 | 20.09 | 86.44 | 98.36 |
| 19 | 11 | 40000 | 150 | 3 | 14.79 | 75.13 | 98.36 |
| 20 | 11 | 40000 | 150 | 3 | 16.95 | 72.12 | 98.29 |
| 21 | 10 | 40000 | 150 | 3 | 20.73 | 79.66 | 98.16 |
| 22 | 12 | 60000 | 250 | 1 | 13.61 | 68.35 | 88.39 |
| 23 | 12 | 20000 | 250 | 1 | 13.07 | 46.50 | 45.63 |
| 24 | 10 | 60000 | 250 | 1 | 15.55 | 76.64 | 98.91 |
| 25 | 12 | 20000 | 250 | 5 | 17.17 | 87.94 | 97.20 |
| 26 | 10 | 20000 | 250 | 1 | 15.98 | 75.89 | 98.77 |
| 27 | 11 | 40000 | 250 | 3 | 19.11 | 71.37 | 97.40 |
| 28 | 10 | 20000 | 250 | 5 | 14.15 | 88.70 | 97.68 |
| 29 | 10 | 60000 | 250 | 5 | 4.43 | 57.80 | 50.00 |
| 30 | 12 | 60000 | 250 | 5 | 11.45 | 60.82 | 32.79 |

Optimization of the Flocculation Step

As illustrated in Table S2 Twenty (20) experimental runs were proposed by the CCD design and were conducted after obtaining the optimal design factors from the coagulation treatment step.

Table S2. DOE matrix with three design factors with actual values of COD, phenol, and turbidity removal percentages.

| Run | Investigated Factors |  |  | Removal (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dose (mg/L) | Mixing rate (RPM) | Mixing time (min) | COD (%) | Toxic phenol (%) | Turbidity (%) |
| 1 | 500 | 30 | 5 | 11.34 | 98.57 | 98.50 |
| 2 | 100 | 30 | 5 | 15.01 | 96.99 | 98.16 |
| 5 | 500 | 30 | 25 | 12.31 | 98.42 | 97.81 |
| 8 | 300 | 30 | 15 | 17.06 | 98.42 | 94.95 |
| 13 | 100 | 30 | 25 | 14.25 | 97.81 | 98.09 |
| 3 | 300 | 140 | 15 | 17.82 | 97.74 | 99.39 |
| 4 | 300 | 140 | 5 | 11.77 | 98.34 | 98.50 |
| 6 | 100 | 140 | 15 | 13.93 | 98.34 | 98.57 |
| 7 | 300 | 140 | 15 | 14.69 | 98.49 | 98.63 |
| 9 | 300 | 140 | 15 | 16.52 | 97.74 | 98.22 |
| 10 | 300 | 140 | 15 | 14.25 | 97.97 | 96.99 |
| 14 | 300 | 140 | 15 | 23.54 | 99.02 | 98.09 |
| 15 | 300 | 140 | 15 | 15.33 | 96.99 | 98.16 |
| 17 | 300 | 140 | 25 | 7.56 | 97.14 | 99.59 |
| 19 | 500 | 140 | 15 | 14.15 | 97.81 | 99.11 |
| 11 | 300 | 250 | 15 | 14.25 | 98.49 | 98.98 |
| 12 | 100 | 250 | 25 | 13.93 | 98.87 | 92.08 |
| 16 | 100 | 250 | 5 | 14.36 | 96.68 | 98.84 |
| 18 | 500 | 250 | 25 | 13.82 | 97.21 | 96.86 |
| 20 | 500 | 250 | 5 | 15.66 | 95.55 | 99.39 |