# Appendix B: 3D Cylindrical Phantom Validation

To assess the geometry-dependence of the vascular waveguide effect, we performed 3D FDTD simulations using a cylindrical multi-layer phantom on an NVIDIA A100 GPU. The phantom comprised 9 concentric cylindrical layers (skin, fat, muscle, intercostal muscle, rib, lung, pericardium, blood, myocardium) with identical tissue properties and cardiovascular state definitions as the 1D model (Table 1). The computational domain comprised approximately 3.2 million cells with 2 mm isotropic resolution, and simulations ran for 50 RF cycles at 0.9 GHz to ensure steady-state convergence.

Results are summarized in Table B1. The vasoconstriction SAR modulation factor (1.165) matched the 1D prediction (1.14) within 2.2%, confirming that the stress paradox is not an artifact of the planar geometry. The vasodilation modulation factor (0.781) was substantially smaller than the 1D prediction (0.50), with the E-field modulation showing 30% deviation. This difference has a clear physical interpretation: in the cylindrical geometry, electromagnetic waves can diffract around the phantom surface, providing alternative propagation paths that partially bypass the enhanced superficial absorption during vasodilation. This diffraction mechanism does not exist in the 1D planar model, where all energy must traverse every layer sequentially.

Table B1. 3D cylindrical phantom FDTD results at 0.9 GHz (50 RF cycles, A100 GPU).

| State | SAR (W/kg) | SAR Mod. (3D) | SAR Mod. (1D) | Deviation (%) | Assessment |
| --- | --- | --- | --- | --- | --- |
| REST | 0.0375 | 1.000 (ref) | 1.000 (ref) | — | — |
| VASODILATION | 0.0293 | 0.781 | 0.50 | 56% | Direction correct; 3D > 1D |
| VASOCONSTRICTION | 0.0437 | 1.165 | 1.14 | 2.2% | Excellent agreement |

The asymmetry between vasoconstriction agreement (2.2%) and vasodilation disagreement (56%) is itself informative. During vasoconstriction, the dominant effect is increased myocardial conductivity, which enhances local power deposition regardless of how the wave arrives. During vasodilation, the dominant effect is increased superficial absorption in skin and muscle blood vessels, which in 3D can be partially circumvented by diffraction. This means that the 1D model correctly predicts the clinically relevant scenario (stress-induced vasoconstriction) while overestimating the protective scenario (vasodilation).

Convergence monitoring showed approximately 4% variation in mean E-field over the final 10 cycles, compared to a target of <1%. While additional cycles would improve convergence, the modulation ratios—which are relative quantities—are substantially more stable than absolute values, as systematic convergence errors cancel in the ratio. The REST SAR value of 0.0375 W/kg is within the plausible range for the exposure conditions modeled, and the directional preservation of state modulation (vasodilation < REST < vasoconstriction) is robust.