Journal of Pharmaceutical Innovation

Supplementary Material

Integrating sustainability metrics into analytical decision-making in biopharmaceutical quality control: a case study

Thallis Martins Souza

Instituto de Tecnologia em Imunobiológicos (Bio-Manguinhos), Fundação Oswaldo Cruz (Fiocruz), Rio de Janeiro, RJ, Brazil

Corresponding author: TMS. E-mail: thallismsouza@gmail.com. ORCID: https://orcid.org/0000-0002-9964-0213

| Table S1. The penalty points (PPs) to calculate Analytical Eco-Scale [1]. |  |  |  |
| --- | --- | --- | --- |
| Category | Subtotal PPs |  | Total PPs |
| Reagents |  |  |  |
| Amount | <10 mL (<10 g) | 1 | Amount PPs x hazard PPs |
|  | 10-100 mL (10-100 g) | 2 |  |
|  | >100 mL (>100 g) | 3 |  |
| Hazard | None | 0 |  |
|  | Less severe hazard | 1 |  |
|  | More severe hazard | 2 |  |
| Instruments |  |  |  |
| Energy | <0.1 kWh per sample |  | 0 |
|  | <1.5 kWh per sample |  | 1 |
|  | >1.5 kWh per sample |  | 2 |
| Occupational hazard | Hermetization of analytical process |  | 0 |
|  | Emission of vapours to the atmosphere |  | 3 |
| Waste | None |  | 0 |
|  | <1 mL (<1 g) |  | 1 |
|  | 1-10 mL (1-10 g) |  | 3 |
|  | >10 mL (>10 g) |  | 5 |
|  | Recycling |  | 0 |
|  | Degradation |  | 1 |
|  | Pasivation |  | 2 |
|  | No treatment |  | 3 |

Table S2. Consumed reagents and sample amounts for assay determination of aromatic amino acids by non-aqueous potentiometric titrations and spectrofluorimetry methods [4].

| Non-aqueous potentiometric titration |  |  |  | Spectrofluorimetry |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Consumed amount* | PHE | TRP | TYR | Consumed amount* | PHE | TRP | TYR |
| Sample (mg) | 300 | 450 | 450 | Sample (mg) | 60.0 | 60.0 | 60.0 |
| Formic acid (mL) | 12.0 | 12.0 | 20.0 | KH2PO4 (mg) | 340 | 340 | 340 |
| Glacial acetic acid (mL) | 120 | 200 | 120 | NaOH 0.1 M (mL) | 4.00 | 4.00 | 4.00 |
| HClO4 0.1 M (mL) | 30.0 | 27.0 | 28.0 | Water (mL) | 200 | 200 | 200 |

*mass and volume considering blank + sample batch in triplicate; PHE: phenylalanine; TRP: tryptophan; TYR: tyrosine.

Table S3. Analytical Eco-Scale penalty points [2] assigned to the non-aqueous potentiometric titrations and spectrofluorimetric methods [4] for aromatic amino acids assays.

| Non-aqueous potentiometric titration |  |  |  | Spectrofluorimetry |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reagents | Amount (A) | Hazardous (H) | Penalty point (AxH) | Reagents | Amount (A) | Hazardous (H) | Penalty point (AxH) |
| Formic acid | 3 | 2 | 6 | Water | 3 | 0 | 0 |
| Glacial acetic acid | 2 | 2 | 4 | NaOH 0.1 M | 1 | 1 | 1 |
| HClO4 0.1 M | 2 | 2 | 4 | KH2PO4 | 1 | 0 | 0 |
| reagents (r) | - | - | 16 | reagents (r) | - | - | 1 |
| Instruments |  |  |  | Instruments |  |  |  |
| Energy consumption | - | - | 0 | Energy consumption | - | - | 0 |
| Occupational hazard | - | - | 3 | Occupational hazard | - | - | 0 |
| Waste generation | - | - | 7 | Waste generation | - | - | 6 |
| instruments (i) | - | - | 10 | instruments (i) | - | - | 6 |
| Total penalty pointsa (TPP) | - | - | 26 | Total penalty pointsa (TPP) | - | - | 7 |
| Analytical Eco-Scale scoreb (AESS) | - | - | 74 | Analytical Eco-Scale scoreb (AESS) | - | - | 93 |

aTPP = r + i; bAESS = 100- TPP

Table S4. Green Analytical Procedure Index (GAPI) parameters for analytical procedures for assay of aromatic amino acids by non-aqueous potentiometric titrations and spectrofluorimetry. [2]

| Category | Non-aqueous potentiometric titrations | Spectrofluorimetry |  |
| --- | --- | --- | --- |
| Sample preparation |  |  |
| Collection (1) | Off-line | Off-line |  |
| Preservation (2) | None | None |  |
| Transport (3) | Required | Required |  |
| Storage (4) | Store the sample extracts at -10°C, protected from light. | Samples must be iced or refrigerated at 4°C, protected from light. |  |
| Type of method: direct or indirect (5) | Extraction required (liquid-liquid extraction) | Extraction required (liquid-solid extraction) |  |
| Scale of extraction (6) | Macroextraction | Macroextraction |  |
| Solvents/reagents used (7) | Non green solvents/reagents used | Non green solvents/reagents used |  |
| Additional treatments (8) | Simple treatments (extract clean up, solvent removal, etc.) | Simple treatments (extract clean up, solvent removal, etc.) |  |
| Reagent and solvents |  |  |
| Amount (9) | 10-100 mL (10-100 g) | 10-100 mL (10-100 g) |  |
| Health hazard (10) | NFPA = 2 or 3 (depends on the reagent) | NFPA = 2 or 3 (depends on the reagent). |  |
| Safety hazard (11) | Highest NFPA flammability or instability score is 4. | Highest NFPA flammability or instability score is 2 or 3, a special hazard is used. |  |
| Instrumentation |  |  |
| Energy (12) | ≤0.1 kWh per sample | ≤0.1 kWh per sample |  |
| Occupational hazard (13) | Emission of vapors to the atmosphere | Hermetization of analytical process |  |
| Waste (14) | <10 mL (cc. 300 mL) | < 10 mL (cc. 350 mL) |  |
| Waste treatment (15) | Recycling possible | Recycling possible |  |
| QUANTIFICATION | Yes | Yes |  |

Table S5. White Analytical Chemistry (WAC) decision profile: R (performance), G (environment), and B (practicality) dimension scores and overall whiteness index for non-aqueous potentiometric titration versus spectrofluorimetry.

| Method: Non-aqueous potentiometric titration |  |  |  |  |  |  | Method: Spectrofluorimetry |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1: Scope of application | 50.0 | G1: Toxicity of reagents | 50.0 | B1: Cost-efficiency | 40.0 |  | R1: Scope of application | 95.0 |  | G1: Toxicity of reagents | 95.0 | B1: Cost-efficiency |  | 90.0 |
| R2: LOD and LOQ | 85.0 | G2: Amount of reagents and waste | 55.0 | B2: Time-efficiency | 60.0 |  | R2: LOD and LOQ | 100.0 |  | G2: Amount of reagents and waste | 60.0 | B2: Time-efficiency |  | 70.0 |
| R3: Precision | 100.0 | G3: Energy and other media | 100.0 | B3: Requirements (mean of B3’-B3’’) | 45.0 |  | R3: Precision | 100.0 |  | G3: Energy and other media | 100.0 | B3: Requirements (mean of B3’-B3’’) |  | 85.0 |
|  |  |  |  | B3’: Sample consumption | 50 |  |  |  |  |  |  | B3’: Sample consumption |  | 90 |
|  |  |  |  | B3’’: Other needs: advanced instruments, skills, facilities | 40 |  |  |  |  |  |  | B3’’: Other needs: advanced instruments, skills, facilities |  | 80 |
| R4: Accuracy | 100.0 | G4: Direct impacts (safety of users) | 98.3 | B4: Operational simplicity* | 0.0 |  | R4: Accuracy | 100.0 |  | G4: Direct impacts (safety of users) | 80.0 | B4: Operational simplicity* |  | 0.0 |
| R: 83.8 (mean of R1 to R4) |  | G: 75.8 (mean of G1 to G4) |  | B: 36.3 (mean of B1 to B4) |  |  | 98.8 (mean of R1-R4) |  | 83.8 (mean of G1-G4) |  |  |  | 61.3 (mean of B1 to B4) |  |
| WAC score: 65.3 (mean of R, G and B) |  |  |  |  |  |  | WAC score: 81.3 (mean of R, G and B) |  |  |  |  |  |  |  |

*Includes miniaturization, integration/automation, and portability parameters.