Supplementary Information 1. Response and selected explanatory variables used in this study

| Variables | Type | Units | Descriptions |
| --- | --- | --- | --- |
| GWL | Response | cm | Groundwater level (depth to water table), averaged to biweekly course |
| Precip_raw | Explanatory/ predictor; dynamic | mm | Current biweekly- summed precipitation |
| Precip_raw_lag1 |  | mm | Precipitation from the previous biweekly period |
| Precip_raw_lag2 |  | mm | Precipitation from two previous biweekly periods |
| Precip_cum_lag2 |  | mm | Cumulative precipitation of the current and two previous biweekly periods |
| S_raw |  | mm | Current biweekly partial storage (LO + S = P – ET) |
| S_raw_lag1 |  | mm | S from the previous biweekly period |
| S_raw_lag2 |  | mm | S from two previous biweekly periods |
| S_cum_lag1 |  | mm | Cumulative S of the current and previous biweekly period |
| S_cum_lag2 |  | mm | Cumulative S of the current and two previous biweekly periods |
| SM_raw |  | %v/v | Current biweekly-averaged soil moisture (SM) |
| SM_raw_lag1 |  | %v/v | SM from the previous biweekly period |
| SM_raw_lag2 |  | %v/v | SM from two previous biweekly periods |
| Peat_thick | Explanatory/ predictor; static | m | Peat thickness (or depth of peat layer) |
| Eucl_dist_ar |  | m | Euclidean distance to the nearest natural streams |
| Filled_surf |  | masl | Surface elevation after depression-filling |
| Filled_subsurf |  | masl | Subsurface elevation (mineral soil below peat layer; surface elevation subtracted by peat thickness) after depression-filling (DEM preprocessing) |
| Slope_surf |  | rad | Slope derived from filled surface DEM |
| Slope_subsurf |  | rad | Slope derived from filled subsurface DEM |
| Coast |  | m | Euclidean distance to the nearest coastline |
| Season_dummy | Explanatory/ predictor; Time-derived hot-encoded-dummy | - | Binary (0, 1); 1 for dry season, 0 for rainy season |
| Year_2019 |  | - | Binary (0, 1); 1 for year 2019, 0 for else |
| Year_2020 |  | - | Binary (0, 1); 1 for year 2020, 0 for else |
| Year_2021 |  | - | Binary (0, 1); 1 for year 2021, 0 for else |
| Year_2022 |  | - | Binary (0, 1); 1 for year 2022, 0 for else |
| Year_2023 |  | - | Binary (0, 1); 1 for year 2023, 0 for else |
| Year_2024 |  | - | Binary (0, 1); 1 for year 2024, 0 for else |
| Year_2025 |  | - | Binary (0, 1); 1 for year 2025, 0 for else |

Supplementary Information 2. Land use change in PHU Pulau Rupat during study period

| Landuse | Area (ha) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 |
| Secondary Swamp Forest | 16,582.25 | 17,614.50 | 16,177.25 | 20,993.50 | 20,921.50 | 20,899.50 |
| Secondary Mangrove Forest | 560.00 | 603.25 | 726.75 | 806.50 | 806.50 | 806.00 |
| Primary Swamp Forest | 4,151.50 | 4,149.25 | 4,144.25 | - | - | - |
| Plantation Forest | 16,338.75 | 20,809.50 | 20,247.50 | 19,307.25 | 20,506.25 | 20,506.25 |
| Shrubland | - | 5.50 | 5.50 | - | - | - |
| Swamp Shrubland | 14,727.25 | 6,814.25 | 10,401.75 | 16,396.00 | 16,216.00 | 16,216.00 |
| Estate Plantation | 15,077.50 | 22,043.50 | 27,088.25 | 26,124.00 | 26,124.00 | 26,124.50 |
| Mixed Agriculture-Shrubland | 11,578.75 | 11,651.50 | 3,381.50 | - | - | - |
| Bare/Open Land | 6,718.00 | 1,934.75 | 3,436.00 | 2,280.50 | 1,333.50 | 1,355.50 |
| Swamp/Marsh | 141.75 | 143.75 | 134.50 | - | - | - |
| Total | 85,875.75 | 85,769.75 | 85,743.25 | 85,907.75 | 85,907.75 | 85,907.75 |

Note: land use classes within study period of early 2025 (Jan-Apr) were assumed same as in 2024.

Supplementary Information 3. Rs-GWL response curve equations used in this study (derived from 14,15)

| Factor | Equation | Valid Range (cm) | R2 |
| --- | --- | --- | --- |
| General | Rs = 117.06-70.911 + exp (-0.10 GWL + 75.78) | −120 ≤ GWL ≤ +10 | 0.49 |
| LU-Based |  |  |  |
| Primary Forest* | Rs =exp⁡3.611 + exp (30.7 GWL100 - 1+0.331 + exp103GWL100 + 0.06 | −80 ≤ GWL ≤ +10 | 0.85 |
| Secondary Forest | Rs = 46.1423 − 0.2393 × GWL × 0.00524 × GWL2 | −80 ≤ GWL ≤ −30 | - |
| Forest Plantation | Rs = 96.9625 − 1.1780 × GWL + 0.01425 × GWL2 | −110 ≤ GWL ≤ −50 | - |
| Estate Plantation | Rs = 0.0151 × GWL2 − 0.7904 × GWL + 52.263 | -80 ≤ GWL ≤ 0 | 0.49 |
| Agriculture and other land uses | Rs = 25.9822 − 1.5543 × GWL − 0.00946 × GWL2 | -90 ≤ GWL ≤ 0 | - |

Note: *CO2-GWL response curve for primary forest is selected from 52

Supplementary Information 4. Uncertainty (upper: SD and lower: SE) of GWL predictions

Supplementary Information 5. Uncertainty (upper: SD and lower: SE) of Rs predictions based on Rs-GWL general equation15

Supplementary Information 6. Uncertainty (upper: SD and lower: SE) of Rs predictions based on land use-based Rs-GWL14