Appendix B

Table B1. Spatial data layers used for computation of  T, Rae, Rrad, and S.

| Indicator | Layer name | Variable | Unit | Portal section | Role in model | Pre-processing |
| --- | --- | --- | --- | --- | --- | --- |
| Socio-economic development | Socio-economic development index of regions | O | dimen-sionless (index) | Socio-economic indicators: Regional development | Component of T (Eq. 2) | Min–max normalization; area-weighted aggregation to districts |
| Industrial pressure | Density of industrial and energy facilities | O | facilities·km⁻² | Technogenic load: Industrial infrastructure | Component of O | Kernel density, district mean |
| Air conta-mination | Atmo-spheric air quality | Zair | μg·m⁻³ (normalized) | Air quality monitoring | Component of Z | Temporal averaging; min–max scaling |
| Surface water pollution | Surface water quality status | Zwater | class / index | Water resources: Monitoring of surface waters | Component of Z | Class, numeric score; normalization |
| Soil conta-mination | Soil contamination and degradation map | Zsoil | relative index | Soils: Degradation and pollution | Com-ponent of Z | Direct use; min–max scaling |
| Fire / explo-sion proxy | Active fires and thermal anomalies | Zfire | counts·km⁻² | Fire monitoring | Proxy for wartime Z | Density calculation; normalization |
| Total contamination | Integrated environ-mental contamination index | Z | dimensionless | - | Component of T (Eq. 2) | Z = Zair + Zwater + Zsoil (+ Zfire proxy) |
| Radio-logical conta-mination | Radio-logical contamination density map | Zr, Zu | Bq·m⁻² (normalized) | Radiation monitoring | Component of Rrad | Normalized to national mean |
| Popu-lation exposure | Population density by admi-nistrative units | Nr, Nu | persons·km⁻² | Population statistics | Component of Rrad | Direct use; normalization |
| Atmo-spheric resilience | Meteorological self-purifi-cation potential | P₁ | index | Climate & atmosphere | Component of S | Normalization |
| Hydro-logical resilience | Hydro-logical stability of surface waters | P₂ | index | Water resources | Component of S | Normalization |
| Soil resilience | Soil self-recovery capacity | P₃ | index | Soils | Component of S | Normalization |
| Biotic resilience | Biodiversity and ecosystem stability index | P₄ | index | Biodiversity | Component of S | Normalization |

Figure В1. Analytical workflow of this study