🧪 STANDARD OPERATING PROCEDURE: Battery Thermal Signature Analysis (SOP 6)

Entropy-Weighted Thermal Response in Li-ion Cells

python

"""

BATTERY-λ-SOP: Thermal Signature Analysis for Li-ion Cells

Purpose: Measure entropy-weighted thermal asymmetry during charge/discharge cycles

Version: 1.0

"""

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

from scipy.optimize import curve_fit

from scipy import stats

from dataclasses import dataclass

from typing import Dict, List, Tuple, Optional

from datetime import datetime

import os

from pathlib import Path

import warnings

warnings.filterwarnings('ignore')

# ============================================

# BATTERY PARAMETERS (Li-ion Reference)

# ============================================

@dataclass

class LiIonParameters:

"""Standard parameters for 18650 Li-ion cells"""

# Physical constants

F: float = 96485.3329      # Faraday constant (C/mol)

R: float = 8.314462618      # Gas constant (J/mol·K)

# Cell specifications

capacity_Ah: float = 2.5     # Nominal capacity (Ah)

voltage_nominal: float = 3.7  # Nominal voltage (V)

mass: float = 45.0            # Cell mass (g)

specific_heat: float = 0.8    # Specific heat capacity (J/g·K)

# Thermal parameters

k_thermal: float = 1.5        # Thermal conductivity (W/m·K)

h_conv: float = 10.0           # Convective coefficient (W/m²·K)

area_surface: float = 0.006    # Surface area (m²)

# Entropy coefficient dU/dT (mV/K) for different chemistries

# Typical values for LCO, NMC, LFP at 50% SOC

entropy_coeff: Dict[str, float] = None

def __post_init__(self):

if self.entropy_coeff is None:

self.entropy_coeff = {

'LCO': 0.5,    # LiCoO₂

'NMC': 0.3,    # LiNiMnCoO₂

'LFP': -0.2,   # LiFePO₄

'LTO': 0.1      # Li₄Ti₅O₁₂

}

# ============================================

# THERMAL MODEL FUNCTIONS

# ============================================

def joule_heating(I: float, R: float) -> float:

"""P_joule = I²R"""

return I**2 * R

def entropic_heating(I: float, T: float, dU_dT: float) -> float:

"""P_entropic = -I·T·(dU/dT)"""

return -I * T * dU_dT / 1000  # dU/dT in mV/K -> V/K

def thermal_response(t: np.ndarray, P: float, tau: float,

T_amb: float, T_init: float) -> np.ndarray:

"""First-order thermal response: T(t) = T_amb + (T_init - T_amb)*exp(-t/tau) + P·R_th·(1 - exp(-t/tau))"""

R_th = 1 / (LiIonParameters().h_conv * LiIonParameters().area_surface)

return T_amb + (T_init - T_amb)*np.exp(-t/tau) + P*R_th*(1 - np.exp(-t/tau))

# ============================================

# CYCLE DATA ANALYZER

# ============================================

class CycleAnalyzer:

"""Analyze charge/discharge cycle data"""

def extract_cycle(self, data: pd.DataFrame, cycle_num: int,

mode: str = 'charge') -> pd.DataFrame:

"""

Extract single charge or discharge cycle

Parameters:

data: full dataset with columns [time, current, voltage, temperature]

cycle_num: cycle number

mode: 'charge' or 'discharge'

"""

# Find cycle boundaries (current sign changes)

current = data['current'].values

time = data['time'].values

# Find zero crossings

zero_crossings = np.where(np.diff(np.signbit(current)))[0]

if mode == 'charge':

# Positive current segments

idx_start = zero_crossings[2*cycle_num]

idx_end = zero_crossings[2*cycle_num + 1]

else:  # discharge

idx_start = zero_crossings[2*cycle_num + 1]

idx_end = zero_crossings[2*cycle_num + 2]

return data.iloc[idx_start:idx_end].copy()

def calculate_capacity(self, cycle_data: pd.DataFrame) -> float:

"""Calculate capacity (Ah) from current and time"""

time = cycle_data['time'].values

current = cycle_data['current'].values

dt = np.diff(time, prepend=time[0])

capacity = np.sum(current * dt) / 3600  # Convert to Ah

return abs(capacity)

def calculate_energy(self, cycle_data: pd.DataFrame) -> float:

"""Calculate energy (Wh)"""

time = cycle_data['time'].values

power = cycle_data['current'].values * cycle_data['voltage'].values

dt = np.diff(time, prepend=time[0])

energy = np.sum(power * dt) / 3600  # Convert to Wh

return abs(energy)

def calculate_efficiency(self, charge_data: pd.DataFrame,

discharge_data: pd.DataFrame) -> Dict:

"""Calculate Coulombic and energy efficiency"""

cap_charge = self.calculate_capacity(charge_data)

cap_discharge = self.calculate_capacity(discharge_data)

energy_charge = self.calculate_energy(charge_data)

energy_discharge = self.calculate_energy(discharge_data)

return {

'coulombic_efficiency': cap_discharge / cap_charge * 100,

'energy_efficiency': energy_discharge / energy_charge * 100,

'capacity_charge': cap_charge,

'capacity_discharge': cap_discharge,

'energy_charge': energy_charge,

'energy_discharge': energy_discharge

}

# ============================================

# THERMAL FITTING

# ============================================

class ThermalFitter:

"""Fit thermal response to extract parameters"""

def fit_thermal_time_constant(self, t: np.ndarray, T: np.ndarray,

P: float, T_amb: float) -> Dict:

"""

Fit thermal transient to extract τ and R_th

"""

# Normalize for fitting

T_norm = T - T_amb

T_init = T_norm[0]

def model(t, tau, R_th):

return T_init * np.exp(-t/tau) + P*R_th*(1 - np.exp(-t/tau))

try:

popt, pcov = curve_fit(

model, t, T_norm,

p0=[100, 10],  # Initial guesses: τ=100s, R_th=10 K/W

bounds=([0, 0], [1000, 100]),

maxfev=5000

)

tau, R_th = popt

tau_err, R_th_err = np.sqrt(np.diag(pcov))

# R²

fitted = model(t, *popt)

ss_res = np.sum((T_norm - fitted)**2)

ss_tot = np.sum((T_norm - np.mean(T_norm))**2)

r_squared = 1 - (ss_res / ss_tot)

return {

'tau': tau,

'tau_err': tau_err,

'R_th': R_th,

'R_th_err': R_th_err,

'r_squared': r_squared,

'success': True

}

except Exception as e:

return {'success': False, 'error': str(e)}

def decompose_heating_sources(self, cycle_data: pd.DataFrame,

R_int: float, dU_dT: float) -> pd.DataFrame:

"""

Decompose total heating into Joule and entropic components

P_joule = I²R

P_entropic = -I·T·(dU/dT)

"""

df = cycle_data.copy()

df['P_joule'] = df['current']**2 * R_int

df['P_entropic'] = -df['current'] * (df['temperature'] + 273.15) * dU_dT / 1000

df['P_total'] = df['P_joule'] + df['P_entropic']

df['P_meas'] = df['temperature'].diff() * df['mass'] * df['specific_heat'] / df['time'].diff()

return df

# ============================================

# MAIN BATTERY ANALYZER

# ============================================

class BatteryLambdaAnalyzer:

"""

Complete analyzer for battery thermal signature experiments

File naming: [Cell]_[Chemistry]_[CRate]C_[Cycle]_[Temp]C.csv

Example: Cell1_NMC_1C_charge_25C.csv

"""

def __init__(self):

self.params = LiIonParameters()

self.cycle_analyzer = CycleAnalyzer()

self.thermal_fitter = ThermalFitter()

self.data = pd.DataFrame()

self.results = {}

self.cycle_results = []

def load_data(self, data_dir: str) -> pd.DataFrame:

"""Load all battery cycle data"""

data_dir = Path(data_dir)

all_data = []

for filepath in data_dir.glob('*.csv'):

filename = filepath.name

parts = filename.replace('.csv', '').split('_')

if len(parts) >= 5:

cell = parts[0]

chemistry = parts[1]

c_rate = float(parts[2].replace('C', ''))

cycle = parts[3]

temp = float(parts[4].replace('C', ''))

# Load data

df = pd.read_csv(filepath)

df['cell'] = cell

df['chemistry'] = chemistry

df['C_rate'] = c_rate

df['cycle_type'] = cycle

df['ambient_temp'] = temp

df['filename'] = filename

all_data.append(df)

self.data = pd.concat(all_data, ignore_index=True)

return self.data

def calculate_thermal_asymmetry(self, cell: str, chemistry: str,

c_rate: float, temp: float) -> Dict:

"""

Calculate thermal asymmetry between charge and discharge

"""

# Get charge and discharge cycles

charge_data = self.data[

(self.data['cell'] == cell) &

(self.data['chemistry'] == chemistry) &

(self.data['C_rate'] == c_rate) &

(self.data['ambient_temp'] == temp) &

(self.data['cycle_type'] == 'charge')

]

discharge_data = self.data[

(self.data['cell'] == cell) &

(self.data['chemistry'] == chemistry) &

(self.data['C_rate'] == c_rate) &

(self.data['ambient_temp'] == temp) &

(self.data['cycle_type'] == 'discharge')

]

if len(charge_data) == 0 or len(discharge_data) == 0:

return {'success': False}

# Calculate thermal metrics

metrics = {}

for phase, df in [('charge', charge_data), ('discharge', discharge_data)]:

# Peak temperature rise

T_amb = df['ambient_temp'].iloc[0]

T_max = df['temperature'].max()

metrics[f'{phase}_T_rise'] = T_max - T_amb

# Temperature at end of cycle

T_end = df['temperature'].iloc[-1]

metrics[f'{phase}_T_end'] = T_end - T_amb

# Integrated temperature rise

time = df['time'].values

T_rise = df['temperature'].values - T_amb

metrics[f'{phase}_integral_T'] = np.trapz(T_rise, time)

# Maximum heating rate

dT_dt = np.gradient(df['temperature'].values, df['time'].values)

metrics[f'{phase}_max_heating_rate'] = np.max(dT_dt)

# Calculate asymmetries

metrics['asymmetry_T_rise'] = metrics['charge_T_rise'] - metrics['discharge_T_rise']

metrics['asymmetry_integral'] = metrics['charge_integral_T'] - metrics['discharge_integral_T']

metrics['asymmetry_heating_rate'] = (metrics['charge_max_heating_rate'] -

metrics['discharge_max_heating_rate'])

# Normalized asymmetry

avg_T_rise = (metrics['charge_T_rise'] + metrics['discharge_T_rise']) / 2

if avg_T_rise > 0:

metrics['asymmetry_normalized'] = metrics['asymmetry_T_rise'] / avg_T_rise

else:

metrics['asymmetry_normalized'] = 0

metrics['success'] = True

return metrics

def fit_thermal_parameters(self, cell: str, chemistry: str,

c_rate: float, temp: float) -> Dict:

"""

Fit thermal response to extract τ and R_th for charge/discharge

"""

results = {}

for phase in ['charge', 'discharge']:

phase_data = self.data[

(self.data['cell'] == cell) &

(self.data['chemistry'] == chemistry) &

(self.data['C_rate'] == c_rate) &

(self.data['ambient_temp'] == temp) &

(self.data['cycle_type'] == phase)

]

if len(phase_data) == 0:

continue

# Get initial thermal transient (first 300 seconds)

t = phase_data['time'].values[:100]  # First 100 points

T = phase_data['temperature'].values[:100]

T_amb = phase_data['ambient_temp'].iloc[0]

# Estimate power

I_avg = np.mean(phase_data['current'].values[:100])

V_avg = np.mean(phase_data['voltage'].values[:100])

P_avg = I_avg * V_avg

# Fit

fit = self.thermal_fitter.fit_thermal_time_constant(t, T, P_avg, T_amb)

if fit['success']:

results[f'{phase}_tau'] = fit['tau']

results[f'{phase}_tau_err'] = fit['tau_err']

results[f'{phase}_R_th'] = fit['R_th']

results[f'{phase}_R_th_err'] = fit['R_th_err']

results[f'{phase}_r_squared'] = fit['r_squared']

results['success'] = len(results) > 0

return results

def calculate_entropy_coefficient(self, cell: str, chemistry: str,

soc_range: Tuple[float, float] = (0.2, 0.8)) -> float:

"""

Estimate dU/dT from voltage-temperature relationship

"""

# Get data at different temperatures

data_subset = self.data[

(self.data['cell'] == cell) &

(self.data['chemistry'] == chemistry)

].copy()

# Bin by SOC

data_subset['SOC'] = data_subset['capacity_removed'] / data_subset['total_capacity']

data_subset = data_subset[(data_subset['SOC'] >= soc_range[0]) &

(data_subset['SOC'] <= soc_range[1])]

# Group by temperature

dU_dT_vals = []

for temp in data_subset['ambient_temp'].unique():

temp_data = data_subset[data_subset['ambient_temp'] == temp]

dU_dT_vals.append(np.polyfit(temp_data['temperature'],

temp_data['voltage'], 1)[0])

return np.mean(dU_dT_vals) if dU_dT_vals else 0

def calculate_lambda(self, chemistry: str, c_rate: float,

temp: float, asymmetry: float) -> float:

"""

Calculate λ from thermal asymmetry

λ = λ_chem + α·(c_rate - c_rate_min) + β·(T - T_ref)

"""

# Base λ from chemistry

lambda_base = {

'LCO': 0.3,

'NMC': 0.2,

'LFP': 0.1,

'LTO': 0.05

}.get(chemistry, 0.0)

# Rate enhancement (α = 0.1 per C-rate)

c_rate_min = 0.1  # Minimum C-rate

lambda_rate = 0.1 * (c_rate - c_rate_min) if c_rate > c_rate_min else 0

# Temperature enhancement (β = 0.01 per °C above 25°C)

T_ref = 25

lambda_temp = 0.01 * (temp - T_ref) if temp > T_ref else 0

lambda_val = lambda_base + lambda_rate + lambda_temp

# Normalize by asymmetry if available

if asymmetry > 0:

lambda_val = lambda_val * (1 + 0.5 * asymmetry)

return min(lambda_val, 1.0)  # Cap at 1.0

def analyze_cell(self, cell: str, chemistry: str) -> pd.DataFrame:

"""

Complete analysis for one cell

"""

print(f"\n🔋 Analyzing Cell {cell} ({chemistry})...")

results = []

cell_data = self.data[self.data['cell'] == cell]

for c_rate in sorted(cell_data['C_rate'].unique()):

for temp in sorted(cell_data['ambient_temp'].unique()):

# Calculate thermal asymmetry

thermal = self.calculate_thermal_asymmetry(cell, chemistry, c_rate, temp)

if thermal['success']:

# Fit thermal parameters

fit = self.fit_thermal_parameters(cell, chemistry, c_rate, temp)

# Calculate λ

lambda_val = self.calculate_lambda(

chemistry, c_rate, temp,

thermal['asymmetry_normalized']

)

results.append({

'cell': cell,

'chemistry': chemistry,

'C_rate': c_rate,

'temperature': temp,

'lambda': lambda_val,

'T_rise_charge': thermal['charge_T_rise'],

'T_rise_discharge': thermal['discharge_T_rise'],

'asymmetry_T_rise': thermal['asymmetry_T_rise'],

'asymmetry_integral': thermal['asymmetry_integral'],

'asymmetry_normalized': thermal['asymmetry_normalized'],

'integral_charge': thermal['charge_integral_T'],

'integral_discharge': thermal['discharge_integral_T'],

'heating_rate_charge': thermal['charge_max_heating_rate'],

'heating_rate_discharge': thermal['discharge_max_heating_rate'],

'tau_charge': fit.get('charge_tau', np.nan),

'tau_discharge': fit.get('discharge_tau', np.nan),

'R_th_charge': fit.get('charge_R_th', np.nan),

'R_th_discharge': fit.get('discharge_R_th', np.nan)

})

print(f"  {c_rate}C, {temp}°C: λ={lambda_val:.3f}, "

f"ΔT={thermal['asymmetry_T_rise']:.2f}°C")

results_df = pd.DataFrame(results)

self.results[f"{cell}_{chemistry}"] = results_df

return results_df

def analyze_all(self) -> pd.DataFrame:

"""Analyze all cells"""

all_results = []

for cell in self.data['cell'].unique():

for chemistry in self.data[self.data['cell'] == cell]['chemistry'].unique():

results = self.analyze_cell(cell, chemistry)

all_results.append(results)

return pd.concat(all_results, ignore_index=True)

def plot_thermal_asymmetry(self, results_df: pd.DataFrame,

chemistry: str = None, save_path: str = None):

"""

Plot thermal asymmetry vs C-rate for different temperatures

"""

if chemistry:

results_df = results_df[results_df['chemistry'] == chemistry]

fig, axes = plt.subplots(1, 2, figsize=(14, 5))

# Left: T_rise asymmetry vs C-rate

ax = axes[0]

for temp in sorted(results_df['temperature'].unique()):

temp_data = results_df[results_df['temperature'] == temp].sort_values('C_rate')

ax.plot(temp_data['C_rate'], temp_data['asymmetry_T_rise'],

'o-', linewidth=2, label=f'{temp}°C')

ax.set_xlabel('C-rate')

ax.set_ylabel('ΔT_rise = T_charge - T_discharge (°C)')

ax.set_title('Thermal Asymmetry vs C-rate')

ax.legend()

ax.grid(True, alpha=0.3)

# Right: Normalized asymmetry vs λ

ax = axes[1]

for chem in results_df['chemistry'].unique():

chem_data = results_df[results_df['chemistry'] == chem].sort_values('lambda')

ax.plot(chem_data['lambda'], chem_data['asymmetry_normalized'],

'o-', linewidth=2, label=chem)

ax.axhline(0, color='k', linestyle='--', alpha=0.5, label='Onsager')

ax.set_xlabel('λ (entropy weighting)')

ax.set_ylabel('Normalized Asymmetry ΔT/⟨T⟩')

ax.set_title('Asymmetry vs λ')

ax.legend()

ax.grid(True, alpha=0.3)

plt.tight_layout()

if save_path:

plt.savefig(save_path, dpi=300, bbox_inches='tight')

plt.show()

return fig

def plot_thermal_profiles(self, cell: str, chemistry: str,

c_rate: float, temp: float, save_path: str = None):

"""

Plot detailed thermal profiles for charge/discharge comparison

"""

charge_data = self.data[

(self.data['cell'] == cell) &

(self.data['chemistry'] == chemistry) &

(self.data['C_rate'] == c_rate) &

(self.data['ambient_temp'] == temp) &

(self.data['cycle_type'] == 'charge')

]

discharge_data = self.data[

(self.data['cell'] == cell) &

(self.data['chemistry'] == chemistry) &

(self.data['C_rate'] == c_rate) &

(self.data['ambient_temp'] == temp) &

(self.data['cycle_type'] == 'discharge')

]

if len(charge_data) == 0 or len(discharge_data) == 0:

print("Insufficient data")

return

fig, axes = plt.subplots(2, 2, figsize=(12, 10))

# Time axis

t_charge = charge_data['time'].values / 3600  # hours

t_discharge = discharge_data['time'].values / 3600

# Top left: Temperature vs time

ax = axes[0, 0]

ax.plot(t_charge, charge_data['temperature'], 'r-', linewidth=2, label='Charge')

ax.plot(t_discharge, discharge_data['temperature'], 'b-', linewidth=2, label='Discharge')

ax.axhline(charge_data['ambient_temp'].iloc[0], color='k', linestyle='--', alpha=0.5)

ax.set_xlabel('Time (hours)')

ax.set_ylabel('Temperature (°C)')

ax.set_title(f'Thermal Profiles\n{cell} {chemistry} {c_rate}C at {temp}°C')

ax.legend()

ax.grid(True, alpha=0.3)

# Top right: Voltage vs SOC

ax = axes[0, 1]

soc_charge = 100 * (1 - charge_data['capacity_removed'] / charge_data['total_capacity'].iloc[-1])

soc_discharge = 100 * discharge_data['capacity_removed'] / discharge_data['total_capacity'].iloc[-1]

ax.plot(soc_charge, charge_data['voltage'], 'r-', linewidth=2, label='Charge')

ax.plot(soc_discharge, discharge_data['voltage'], 'b-', linewidth=2, label='Discharge')

ax.set_xlabel('State of Charge (%)')

ax.set_ylabel('Voltage (V)')

ax.set_title('Voltage Profiles')

ax.legend()

ax.grid(True, alpha=0.3)

# Bottom left: Temperature rise vs SOC

ax = axes[1, 0]

T_amb = charge_data['ambient_temp'].iloc[0]

T_rise_charge = charge_data['temperature'] - T_amb

T_rise_discharge = discharge_data['temperature'] - T_amb

ax.plot(soc_charge, T_rise_charge, 'r-', linewidth=2, label='Charge')

ax.plot(soc_discharge, T_rise_discharge, 'b-', linewidth=2, label='Discharge')

ax.fill_between(soc_charge, 0, T_rise_charge, alpha=0.3, color='red')

ax.fill_between(soc_discharge, 0, T_rise_discharge, alpha=0.3, color='blue')

ax.set_xlabel('State of Charge (%)')

ax.set_ylabel('Temperature Rise (°C)')

ax.set_title('Temperature Rise Profiles')

ax.legend()

ax.grid(True, alpha=0.3)

# Bottom right: Heating rate

ax = axes[1, 1]

dT_dt_charge = np.gradient(charge_data['temperature'], charge_data['time'])

dT_dt_discharge = np.gradient(discharge_data['temperature'], discharge_data['time'])

ax.plot(t_charge, dT_dt_charge * 3600, 'r-', linewidth=2, label='Charge')

ax.plot(t_discharge, dT_dt_discharge * 3600, 'b-', linewidth=2, label='Discharge')

ax.set_xlabel('Time (hours)')

ax.set_ylabel('Heating Rate (°C/hour)')

ax.set_title('Heating Rates')

ax.legend()

ax.grid(True, alpha=0.3)

plt.tight_layout()

if save_path:

plt.savefig(save_path, dpi=300, bbox_inches='tight')

plt.show()

return fig

def plot_lambda_dependence(self, results_df: pd.DataFrame, save_path: str = None):

"""

Plot asymmetry vs λ for all chemistries

"""

fig, ax = plt.subplots(figsize=(10, 6))

for chem in results_df['chemistry'].unique():

chem_data = results_df[results_df['chemistry'] == chem].sort_values('lambda')

ax.errorbar(chem_data['lambda'], chem_data['asymmetry_normalized'],

yerr=chem_data['asymmetry_normalized'] * 0.1,  # 10% error estimate

fmt='o-', capsize=5, linewidth=2, label=chem)

# Fit linear trend

all_lambda = results_df['lambda'].values

all_asym = results_df['asymmetry_normalized'].values

mask = ~np.isnan(all_lambda) & ~np.isnan(all_asym)

if np.sum(mask) > 2:

slope, intercept, r_value, p_value, _ = stats.linregress(all_lambda[mask], all_asym[mask])

x_fit = np.linspace(0, np.max(all_lambda), 100)

y_fit = slope * x_fit + intercept

ax.plot(x_fit, y_fit, 'k--', linewidth=1.5,

label=f'Fit: ΔT/⟨T⟩ = {slope:.2f}λ + {intercept:.2f} (R²={r_value**2:.3f})')

ax.axhline(0, color='k', linestyle='-', alpha=0.3)

ax.set_xlabel('λ (entropy weighting parameter)')

ax.set_ylabel('Normalized Thermal Asymmetry ΔT/⟨T⟩')

ax.set_title('Battery Thermal Asymmetry vs λ')

ax.legend()

ax.grid(True, alpha=0.3)

if save_path:

plt.savefig(save_path, dpi=300, bbox_inches='tight')

plt.show()

return fig

def generate_report(self, output_dir: str = './battery_analysis'):

"""

Generate complete analysis report

"""

os.makedirs(output_dir, exist_ok=True)

# Save raw data summary

self.data.to_csv(f'{output_dir}/raw_data_summary.csv', index=False)

# Analyze all

results = self.analyze_all()

results.to_csv(f'{output_dir}/lambda_results.csv', index=False)

# Generate plots

self.plot_thermal_asymmetry(results, save_path=f'{output_dir}/thermal_asymmetry.png')

self.plot_lambda_dependence(results, save_path=f'{output_dir}/lambda_dependence.png')

# Individual cell plots

for cell in results['cell'].unique():

for chem in results[results['cell'] == cell]['chemistry'].unique():

cell_data = results[(results['cell'] == cell) & (results['chemistry'] == chem)]

for _, row in cell_data.iterrows():

self.plot_thermal_profiles(

row['cell'], row['chemistry'],

row['C_rate'], row['temperature'],

save_path=f"{output_dir}/profile_{cell}_{chem}_{row['C_rate']}C_{row['temperature']}C.png"

)

# Text report

with open(f'{output_dir}/report.txt', 'w') as f:

f.write("BATTERY THERMAL SIGNATURE λ-FRAMEWORK REPORT\n")

f.write("="*60 + "\n\n")

f.write(f"Date: {datetime.now().strftime('%Y-%m-%d %H:%M')}\n")

f.write(f"Cells analyzed: {', '.join(results['cell'].unique())}\n")

f.write(f"Chemistries: {', '.join(results['chemistry'].unique())}\n\n")

f.write("="*60 + "\n")

f.write("THERMAL ASYMMETRY RESULTS\n")

f.write("="*60 + "\n\n")

for chem in results['chemistry'].unique():

chem_data = results[results['chemistry'] == chem]

f.write(f"\n{chem}:\n")

f.write(f"  Avg asymmetry: {chem_data['asymmetry_normalized'].mean():.4f} ± {chem_data['asymmetry_normalized'].std():.4f}\n")

f.write(f"  λ range: {chem_data['lambda'].min():.3f} - {chem_data['lambda'].max():.3f}\n")

# λ correlation

corr = results[['lambda', 'asymmetry_normalized']].corr().iloc[0,1]

f.write(f"\n\nλ-asymmetry correlation: {corr:.3f}\n")

if abs(corr) > 0.7:

f.write("✓ Strong correlation - entropy-weighting confirmed\n")

else:

f.write("⚠ Weak correlation\n")

# Theory comparison

f.write("\n" + "="*60 + "\n")

f.write("THEORETICAL PREDICTION\n")

f.write("="*60 + "\n")

f.write("Onsager symmetry at λ=0: ΔT = 0\n")

f.write("Finite λ → ΔT ∝ λ\n")

# Check λ=0 case

lambda_zero = results[np.abs(results['lambda']) < 0.05]

if len(lambda_zero) > 0:

avg_zero = lambda_zero['asymmetry_normalized'].mean()

f.write(f"\nλ≈0 asymmetry: {avg_zero:.4f}\n")

if np.abs(avg_zero) < 0.05:

f.write("✓ Onsager symmetry confirmed at λ=0\n")

else:

f.write("⚠ Onsager violation at λ=0 - check baseline\n")

f.write("\n" + "="*60 + "\n")

f.write("CONCLUSION\n")

f.write("="*60 + "\n")

if abs(corr) > 0.7 and np.abs(avg_zero) < 0.05:

f.write("\n✅ λ-FRAMEWORK VALIDATED IN BATTERY THERMAL RESPONSE\n")

f.write("Thermal asymmetry scales with entropy weighting parameter λ\n")

f.write("Onsager symmetry holds at λ=0, breaks at λ>0\n")

elif abs(corr) > 0.7:

f.write("\n⚠ PARTIAL VALIDATION\n")

f.write("Asymmetry correlates with λ but baseline Onsager may be broken\n")

else:

f.write("\n⚠ INCONCLUSIVE\n")

f.write("Further measurements needed\n")

print(f"\n📊 Report generated in {output_dir}/")

return results

# ============================================

# MAIN EXECUTION

# ============================================

def main():

"""

Run complete battery thermal signature analysis

"""

print("="*60)

print("BATTERY THERMAL SIGNATURE λ-FRAMEWORK ANALYSIS")

print("Testing Entropy-Weighted Thermodynamic Geometry")

print("="*60)

# Initialize analyzer

analyzer = BatteryLambdaAnalyzer()

# Load data (modify path to your data directory)

data_dir = "./battery_data"

if os.path.exists(data_dir):

print(f"\n📂 Loading data from {data_dir}...")

analyzer.load_data(data_dir)

print(f"\nLoaded {len(analyzer.data)} measurements")

print("\nData summary:")

print(analyzer.data.groupby(['cell', 'chemistry', 'C_rate'])['ambient_temp'].agg(['min', 'max', 'count']))

# Run complete analysis

results = analyzer.generate_report(output_dir='./battery_analysis')

print("\n" + "="*60)

print("✅ ANALYSIS COMPLETE")

print("="*60)

else:

print(f"\n❌ Data directory {data_dir} not found.")

print("\nPlease create directory with file naming convention:")

print("  [Cell]_[Chemistry]_[CRate]C_[Cycle]_[Temp]C.csv")

print("\nExample:")

print("  Cell1_NMC_1C_charge_25C.csv")

print("  Cell1_NMC_1C_discharge_25C.csv")

print("  Cell2_LFP_0.5C_charge_30C.csv")

print("  etc.")

if __name__ == "__main__":

main()

📋 EXPERIMENTAL PROTOCOL

1. Cell Preparation

| Step | Action | Acceptance |
| --- | --- | --- |
| 1.1 | Select 5 cells per chemistry (matched capacity ±2%) | ✓ |
| 1.2 | Mount thermocouple at cell center (K-type, 0.1°C accuracy) | ✓ |
| 1.3 | Place in thermal chamber with 25°C ±0.5°C | ✓ |
| 1.4 | Connect to cycler (4-wire sense) | ✓ |
| 1.5 | Stabilize for 2 hours (voltage drift <1 mV) | ✓ |

2. Characterization Cycles

| Step | Action | Data to Record |
| --- | --- | --- |
| 2.1 | CC charge at 0.1C to 4.2V | time, I, V, T |
| 2.2 | CV hold until I < 0.05C | time, I, V, T |
| 2.3 | Rest 1 hour | time, T |
| 2.4 | CC discharge at 0.1C to 2.5V | time, I, V, T |
| 2.5 | Rest 1 hour | time, T |
| 2.6 | Repeat for 0.2C, 0.5C, 1C, 2C | All data |
| 2.7 | Repeat at 30°C, 35°C, 40°C | All data |

3. Key Observables

| Symbol | Definition | Units |
| --- | --- | --- |
| ΔT_charge | Peak temperature rise during charge | °C |
| ΔT_discharge | Peak temperature rise during discharge | °C |
| A_thermal = ∫ΔT dt | Integrated temperature rise | °C·s |
| λ | (C-rate - min_rate)/min_rate + (T - 25)/100 | dimensionless |
| τ | Thermal time constant | s |
| R_th | Thermal resistance | K/W |

4. Acceptance Criteria

| Test | Criterion |
| --- | --- |
| Capacity | Within 5% of nominal |
| Thermal stability | Drift <0.1°C/min during rest |
| Cycle reproducibility | ΔT variation <5% across cycles |
| λ correlation | R² > 0.8 |
| Onsager at λ=0 | ΔT < 0.1°C |

📊 Expected Results

| Chemistry | λ range | ΔT_asymmetry | Scaling |
| --- | --- | --- | --- |
| LCO | 0.3 - 0.8 | 0.5 - 2.0°C | ΔT ∝ 2.5λ |
| NMC | 0.2 - 0.7 | 0.3 - 1.5°C | ΔT ∝ 2.0λ |
| LFP | 0.1 - 0.5 | 0.1 - 0.8°C | ΔT ∝ 1.5λ |
| LTO | 0.05 - 0.4 | 0.05 - 0.4°C | ΔT ∝ 1.0λ |

📁 File Naming Convention

text

[CellID]_[Chemistry]_[CRate]C_[Cycle]_[Temp]C.csv

Examples:

Cell1_NMC_1C_charge_25C.csv

Cell1_NMC_1C_discharge_25C.csv

Cell2_LFP_0.5C_charge_30C.csv

📈 What This Tests

- Entropy-weighting → Thermal asymmetry ∝ λ

- Onsager symmetry → ΔT = 0 at λ = 0

- Chemistry dependence → Different dU/dT → different slopes

- Rate dependence → λ ∝ C-rate

- Temperature dependence → λ ∝ (T - T_ref)