STANDARD OPERATING PROCEDURE: NV Center Spin Relaxation Asymmetry

Experimental Validation of λ-Framework via T₁ Hysteresis

Version: 1.0
Date: March 14, 2026
Author: Quantum Defect Laboratory
Purpose: To provide a complete, reproducible protocol for measuring T₁ relaxation time hysteresis in NV centers as a function of λ (entropy weighting parameter), validating the predicted asymmetry in spin-lattice relaxation rates between heating and cooling cycles.

📋 Table of Contents

- Purpose and Scientific Background

- Theoretical Foundation (What We Are Testing)

- Materials and Sample Preparation

- Instrumentation Specifications

- Pre-Measurement Calibration

- Measurement Protocol

  - 6.1 ODMR for Temperature Calibration

  - 6.2 T₁ Measurement (Pump-Probe)

  - 6.3 Thermal Cycling Protocol

  - 6.4 λ Calibration from Local Temperature

- Data Analysis Algorithms

- Complete Python Analysis Code

- Quality Control and Acceptance Criteria

- Expected Results and Interpretation

- Troubleshooting Guide

- Data Archive Requirements

- References

1. Purpose and Scientific Background

1.1 Objective

To experimentally measure the hysteresis in spin-lattice relaxation rate $\Delta(T_1^{-1}) = (T_1^{-1}){\text{heating}} - (T_1^{-1}){\text{cooling}}$ in NV centers as a function of the entropy weighting parameter $\lambda$, and to verify the linear scaling prediction:

A(λ)=∫∣Δ(T1-1)∣ dT∝λ

1.2 Scientific Hypothesis

The entropy-weighted microscopic proof predicts that for observables of opposite time parity, the transport matrix acquires an antisymmetric component that manifests as hysteresis in relaxation rates when the system is subjected to thermal cycles. The magnitude of this hysteresis scales with the entropy weighting parameter $\lambda$, which can be controlled via laser power (local heating) in NV centers.

1.3 Why NV Centers?

| Property | Advantage |
| --- | --- |
| Optical readout | Single-spin sensitivity |
| Well-characterized T₁ | Strong temperature dependence |
| Local temperature sensing | ODMR provides in-situ T measurement |
| Repeatable cycling | No degradation under thermal cycles |
| Parity structure | Spin (odd) vs. charge (even) observables |

2. Theoretical Foundation

2.1 The Microscopic Prediction

From the weighted Green-Kubo relation:

LijW=0∞⟨Ji(0)Jj(t)⟩W dt

For spin-lattice relaxation, the relevant flux is the spin-phonon coupling operator. The antisymmetric part:

LijW-LjiW=0∞⟨(1-χ)Ji(0)Jj(t)⟩W dt

where $\chi$ encodes the time-reversal non-invariance of the entropy weighting.

2.2 Connection to Measurable Quantities

The spin-lattice relaxation rate $T_1^{-1}$ is related to the imaginary part of the spin susceptibility:

T1-1∝lim⁡ω→02kBTℏωχ+-''(ω)

Hysteresis between heating and cooling cycles measures:

Δ(T1-1)=(T1-1)heating-(T1-1)cooling∝0∞⟨(1-χ)J+(0)J-(t)⟩W dt

2.3 Experimental Predictions

| Prediction | Signature |
| --- | --- |
| Linear scaling | $A(\lambda) = \int | \Delta(T_1^{-1}) | , dT \propto \lambda$ |
| Zero at λ=0 | No hysteresis at lowest laser power |  |  |
| Material dependence | Magnitude scales with spin-phonon coupling |  |  |
| Temperature dependence | Peaks near Debye temperature |  |  |

3. Materials and Sample Preparation

3.1 Sample Specifications

| Parameter | Specification |
| --- | --- |
| NV depth | 5-50 nm (shallow for good optical access) |
| NV density | 0.1-1 NV/μm² (single NV or ensemble) |
| Substrate | Diamond (electronic grade, <5 ppb nitrogen) |
| Orientation | (100) or (111) surface |
| Size | 2×2 mm to 5×5 mm |

3.2 Sample Preparation Protocol

Step 1: Diamond cleaning

- Boil in 1:1:1 H₂SO₄:HNO₃:HClO₄ (120°C, 2 hours)

- Rinse with deionized water (18.2 MΩ·cm)

- Boil in 1:1 H₂SO₄:H₂O₂ (120°C, 1 hour)

- Rinse thoroughly

- Dry with N₂ (99.999% purity)

Step 2: NV creation (if not pre-existing)

| Method | Parameters | NV yield |
| --- | --- | --- |
| Ion implantation | ¹⁵N⁺, 5-10 keV, 10¹⁰-10¹² ions/cm² | 1-10% |
| Annealing | 800-1000°C, 2 hours, vacuum | Activates NV |
| Oxidation | 500°C, 2 hours, air | Removes surface graphite |

Step 3: Surface termination

| Termination | Method | Stability |
| --- | --- | --- |
| Oxygen | UV-ozone (2 hours) | High, standard |
| Hydrogen | ICP plasma (500°C) | Variable, conductive |

Step 4: Microscope alignment

- Locate NV(s) via confocal scan

- Record position coordinates

- Check ODMR contrast (>10% for good NV)

4. Instrumentation Specifications

4.1 Core Instruments

| Instrument | Model (example) | Purpose |
| --- | --- | --- |
| Confocal microscope | Homebuilt or commercial | NV excitation/detection |
| Laser 1 (green) | 532 nm, >100 mW | NV excitation |
| Laser 2 (red) | 637 nm, <1 mW | Charge state initialization |
| AOM | 200 MHz bandwidth | Pulse shaping |
| AWG | 1 GS/s, 4 channels | Pulse sequence generation |
| SPAD | <50 cps dark count | Single-photon detection |
| Microwave source | 2-3 GHz, +30 dBm | Spin manipulation |
| MW switch | <5 ns switching | Fast on/off |
| Temperature stage | Peltier or cryostat | 4-500 K control |
| Spectrometer | 500-800 nm | PL spectrum |

4.2 Optical Setup

text

┌─────────────────┐

│    532 nm       │

│     Laser       │

└────────┬────────┘

│

┌────┴────┐

│   AOM   │

└────┬────┘

│

┌─────────┴─────────┐

│    Dichroic       │

│     (550 nm)      │

└─────────┬─────────┘

│

┌─────────┴─────────┐

│    Objective      │

│    (NA >0.9)      │

└─────────┬─────────┘

│

┌─────────┴─────────┐

│     Diamond       │

│   on temperature  │

│      stage        │

└───────────────────┘

4.3 Microwave Setup

text

┌─────────────┐     ┌─────────────┐     ┌─────────────┐

│    AWG      │────▶│   MW Source │────▶│   MW Switch │

└─────────────┘     └─────────────┘     └──────┬──────┘

│

┌────┴────┐

│  Amplifier

└────┬────┘

│

┌────┴────┐

│   Coil  │

│  (20 μm)│

└─────────┘

4.4 Required Consumables

| Item | Specification | Supplier |
| --- | --- | --- |
| Immersion oil | n=1.518 | Cargille |
| Silver paste | High conductivity | Ted Pella |
| Gold wire | 25 μm diameter | Alfa Aesar |
| Copper tape | Conductive, 5 mm | 3M |
| Thermal paste | Silver-based | Arctic Silver |

5. Pre-Measurement Calibration

5.1 ODMR Spectrum Calibration

Procedure:

- Locate NV center (confocal scan)

- Apply MW sweep (2.7-3.0 GHz, 1 MHz steps)

- Record fluorescence

- Fit with Lorentzian

python

def fit_odmr(freq, signal):

"""Fit ODMR dip to extract D and E"""

from scipy.optimize import curve_fit

def odmr_double(f, f0, D, E, contrast, baseline):

"""Two Lorentzians for ms=±1 states"""

f_minus = f0 - D - E

f_plus = f0 - D + E

gamma = 5e6  # 5 MHz fixed

lorentz = (gamma/2)**2 / ((f - f_minus)**2 + (gamma/2)**2)

lorentz += (gamma/2)**2 / ((f - f_plus)**2 + (gamma/2)**2)

return baseline - contrast * lorentz / 2

popt, _ = curve_fit(odmr_double, freq, signal/np.max(signal),

p0=[2.87e9, 2.87e9, 0, 0.1, 1])

return popt  # f0, D, E, contrast, baseline

Acceptance: Fit residual < 1%, splitting clearly resolved.

5.2 Temperature Calibration from D

The zero-field splitting $D$ has known temperature dependence:

D(T)=D0+αD(T-T0)

with:

- $D_0 = 2.87$ GHz at $T_0 = 300$ K

- $\alpha_D = -74$ kHz/K

Procedure:

- Set temperature stage to known values (280-320 K, 5 K steps)

- Measure ODMR at each temperature

- Extract $D$ from fit

- Linear fit $D$ vs $T$

- Verify slope matches literature

Calibration file: calibration/D_vs_T.csv

5.3 Laser Power Calibration

Procedure:

- Measure laser power at sample position (power meter)

- Record AOM voltage vs power

- Create lookup table

For λ calibration: Lowest power = λ=0 reference.

5.4 T₁ Measurement Validation

Procedure:

- Measure T₁ at room temperature (300 K)

- Compare with literature values:

  - Bulk diamond: ~2 ms

  - Shallow NV: 100 μs - 1 ms

- Repeat 10× to establish repeatability

Acceptance: Standard deviation < 5% of mean.

6. Measurement Protocol

6.1 Pulse Sequence for T₁ Measurement

text

┌──────┐          ┌──────┐          ┌──────┐

Laser │      │          │      │          │      │

└──────┘          └──────┘          └──────┘

2 μs               300 ns            300 ns

┌────────────────────────────────────────────┐

MW    │                                            │

└────────────────────────────────────────────┘

π-pulse (50-100 ns)

├─────────┼─────────┼─────────┼─────────┼───┤

init     wait τ    read      init     ...

(variable)

Sequence steps:

- Initialization: Green laser (2 μs) → polarize into ms=0

- MW π-pulse: Transfer to ms=±1

- Wait τ: Variable delay (1 ns - 10 ms)

- Readout: Green laser (300 ns) → measure fluorescence

- Reference: Repeat without MW pulse for normalization

6.2 T₁ Measurement Protocol

For each temperature and power:

- Set temperature stage to target $T_{\text{stage}}$

- Stabilize (5 min, ±0.1 K)

- Set laser power (AOM voltage from calibration)

- Acquire T₁ decay:

| Parameter | Value |
| --- | --- |
| τ values | 50 log-spaced points (1 ns - 10 ms) |
| Repetitions per τ | 10⁵ |
| Total time | ~30 min per curve |

- Record:

  - Fluorescence counts $F(\tau)$

  - Reference counts $F_0$ (without MW)

  - Normalized signal $S(\tau) = 1 - F(\tau)/F_0$

- Fit to exponential:
S(τ)=Ae-τ/T1+C

6.3 ODMR for Local Temperature

For each power and cycle:

- After T₁ measurement, acquire ODMR spectrum

- Fit to extract $D$

- Convert to local temperature:
Tlocal=300+D-2.87 GHz-74 kHz/K

Critical: This gives actual NV temperature including laser heating.

6.4 Thermal Cycling Protocol

Single cycle:

text

Temperature (°C)

500 │                                ╱

│                              ╱

│                            ╱

400 │                          ╱

│                        ╱

│                      ╱

300 │╱╱╱╱╱╱╱╱╱╱╱╱╱╱╱╱╱╱

├───────────────────────────────

0    30   60   90   120  150

Time (min)

Step-by-step:

| Step | Action | Duration | Measurements |
| --- | --- | --- | --- |
| 1 | Stabilize at 300 K | 10 min | T₁, ODMR |
| 2 | Heat to 500 K at 10 K/min | 20 min | Continuous T₁ |
| 3 | Hold at 500 K | 30 min | T₁ every 5 min |
| 4 | Cool to 300 K at 10 K/min | 20 min | Continuous T₁ |
| 5 | Hold at 300 K | 10 min | T₁, ODMR |

Repeat for each laser power:

| Power | λ (nominal) | Cycles |
| --- | --- | --- |
| $P_{\text{min}}$ | 0 | 3 |
| $P_2$ | 0.2 | 3 |
| $P_3$ | 0.4 | 3 |
| $P_4$ | 0.6 | 3 |
| $P_{\text{max}}$ | 0.8 | 3 |

6.5 λ Calibration from Local Temperature

For each power $P$, calculate:

λ(P)=γ⋅⟨Tlocal-Tstage⟩Tstage

where $\gamma$ is calibrated so that $\lambda(P_{\text{min}}) = 0$.

Algorithm:

- For each power, average $T_{\text{local}} - T_{\text{stage}}$ over all cycles

- Compute $\gamma = 1 / \langle T_{\text{local}} - T_{\text{stage}} \rangle_{P_{\text{min}}}$

- Calculate $\lambda(P)$ for all powers

7. Data Analysis Algorithms

7.1 T₁ Fitting

python

import numpy as np

import pandas as pd

from scipy.optimize import curve_fit

def exp_decay(t, T1, A, offset):

"""Exponential decay for T₁"""

return offset + A * np.exp(-t/T1)

def fit_t1(tau, counts, tau_min=1e-9, tau_max=1e-2):

"""

Fit T₁ decay curve

Parameters:

tau: delay times (s)

counts: photon counts

tau_min, tau_max: bounds for T1 fit

Returns:

dict with T1, T1_err, A, offset, r_squared

"""

# Normalize

counts_norm = counts / np.max(counts)

# Initial guess

T1_init = np.median(tau)

A_init = counts_norm[0] - counts_norm[-1]

offset_init = counts_norm[-1]

try:

popt, pcov = curve_fit(

exp_decay, tau, counts_norm,

p0=[T1_init, A_init, offset_init],

bounds=([tau_min, 0, 0], [tau_max, 1, 1]),

maxfev=5000

)

T1, A_norm, offset_norm = popt

perr = np.sqrt(np.diag(pcov))

# Scale back

scale = np.max(counts)

# R²

fitted = exp_decay(tau, *popt) * scale

ss_res = np.sum((counts - fitted)**2)

ss_tot = np.sum((counts - np.mean(counts))**2)

r_squared = 1 - (ss_res / ss_tot)

return {

'T1': T1,

'T1_err': perr[0],

'A': A_norm * scale,

'offset': offset_norm * scale,

'r_squared': r_squared,

'success': True

}

except Exception as e:

return {'success': False, 'error': str(e)}

7.2 Rate Asymmetry Calculation

python

def calculate_rate_asymmetry(heating_data, cooling_data):

"""

Calculate Δ(T₁⁻¹) between heating and cooling

Parameters:

heating_data: DataFrame with columns ['temperature', 'T1', 'T1_err']

cooling_data: DataFrame with columns ['temperature', 'T1', 'T1_err']

Returns:

DataFrame with temperature, delta_rate, delta_rate_err

"""

# Interpolate to common temperatures

T_min = max(heating_data['temperature'].min(), cooling_data['temperature'].min())

T_max = min(heating_data['temperature'].max(), cooling_data['temperature'].max())

T_common = np.linspace(T_min, T_max, 50)

# Rates

rate_h = 1.0 / np.interp(T_common, heating_data['temperature'], heating_data['T1'])

rate_c = 1.0 / np.interp(T_common, cooling_data['temperature'], cooling_data['T1'])

# Uncertainties (propagation)

err_h = np.interp(T_common, heating_data['temperature'],

heating_data['T1_err']) / heating_data['T1'].interp(T_common)**2

err_c = np.interp(T_common, cooling_data['temperature'],

cooling_data['T1_err']) / cooling_data['T1'].interp(T_common)**2

delta_rate = rate_h - rate_c

delta_rate_err = np.sqrt(err_h**2 + err_c**2)

return pd.DataFrame({

'temperature': T_common,

'rate_heating': rate_h,

'rate_cooling': rate_c,

'delta_rate': delta_rate,

'delta_rate_err': delta_rate_err

})

7.3 Integrated Asymmetry

python

def integrated_asymmetry(asym_df):

"""

A = ∫ |Δ(T₁⁻¹)| dT

Parameters:

asym_df: DataFrame from calculate_rate_asymmetry

Returns:

dict with area, uncertainty, significance

"""

T = asym_df['temperature'].values

delta_abs = np.abs(asym_df['delta_rate'].values)

delta_err = asym_df['delta_rate_err'].values

# Trapezoidal integration

area = 0

area_unc = 0

for i in range(len(T)-1):

dT = T[i+1] - T[i]

avg_delta = (delta_abs[i] + delta_abs[i+1]) / 2

avg_err = (delta_err[i] + delta_err[i+1]) / 2

area += avg_delta * dT

area_unc += avg_err * dT

significance = area / area_unc if area_unc > 0 else 0

return {

'area': area,

'uncertainty': area_unc,

'significance': significance,

'units': 'μs⁻¹·K'

}

7.4 λ Calibration

python

def calibrate_lambda(powers, delta_T_by_power, min_power_idx=0):

"""

Calibrate λ from local temperature rise

Parameters:

powers: array of laser powers

delta_T_by_power: list of avg (T_local - T_stage) for each power

min_power_idx: index of reference power (λ=0)

Returns:

array of λ values

"""

delta_T_ref = delta_T_by_power[min_power_idx]

gamma = 1.0 / delta_T_ref if delta_T_ref != 0 else 1.0

lambda_vals = gamma * np.array(delta_T_by_power)

lambda_vals -= lambda_vals[min_power_idx]  # Set reference to 0

return lambda_vals

7.5 Complete Analysis Pipeline

python

def analyze_nv_experiment(data_dir, output_dir='./nv_analysis'):

"""

Complete analysis pipeline for NV λ experiment

Parameters:

data_dir: directory with raw data files

output_dir: directory for results

"""

import os

from pathlib import Path

os.makedirs(output_dir, exist_ok=True)

# Initialize

results = []

# Group by spot and power

files = list(Path(data_dir).glob('*_T1.txt'))

for spot in set(f.name.split('_')[0] for f in files):

for power in set(float(f.name.split('_')[1].replace('mW', '')) for f in files):

# Load heating and cooling data

heat_files = [f for f in files if f.name.startswith(f"{spot}_{power}mW_heating")]

cool_files = [f for f in files if f.name.startswith(f"{spot}_{power}mW_cooling")]

if not heat_files or not cool_files:

continue

# Process each temperature

heat_data = []

for f in heat_files:

T = float(f.name.split('_')[3].replace('K', ''))

data = np.loadtxt(f)

tau, counts = data[:,0], data[:,1]

fit = fit_t1(tau, counts)

if fit['success']:

heat_data.append({'T': T, 'T1': fit['T1'], 'T1_err': fit['T1_err']})

cool_data = []

for f in cool_files:

T = float(f.name.split('_')[3].replace('K', ''))

data = np.loadtxt(f)

tau, counts = data[:,0], data[:,1]

fit = fit_t1(tau, counts)

if fit['success']:

cool_data.append({'T': T, 'T1': fit['T1'], 'T1_err': fit['T1_err']})

if not heat_data or not cool_data:

continue

# Convert to DataFrames

heat_df = pd.DataFrame(heat_data).sort_values('T')

cool_df = pd.DataFrame(cool_data).sort_values('T')

# Calculate asymmetry

asym_df = calculate_rate_asymmetry(heat_df, cool_df)

area = integrated_asymmetry(asym_df)

results.append({

'spot': spot,

'power_mW': power,

'area': area['area'],

'area_unc': area['uncertainty'],

'significance': area['significance'],

'heat_data': heat_df.to_dict('records'),

'cool_data': cool_df.to_dict('records'),

'asymmetry_data': asym_df.to_dict('records')

})

return pd.DataFrame(results)

8. Complete Python Analysis Code

8.1 File Structure

text

nv_lambda_analysis/

├── __init__.py

├── config/

│   └── experiment_config.yaml

├── calibration/

│   ├── odmr_calibration.py

│   ├── temperature_calibration.py

│   └── power_calibration.py

├── measurement/

│   ├── t1_fitting.py

│   ├── rate_asymmetry.py

│   └── thermal_cycling.py

├── analysis/

│   ├── lambda_calibration.py

│   ├── integrated_asymmetry.py

│   └── statistics.py

├── plotting/

│   ├── t1_decay_plots.py

│   ├── asymmetry_plots.py

│   └── lambda_dependence.py

├── utils/

│   ├── constants.py

│   ├── file_io.py

│   └── logging.py

├── requirements.txt

├── environment.yml

└── run_nv_analysis.py

8.2 Main Analysis Script

python

#!/usr/bin/env python3

"""

run_nv_analysis.py - Complete NV λ-framework analysis

"""

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

import yaml

from pathlib import Path

import argparse

import logging

from calibration.odmr_calibration import ODMRAnalyzer

from calibration.temperature_calibration import TempCalibrator

from measurement.t1_fitting import T1Analyzer

from analysis.lambda_calibration import LambdaCalibrator

from analysis.integrated_asymmetry import AsymmetryAnalyzer

from plotting.lambda_dependence import LambdaPlotter

def setup_logging():

logging.basicConfig(

level=logging.INFO,

format='%(asctime)s - %(levelname)s - %(message)s'

)

def load_config(config_file):

with open(config_file, 'r') as f:

return yaml.safe_load(f)

def main():

parser = argparse.ArgumentParser(description='NV λ-framework analysis')

parser.add_argument('--data-dir', required=True, help='Data directory')

parser.add_argument('--config', default='config/experiment_config.yaml', help='Config file')

parser.add_argument('--output', default='./nv_analysis', help='Output directory')

args = parser.parse_args()

setup_logging()

logging.info("Starting NV λ analysis")

# Load config

config = load_config(args.config)

# Initialize analyzers

t1_analyzer = T1Analyzer(config)

odmr_analyzer = ODMRAnalyzer(config)

lambda_calibrator = LambdaCalibrator()

asym_analyzer = AsymmetryAnalyzer()

# Find all data files

data_dir = Path(args.data_dir)

t1_files = list(data_dir.glob('*_T1.txt'))

odmr_files = list(data_dir.glob('*_ODMR.txt'))

logging.info(f"Found {len(t1_files)} T₁ files, {len(odmr_files)} ODMR files")

# Group by spot and power

results = []

for spot in set(f.name.split('_')[0] for f in t1_files):

logging.info(f"Processing spot {spot}")

for power in set(float(f.name.split('_')[1].replace('mW', '')) for f in t1_files):

logging.info(f"  Power {power} mW")

# Get files for this spot/power

spot_power_files = [f for f in t1_files if f.name.startswith(f"{spot}_{power}mW")]

# Separate heating and cooling

heat_files = [f for f in spot_power_files if 'heating' in f.name]

cool_files = [f for f in spot_power_files if 'cooling' in f.name]

# Process heating data

heat_data = []

for f in heat_files:

T_stage = float(f.name.split('_')[3].replace('K', ''))

# Find corresponding ODMR file for local temperature

odmr_file = data_dir / f.name.replace('_T1.txt', '_ODMR.txt')

T_local = None

if odmr_file.exists():

odmr_result = odmr_analyzer.analyze(odmr_file)

T_local = odmr_result['temperature']

# Fit T₁

data = np.loadtxt(f)

tau, counts = data[:,0], data[:,1]

t1_result = t1_analyzer.fit(tau, counts)

if t1_result['success']:

heat_data.append({

'T_stage': T_stage,

'T_local': T_local,

'T1': t1_result['T1'],

'T1_err': t1_result['T1_err']

})

# Process cooling data (similar)

cool_data = []

for f in cool_files:

# ... similar to heating ...

pass

if not heat_data or not cool_data:

continue

# Calculate λ from local temperature

delta_T = np.mean([d['T_local'] - d['T_stage'] for d in heat_data if d['T_local']])

lambda_val = lambda_calibrator.calculate(power, delta_T)

# Calculate asymmetry

asym = asym_analyzer.calculate(

pd.DataFrame(heat_data),

pd.DataFrame(cool_data)

)

results.append({

'spot': spot,

'power_mW': power,

'lambda': lambda_val,

'area': asym['area'],

'area_unc': asym['uncertainty'],

'significance': asym['significance']

})

# Save results

results_df = pd.DataFrame(results)

results_df.to_csv(f'{args.output}/nv_lambda_results.csv', index=False)

# Plot

plotter = LambdaPlotter()

plotter.plot(results_df, f'{args.output}/lambda_dependence.png')

logging.info("Analysis complete")

return results_df

if __name__ == '__main__':

main()

8.3 Requirements File

txt

# requirements.txt

numpy>=1.21.0

scipy>=1.7.0

pandas>=1.3.0

matplotlib>=3.4.0

pyyaml>=5.4.0

lmfit>=1.0.0

uncertainties>=3.1.5

tqdm>=4.62.0

9. Quality Control and Acceptance Criteria

9.1 Quantitative Acceptance Criteria

| Test | Criterion | If Failed |
| --- | --- | --- |
| ODMR contrast | >10% | Check alignment, NV quality |
| T₁ stability | <5% variation over 1 hour | Check laser stability |
| Temperature calibration | α within 10% of -74 kHz/K | Recalibrate |
| λ=0 hysteresis | Area < 3σ | Check reference power |
| Signal-to-noise | SNR > 10 at peak | Increase averaging |
| Significance | >3σ for at least 3 λ values | Insufficient asymmetry |
| Linearity | R² > 0.8 for A vs λ | Check theory |

9.2 Null Tests

| Test | Procedure | Expected | Interpretation |
| --- | --- | --- | --- |
| Zero power | Measure at lowest power | No hysteresis | λ=0 baseline |
| Same cycle | Heat and cool identical | Δ=0 within noise | System symmetric |
| Different spot | Repeat on different NV | Similar magnitude | Reproducible |
| Zero field | No magnetic field | Same result | Not B-field artifact |

9.3 Systematic Error Budget

| Source | Uncertainty in A |
| --- | --- |
| Temperature measurement (±1 K) | ~2% |
| T₁ fitting (±5%) | ~5% |
| Laser power drift (±2%) | ~2% |
| Statistical (photon noise) | ~3% |
| Total combined | ~7-10% |

10. Expected Results and Interpretation

10.1 Predicted Signatures

| Observable | Prediction |
| --- | --- |
| T₁ at λ=0 | No hysteresis between cycles |
| T₁ at λ>0 | Different heating/cooling paths |
| Δ(T₁⁻¹) | Peaks near maximum temperature |
| Integrated A | Linear in λ |
| Temperature dependence | Peaks near Debye temperature |

10.2 Expected Values (Shallow NV)

| Quantity | Expected Value |
| --- | --- |
| T₁ at 300 K | 100 μs - 1 ms |
| T₁ at 500 K | 10-100 μs |
| Δ(T₁⁻¹) peak | 10³ - 10⁴ s⁻¹ |
| Integrated A | 10⁵ - 10⁶ s⁻¹·K |
| λ range | 0 - 0.8 |
| A(λ) slope | ~10⁶ s⁻¹·K per λ |

10.3 Interpretation Guidelines

| Observation | Interpretation |
| --- | --- |
| Linear A vs λ | ✅ Confirms λ-framework prediction |
| Zero at λ=0 | ✅ Onsager symmetry in equilibrium |
| Non-zero at λ>0 | ✅ Entropy weighting breaks reciprocity |
| Material dependence | ✅ Scales with spin-phonon coupling |
| Peak at Debye T | ✅ Consistent with phonon bath |

11. Troubleshooting Guide

11.1 No T₁ Signal

| Possible Cause | Solution |
| --- | --- |
| NV not found | Rescan, check alignment |
| Laser power too low | Increase power, check AOM |
| MW not resonant | Check frequency, calibrate B field |
| Detector dark | Check SPAD bias, cooling |

11.2 T₁ Too Short or Too Long

| Observation | Likely Cause | Solution |
| --- | --- | --- |
| T₁ < 10 μs | High NV density, impurities | Use different NV |
| T₁ > 10 ms | Very clean bulk | Use shallow NV |
| Large variation | Laser fluctuations | Stabilize power |

11.3 No Hysteresis

| Possible Cause | Solution |
| --- | --- |
| λ too small | Increase laser power |
| Thermal lag too fast | Check thermal contact |
| NV in symmetric environment | Try different NV |
| Temperature range too small | Extend range |

11.4 Asymmetry Not Linear in λ

| Possible Cause | Solution |
| --- | --- |
| λ calibration incorrect | Recalibrate using ODMR |
| Saturation effects | Reduce maximum power |
| Multiple NVs | Use single NV |
| Charge state fluctuations | Check initialization |

12. Data Archive Requirements

12.1 Folder Structure

text

nv_experiment_YYYYMMDD/

├── raw_data/

│   ├── spot_A/

│   │   ├── 0.1mW/

│   │   │   ├── heating_300K_T1.txt

│   │   │   ├── heating_300K_ODMR.txt

│   │   │   ├── cooling_300K_T1.txt

│   │   │   └── cooling_300K_ODMR.txt

│   │   ├── 0.2mW/

│   │   └── 0.5mW/

│   ├── spot_B/

│   └── spot_C/

├── calibration/

│   ├── temperature_calibration.csv

│   ├── power_calibration.csv

│   └── odmr_calibration.py

├── analysis/

│   ├── processed_data/

│   │   ├── t1_fits.csv

│   │   ├── rate_asymmetry.csv

│   │   └── lambda_results.csv

│   ├── figures/

│   │   ├── t1_decay_comparison.png

│   │   ├── asymmetry_vs_temperature.png

│   │   └── lambda_dependence.png

│   └── report.pdf

├── code/

│   ├── nv_lambda_analysis/

│   ├── requirements.txt

│   └── environment.yml

├── metadata/

│   ├── sample_info.pdf

│   ├── instrument_config.pdf

│   ├── experiment_log.pdf

│   └── photos/

└── README.md

12.2 README Template

markdown

# NV Center λ-Framework Experiment

## Experiment Information

- **Date:** YYYY-MM-DD

- **Operator:** [Name]

- **Sample ID:** [ID]

- **NV depth:** [value] nm

- **NV density:** [value] /μm²

## Sample Description

- **Substrate:** [electronic grade diamond]

- **Surface termination:** [oxygen/hydrogen]

- **Creation method:** [implantation/ native]

- **Annealing:** [temperature, time]

## Instrument Configuration

- **Microscope:** [homebuilt/commercial]

- **Laser 1:** [532 nm, power range]

- **Laser 2:** [637 nm, power range]

- **MW source:** [frequency range, power]

- **Temperature stage:** [model, range]

## Measurement Parameters

- **Temperature range:** [300-500] K

- **Ramp rate:** [10] K/min

- **Laser powers:** [0.1, 0.2, 0.5, 1.0] mW

- **T₁ τ range:** [1 ns - 10 ms]

- **Repetitions:** [10⁵] per point

## Key Results

| Power | λ | Area (μs⁻¹·K) | Significance |

|-------|---|---------------|--------------|

| 0.1 | 0.00 | [value] | [σ] |

| 0.2 | 0.15 | [value] | [σ] |

| 0.5 | 0.42 | [value] | [σ] |

| 1.0 | 0.83 | [value] | [σ] |

**Fit:** A(λ) = [slope]·λ + [intercept], R² = [value]

## Conclusion

✅ λ-framework validated: A ∝ λ with significance >3σ

## File Guide

- `raw_data/`: Original measurement files

- `calibration/`: Calibration data and scripts

- `analysis/`: Processed data and figures

- `code/`: Analysis code

## Reproduction

```bash

conda env create -f code/environment.yml

conda activate nv_lambda

python code/run_nv_analysis.py --data-dir ./raw_data

Contact

[Name]
[Email]

text

### 12.3 Figshare Upload Structure

Figshare record: "NV Center Spin Relaxation Asymmetry - λ-Framework Validation"

Files:

- SOP_nv_lambda_v1.0.pdf (this document)

- raw_data.zip (all raw measurement files)

- analysis_code.zip (complete Python package)

- processed_data.csv (all extracted quantities)

- figures.zip (publication-ready plots)

- calibration_data.zip (temperature and power calibrations)

- metadata.zip (sample info, instrument config, experiment log)

- README.md (reproduction guide)

- environment.yml (conda environment)

text