Appendices

Appendix 1 Flame/manual measurement conditions for AAS (Hitachi Z-2000)

| Device conditions | Zn | Cu | Pb | Cd | Sn |
| --- | --- | --- | --- | --- | --- |
| Measurement signal | BKG correction | BKG correction | BKG correction | BKG correction | BKG correction |
| Signal calculation | Integral | Integral | Integral | Integral | Integral |
| Measurement wavelength (nm) | 213.9 | 324.8 | 283.3 | 228.8 | 224.6 |
| Wavelength setting method | Automatic | Automatic | Automatic | Automatic | Automatic |
| Slit width (nm) | 1.3 | 1.3 | 1.3 | 1.3 | 1.3 |
| Time constant (s) | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| Lamp current (mA) | 5.0 | 7.5 | 7.5 | 7.5 | 12.0 |
| Photomultiplier voltage (V) | 352 | 243 | 274 | 321 | 392 |
| Analysis Conditions |  |  |  |  |  |
| Atomization device | Standard burner | Standard burner | Standard burner | Standard burner | Standard burner |
| Flame type | Air-C2H2 | Air-C2H2 | Air-C2H2 | Air-C2H2 | Air-C2H2 |
| Fuel gas flow rate (L/min) | 1.8 | 2.0 | 2.0 | 1.8 | 2.8 |
| Combustion support gas pressure (kPa) | 160 | 160 | 160 | 160 | 160 |
| Combustion-supporting gas flow rate (L/min) | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 |
| Burner height (mm) | 7.5 | 7.5 | 7.5 | 5.0 | 7.5 |
| Delay time (s) | 5 | 5 | 5 | 5 | 5 |
| Data acquisition time (s) | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 |

Appendix 2 Correlation values (r) of general sediment profiles

|  | Moisture | Redox | Sand fraction | Silt/clay fraction | TOM |
| --- | --- | --- | --- | --- | --- |
| Moisture | 1 |  |  |  |  |
| Redox | -0.51* | 1 |  |  |  |
| Sand fraction | -0.88* | 0.58* | 1 |  |  |
| Silt/clay fraction | 0.90* | -0.59* | -0.99* | 1 |  |
| TOM | 0.94* | -0.45* | -0.87* | 0.88* | 1 |

* = significant at p < 0.05 level

Appendix 3 Correlation (r) values of sediment profiles and Zn, Cu, Cd, Pb, Sn, Se

concentrations in sediment

| Sediment Profile | Correlation coefficient (r) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Zn | Cu | Cd | Pb | Se | Sn |
| Moisture | 0.76*** | 0.77*** | -0.11 | 0.90*** | -0.12 | -0.09 |
| Redox | -0.21 | -0.21 | 0.46* | -0.40* | 0.14 | -0.46* |
| Very coarse sand | -0.39* | -0.29 | 0.51** | -0.46* | -0.02 | -0.47** |
| Coarse sand | -0.49** | -0.45* | 0.39* | -0.60*** | -0.10 | -0.31 |
| Medium sand | -0.42* | -0.46* | 0.07 | -0.54** | -0.09 | 0.02 |
| Fine sand | -0.47** | -0.54** | -0.08 | -0.62*** | -0.003 | 0.25 |
| Very fine sand | 0.04 | -0.07 | -0.45* | 0.02 | 0.50** | 0.64*** |
| Silt/clay fraction | 0.59*** | 0.60*** | -0.24 | 0.74*** | -0.01 | 0.09 |
| Total organic matter | 0.72*** | 0.71*** | -0.12 | 0.85*** | -0.13 | -0.11 |
| Zn | 1 |  |  |  |  |  |
| Cu | 0.96*** | 1 |  |  |  |  |
| Cd | 0.28 | 0.27 | 1 |  |  |  |
| Pb | 0.92*** | 0.94*** | 0.05 | 1 |  |  |
| Se | 0.15 | 0.01 | 0.32 | -0.11 | 1 |  |
| Sn | -0.30 | -0.37* | -0.61*** | -0.21 | 0.19 | 1 |

***Correlation is significant at the 0.001 level. **Correlation is significant at the 0.01 level. *Correlation is significant at the 0.05 level.

Appendix 4 Eigenvalues screeplot for Principal Component Analysis

Supplementary Tables

Table S1 Sampling site coordinates and description

| Sampling Station | Latitude | Longitude | Distance from the mainland shoreline (m) | Depth (m) | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 11°11'25"N | 123°07'35"E | 317 | 9 | 317 m from the coal power plant |
| 2 | 11°11'41"N | 123°07'23"E | 159 | 6 | 150 m from the coast; 639 m from the coal power plant |
| 3 | 11°11'54"N | 123°07'46"E | 649 | 10 | 1,196 m from the coal power plant; 854 m from the coast |
| 4 | 11°12'09"N | 123°07'15"E | 170 | 4 | 149 m from the residential area; 96 m from the docking site of squid fishing vessel |
| 5 | 11°12'24"N | 123°07'33"E | 922 | 8 | 896 m from the coast; navigational path |
| 6 | 11°12'44"N | 123°07'27"E | 653 | 8 | 70 m from the seaweed farm; navigational path |
| 7 | 11°12'56"N | 123°07'40"E | 1,201 | 8 | 217 m from a mariculture farm |
| 8 | 11°12'47"N | 123°07'17"E | 498 | 1 | Within the seaweed farm |
| 9 | 11°12'47"N | 123°07'18"E | 529 | 1 | Within the seaweed farm |
| 10 | 11°12'48"N | 123°07'19"E | 576 | 1 | Within the seaweed farm |

Table S2 Tabular result of principal component analysis (PCA)

| Component | PC1 | PC2 | PC3 | PC4 |
| --- | --- | --- | --- | --- |
| Eigenvalue | 7.401 | 3.259 | 1.586 | 1.218 |
| Percentage of variance | 49.338 | 21.728 | 10.576 | 8.117 |
| Cumulative percentage | 49.338 | 71.066 | 81.641 | 89.759 |
| Redox | -0.672 | 0.503 | 0.220 | 0.098 |
| Very coarse sand | -0.631 | 0.499 | 0.173 | -0.472 |
| Coarse sand | -0.848 | 0.383 | -0.072 | -0.047 |
| Medium sand | -0.741 | -0.003 | -0.255 | 0.580 |
| Fine sand | -0.716 | -0.288 | -0.071 | 0.523 |
| Very fine sand | 0.295 | -0.705 | 0.541 | -0.023 |
| Silt/clay fraction | 0.958 | -0.116 | 0.011 | -0.224 |
| Moisture | 0.954 | 0.109 | -0.148 | 0.092 |
| Total organic matter | 0.927 | 0.123 | -0.137 | -0.019 |
| Zn | 0.768 | 0.437 | 0.180 | 0.387 |
| Cu | 0.764 | 0.512 | 0.069 | 0.276 |
| Cd | -0.170 | 0.780 | 0.414 | 0.063 |
| Pb | 0.888 | 0.334 | -0.071 | 0.208 |
| Se | -0.007 | -0.111 | 0.937 | 0.167 |
| Sn | 0.023 | -0.897 | 0.094 | 0.116 |

Extraction method: Principal Component Analysis