SUPPLEMENTARY MATERIAL

Development of Solid Lipid Nanoparticles Containing Budesonide for Inflammatory Bowel Disease (IBD) Based on the Quality by Design (QbD) Approach

Evaluation and Selection of Solidification Adsorbents for BUD-SLNs

Table S1. Flowability assessment of tested adsorbents, pure BUD, and BUD-SLN formulation (mean±SD, n=3).

| Powder | Bulk Density (g/mL) | Tapped Density (g/mL) | Hausner Ratio | Carr’s Index | Angle of repose (θ) | Flowability |
| --- | --- | --- | --- | --- | --- | --- |
| Florite® R | 0.08 ±0.001 | 0.12 ±0.001 | 1.5 | 33.3 | 47.0 | Poor |
| Florite PS-200 | 0.08 ±0.001 | 0.11 ±0.002 | 1.4 | 27.3 | 40.0 | Fair |
| Neusilin US2 | 0.17 ±0.02 | 0.2 ±0.01 | 1.2 | 15.0 | 30.0 | Excellent |
| Neusilin UFL2 | 0.1 ±0.01 | 0.13 ±0.00 | 1.3 | 23.0 | 45.0 | Passable |
| Syloid® XDP 3050 | 0.24 ±0.02 | 0.29 ±0.00 | 1.2 | 17.2 | 26.0 | Excellent |
| Syloid® XDP 3150 | 0.24 ±0.07 | 0.28 ±0.01 | 1.2 | 14.3 | 26.0 | Excellent |
| Pure BUD | 0.3 ±0.00 | 0.5 ±0.00 | 1.7 | 40.0 | 43.0 | Passable |
| BUD-SLNs: Syloid® XDP 3150 (1:0.1g) | 0.59 ±0.05 | 0.6 ±0.00 | 1.0 | 3.3 | 37.0 | Fair |

QbD Approach and Preliminary Studies

Table S2. QTPP for the capsule of BUD-SLNs.

| QTPP Elements | Target | Justification |
| --- | --- | --- |
| Dosage form | SLNs inside Capsule | Increasing the therapeutic effect with a low risk of adverse effects, a solid dosage form is a desired choice for the patient |
| Dosage design | Delayed release capsule (pH-dependent) | The most suitable for ileum targeting |
| Route of administration | Oral | Dosage form designed to administer orally, the most preferred route for delivery |
| Dosage strength | 3mg budesonide | Commonly accepted strength |
| Stability (physically, chemically, biologically) | During the preparation and after 3 months under different conditions no vision of gelation or aggregation, maintenance of the same content of uniformity during the storage period | Retain the therapeutic effect during the shelf-life |
| Pharmacokinetics | Delayed release enabling Tmax in 1-10 hours or less bioequivalent to RLD* | Bioequivalence requirement |
| Drug product quality attributes: Physical Attributes Particle Size Polydispersity Index Zeta Potential Entrapment Efficiency Lipolysis Dissolution | Must meet the same compendial or other applicable (quality) standards |  |
| Container closure system | Appropriate for storage of dosage form | Required to achieve the target shelf life |

*RLD: Reference Listed Drug

Table S3. Risk assessment of BUD-SLNs.

| Target | Material attribute, process parameter, failure mode, or independent variable | Critical quality attribute (CQAs), failure effect, response, or dependent variable | Risk Description | Severity | Occurrence | Detectability | RPN | Risk degree | Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLNs | Homogenization time (HT) (min) | Particle size and polydispersity index | Difficult to obtain nanoparticles, uniformity of size distribution, and a stable system. Particle size can affect directly drug release | 5 | 5 | 5 | 125 | Medium | Regular monitoring |
|  | Homogenization speed (rpm) | Particle size and polydispersity index | Insufficient mixing can lead to gelation and an unstable system | 7 | 1 | 5 | 35 | Low | Regular monitoring and maintenance |
|  | Speed of magnetic stirrer | Particle size, polydispersity index, and entrapment efficiency | Insufficient mixing might lead to preparing non-homogeneous emulsion and depositing lipid | 5 | 3 | 1 | 15 | Low | No action |
|  | Temperature | Particle size and polydispersity index | Low temperature might lead to lipid solidification, high temperature might lead to any effect | 3 | 5 | 1 | 15 | Low | No action |
|  | Concentration of lipid | Particle size, entrapment efficiency, and dissolution | Lipid's concentration can affect directly particle size, drug encapsulation, and drug release | 9 | 5 | 7 | 315 | High | Chosen solid lipid with high solubility of drug, small particle size, and high entrapment efficiency |
|  | Concentration of surfactant | Particle size, polydispersity index | Surfactant's concentration can affect directly particle size, drug release, and stability | 9 | 5 | 5 | 225 | High | Chosen surfactant with high solubility of drug, providing small particle size and stable system |

*RPN: Risk Priority Number

| Low | Additional investigation is not necessary since the risk is widely acceptable. |
| --- | --- |
| Medium | Additional investigation is recommended to reduce risk although the risk is acceptable. |
| High | Additional investigation is required to reduce the risk since the risk is unacceptable. |

Note: The RPN = (Severity) * (Occurrence) *(Detectability)

According to the RPN, the risk was classified as low, medium, and high, (1–97, indicated as green), (98–218, indicated as yellow), (219–729, indicated as red), respectively

Particle Size (PS) and Polydispersity Index (PDI) Determination

Figure S1. PS and PDI analysis of optimized liquid BUD-SLNs.

Differential Scanning Calorimetry (DSC)

Pure Bud

Optimized Bud-SLNs

Figure S2. DSC study of Pure BUD and Optimized BUD-SLNs.

In Vitro Cytotoxicity Studies

(b)

(a)

Figure S3. (a) IC50 value for BUD in Caco-2 cells, and (b) the effect of formulations containing BUD on cellular viability.