Occurrence and risk assessment of polybrominated diphenyl ethers in surface water and sediment of Nahoon River Estuary, South Africa

Chinemerem Ruth Ohoro1,2, Abiodun Olagoke Adeniji1,2,4, Lucy Semerjian5, Anthony Ifeanyi Okoh1,3,5 and Omobola Oluranti Okoh1,2.

1 SAMRC Microbial Water Quality Monitoring Centre, University of Fort Hare, Alice, 5700, South Africa.

2 Department of Pure and Applied Chemistry, University of Fort Hare, Alice, 5700, South Africa.

3Applied and Environmental Microbiology Research Group, Department of Biochemistry and Microbiology, University of Fort Hare, Alice, 5700, South Africa.

4 Department of Chemistry and Chemical Technology, National University of Lesotho. P.O.  180, Roma, Lesotho

5Department of Environmental Health Sciences, College of Medical of Health Sciences, University of Sharjah, Sharjah, United Arab Emirates.

Correspondence should be addressed to Chinemerem Ruth Ohoro; greatnemerem@yahoo.co.uk

Supplementary Tables

Table S1: Calibration and linearity of PBDEs method

| PBDEs | Retention time (mins) | Surface water recovery(%) | Sediment recovery(%) | Linearity (R2) | LOD (ng/L) | LOQ (ng/L) | RSD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BDE 17 | 15.414 | 88.88±2.85 | 64.48±0.38 | 0.9989 | 0.002 | 0.0063 | 3.61 |
| BDE 47 | 19.46 | 117.37±0.10 | 97.00±0.55 | 0.9979 | 0.0014 | 0.0042 | 4.18 |
| BDE 66 | 19.956 | >140 | 94.32±0.96 | 0.9976 | 0.0020 | 0.0060 | 5.03 |
| BDE 100 | 22.227 | 102.47±0.46 | 40.65±0.01 | 0.9979 | 0.0011 | 0.0033 | 2.47 |
| PCB 209 | 24.52 | 45.30±0.27 | 47.58± 1.65 | 0.982 | 0.0247 | 0.0748 | 1.11 |
| BDE 153 | 26.965 | 118.74±0.01 | 72.16±1.60 | 0.997 | 0.0021 | 0.0063 | 3.05 |
| BDE 183 | 31.833 | 122.67±0.05 | 68.22±1.82 | 0.9967 | 0.0030 | 0.0092 | 1.74 |

LOD =  Limit of detection; LOQ = limit of quantification; RSD- relative standard deviation

Table S2: Matrix of Pearson correlation among analytes and water quality  parameters

|  | BDE 17 | BDE 47 | BDE 100 | BDE 153 | BDE 183 | Temp.[°C] | pH | EC[MS/cm] | TDS [g/L] | Sal.[psu] | Turb.FNU | ORP[mV] | RES[Ohm-cm ] | DO | TSS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BDE 17 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BDE 47 | 0.912 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BDE 100 | 0.786 | 0.933 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| BDE 153 | 0.287 | 0.338 | 0.474 | 1 |  |  |  |  |  |  |  |  |  |  |  |
| BDE 183 | 0.78 | 0.816 | 0.731 | -0.22 | 1 |  |  |  |  |  |  |  |  |  |  |
| pH | 0.369 | 0.036 | 0.017 | -0.25 | 0.348 | 1 |  |  |  |  |  |  |  |  |  |
| EC | -0.87 | -0.77 | -0.77 | -0.09 | -0.86 | -0.64 | 1 |  |  |  |  |  |  |  |  |
| TDS | -0.81 | -0.86 | -0.79 | 0.127 | -1 | -0.33 | 0.883 | 1 |  |  |  |  |  |  |  |
| Sal | -0.81 | -0.86 | -0.79 | 0.128 | -1 | -0.34 | 0.883 | 1 | 1 |  |  |  |  |  |  |
| Turb | -0.81 | -0.86 | -0.79 | 0.13 | -1 | -0.34 | 0.884 | 1 | 1 | 1 |  |  |  |  |  |
| Temp | 0.215 | 0.035 | 0.252 | 0.285 | 0.104 | 0.73 | -0.55 | -0.14 | -0.15 | -0.15 | 1 |  |  |  |  |
| Mvorp | 0.787 | 0.905 | 0.991 | 0.438 | 0.754 | 0.13 | -0.83 | -0.81 | -0.81 | -0.81 | 0.361 | 1 |  |  |  |
| RES | 0.449 | 0.529 | 0.716 | 0.037 | 0.704 | 0.424 | -0.79 | -0.73 | -0.73 | -0.73 | 0.629 | 0.791 | 1 |  |  |
| TSS | -0.5 | -0.39 | -0.53 | -0.05 | -0.58 | -0.77 | 0.851 | 0.606 | 0.608 | 0.609 | -0.864 | -0.638 | -0.88 | 1 |  |
| DO | 0.996 | 0.934 | 0.809 | 0.342 | 0.756 | 0.284 | -0.83 | -0.79 | -0.79 | -0.79 | 0.162 | 0.799 | 0.416 | -0.44 | 1 |

Table S3: Matrix of Pearson correlation among sediment parameters

|  | BDE 17 | BDE 47 | BDE 66 | BDE 100 | BDE 153 | BDE 183 | MC | OC | OM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BDE 17 | 1 |  |  |  |  |  |  |  |  |
| BDE 47 | 0.589 | 1 |  |  |  |  |  |  |  |
| BDE 66 | 0.379 | 0.964 | 1 |  |  |  |  |  |  |
| BDE 100 | 0.391 | 0.972 | 0.988 | 1 |  |  |  |  |  |
| BDE 153 | 0.37 | 0.952 | 0.966 | 0.991 | 1 |  |  |  |  |
| BDE 183 | 0.262 | 0.921 | 0.963 | 0.985 | 0.993 | 1 |  |  |  |
| MC | -0.73 | -0.94 | -0.836 | -0.86 | -0.82 | -0.77 | 1 |  |  |
| OC | 0.351 | 0.396 | 0.353 | 0.294 | 0.171 | 0.16 | -0.564 | 1 |  |
| OM | 0.351 | 0.396 | 0.353 | 0.294 | 0.171 | 0.16 | -0.564 | 1 | 1 |

Table S4: Concentrations of PBDEs in surface water and sediments of sampling sites in Nahoon River Estuary in ng/L and ng/g, respectively

|  | Spring |  |  |  |  |  |  | Summer |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Surface water |  |  |  |  |  |  |  |  |  |  |  |
| Congener | NH1 | NH2 | NH3 | NH4 | NH5 |  | NH1 | NH2 | NH3 | NH4 | NH5 |
| BDE 17 | 246.96 | 192.15 | 121.70 | BDL | 136.80 |  | 70.20 | 47.43 | BDL | BDL | BDL |
| BDE 47 | 189.90 | 89.32 | 4.94 | 4.27 | 56.33 |  | 6.52 | 6.41 | 12.39 | 8.87 | 5.43 |
| BDE 100 | 178.34 | 93.95 | 4.25 | 4.43 | 5.69 |  | 8.68 | 9.82 | 11.53 | 7.36 | 7.38 |
| BDE 153 | 6.39 | 4.97 | 5.11 | 5.59 | 7.31 |  | 7.31 | 5.82 | 8.00 | 5.94 | 5.86 |
| BDE 183 | 5.56 | 5.81 | 5.72 | 5.41 | 6.42 |  | 8.80 | 8.36 | 5.97 | 6.83 | 5.14 |
| Sediment |  |  |  |  |  |  |  |  |  |  |  |
| BDE 17 | 1.96 | 2.77 | 0.50 | 0.45 | - |  | 13.87 | 15.35 | 32.10 | 0.14 | 25.47 |
| BDE 47 | 0.29 | 0.24 | 0.29 | 0.27 | - |  | 2.43 | 2.22 | 8.75 | 0.28 | 2.01 |
| BDE 66 | 3.42 | 1.18 | 0.27 | 0.99 | - |  | 6.44 | 2.72 | 31.39 | 0.29 | 3.54 |
| BDE 100 | 0.26 | 0.27 | 0.21 | 0.41 | - |  | 4.29 | 3.44 | 25.69 | 0.26 | 2.57 |
| BDE 153 | 0.08 | 0.27 | 0.16 | 0.23 | - |  | 1.01 | 1.29 | 25.12 | 0.63 | 1.96 |
| BDE 183 | 0.54 | 0.59 | 0.47 | 0.64 | - |  | 0.25 | 0.69 | 111.98 | 0.37 | 0.36 |

BDL: below detection limit