!# Purpose: Iterate over different contact lens configurations and simulate wide-field retinal images under a controlled set of contact lens positions as a function of emulated blink action, modelled using decentration and tilt conditions, for multiple pupils. Outputs image frames as montages can then be used for downstream virtual retina processing. #!

!# Lens design labels, serves as example only

LENS_SET            = ["SV"]

!# Example stimulus, replace as needed

INPUT_SCENE         = "lena_hr.png"

!# Pupils to simulate, example only

PUPIL_LIST_MM       = [4, 5, 6]

!# Vertical decentration sweep, example only

DECY_RANGE_MM       = [-0.50 : +0.50 : 0.025]

!# Horizontal decentration drawn from population Gaussian(σ), example only

DECX_STD_MM         = 0.30

!# Coupling tilt-x per mm horizontal decentration, example only

TILT_GAIN_X         = 7.0

!# Coupling tilt-y per mm vertical decentration, example only

TILT_GAIN_Y         = -7.0

!# Image sampling parameters (generic), example only

ISM_PSAMP = 4

ISM_ISAMP = 4

ISM_PSFX = 21

ISM_PSFY = 21

OVERSAMPLING = 0

GUARDBAND = 0

!# Note: The complete lens geometry and all feature dimensions for the SV and BF designs are provided through fully transparent ZOS files, including configuration files. The AR design details, which represent the novel aspects of this work, are described comprehensively in the manuscript. Transmittance maps and proprietary UDS files are intentionally omitted to protect intellectual property. However, a clear description of the design principles and the pipeline required to reproduce such designs is included, enabling readers to understand the process and, if desired, replicate the methodology using their own designs.#!

!# High-level procedure

FOR EACH lens_label IN LENS_SET DO

!# LOADLENS: abstracted; in practice, load the. ZMX/.ZOS file associated with lens_label.

LOAD_LENS_FOR(lens_label)

!# Derive a run-specific settings file name

settings_cfg = MAKE_TEMP_SETTINGS(lens_label)

!# Configure image simulation module (ISM) with neutral parameters.

ISM_SET_INPUT(settings_cfg, INPUT_SCENE)

ISM_SET_SAMPLING(settings_cfg, PSAMP=ISM_PSAMP, ISAMP=ISM_ISAMP)

ISM_SET_PSF(settings_cfg, PSFX=ISM_PSFX, PSFY=ISM_PSFY)

ISM_SET_GUARDS(settings_cfg, oversampling=OVERSAMPLING, guardband=GUARDBAND)

FOR EACH pupil_mm IN PUPIL_LIST_MM DO

!# Generic pupil control; avoids system-specific calls

SET_PUPIL(pupil_mm)

frame_index = 1

FOR EACH decy_mm IN DECY_RANGE_MM DO

decx_mm = GAUSSIAN( mean=0.0, std=DECX_STD_MM )

!# Decentration–tilt coupling (safe abstraction of SURP calls)

!# If no decentration, no tilt. Otherwise, apply proportional tilt.

IF (decy_mm == 0 AND decx_mm == 0) THEN

tilx = 0.0

tily = 0.0

ELSEIF (decy_mm == 0 AND decx_mm != 0) THEN

tilx = TILT_GAIN_X * decx_mm

tily = 0.0

ELSEIF (decy_mm != 0 AND decx_mm == 0) THEN

tilx = 0.0

tily = TILT_GAIN_Y * decy_mm

ELSE

tilx = TILT_GAIN_X * decx_mm

tily = TILT_GAIN_Y * decy_mm

ENDIF

!# Apply transforms to contact lens surfaces (front/back).

SET_CL_FRONT( decx=decx_mm, decy=decy_mm, tilx=tilx, tily=tily, tilz=0.0 )

SET_CL_BACK ( decx=decx_mm, decy=decy_mm, tilx=tilx, tily=tily, tilz=0.0 )

!# Image simulation

out_name = MAKE_SAFE_FILENAME(lens_label, pupil_mm, frame_index)

!# e.g., "<lens>_P<mm>_F<idx>.png"

ISM_SET_OUTPUT(settings_cfg, out_name)

!# Abstract ZPL OPENANALYSISWINDOW

OPEN_SIM_WINDOW(settings_cfg)

!# Abstract engine call

RUN_SIMULATION(settings_cfg)

!# Abstract ZPL CLOSEWINDOW

CLOSE_SIM_WINDOW()

frame_index = frame_index + 1

NEXT decy_mm

NEXT pupil_mm

!# Restore lens to neutral configuration before moving to next design

!# Decenter=0, Tilt=0, Mode=default

RESET_CL_FRONT()

RESET_CL_BACK()

SAVE_LENS_STATE(lens_label)

!# Optional: final neutral save

NEXT lens_label