""" Python Code to generate 40 dynamic grating images and an optional GIF for retinal image simulations.

Outputs:

- 01.png, 02.png, ..., 40.png  (in ./dynamic_gratings_40/)

- sinusoidal_grating.gif

Parameters:

- size_px: image size in pixels (default 4096)

- envelope_sigma: Gaussian window sigma in normalised units [-1, +1]

- freq sweep: linearly from 1 to 30 (40 steps)

- angle sweep: 0 to 360 degrees (40 steps)

Notes: Frequency is applied as cycles per unit along the rotated X-axis of the normalised coordinate grid; reviewers may map this to cpd in their own optical model by calibrating units to their retinal sampling scale. """

import os

import numpy as np

import matplotlib.pyplot as plt

import matplotlib.animation as animation

# Deterministic behaviour

np.random.seed(0)

# Configurable parameters

SIZE_PX = 4096

# Gaussian sigma (in normalised units)

ENVELOPE_SIGMA = 0.50

OUT_DIR = "dynamic_gratings_40"

# Set False if GIF is not needed

MAKE_GIF = True

def generate_grating(freq, angle_rad, size_px=SIZE_PX, envelope_sigma=ENVELOPE_SIGMA):

# Normalised coordinates in [-1, 1]

x = np.linspace(-1, 1, size_px)

y = np.linspace(-1, 1, size_px)

X, Y = np.meshgrid(x, y)

# Rotate coordinate system by angle_rad

X_rot = X * np.cos(angle_rad) - Y * np.sin(angle_rad)

# Y_rot is unused here, but kept to show full rotation logic for reviewers

Y_rot = X * np.sin(angle_rad) + Y * np.cos(angle_rad)

# Sinusoidal grating along rotated X axis

grating = np.sin(np.pi * freq * X_rot)

# Gaussian envelope to reduce edge artefacts in downstream pipelines

gaussian_envelope = np.exp(-(X**2 + Y**2) / (2 * (envelope_sigma ** 2)))

return grating * gaussian_envelope

def main():

os.makedirs(OUT_DIR, exist_ok=True)

# Prepare for optional GIF

fig, ax = plt.subplots(figsize=(SIZE_PX / 100.0, SIZE_PX / 100.0), dpi=100)

ax.axis("off")

ims = []

for i in range(40):

# Frequency: linear sweep 1..30 across 40 frames

freq = 1 + (29 / 39) * i

# Rotation: 0..360 degrees across 40 frames

angle_rad = (2 * np.pi / 40) * i

grating = generate_grating(freq=freq, angle_rad=angle_rad)

# Save PNG frame with 2-digit numbering

png_name = os.path.join(OUT_DIR, f"{i + 1:02}.png")

plt.imsave(png_name, grating, cmap="gray", vmin=-1, vmax=1)

if MAKE_GIF:

im = ax.imshow(grating, animated=True, cmap="gray", extent=[-1, 1, -1, 1], vmin=-1, vmax=1)

ims.append([im])

if MAKE_GIF:

ani = animation.ArtistAnimation(fig, ims, interval=50, blit=True)

ani.save(os.path.join(OUT_DIR, "sinusoidal_grating.gif"), writer="pillow")

plt.close(fig)

print(f"Generated {len(os.listdir(OUT_DIR))} files in {OUT_DIR}")

if __name__ == "__main__":