SUPPLEMENTARY MATERIAL

Occurrence and short-term variability of psychoactive substances in wastewater from a mixed urban catchment in central Chile

Wendy Calzadilla1*, Karla Montenegro2,3, Andrés Yar2,3, Sebastián Campos2,3, Ricardo Salazar-González2,3*

1 Departamento de Química, Universidad Técnica Federico Santa María, Campus San Joaquín, Santiago. Chile

2 Grupo de Investigación de Análisis, Tratamiento, Electroquímica, Recuperación y Reúso de Agua (WATER2), Departamento de Química Inorgánica, Facultad de Química y de Farmacia, Pontificia Universidad Católica de Chile, Santiago. Chile

3Centro de Derecho y Gestión de Aguas. Pontificia Universidad Católica de Chile, Santiago, Chile

*Corresponding author: wendy.calzadilla@usm.cl, https://orcid.org/0000-0002-1082-7875

*Corresponding author: ricardo.salazar@uc.cl, https://orcid.org/0000-0003-2180-6022

Summary

Pages S1-S14, Tables S1-S5, Figure S1.

Chemical and materials

Standard stock solutions of each compound: amphetamine (AMP), methamphetamine (METH), MDMA, cocaine (COC), benzoylecgonine (BE), ketamine (KET), phentermine (PHT), fentanyl (FEN), and norfentanyl (NOR) were prepared at 100 mg L−1 in MeOH or ACN. Intermediate solutions (20 mg L−1 and 0,5 mg L−1) were prepared by diluting the stock solution with MeOH. Mixed working solutions containing all analytes were prepared by diluting intermediate solutions with Milli-Q water. These mixtures were then used to prepare calibration standards, internal quality controls, and to spike samples during the validation study.

Individual stock solutions of isotopically labelled internal standards (ILIS) of amphetamine-d6, methamphetamine-d5, MDMA-d5, cocaine-d3, benzoylecgonine-d3, phentermine-d5, fentanyl-d5, norfentanyl-d5, and ketamine-d4 were prepared in MeOH or ACN at a concentration of 200 and 20 mg L−1. Standard working solutions mixed at 10 and 20 μg L−1 were used as a surrogate internal standard. All standard solutions were stored at −20 °C. SPE cartridges used were Chromabond HLB (6 mL, 200 mg) from Macherey-Nagel (Düren, Germany).

Sample collection

All wastewater (WW) samples were collected in high-density polyethylene bottles and transported directly to the laboratory, where they were fractionated, acidified to pH 2, and immediately stored in the dark at -20 °C until analysis.

The ammonium ion (NH4+) was measured in the separate fraction of each sample and converted to ammonia nitrogen (NH₃–N) for population normalization, considering a 1:2 dilution, using the specific HI93733-01 kit purchased from Hanna Instruments for ranges between 0 and 50 mg·L-1. The measures were taken in a Hanna spectrophotometer, model HI83214. The instrument directly displays the concentration of NH4+ in mg L-1. To convert the reading to mg L-1 of ammonia nitrogen (NH3-N), it was multiplied by the factor 0.778 (Hanna Instrument, 1999). Based on the NH3-N values, equivalent population (EP) estimates (Table 1SM, Supplementary Material) were calculated independently for each sampling day using Equation S1 in order to capture short-term fluctuations in the contributing population.

Equation S1

IENH3-N=Q⋅CNH3-NLNH3-N,inh

Where, CNH3-N refers to the concentration of NH3-N expressed in g·L-1, Q corresponds to the daily flow rates (L·day−1), and LNH3-N,inh is the average daily NH3-N load per inhabitant (g day-1 inhabitants-1). Table S2 in Supplementary Material presents estimated average daily NH3-N load values per inhabitant for several cities worldwide.

As no Chile-specific per-capita NH₃–N load values are currently available, a reference value of 8 g· day⁻¹·· inh⁻¹ was adopted. This value lies within the range commonly reported for wastewater treatment plants (WWTP) receiving mixed domestic and industrial WW (5–12 g· day⁻¹ · inh⁻¹) and has been widely applied in WBE studies for population normalization (Been et al., 2014; Metcalf & Eddy, 2013; Si-Yu Liu et al., 2021). Although the use of an externally derived per-capita load introduces uncertainty in the absolute EP estimates, this approach allows for consistent normalization across sampling days and supports the interpretation of short-term temporal variability in population size.

| Sampling date | 2024 |  |  |  |  |  |  |  | 2025 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mon | Tues | Wed | Thur | Mon | Tues | Wed | Average ± SD |  | Tues | Wed | Thur | Fri | Sat | Sun | Mon | Average ± SD |
|  | 29-04-2024 | 30-04-2024 | 01-05-2024 | 02-05-2024 | 08-07-2024 | 09-07-2024 | 10-07-2024 |  |  | 13-05-2025 | 14-05-2025 | 15-05-2025 | 16-05-2025 | 17-05-2025 | 18-05-2025 | 19-05-2025 |  |
| NH3-N (g L-1) | 0.037 | 0.042 | 0.033 | 0.045 | 0.041 | 0.042 | 0.042 | 0.040 ± 0.004 |  | 0.042 | 0.044 | 0.043 | 0.034 | 0.033 | 0.023 | 0.037 | 0.036 ± 0.007 |
| Flow rate (m3 day-1) | 3797 | 3363 | 3309 | 3729 | 3576 | 3373 | 3671 | 3545 ± 197 |  | 4231 | 3719 | 3969 | 3637 | 2984 | 4058 | 4251 | 3836 ± 443 |
| EP | 17615 | 17512 | 13596 | 21017 | 18127 | 17550 | 19226 | 17806 ± 2249 |  | 22162 | 20252 | 21102 | 15350 | 12303 | 11774 | 19487 | 17490 ± 3976 |

Table S1. Measured NH₃–N concentrations, daily influent flow rates, and estimated equivalent population.

EP: equivalent population

Table S2. Per-capita NH₃–N loads (g·day⁻¹·inhabitants⁻¹) reported for different wastewater catchments worldwide and used to define the sensitivity range applied in this study.

| Country | City | Industry contributions | NH3-N (g·day-1· inh-1) | References |
| --- | --- | --- | --- | --- |
| Switzerland | Several cities | High | 8.1 | (Been et al., 2014) |
| Switzerland | Lausanne | Low | 6.9 | (Been et al., 2014) |
| China | Several cities | n.s | 5-11 | (Si-Yu Liu et al., 2021) |
| Greece | n.s | n.s | 8-10 | (Metcalf & Eddy, 2013) |
| United States | n.s | n.s | 5-12 | (Metcalf & Eddy, 2013) |

n.s: not specified by the authors.

Analytical procedure

For both procedures, WW samples were adjusted to pH 7 and centrifuged for 10 minutes at 9500 rpm. For the direct injection procedure, 250 µL of supernatant was taken, and 100 µL of ILIS solution (10 μg L−1) was added, making up to 1mL, resulting in four-fold diluted samples.

For the SPE procedure, a 25-mL aliquot was taken for the supernatant, then 50 µL of ILIS solution (20 μg L−1) was added, and the solution was made up to 100 mL, resulting in four-fold diluted samples. The SPE was carried out using Chromabond HLB (6 mL, 200 mg) cartridges from Macherey-Nagel (Düren, Germany). The cartridges were conditioned by washing and rinsing with 6 mL of MeOH and 6 mL of Milli-Q water (3 mL for 2 times). After conditioning, the samples were passed through the cartridges under gravity and then dried under vacuum for 30 min. The analytes were eluted by gravity using 8 mL of MeOH (4 mL twice), and the extract was evaporated to dryness at 35 °C under a gentle stream of nitrogen. Finally, the residue was redissolved in 1 mL Milli-Q water:MeOH (90:10, v/v), sonicated in an ultrasonic bath for 5 minutes, and transferred to a glass vial for liquid chromatography-tandem mass spectrometry (LC-MS/MS) analysis. The procedural and field blanks, consisting of 100 mL of Milli-Q water and Milli-Q water stored under the same conditions as the actual samples, respectively, were processed together with the WW samples to verify that there was no contamination.

Instrumentation

LC-MS/MS using an Ekspert ultraLC 100- XL HPLC system, coupled to a triple quadruple mass spectrometer TripleQuad 4500 AB SCIEX (Ontario, Canada) fitted with a Turbo Ion Spray source. Chromatographic separation was conducted in an Inertsil ODS-4 HP (100 x 2.1 mm, 3 μm) from GL Sciences (Tokyo, Japan) with a mobile phase composed of (A) 0.1% formic acid in Milli-Q water and (B) methanol. The elution gradient for analysis was as follows: 0 min (5% B), 8 min (90% B), 8.1 min (5% B), and 10 min (5% B). The flow rate was 0.4 mL min–1 and the injection volume was 10 μL. The MS analysis was done in positive mode under the following conditions: ion spray voltage (ISV), +5500 V; curtain gas (CUR), 30; collision gas (CAD), 9; source temperature, 400°C; ion source gas 1 (GS1), 70, and gas 2 (GS2), 50. Dwell times of 0.015 s/transition were selected. Data were acquired and processed using Analyst® 1.6.2 and MultiQuantTM 3.0 software.  The three most abundant fragments of the protonated pseudomolecular ions of each analyte were selected. The MS/MS parameters and RT for all compounds are provided in Table S3, Supplementary Material. Identification and confirmation of positives were performed considering the ion ratio (q/Q ratio) and RT. The ratio between the qualitative and quantitative transitions (q/Q ratio) between samples and analytical standards should be lower than 30%, and RT error less than ± 0.2 min (SANTE, 2021). The quantification of the analytes was performed using their respective ILIS analogues, which are also reported in Table S3.

Table S3. MS/MS parameters: precursor and product ions, collision energy (CE), cell exit potential (CXP), declustering potential (DP) and retention time (RT).

| Analyte | ILIS | Retention time (min) | Precursor (m/z) | Product (m/z) | CE | CXP | DP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Amphetamine | Amphetamine-d6 | 3.67 | 136 | 119 | 11 | 20 | 11 |
|  |  |  |  | 91* | 30 | 9 |  |
|  |  |  |  | 65 | 53 | 11 |  |
| Benzoylecgonine | Benzoylecgonine-d3 | 4.67 | 290 | 168.2* | 25 | 16 | 66 |
|  |  |  |  | 105 | 38 | 14 |  |
|  |  |  |  | 82 | 45 | 17 |  |
| Cocaine | Cocaine-d3 | 4.90 | 304 | 182.2* | 25 | 24 | 26 |
|  |  |  |  | 82 | 41 | 4 |  |
|  |  |  |  | 77 | 86 | 12 |  |
| Ketamine | Ketamine-d4 | 4.53 | 238 | 125* | 42 | 11 | 43 |
|  |  |  |  | 179.1 | 22 | 11 |  |
|  |  |  |  | 207 | 20 | 10 |  |
| Methamphetamine | Methamphetamine-d5 | 3.80 | 150 | 91* | 28 | 13 | 31 |
|  |  |  |  | 119 | 15 | 17 |  |
|  |  |  |  | 65 | 52 | 14 |  |
| MDMA | MDMA-d5 | 3.96 | 194 | 136.2* | 18 | 19 | 42 |
|  |  |  |  | 105.1 | 34 | 13 |  |
|  |  |  |  | 77 | 56 | 19 |  |
| Phentermine | Phentermine-d5 | 3.80 | 150 | 91.1* | 28 | 13 | 31 |
|  |  |  |  | 65.1 | 52 | 14 |  |
|  |  |  |  | 119.1 | 15 | 17 |  |
| Fentanyl | Fentanyl-d5 | 5.61 | 336.7 | 188* | 30 | 17 | 93 |
|  |  |  |  | 155 | 55 | 16 |  |
|  |  |  |  | 216 | 31 | 12 |  |
| Norfentanyl | Norfentanyl-d5 | 4.59 | 233.2 | 150* | 28 | 16 | 10 |
|  |  |  |  | 84 | 25 | 13 |  |
|  |  |  |  | 177 | 22 | 18 |  |
| Amphetamine-d6 |  | 3.63 | 142.2 | 93 | 22 | 6 | 41 |
| Benzoylecgonine-d3 |  | 4.60 | 293.1 | 171.1 | 29 | 15 | 35 |
| Cocaine-d3 |  | 4.90 | 307.1 | 185.2 | 25 | 11 | 33 |
| Ketamine-d4 |  | 4.49 | 242.2 | 129 | 33 | 8 | 12 |
| Methamphetamine-d5 |  | 3.78 | 155 | 121 | 15 | 17 | 33 |
| MDMA-d5 |  | 3.93 | 199 | 165 | 19 | 11 | 41 |
| Phentermine-d5 |  | 4.31 | 155.1 | 96 | 28 | 10 | 37 |
| Fentanyl-d5 |  | 5.59 | 342.2 | 188.1 | 31 | 11 | 33 |
| Norfentanyl-d5 |  | 4.55 | 238 | 84 | 25 | 12 | 80 |

Method validation and quality assurance

Linearity was evaluated by analyzing both standard solutions in solvent (Milli-Q water:MeOH, 90:10) and matrix (untreated WW) at twelve concentration levels ranging from 0.005 to 15 μg L−1 to determine whether there were differences in the slope (>20%) that could affect quantification. Linearity was considered acceptable by applying a linear regression model with a 1/x weighting factor, and the coefficient of correlation (r) was ≥ 0.99.  For each analytical batch, a complete calibration curve was injected at the beginning and end to verify linearity (correlation coefficients).

The precision and accuracy of the method were evaluated using recoveries and repeatability by analyzing extracted untreated WW samples in quintuplicate (n=5) at three concentration levels, 0.5, 2, and 10 µg L-1. In the direct injection method, precision was calculated as the relative standard deviation (RSD) of the repeated injections. A tolerance interval of 70–120% was applied to define acceptable recoveries, under the condition that the method demonstrated satisfactory precision, expressed as a repeatability RSD < 20%. In addition, quality control (QC) samples were processed and analyzed at two levels, 0.5 and 2 µg L-1, for each different sample injected, to ensure the reliability of the analytical procedure. Results were considered acceptable when the QCs samples showed apparent recoveries within the 60–140% range. Furthermore, blanks were injected after every six sample injections to detect potential contamination and apply any necessary corrections.

The limits of detection (LOD) and limits of quantification (LOQ) for the whole method were determined using the quantification transition from extracted or non-extracted WW samples when the “natural” concentrations were sufficiently high to be measured directly. In cases where the samples exhibited low concentrations, the LOD and LOQ were obtained by spiking WW with low levels of psychoactive substances, followed by the corresponding extraction. The LOD was defined as the concentration yielding a signal-to-noise ratio of 3, and the LOQ as the concentration yielding a signal-to-noise ratio of 10.

Matrix effects were evaluated by comparing the peak areas (uncorrected by internal standard) obtained for extracted WW samples post spiked at three concentration levels, 0.5, 2, and 10 µg L⁻¹ (same levels as in the accuracy and precision study) after subtracting the blank (n=3), with those from solvent (Milli-Q water:MeOH, 90:10) fortified at the same levels. According to (Cortese et al., 2020; Matuszewski et al., 2003) a matrix effect of 100% reflects the absence of signal alteration (no matrix effect), whereas values above 100% indicate signal enhancement, and values below 100% correspond to signal suppression due to the matrix.

Back-calculation of normalized mass loads and consumption of licit and illicit psychoactive substances

Equation S2

Daily Consumption mg⋅day-1⋅1000 inhabitants-1=C⋅Q⋅10-6⋅1000P⋅Cf

Where C correspond to measured concentrations in (ng L−1), Q correspond to the daily WW flow rates (L·day−1), P represents the equivalent population (EP) estimated from NH₃–N according to Equation S1 and Cf is the specific correction factor used to convert the normalized mass loads found in influent into the amounts of drugs daily consumed for each psychoactive substance. Table S4 in Supplementary Material lists the urinary excretion rates and the referenced Cf used from literature or calculated using Equation S3.

Equation S3

Cf=1Excretion⋅MWparentMWbiomarker

where Excretion is the fractional urinary excretion of the parent drug or its metabolite, and MWparent and MWbiomarker are the molar masses of the parent drug and the target biomarker, respectively.

Equation S4

Equivalent doses (ED) per 1000 inhabitants=(mg . day-1 .1000 inhabitants-1)D mg day-1

This conversion was performed by dividing the mass load by the usual therapeutic daily dose D reported in Table S4, which is the most prescribed regimen for PHT both in Chile and internationally.

This approach provides a population normalized metric that enables comparison with clinical consumption patterns.

Equation S5

% treated=ED1000×100

The proportion of the population potentially treated with PHT was estimated using Equation S5. This metric expresses the fraction of the population whose consumption is compatible with one therapeutic daily dose and was calculated by dividing the equivalent doses (ED) per 1000 inhabitants. The resulting percentage reflects the treated population equivalent inferred from WW and allows for an indirect comparison with epidemiological indicators such as the national prevalence of overweight and obesity.

Pharmacokinetic considerations and correction factor assumptions

Cf applied in this study were selected based on published urinary excretion data and commonly adopted practices in WBE. A summary of biomarkers, excretion fractions, Cf and dose assumptions is provided in Table S4.

In the case of COC, urinary excretion of BE substantially exceeds that of the parent compound and shows greater stability in wastewater matrices. Reported excretion fractions vary widely depending on the route of administration; therefore, a mean value reported in the literature was adopted to derive the Cf used in this study(Castiglioni et al., 2013).

For ATS, reported urinary excretion data indicates that the parent compounds represent the dominant fraction excreted, supporting their use as biomarkers for back-calculation (Gracia-Lor et al., 2016; van Nuijs et al., 2011). Cf applied for these substances were taken from established WBE studies and are summarized in Table S4.

PHT excretion is strongly influenced by urinary pH. While higher unchanged excretion fractions have been reported under experimentally acidified conditions, studies conducted under uncontrolled urinary pH indicate substantially lower unchanged fractions (Baselt, 1989). To avoid overestimation of population-level consumption, a conservative excretion fraction representative of uncontrolled conditions was adopted. Based on this assumption, mass loads were further converted into therapeutic-equivalent doses to estimate the potentially treated fraction of the population.

For KET, available pharmacokinetic data and wastewater observations remain inconsistent with respect to the relative abundance of the parent compound and its metabolites. In addition, potential deconjugation processes and direct disposal may contribute to elevated WW concentrations (Castiglioni et al., 2015; Du et al., 2020; Lai et al., 2013; Van Wichelen et al., 2025). For these reasons, KET mass loads are frequently reported without correction in WBE studies and are presented as measured in this work.

Table S4. Biomarkers used, excretion rates, Cf and average doses used for back-calculation of consumption of psychoactive substances.

| Compound | Biomarker | Excretion rate (%) | Cf | Typical dose (mg) |
| --- | --- | --- | --- | --- |
| PHT | PHT | 48a | 2.08 | 18.75 td 37.5 od |
| COC | BE | 29b | 3.59b |  |
| KET | KET | - | - |  |
| AMP | AMP | 36c | 2.77c |  |
| METH | METH | 40.9c | 2.44c |  |
| MDMA | MDMA | 22.5c | 4.4c |  |

td: twice a day, od: one a day

a (Lemas et al., 2021), b (Castiglioni et al., 2013), c (Gracia-Lor et al., 2016).

Table S5. Matrix-effect corrected values and calibration linearity (R²) in solvent (Milli-Q water: MeOH, 90:10) and wastewater, with percentage differences in calibration slopes.

| Analyte | Matrix effects corrected (%) |  |  | Linearity (Solvent) | Linearity (Wastewater) | Slope difference (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0.5 (µg L-1) | 2 (µg L-1) | 10 (µg L-1) |  |  |  |
| AMP | 136 (2) | 82 (3) | 93 (14) | 0.99838 | 0.99667 | 0 |
| BE | 117 (7) | 157 (9) | 102 (5) | 0.99898 | 0.99754 | 3 |
| COC | 149 (4) | 119 (7) | 100 (6) | 0.99721 | 0.99749 | 14 |
| FEN | 100 (7) | 104 (0) | 88 (4) | 0.99784 | 0.99599 | 4 |
| KET | 111 (3) | 110 (12) | 101 (6) | 0.99940 | 0.99750 | 0 |
| MDMA | 88 (13) | 83 (21) | 72 (7) | 0,99777 | 0.99319 | 16 |
| METH | 88 (8) | 117 (26) | 105 (14) | 0,99665 | 0.99124 | 26 |
| NOR | NC | NC | NC | 0,99912 | 0.99633 | 2 |
| PHT | 126 (20) | 115 (11) | 98 (3) | 0,99251 | 0.99670 | 7 |

NC: not calculated. An isotopically labeled deuterated standard for NOR was not available when the method was validated.

Figure S1

Fig. S1 Confirmation of (a) KET and (b) BE in composite influent WW collected on 1 May 2024

Extracted ion chromatograms of quantitative (Q) and qualitative (q1 and q2) transitions are shown for analytical standards and corresponding wastewater samples

Analyte identification was based on retention time agreement (±0.2 min) and relative ion ratio (q/Q) deviations below 30%, in accordance with SANTE (2021) guidelines

Percent deviations of q/Q ratios between standards and samples are indicated in the figure

References:

Baselt, R.C.C.R.H., 1989. Disposition of toxic drugs and chemicals in man, 3rd ed. Chicago : Year Book Medical Publishers.

Been, F., Rossi, L., Ort, C., Rudaz, S., Delémont, O., Esseiva, P., 2014. Population normalization with ammonium in wastewater-based epidemiology: Application to illicit drug monitoring. Environ. Sci. Technol. 48, 8162–8169. https://doi.org/10.1021/es5008388

Castiglioni, S., Bijlsma, L., Covaci, A., Emke, E., Hernández, F., Reid, M., Ort, C., Thomas, K. V., Van Nuijs, A.L.N., De Voogt, P., Zuccato, E., 2013. Evaluation of uncertainties associated with the determination of community drug use through the measurement of sewage drug biomarkers. Environ. Sci. Technol. 47, 1452–1460. https://doi.org/10.1021/es302722f

Castiglioni, S., Borsotti, A., Senta, I., Zuccato, E., 2015. Wastewater analysis to monitor spatial and temporal patterns of use of two synthetic recreational drugs, Ketamine and Mephedrone, in Italy. Environ. Sci. Technol. 49, 5563–5570. https://doi.org/10.1021/es5060429

Cortese, M., Gigliobianco, M.R., Magnoni, F., Censi, R., Di Martino, P., 2020. Compensate for or minimize matrix effects? strategies for overcoming matrix effects in liquid chromatography-mass spectrometry technique: A tutorial review. Molecules. https://doi.org/10.3390/molecules25133047

Du, P., Zheng, Q., Thomas, K. V., Li, X., Thai, P.K., 2020. A revised excretion factor for estimating ketamine consumption by wastewater-based epidemiology – Utilising wastewater and seizure data. Environ. Int. 138. https://doi.org/10.1016/j.envint.2020.105645

Gracia-Lor, E., Zuccato, E., Castiglioni, S., 2016. Refining correction factors for back-calculation of illicit drug use. Science of the Total Environment 573, 1648–1659. https://doi.org/10.1016/j.scitotenv.2016.09.179

Hanna Instrument, 1999. Instruction Manual HI 93733 Ammonia ISM [WWW Document]. URL https://www.manualslib.com/manual/464480/Hanna-Instruments-Hi-93733.html?page=2#manual (accessed 3.25.26).

Lai, F.Y., Bruno, R., Leung, H.W., Thai, P.K., Ort, C., Carter, S., Thompson, K., Lam, P.K.S., Mueller, J.F., 2013. Estimating daily and diurnal variations of illicit drug use in Hong Kong: A pilot study of using wastewater analysis in an Asian metropolitan city. Forensic Sci. Int. 233, 126–132. https://doi.org/10.1016/j.forsciint.2013.09.003

Lemas, D.J., Loop, M.S., Duong, M., Schleffer, A., Collins, C., Bowden, J.A., Du, X., Patel, K., Ciesielski, A.L., Ridge, Z., Wagner, J., Subedi, B., Delcher, C., 2021. Estimating drug consumption during a college sporting event from wastewater using liquid chromatography mass spectrometry. Science of the Total Environment 764, 143963. https://doi.org/10.1016/j.scitotenv.2020.143963

Matuszewski, B.K., Constanzer, M.L., Chavez-Eng, C.M., 2003. Strategies for the assessment of matrix effect in quantitative bioanalytical methods based on HPLC-MS/MS. Anal. Chem. 75, 3019–3030. https://doi.org/10.1021/ac020361s

Metcalf & Eddy, Inc.M.& E., 2013. Wastewater Flowrates and Constituent Loadings, in: Wastewater Engineering: Treatment and Resource Recovery. McGraw-Hill Higher Education, pp. 183–261.

SANTE, 2021. Analytical quality control and method validation procedures for pesticide residues analysis in food and feed SANTE 11312/2021, Document N° SANTE 11312/2021.

Si-Yu Liu, Wen-Jing Yu, Yi-Ru Wang, Xue-Ting Shao, De-Gao Wang, 2021. Tracing consumption patterns of stimulants, opioids, and ketamine in China by wastewater-based epidemiology. Environmental Science and Pollution Research 28, 16754–16766. https://doi.org/10.1007/s11356-020-12035-w

van Nuijs, A.L.N., Castiglioni, S., Tarcomnicu, I., Postigo, C., de Alda, M.L., Neels, H., Zuccato, E., Barcelo, D., Covaci, A., 2011. Illicit drug consumption estimations derived from wastewater analysis: A critical review. Science of the Total Environment. https://doi.org/10.1016/j.scitotenv.2010.05.030

Van Wichelen, N., Boogaerts, T., Quireyns, M., Dermitzaki, R., Delputte, P., Hudda, N.U., De Roeck, N., Verhaegen, B., Van Hoorde, K., Maloux, H., Hutse, V., Gys, C., Covaci, A., van Nuijs, A.L.N., 2025. Ketamine, a new (or old) kid on the block: A comprehensive three-year spatio-temporal study in Belgium through wastewater-based epidemiology. Water Res. 276. https://doi.org/10.1016/j.watres.2025.123269