Appendix 1, Table A1. Comprehensive Inventory of Morphometric Attributes Identified Across 24 Reviewed Studies: Operational Definitions, Spatial Scales, and References

Note: Dimensions marked with an asterisk (*) contain attributes used exclusively for external validation, socioeconomic correlation, or performance assessment—such as traffic indicators, emission metrics, or model performance proxies—rather than intrinsic morphometric descriptors of physical urban form. These attributes are documented for methodological transparency but fall outside the scope of core morphometric attributes focused on geometric, topological, or shape properties of urban elements. Attributes are assigned to the finest operational scale at which they are computed. Operational definitions are reproduced verbatim from source extractions where provided. References follow the shorthand convention: J. Zhang et al., 2025 = Zhang et al. (Traditional Villages); L. Zhang et al., 2025 = Zhang et al. (Grids to Dendrites); Tian et al., 2025 = Tian et al. (Non-isomorphism).

| Morphometric Dimension | Attribute Name | Operational Definition | Spatial Scale | References |
| --- | --- | --- | --- | --- |
| Geometric Extent | Footprint area | Planar surface area of building footprint or block/mesh polygon | Building/Block/Mesh | (Bei et al., 2019; Chen & Qian, 2025; Huang et al., 2024; Jia et al., 2024; T. Liu et al., 2025; Y. Liu et al., 2025; Z. Liu et al., 2026; Lu & Tong, 2025; Yan et al., 2019, 2022; Yang et al., 2025; Zhao et al., 2020) |
|  | Perimeter | Total boundary length of footprint or polygon | Building/Block/Mesh | (Bei et al., 2019; Chen & Qian, 2025; Jia et al., 2024; T. Liu et al., 2025; Y. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Lu & Tong, 2025; Yan et al., 2019, 2022) |
|  | Mean radius | Average Euclidean distance from centroid to boundary vertices | Building | (Chen & Qian, 2025; T. Liu et al., 2025; Z. Liu et al., 2026; Yan et al., 2019, 2022) |
|  | Height / Elevation | Vertical dimension (absolute height in meters or normalized elevation) | Building | (Jia et al., 2024; Z. Liu et al., 2026; Z. Liu & Song, 2025; Lu & Tong, 2025; Yang et al., 2025) |
|  | Stories / Number of floors | Discrete count of building levels | Building | (Z. Liu & Song, 2025) |
|  | Volume | Three-dimensional spatial extent (footprint area × height) | Building | (Jia et al., 2024; Z. Liu et al., 2026; Z. Liu & Song, 2025; Lu & Tong, 2025) |
|  | Slenderness ratio | Height-to-area ratio (H/A) or height-to-radius ratio quantifying vertical proportionality | Building | (Z. Liu et al., 2026; Z. Liu & Song, 2025) |
|  | Segment length | Euclidean length of road segment | Street segment | (De Sabbata et al., 2023; Gharaee et al., 2021; M. E. Molefe & Tapamo, 2023, 2024; Wu et al., 2025; L. Zhang et al., 2025) |
|  | Midpoint coordinates | Geographic position (longitude/latitude) of segment centroid or node location | Street segment /Intersection | (Gharaee et al., 2021; M. E. Molefe & Tapamo, 2023, 2024; Neira & Murcio, 2025) |
|  | Block area | Planar surface area of street block or mesh polygon | Block/Mesh | (Huang et al., 2024; Y. Liu et al., 2025) |
|  | Block perimeter | Total boundary length of street block or mesh polygon | Block/Mesh | (Huang et al., 2024; Y. Liu et al., 2025) |
| Shape Configuration | Compactness | Shape regularity measured as area-to-perimeter squared ratio | Building/Block | (Chen & Qian, 2025; Jia et al., 2024; T. Liu et al., 2025; Y. Liu et al., 2025; Yan et al., 2019, 2022; Yang et al., 2025) |
|  | Circularity (IPQ) | Isoperimetric quotient: 4πA/P² measuring boundary efficiency | Building/Block | (Bei et al., 2019; Y. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Yan et al., 2019, 2022; Yang et al., 2025; Zhao et al., 2020) |
|  | Max circularity / Gibbs compactness | Alternative circularity formulations beyond standard IPQ | Building | (Z. Liu & Song, 2025) |
|  | Fractality index | Boundary complexity: 1−log(A)/2log(P) or log(A)/log(P) | Building | (Bei et al., 2019; Chen & Qian, 2025; Jia et al., 2024; T. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Yan et al., 2019, 2022) |
|  | Shape coefficient / index | Generic boundary complexity metric (Lu et al. formulation) | Building | (Lu & Tong, 2025) |
|  | Elongation | Aspect ratio of smallest bounding rectangle (length/width) | Building/Block | (Bei et al., 2019; Chen & Qian, 2025; Jia et al., 2024; T. Liu et al., 2025; Y. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Yan et al., 2019, 2022) |
|  | Concavity | Ratio of actual area to convex hull area (A/convex hull) | Building | (Bei et al., 2019; Chen & Qian, 2025; T. Liu et al., 2025; Yan et al., 2019, 2022) |
|  | Convexity | Complement of concavity; regularity of boundary shape | Block/Mesh | (Huang et al., 2024; Y. Liu et al., 2025) |
|  | Form factor | Ratio of area to area of minimum enclosing circle | Block | (Y. Liu et al., 2025) |
|  | Rectangularity (N1-R) | Ratio of polygon area to area of minimum enclosing rectangle | Mesh | (Huang et al., 2024) |
|  | Shape ratio (N2-SR) | Compactness via aspect ratio of enclosing rectangle (mesh-specific) | Mesh | (Huang et al., 2024) |
|  | Overlap index | Intersection-to-union ratio with equal-area reference circle | Building | (Chen & Qian, 2025; T. Liu et al., 2025; Yan et al., 2019, 2022) |
|  | Shifting degree of adjacency weight | 2D metric (D<sub>w</sub>, D<sub>h</sub>) quantifying SBR offset from centroid | Building | (Bei et al., 2019) |
|  | Boundary points | 20 equidistant points sampled along segment boundary to capture curvature | Street segment | (Gharaee et al., 2021; M. E. Molefe & Tapamo, 2023, 2024) |
|  | Grid index (N4-GI) | Average right-angledness of interior corners capturing orthogonality | Mesh | (Huang et al., 2024) |
|  | Maximum containment ratio (N6-MCAR) | Overlap potential with neighboring meshes | Mesh | (Huang et al., 2024) |
|  | Maximum fusion rectangularity (N7-MFR) | Rectangularity after merging mesh with neighbors | Mesh | (Huang et al., 2024) |
|  | Maximum fusion grid index (N8-MFGI) | Orthogonality after neighbor fusion | Mesh | (Huang et al., 2024) |
| Spatial Orientation | SBRO | Orientation angle of smallest bounding rectangle's long axis | Building/Block | (Bei et al., 2019; Chen & Qian, 2025; Jia et al., 2024; T. Liu et al., 2025; Y. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Yan et al., 2019, 2022; Yang et al., 2025; Zhao et al., 2020) |
|  | WSWO | Most frequent orientation angle among wall segments | Building | (Chen & Qian, 2025; T. Liu et al., 2025; Yan et al., 2019, 2022) |
|  | Wall orientation frequency | Frequency distribution of wall segment orientations | Building | (Chen & Qian, 2025) |
|  | Bearing distribution | Orientation angle distribution of road segments | Street segment | (Tian et al., 2025) |
| Contextual Density | FAR | Floor Area Ratio: total floor area divided by plot/block area | Building/Plot /Block | (Jia et al., 2024; T. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Lu & Tong, 2025) |
|  | GSI / GCR / BCR | Ground/Building Coverage Ratio: footprint area divided by plot/block area | Building/Plot /Block | (Jia et al., 2024; T. Liu et al., 2025; Z. Liu et al., 2026; Z. Liu & Song, 2025; Lu & Tong, 2025) |
|  | Voronoi influence ratio | Building area divided by area of Voronoi-derived influence zone | Building | (Chen & Qian, 2025; T. Liu et al., 2025; Lu & Tong, 2025; Yan et al., 2019, 2022) |
|  | Open Space Ratio / Porosity | Proportion of non-built space within influence zone or block | Building/Block | (Lu & Tong, 2025) |
|  | Volume-Area Ratio | Ratio of building volume to footprint area | Building | (Lu & Tong, 2025) |
|  | Building count | Number of buildings within block or graph | Block | (T. Liu et al., 2025; Z. Liu et al., 2026) |
|  | Lot coverage limit | Regulatory constraint on maximum plot development ratio | Block | (Jia et al., 2024) |
|  | Average shortest path | Mean Euclidean distance between building centroids within block | Block | (Jia et al., 2024) |
|  | Footprint area ratio (building/block) | Ratio of individual building footprint to containing block area | Building/Block | (T. Liu et al., 2025) |
|  | Building area ratio (floor area/block) | Ratio of total floor area to block area | Building/Block | (T. Liu et al., 2025) |
| Block/Mesh Topology | Dead-end count | Number of cul-de-sac segments bounding a block | Block | (Y. Liu et al., 2025) |
|  | Number of dead ends | Count of terminal edges in block/mesh boundary | Block/Mesh | (Y. Liu et al., 2025) |
| Topological (Node-Level) | Degree centrality | Number of direct connections (used as node feature) | Intersection /Segment | (De Sabbata et al., 2023; M. Molefe, 2025; Sugama, 2025; Tian et al., 2025; Wu et al., 2025) |
|  | Topological centralities suite | Eigenvector, betweenness, closeness centrality, clustering coefficient | Segment | (M. Molefe, 2025) |
|  | Degree distribution metrics | Percentage of nodes with degree ≤2, =3, =4, ≥5 | Intersection /Node | (De Sabbata et al., 2023; M. Molefe, 2025; Sugama, 2025; Tian et al., 2025; Wu et al., 2025) |
|  | Average node degree (k_avg) | Mean number of edges connected to each node | Intersection /Node | (Tian et al., 2025) |
|  | Circuity average | Average ratio of network path distance to straight-line distance | Network/Path | (Tian et al., 2025) |
|  | Orientation entropy | Measure of directional diversity in road network | Network | (Y. Liu et al., 2025) |
|  | Intersection proportions | Proportion of nodes by degree category | Network | (Y. Liu et al., 2025) |
| Edge/Relationship Features | Edge type encoding | 10 distinct relationship types: Main/Auxiliary roads + 8 compass directions (N,S,E,W,NE,NW,SE,SW) | Edge/Road segment | (L. Zhang et al., 2025) |
|  | Normalized geodesic distance | Normalized shortest-path distance between intersections | Edge | (Wu et al., 2025) |
|  | Normalized angular deviation | Normalized sum of left/right angles between adjacent segments ÷ 360° | Edge | (Wu et al., 2025) |
|  | Shape similarity (edge) | Intersection-to-union ratio of adjacent blocks | Edge (Block adjacency) | (Y. Liu et al., 2025) |
|  | Arrangement degree | Angular alignment of block orientations | Edge (Block adjacency) | (Y. Liu et al., 2025) |
|  | Bridge count/percentage | Number/proportion of edges whose removal disconnects the graph | Edge/Network | (L. Zhang et al., 2025) |
|  | Dead-end percentage | Proportion of edges with a node of degree 1 | Edge/Node | (L. Zhang et al., 2025) |
|  | Average triangles per node | Mean number of 3-node cycles per node, representing local clustering | Node/Local neighborhood | (L. Zhang et al., 2025) |
|  | Sinuosity | Ratio of path length to end-to-end Euclidean distance | Path/Segment | (L. Zhang et al., 2025) |
| Graph Embedding / Heterogeneity* | Hits@1 Score | Proportion of test queries where correct entity is ranked first; used as proxy for spatial heterogeneity | Network/Grid cell | (L. Zhang et al., 2025) |
|  | Graph classification accuracy | Accuracy of GNN/graph kernel in distinguishing road network structures; used to quantify non-isomorphism | Network/Grid cell | (Tian et al., 2025) |
|  | Node embedding dimensionality | Low-dimensional vector representation (e.g., 100-dim) capturing topological and attribute similarity | Node /Intersection | (J. Zhang et al., 2025; L. Zhang et al., 2025) |
| Multi-Modal Semantic* | Image features (CLIP) | Frozen CLIP image encoder outputs (512-dim) from satellite/topographic/texture images | Settlement /Grid cell | (J. Zhang et al., 2025) |
|  | Text features (CLIP+GPT-4o) | Summarized village introduction text embedded via CLIP text encoder (≤248 tokens) | Settlement | (J. Zhang et al., 2025) |
|  | Humanities factors | Categorical/tabular data: Folklore, Architecture, Clan, Village History, Famous People | Settlement | (J. Zhang et al., 2025) |
|  | Geographic factors | Continuous/tabular: Altitude, Elevation, Water network density, Distance to waterbody, Climate variables, NDVI, Arable land % | Settlement /Grid cell | (J. Zhang et al., 2025) |
|  | Societal factors | Continuous/tabular: Population density, Night light index, Urbanization rate, POI count, GDP, Road length, Distance to ancient stage road, Texture dispersion/density | Settlement /Grid cell | (J. Zhang et al., 2025) |
| Settlement–Landscape Relational | Village–Mountain Relationship | Spatial location of settlement boundaries relative to hills; quantified by distance (D) and enclosure (C) | Settlement | (J. Zhang et al., 2025) |
|  | Village–Water Relationship | Geometric relationship between settlement physical boundary and river system | Settlement | (J. Zhang et al., 2025) |
|  | Village–Field Relationship | Positional relationship between village boundary and cultivated field space; quantified by distance (D) and enclosure (C) | Settlement | (J. Zhang et al., 2025) |
|  | Natural environment type | Dominant natural environment type defined by comparative area coverage of mountains, water, and fields | Settlement | (J. Zhang et al., 2025) |
|  | Water–Field Pattern | Abstract geometric pattern of settlement-field morphology; quantified by average field side length (D) and area (A) | Settlement | (J. Zhang et al., 2025) |
|  | Typical pattern sequence | Interface ordering characteristics of village-mountain-water-field elements centered on settlement boundary | Settlement /Village | (J. Zhang et al., 2025) |
|  | Shape index of settlement boundary | S = P/(1.5√λ−√λ+1.5)√(A/π)/π, where P=perimeter, A=area, λ=long-to-wide side ratio | Settlement /Block | (J. Zhang et al., 2025) |
|  | Degree of boundary fragmentation | D = 2log₁₀(P/4)/log₁₀(A), where P=perimeter, A=area of settlement boundary | Settlement | (J. Zhang et al., 2025) |
|  | Degree of plot dispersion | S = (1/n)∑√[(xᵢ−x̄)²+(yᵢ−ȳ)²]; R = S/d, where (x,y)=centroid coordinates, d=diagonal length | Settlement/Plot system | (J. Zhang et al., 2025) |
|  | Colony density | Proportion of total plot area within settlement boundaries to total settlement study boundary area | Settlement /Block | (J. Zhang et al., 2025) |
|  | Mean area of cluster plots | Average area of individual plots within settlement boundaries | Settlement/Plot | (J. Zhang et al., 2025) |
|  | Road network curvature | C = D/D₀, where D = actual road length, D₀ = road network length in topological model | Road segment /Network | (J. Zhang et al., 2025) |
|  | Road network density | Total length of road segments per unit area | Network/Grid cell | (J. Zhang et al., 2025; L. Zhang et al., 2025) |
|  | Average road width | Average width of primary and secondary road networks within settlement | Road segment | (J. Zhang et al., 2025) |
|  | Road network alignment-landscape | Geometric relationship between road network characterization and landscape morphology | Network | (J. Zhang et al., 2025) |
|  | Road network alignment-wind direction | Geometric relationship between main road alignment and prevailing wind direction | Network | (J. Zhang et al., 2025) |
|  | Road network structure typology | Qualitative classification: Grid pattern, Fishbone pattern, or Mixed pattern | Network/Grid cell | (J. Zhang et al., 2025) |
| Socioeconomic / Performance* | Traffic index | Composite measure of commute time, dissatisfaction, CO₂ emissions, and system inefficiencies | City/Region | (L. Zhang et al., 2025) |
|  | CO₂ emission index | Grams of CO₂ attributable to daily commute by passenger (return trip basis) | City/Region | (L. Zhang et al., 2025) |
|  | Time index / Inefficiency index | Average one-way commute time (min); estimated dissatisfaction from commute duration | City/Region | (L. Zhang et al., 2025) |
|  | Intersection density | Number of road intersections per square kilometer | Grid cell /Network | (L. Zhang et al., 2025) |
|  | Road length / Road number | Total length (km) or count of road segments within study area | Network/Grid cell | (L. Zhang et al., 2025) |

Appendix 2 Table A2. Graph Construction Methodologies Across 24 Reviewed Studies: Node Definitions, Representation Paradigms, Edge Logic, and Spatial Constraints

Note: This inventory documents the methodological choices underlying graph representation in each reviewed study. Columns are organized as follows: (1) Node Element: the physical or abstract urban entity represented as a graph node (e.g., building centroid, road intersection, data modality); (2) Representation Paradigm: the graph-theoretic framing (primal, dual, constrained triangulation, hierarchical fusion, etc.); (3) Edge Definition Logic: the rule or algorithm determining node connectivity (e.g., Delaunay adjacency, topological connectivity, Voronoi influence, attention-based); (4) Spatial Constraints / Boundary Handling: how natural or administrative boundaries (roads, rivers, parcels, grid cells) constrain graph construction; (5) Edge Weighting Scheme: the function or attribute used to weight connections (e.g., Gaussian distance decay, unweighted adjacency, GAT attention); (6) Graph Scope: the spatial extent of the constructed graph (intra-block, city-wide, grid-cell, ego-network, multi-modal scaffold).

| Study | Node Element | Representation Paradigm | Edge Definition Logic | Spatial Constraints / Boundary Handling | Edge Weighting Scheme | Graph Scope |
| --- | --- | --- | --- | --- | --- | --- |
| (Z. Liu et al., 2026) | Building centroids | Proximity graph | Fully connected intra-block (all-to-all connections) | Road-defined urban blocks; graphs constructed independently per block | Gaussian distance decay: w<sub>ij</sub> = exp(−(d<sub>ij</sub>/σ)²), d<sub>ij</sub> = centroid distance, σ = block std. dev. | Intra-block (each block = independent graph) |
| (L. Zhang et al., 2025) | Road intersections | Primal graph | Topological connectivity: edges = road segments linking intersections | None specified (city-wide network) | Unweighted edges; 10 discrete relationship types encoding functional hierarchy + compass direction | City-wide directed graph |
| (J. Zhang et al., 2025) | Input/communication nodes (data modalities) | Abstract fusion graph | Predefined input→communication connections + GAT-learned communication edges | None (semantic relationships, not spatial) | Dynamically weighted via GAT attention mechanism | Multi-modal fusion scaffold (non-spatial) |
| (Yang et al., 2025) | Building centroids | Constrained Delaunay Triangulation (CDT) | Delaunay edges retaining only Type 1 (≥2 buildings) & Type 2 (outside buildings) triangles; Type 3 (within-building) removed | Road/river networks partition space; CDT applied within natural boundaries | Nearest distance between building footprints (not centroid-to-centroid) | Intra-block constrained graphs |
| (Wu et al., 2025) | Road intersections | Primal graph | Topological connectivity: edges = road segments linking intersections | Optional 1km² grid-cell partitioning for localized analysis | Two edge attributes: (1) normalized geodesic distance; (2) normalized angular deviation (sum of left/right angles ÷ 360°) | City-wide or grid-cell networks |
| (Tian et al., 2025) | Intersections & segment endpoints | Primal graph | Topological connectivity after topology simplification (simplify=True) | 1km × 1km grid cells (400 cells per city) | Unweighted undirected edges | Localized (1km² cells) |
| (Sugama, 2025) | Road intersections | Primal graph | Topological connectivity (all roads modeled uniformly) | 15km radius city networks | Unweighted undirected edges (explicitly excludes functional differentiation) | City-wide (15km radius) |
| (Neira & Murcio, 2025) | Street junctions | Primal graph | Topological connectivity after node merging (<10m threshold) | 1km × 1km grid cells centered on city centroids (39,364 cities/towns) | Unweighted adjacency matrix; spatial extent implicitly encoded via normalized (x,y) coordinates | Localized (1km² cells) |
| (M. Molefe, 2025) | Road segments | Dual graph (line graph transformation) | Two segments connect if sharing a common intersection in primal graph | None specified | Unweighted edges; morphological signal isolated via pure topology | City-wide segment adjacency |
| (Lu & Tong, 2025) | Building centroids | Voronoi influence zone adjacency | Buildings connect if Voronoi-derived influence zones share boundary adjacency | Block-wise construction: process repeated independently within pre-divided urban blocks | Unweighted edges (weight = 1) | Intra-block constrained graphs |
| (Z. Liu & Song, 2025) | Building centroids | Intra-plot Delaunay Triangulation | Delaunay edges within cadastral parcel boundaries | Cadastral parcels (plots); edges >200m trimmed; plots with <3 buildings excluded | Euclidean centroid-to-centroid distance | Intra-plot constrained graphs |
| (Y. Liu et al., 2025) | Street blocks (meshes) | Mesh adjacency graph | Rook adjacency: blocks connect if sharing common boundary edge (not corner) | 1km × 1km patches; grid-boundary intersections included as blocks | Edge features: (1) shape similarity (intersection/union ratio); (2) arrangement degree (angular alignment) | Patch-level (1km²) tissue graphs |
| (T. Liu et al., 2025) | Building centroids | Multi-variant proximity graphs | Three variants compared: (1) k-NN; (2) Delaunay Triangulation; (3) Minimum Spanning Tree | Road/river networks partition space into blocks; graphs constructed within blocks | Euclidean centroid-to-centroid distance | Intra-block constrained graphs |
| (Chen & Qian, 2025) | Building centroids | Road-constrained Delaunay Triangulation | First-order Delaunay neighbors within road-constrained sub regions | Road networks partition space; 0.01m inward buffer ensures min. inter-building distance >0; 0.01m outward convex hull buffer ensures graph closure | Euclidean centroid-to-centroid distance | Intra-subregion constrained graphs |
| (M. E. Molefe & Tapamo, 2024) | Road segments | Dual graph (line graph transformation) | Two segments connect if sharing a common intersection in primal graph | None specified | Unweighted edges; geometric signal encoded via node features (length, 20 boundary points, coordinates, speed limit) | City-wide segment adjacency |
| (Jia et al., 2024) | Residential building centroids | Delaunay Triangulation | Delaunay edges on building centroids | Cadastral parcels: each residential block forms independent graph; no cross-block edges | Euclidean centroid-to-centroid distance | Intra-block constrained graphs (2,196 blocks Shenzhen; 1,742 blocks Hong Kong) |
| (Huang et al., 2024) | Road meshes (polygons) | Mesh adjacency graph | Meshes connect if sharing common boundary edge or intersection point | Road network polygonization defines mesh cells | Unweighted edges; contextual signal via mesh fusion features (N6–N8) | City-wide mesh tissue graph |
| (M. E. Molefe & Tapamo, 2023) | Road segments | Dual graph (line graph transformation) | Two segments connect iff sharing a common intersection in primal graph | None specified | Unweighted edges; geometric signal encoded via node features (length, 20 boundary points, coordinates, speed limit) | City-wide segment adjacency |
| (De Sabbata et al., 2023) | Street junctions | Ego-network sampling | Topological connectivity within 500m street-distance radius | Ego-graphs centered on each junction (min. 8 nodes retained) | Edge attribute: street segment length (min-max normalized 50–500m); node attribute: degree (min-max normalized 1–4) | Localized ego-networks (500m radius) |
| (Yan et al., 2022) | Building centroids | Voronoi-like tessellation | Buildings connect if Voronoi partitions share boundary adjacency | Physical constraints (roads, rivers) partition space into blocks before graph construction | Unweighted edges; 0.01m negative buffer handles touching boundaries | Intra-block constrained graphs |
| (Gharaee et al., 2021) | Road segments | Dual graph (line graph transformation) | Two segments connect if sharing a common intersection in primal graph | Two-view neighborhood sampling: (1) fixed-length local view L<sub>l</sub>; (2) random walk global view L<sub>g</sub> = 2×L<sub>l</sub> | Unweighted edges; geometric signal via 56–58-dim node features (length, 20 boundary points, coordinates, speed limit) | City-wide segment adjacency |
| (Zhao et al., 2020) | Building centroids | Constrained Delaunay Triangulation + skeletonization | Buildings connect if sharing ≥1 triangle edge in CDT | Building outlines + surrounding street segments form CDT constraints; skeletonization of gap space | Inverse squared weighted distance: W = 1/d² (incorporating skeleton length & triangle height) | Intra-block constrained graphs |
| (Yan et al., 2019) | Building centroids | Multi-variant proximity graphs | Two variants compared: (1) Delaunay Triangulation (fully connected); (2) Minimum Spanning Tree (sparse subset of DT) | Road/river networks used as hard boundaries to partition space before graph construction | Nearest distance between building outlines (not centroid-to-centroid) | Intra-block constrained graphs |
| (Bei et al., 2019) | Building centroids / contour key points | Constrained Delaunay Triangulation | Delaunay neighbors within block boundaries | CDT partitions buildings into blocks; each block forms independent graph | Euclidean distance or shifting degree of adjacency weight (D<sub>w</sub>, D<sub>h</sub>) | Intra-block constrained graphs |

Appendix 3 Table A3: Complete inventory of graph-based learning approaches across the 24 reviewed studies, organized by learning paradigm

Note: Studies are grouped by learning paradigm (Unsupervised: n=8; Supervised: n=14; Semi-supervised: n=2). Output granularity indicates the primary prediction unit: Node = individual urban element (building, segment); Graph = aggregated unit (block, network patch); Edge = relationship between elements. Full methodological details, including hyperparameters and evaluation metrics, are available in the original publications.

| Study | Paradigm | Learning Objective | Model Family & Key Innovation | Output Granularity |
| --- | --- | --- | --- | --- |
| Unsupervised Approaches |  |  |  |  |
| Liu et al. (2026) | Unsupervised | Discover block-level morphological typologies through joint structural and metric analysis | Contrastive GAT + Autoencoder: Multi-head attention with contrastive loss on augmented intra-block graphs; fused with block-level morphometrics | Graph (urban block) |
| Yang et al. (2025) | Unsupervised | Cluster buildings into perceptually coherent groups for cartographic generalization | Distance-biased GAT + Spectral Clustering: Attention enforcing spatial proximity; clustering on learned similarities | Node (building) |
| Wu et al. (2025) | Unsupervised | Quantify similarity between city-scale road networks via dynamic topological response | Graph Cellular Automata: Differentiable logic gates; similarity from Jensen-Shannon divergence of state evolution | Graph (road network) |
| Sugama (2025) | Unsupervised | Embed global street networks to reveal regional topological patterns | graph2vec: WL kernel + doc2vec; pure topology, no geometry or semantics | Graph (city network) |
| Neira & Murcio (2025) | Unsupervised | Learn generative representations of street networks for synthesis and classification | VGAE + Transformer: Autoregressive coordinate modeling + GCN-based structural embedding | Graph (1km² segment) |
| Lu & Tong (2025) | Unsupervised | Identify urban form types at building/block scale using spatial relationships | GCN + K-means: 3-layer spectral convolution on Voronoi-derived graphs; clustering on embeddings | Node (building) |
| Liu & Song (2025) | Unsupervised | Classify cadastral plots by internal building configuration | Distance-decay GAT: Attention on intra-plot Delaunay graphs; attention pooling for plot-level representation | Graph (cadastral plot) |
| De Sabbata et al. (2023) | Unsupervised | Embed street junctions to characterize urban-suburban transitions | Graph Autoencoder (GIN): Modified GIN with edge attributes; 2D embeddings for visual interpretation | Node/Graph (junction + neighborhood) |
| Supervised Approaches |  |  |  |  |
| Yan et al. (2019) | Supervised | Classify building groups as regular vs. irregular patterns | Spectral GCN: Chebyshev polynomial approximation (K=3); global mean pooling + FC layers | Graph (building group) |
| Yan et al. (2022) | Supervised | Predict centroid positions for building grouping (cartographic generalization) | Spectral GCN: 5-layer Chebyshev GCN with skip connections; regression output + k-means | Node → Graph (via post-processing) |
| Zhao et al. (2020) | Supervised | Semantic segmentation of buildings into 5 pattern types (e.g., grid-like, curvilinear) | Spectral GCN + SpecTN: Chebyshev convolution with spectral transformer; multi-scale skip connections | Node (individual building) |
| Gharaee et al. (2021) | Supervised | Road-type classification using structural and visual features | GAIN: Attention-weighted SUM aggregation with cross-entropy loss | Node (road segment) |
| Liu et al. (2025) | Supervised | Classify building groups into 3 patterns (irregular/grid/linear) | DGCNN: 17 GCN layers + SortPooling + knowledge graph embedding layer (bilinear fusion) | Graph (building group) |
| Jia et al. (2024) | Supervised | Classify residential blocks into 3 high-density Asian patterns | Spectral GCN: 2-layer GCN with Delaunay graphs; features at building and block scales | Graph (residential block) |
| Huang et al. (2024) | Supervised | Detect road grid patterns via hierarchical mesh-to-segment cognition | TAGCN: Topology-adaptive GCN (K=3) on road meshes; rule-based non-grid exclusion | Graph (road mesh) |
| Liu et al. (2025) | Supervised | Classify street networks into 6 morphology types (grid, dendritic, etc.) | Hybrid SBGNet: Graph Transformer (edge-aware) + Chebyshev GCN + ResNet-34 (image) + MLP (global) | Graph (street network patch) |
| Zhang et al. (2025) | Supervised | Multi-task classification of 17 traditional village spatial subtypes | Hierarchical GNN: GCN + GAT + CLIP fusion; relation-aware pooling per subtype | Graph (village) |
| Zhang et al. (2025) | Supervised | Quantify spatial heterogeneity via road network edge classification | Relational GCN: 10 discrete edge types encoding functional/directional road properties; DistMult scoring | Edge/Node (intersection embeddings) |
| Molefe (2025) | Supervised | Classify road segments into 5 functional types using structural features only | Standard GCN: 2-hop aggregation with mean pooling; 5D topological features (centrality metrics) | Node (road segment) |
| Molefe & Tapamo (2024) | Supervised | Road-type classification robust to noisy OSM attributes | DAE + GNN: DAE compresses 58D raw features → GNN (GCN/GraphSAGE/GAT) with 2-hop aggregation | Node (road segment) |
| Molefe & Tapamo (2023) | Supervised | Road-type classification using compact structural representations | DAE + GCN: DAE preprocessor + 2-hop GCN with max-pooling aggregation | Node (road segment) |
| Tian et al. (2025) | Supervised | Quantify structural non-isomorphism via city identification | EdgeCNN: Edge-based convolutions on continuous node coordinates; outperforms WL kernel | Graph (1km² road network cell) |
| Semi-Supervised Approaches |  |  |  |  |
| Chen & Qian (2025) | Semi-supervised | Predict optimal number of building clusters per block to guide cartographic generalization, using minimal expert annotations | Two-stage hybrid: (1) Supervised GAT predicts graph-level cluster count (2) Unsupervised spectral clustering guided by predicted count | Graph → Node (graph-level prediction informs node-level clustering) |
| Bei et al. (2019) | Semi-supervised | Jointly classify building spatial states (edge/inner/free) and recognize block-level morphological patterns (I/L/grid/single) using limited labeled examples | Multi-task spectral GCN: Single Chebyshev GCN backbone with two output heads (node-level state + graph-level pattern) trained with partial labels | Node + Graph (simultaneous node- and graph-level predict |

Appendix 4, Table A4. Cross-study performance summary across the 24 reviewed studies, organized by learning paradigm.

Note: Studies are grouped by learning paradigm (Unsupervised: n=8; Supervised: n=14; Semi-supervised: n=2). Performance metrics vary by paradigm: unsupervised studies report clustering quality indices (Silhouette, NMI, ARI) or embedding fidelity measures, while supervised studies report classification accuracy, F1 score, or task-specific metrics. Direct cross-study comparison should be interpreted with caution given heterogeneity in datasets, evaluation protocols, and task definitions. All values are reproduced from the original publications.

| # | Study | Node | Edge Strategy | Paradigm | What to Extract | Already Known |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Z. Liu et al. (2026) | Building | Fully connected intra-block | Unsup. (GCL+AE) | Clustering quality: Purity, NMI, ARI (land-use reference); Silhouette score; Random Forest accuracy/F1 for functional prediction | 13 block types via K-means on 10-dim AE embeddings; ~30% of discriminative info in inter-building spatial relationships |
| 2 | Yang et al. (2025) | Building | CDT | Unsup. (LA-GAT) | Cluster Compactness, Silhouette Coefficient, Adjusted Rand Index (ARI) vs. manual annotations | LA-GATs: Silhouette 0.65, ARI 0.87; ~21% ARI improvement over K-means in residential areas; distance bias +15% ARI |
| 3 | Wu et al. (2025) | Intersection | Primal street | Unsup. (GCA) | Silhouette (>0.9 target), Soft-Silhouette, CH, DB indices; Internal consistency index I_C(c); Spearman correlations with land value/space syntax | GCA-Sim achieves Silhouette >0.9; 20% of parameter settings induce strong structural amplification ("network resonance") |
| 4 | Sugama (2025) | Intersection | Primal street | Unsup. (graph2vec) | Cosine similarity/distance (0–2 range) between city embeddings; MDS clustering coherence; cumulative similarity distributions | Regional clustering: Europe (0.777–0.838), NA (0.868–0.931), Asia-Pacific (0.807–0.887); Asia shows exceptional internal diversity |
| 5 | Neira & Murcio (2025) | Intersection | Primal street | Unsup. (VGAE) | Topological fidelity (KDE: street length, degree, circuity); Geometric fidelity (block area/form factor); K-means cluster coherence (K=7) | Synthetic circuity matches real (1.039 vs. 1.033); 7 morphological types with regional coherence; megacity centers cluster in rare type 6 |
| 6 | Lu & Tong (2025) | Building | Voronoi adjacency | Unsup. (GCN+K-means) | Silhouette Score, CH Index, DB Index, BIC, Elbow Method (optimal k); Patch aggregation count post-merging | Graph-based clustering yields 46% fewer patches (1,791 vs. 3,337); significant inter-cluster differences in FSI, Unbuilt Area, GSI |
| 7 | Z. Liu & Song (2025) | Building | Delaunay intra-plot | Unsup. (GAT) | Silhouette score for k-means (optimal k=7); Space Matrix (FAR vs. GSI) separation of identical-density plots | 7 plot types identified; framework separates plots with identical FAR-GSI but different internal configurations (tower/slab/composite) |
| 8 | De Sabbata et al. (2023) | Junction | Primal street | Unsup. (GAE) | Qualitative embedding mapping (DBSCAN clusters, urban-suburban gradients); Kendall's τ with centrality metrics | Embeddings capture distinct urban form patterns; strongest correlations with segment lengths and junction degree |
| 9 | Yan et al. (2019) | Building | Delaunay (DT) | Sup. (GCN) | Overall accuracy, per-class accuracy, kappa coefficient for regular vs. irregular patterns; cross-city generalization | 98.68% test accuracy (Guangzhou); 94.71% cross-city (Shanghai); outperformed SVM (92.93%/75.59%) and RF (95.14%/87.75%) |
| 10 | Yan et al. (2022) | Building | Voronoi-like tessellation | Sup. (GCN) | Adjusted Rand Index (ARI) for grouping accuracy; comparison vs. density-based and ArcGIS polygon aggregation | Guangzhou 1:2000: ARI=0.729 (79% >0.6); cross-scale: ARI=0.383; cross-city Shenzhen: ARI=0.667; outperformed baselines |
| 11 | Zhao et al. (2020) | Building | CDT + skeletonization | Sup. (GCN+SpecTN) | Classification accuracy and mean IoU for 5 pattern types (collinear/curvilinear/grid-like/rectangular/high-density) | 90.24–93.12% accuracy; mean IoU 77.01–83.40%; outperformed MST and GSCARF in boundary precision |
| 12 | Gharaee et al. (2021) | Road segment (dual) | Dual street | Sup. (GAIN) | Micro-averaged F1 for 5 road types in transductive/inductive settings; architecture comparison (GCN/GraphSAGE/GAT/GIN/GAIN) | GAIN: 0.71 F1 unsupervised transductive (+20% vs. baseline); 0.81 F1 supervised transductive; 0.59 F1 inductive |
| 13 | T. Liu et al. (2025) | Building | k-proximity / DT / MST | Sup. (DGCNN) | Accuracy for 3 patterns (irregular/grid/linear); comparison across graph construction methods | 99.03% train / 95.89% test accuracy; outperformed spectral GCN (60.54%) and original DGCNN (93.75%) |
| 14 | Jia et al. (2024) | Building | Delaunay (DT) | Sup. (GCN) | Classification accuracy for 3 Asian residential patterns; cross-city generalization on Hong Kong dataset | 97.95% train / 94.91% test (Shenzhen); 93.77% accuracy on Hong Kong; depth=2 architecture optimal |
| 15 | Huang et al. (2024) | Road mesh | Dual street | Sup. (TAGCN) | Precision, recall, F1 for grid detection; cross-city validation on Amsterdam/Denver | Changchun: 97.68% accuracy, 96.99% precision, 90.09% recall, 93.41% F1; outperformed baselines by 0.79–44.87% |
| 16 | Y. Liu et al. (2025) | Block/Intersection | Multi-branch (rook adjacency) | Sup. (Hybrid SBGNet) | Overall accuracy for 6 morphology types; per-branch ablation (graph/image/global); ROC/PR AUC, F1 | 0.91±0.01 train / 0.80±0.01 test accuracy; ROC AUC 0.97, PR AUC 0.88, F1 0.80; 5–33% PR AUC improvement over baselines |
| 17 | J. Zhang et al. (2025) | Abstract nodes | Semantic (modality-based) | Sup. (Hierarchical GNN) | Accuracy/F1 per subtype and S/V/R groups; multi-modal vs. single-modal; ablation on fusion coefficient β | Overall: 0.821 accuracy / 0.907 F1 (best prior: 0.806/0.892); joint training +6% gain for parcel tasks; GAT communication edges +4% avg accuracy |
| 18 | L. Zhang et al. (2025) | Intersection | Primal (relational, 10 edge types) | Sup. (RGCN) | Hits@1 as heterogeneity metric; typology differentiation via K-means; Pearson correlations with traffic/CO₂/road density | Hits@1 distinguishes typologies: Dendritic ≈0.57 > Mixed ≈0.51 > Polygonal ≈0.48 > Grid ≈0.42; correlations: traffic R=0.36, CO₂ R=0.43, density R=0.48 |
| 19 | M. Molefe (2025) | Road segment (dual) | Dual street | Sup. (GCN) | Micro-averaged F1 for 5 road types; comparison vs. raw features only and raw+GCN baselines | 73% test F1 (Johannesburg, mean aggregator, M=4); 71% (Linköping, M=8); outperforms raw-only (59%) and raw+GCN (58%); within 4% of DAE+GCN (75%) |
| 20 | M. E. Molefe & Tapamo (2024) | Road segment (dual) | Dual street | Sup. (DAE+GNN) | Micro F1 comparing GCN/GraphSAGE/GAT with 58D→8D DAE compression; aggregator function comparison | Best: GSAGE-Max 78% (Linköping), GSAGE-Mean 84% (Johannesburg); DAE preprocessing +19–20% F1 over raw baseline |
| 21 | M. E. Molefe & Tapamo (2023) | Road segment (dual) | Dual street | Sup. (DAE+GCN) | Classification accuracy with DAE preprocessing; micro F1 for 5 road types | 75% test F1 (Linköping); outperforms baseline (59%) and GCNN-only (58%); DAE compresses 58D→8D before convolution |
| 22 | Tian et al. (2025) | Intersection | Primal street | Sup. (EdgeCNN) | Classification accuracy as non-isomorphism metric; regional patterns; correlations with road density/CO₂/traffic | EdgeCNN: 85% accuracy (vs. WL 80%, GIN 75%); Europe: 0.777–0.838, NA: 0.868–0.931, Asia-Pacific: 0.807–0.887 |
| 23 | Chen & Qian (2025) | Building | Voronoi-like (road-constrained) | Semi-sup. (GAT+SC) | ARI, ACC, NMI for building clustering; comparison with K-means, DBSCAN, VGAE/ARVGA/DAEGC | Two Ningbo datasets: ARI 0.6851/0.5348, ACC 0.4632/0.5531, NMI 0.7523/0.5941; outperformed all baselines; better orthogonal feature preservation |
| 24 | Bei et al. (2019) | Building | CDT | Semi-sup. (GCN) | Node-level state accuracy (edge/inner/free); graph-level pattern accuracy (I/L/grid/single); kappa; cross-city generalization | 92.71% node state accuracy; 89.83% pattern recognition (Xi'an); kappa 0.832/0.847; cross-city tested (Beijing→Xi'an) |

References

Bei, W., Guo, M., & Huang, Y. (2019). A Spatial Adaptive Algorithm Framework for Building Pattern Recognition Using Graph Convolutional Networks. Sensors, 19(24), 5518. https://doi.org/10.3390/s19245518

Chen, G., & Qian, H. (2025). Building clustering method that integrates graph attention networks and spectral clustering. Geocarto International, 40(1), 2471091. https://doi.org/10.1080/10106049.2025.2471091

De Sabbata, S., Ballatore, A., Liu, P., & Tate, N. (2023). Learning urban form through unsupervised graph-convolutional neural networks. https://doi.org/10.17605/OSF.IO/H2AWQ'%255D

Gharaee, Z., Kowshik, S., Stromann, O., & Felsberg, M. (2021). Graph representation learning for road type classification. Pattern Recognition, 120, 108174. https://doi.org/10.1016/j.patcog.2021.108174

Huang, Z., Qian, H., Cai, Z., Wang, A., Wang, J., & Xiong, F. (2024). Intelligent recognition method for urban road grid patterns by fusing mesh and road features. International Journal of Digital Earth, 17(1), 2433647. https://doi.org/10.1080/17538947.2024.2433647

Jia, M., Zhang, K., & Narahara, T. (2024). CHARACTERIZING RESIDENTIAL BUILDING PATTERNS IN HIGH-DENSITY CITIES USING GRAPH CONVOLUTIONAL NEURAL NETWORKS (N. Gardner, C. Herr, L. Wang, H. Toshiki, & S. Khan, Eds.; pp. 39–48). (WOS:001248011500004).

Liu, T., Zhang, Z., Du, P., Wang, W., Yan, H., Qiang, B., & Xu, S. (2025). Recognition of building group patterns using GCN and knowledge graph. Geocarto International. (world). https://www.tandfonline.com/doi/abs/10.1080/10106049.2024.2436906

Liu, Y., Guo, Q., & Zheng, C. (2025). Urban street network morphology classification through street-block based graph neural networks and multi-model fusion. INTERNATIONAL JOURNAL OF DIGITAL EARTH, 18(1). (WOS:001478785100001). https://doi.org/10.1080/17538947.2025.2497490

Liu, Z., & Song, Y. (2025). Unsupervised Plot Morphology Classification via Graph Attention Networks: Evidence from Nanjing’s Walled City. Land, 14(7). Scopus. https://doi.org/10.3390/land14071469

Liu, Z., Wang, L., Xu, H., Wang, X., Han, D., & Song, Y. (2026). From buildings to blocks: Graph contrastive learning for integrated urban architectural morphology classification. Frontiers of Architectural Research. https://doi.org/10.1016/j.foar.2025.11.004

Lu, T., & Tong, Z. (2025). Graph-Based Typology Identification and Analysis of Urban Form: A case study of Nanjing main district. Proc. Int. Conf. Comput.-Aided Architect. Des. Res. Asia, 4, 39–48. Scopus. https://doi.org/10.52842/conf.caadria.2025.4.039

Molefe, M. (2025). Modelling Road Networks with Node Structural Features and Graph Convolutional Networks. In T. M. N. Ngatched, I. Woungang, J.-R. Tapamo, & S. Viriri (Eds.), Pan-African Artificial Intelligence and Smart Systems (pp. 309–325). Springer Nature Switzerland. https://doi.org/10.1007/978-3-031-94439-0_18

Molefe, M. E., & Tapamo, J. R. (2023). A New Approach for Road Type Classification Using Multi-stage Graph Embedding Method. Pattern Recognition, 23–35. https://doi.org/10.1007/978-3-031-33783-3_3

Molefe, M. E., & Tapamo, J. R. (2024). Classifying Roads with Multi-Step Graph Embeddings. Computación y Sistemas, 28(1). https://doi.org/10.13053/cys-28-1-4891

Neira, M., & Murcio, R. (2025). Graph Representation Learning on Street Networks. ISPRS INTERNATIONAL JOURNAL OF GEO-INFORMATION, 14(8). (WOS:001557700000001). https://doi.org/10.3390/ijgi14080284

Sugama, A. (2025). Analysis of the Topological Similarity of Street Networks in Cities Around the World Using Graph2vec. Journal of the Eastern Asia Society for Transportation Studies, 16. https://doi.org/10.11175/easts.16.PP4273

Tian, L., Rao, W., Zhao, K., & Vo, H. T. (2025). Quantifying the non-isomorphism of global urban road networks using GNNs and graph kernels. Scientific Reports, 15(1), 6485. https://doi.org/10.1038/s41598-025-90839-x

Wu, P., Zhai, M., & Zhang, L. (2025). A Framework Based on Graph Cellular Automata for Similarity Evaluation in Urban Spatial Networks (arXiv:2511.00768). arXiv. https://doi.org/10.48550/arXiv.2511.00768

Yan, X., Ai, T., Yang, M., Tong, X., & Liu, Q. (2022). A graph deep learning approach for urban building grouping. Geocarto International, 37(10), 2944–2966. https://doi.org/10.1080/10106049.2020.1856195

Yan, X., Ai, T., Yang, M., & Yin, H. (2019). A graph convolutional neural network for classification of building patterns using spatial vector data. ISPRS Journal of Photogrammetry and Remote Sensing, 150, 259–273. https://doi.org/10.1016/j.isprsjprs.2019.02.010

Yang, X., Xie, X., & Liu, D. (2025). LA-GATs: A Multi-Feature Constrained and Spatially Adaptive Graph Attention Network for Building Clustering. ISPRS International Journal of Geo-Information, 14(11), 415. https://doi.org/10.3390/ijgi14110415

Zhang, J., Zhu, Z., Deng, J., Li, Y., & Wang,  and B. (2025). Multi-Modal Feature Fusion for Spatial Morphology Analysis of Traditional Villages via Hierarchical Graph Neural Networks. https://arxiv.org/abs/2510.27208v1

Zhang, L., Li, S., Liu, Y., Huang, H., & Van De Weghe, N. (2025). From grids to dendrites: Quantifying spatial heterogeneity in urban road networks. Computers, Environment and Urban Systems, 121. Scopus. https://doi.org/10.1016/j.compenvurbsys.2025.102309

Zhao, R., Ai, T., Yu, W., He, Y., & Shen, Y. (2020). Recognition of building group patterns using graph convolutional network. Cartography and Geographic Information Science, 47(5), 400–417. https://doi.org/10.1080/15230406.2020.1757512