Supplementary Material

for

K–R Adaptive Flight Control: Physics-Informed Nonlinear Residual Correction for Robust Trajectory Tracking Under Model Uncertainty

This document provides extended data tables, parameter tuning details, Monte Carlo statistics, reproducibility information, and code documentation that support the main manuscript. All tables referenced as “Table S-N” correspond to supplementary tables. The main manuscript references these as supplementary material.

S-1. Extended Performance Metrics

Tables S-1 through S-7 report all 11 performance metrics for each of the 7 scenarios. These extend the summary RMSE values in Table 3 of the main manuscript. Metrics are defined in Section S-8.

Table S-1. Extended Metrics: S1: Extreme Turbulence + Model Mismatch

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 0.145 | 0.076 | 1.875 | 1.120 | 0.400 | 2.422 | 25 | 1.190 | 2 | 0.020 | 0.135 |
| LQR | 0.291 | 0.195 | 1.932 | 14.980 | 2.700 | 1.983 | 13.710 | 0.450 | 2 | 0.051 | 0.401 |
| MPC | 0.921 | 0.848 | 1.992 | 14.980 | 0 | 1.821 | 1.980 | 0.010 | 1 | 0.222 | 1.720 |
| L1 | 1.011 | 0.828 | 1.939 | 14.980 | 2.200 | 1.931 | 11.710 | 1.340 | 2 | 0.217 | 1.192 |
| K-only | 0.291 | 0.195 | 1.932 | 14.980 | 2.700 | 1.983 | 13.710 | 0.450 | 2 | 0.051 | 0.401 |
| K–R | 0.179 | 0.075 | 1.932 | 1.220 | 2 | 2.029 | 13.720 | 0.560 | 6 | 0.020 | 0.123 |

Table S-2. Extended Metrics: S2: Near-Stall + Actuator Saturation

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 3.168 | 2.670 | 7.142 | 14.980 | 115.400 | 14.164 | 15.750 | 14.310 | 27 | 0.699 | 4.321 |
| LQR | 2.656 | 2.106 | 5.839 | 14.980 | 83.600 | 13.429 | 15.750 | 18.030 | 31 | 0.551 | 3.640 |
| MPC | 4.048 | 3.309 | 7.997 | 14.980 | 74.900 | 1.607 | 2.570 | 0.210 | 3 | 0.866 | 6.785 |
| L1 | 3.663 | 2.998 | 7.369 | 14.980 | 72.100 | 14.249 | 15.750 | 18.470 | 15 | 0.785 | 5.657 |
| K-only | 2.656 | 2.106 | 5.839 | 14.980 | 83.600 | 13.429 | 15.750 | 18.030 | 31 | 0.551 | 3.640 |
| K–R | 3.614 | 3.042 | 7.378 | 14.820 | 99.100 | 13.996 | 15.750 | 13.880 | 26 | 0.796 | 5.539 |

Table S-3. Extended Metrics: S3: Sensor Failure + Noise

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 0.222 | 0.167 | 1.597 | 14.980 | 4.700 | 10.761 | 25 | 161.140 | 80 | 0.044 | 0.342 |
| LQR | 0.453 | 0.375 | 1.866 | 12.900 | 1 | 3.182 | 13.490 | 44.570 | 10 | 0.098 | 0.670 |
| MPC | 1.004 | 0.849 | 2.027 | 14.980 | 3.700 | 0.955 | 1.500 | 1.670 | 5 | 0.222 | 1.758 |
| L1 | 0.908 | 0.715 | 1.940 | 12.780 | 3.300 | 5.311 | 17.850 | 80.960 | 37 | 0.187 | 0.992 |
| K-only | 0.453 | 0.375 | 1.866 | 12.900 | 1 | 3.182 | 13.490 | 44.570 | 10 | 0.098 | 0.670 |
| K–R | 0.262 | 0.184 | 1.797 | 14.980 | 4.900 | 5.784 | 18.770 | 79.550 | 50 | 0.048 | 0.355 |

Table S-4. Extended Metrics: S4: Parameter Drift (Fuel Burn)

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 0.272 | 0.232 | 1.663 | 14.040 | 2 | 1.867 | 25 | 1.360 | 3 | 0.061 | 0.521 |
| LQR | 0.966 | 0.812 | 1.600 | 14.980 | 13 | 1.300 | 12.770 | 0.490 | 3 | 0.213 | 1.381 |
| MPC | 1.209 | 0.993 | 2.367 | 14.980 | 8.300 | 1.004 | 1.560 | 0.130 | 8 | 0.260 | 2.024 |
| L1 | 2.447 | 1.990 | 4.362 | 14.980 | 35.400 | 1.439 | 10.920 | 1.340 | 3 | 0.521 | 3.713 |
| K-only | 0.966 | 0.812 | 1.600 | 14.980 | 13 | 1.300 | 12.770 | 0.490 | 3 | 0.213 | 1.381 |
| K–R | 0.261 | 0.166 | 1.585 | 5.700 | 7.100 | 1.268 | 14.380 | 0.690 | 10 | 0.043 | 0.162 |

Table S-5. Extended Metrics: S5: Adversarial Disturbance

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 0.159 | 0.094 | 1.846 | 1.840 | 1 | 1.824 | 25 | 1.180 | 1 | 0.025 | 0.167 |
| LQR | 0.452 | 0.400 | 1.935 | 13.020 | 0 | 1.121 | 12.760 | 0.450 | 0 | 0.105 | 0.731 |
| MPC | 1.175 | 0.924 | 2.108 | 14.780 | 4.300 | 0.974 | 1.330 | 0.110 | 9 | 0.242 | 1.956 |
| L1 | 1.359 | 1.286 | 1.959 | 14.980 | 0 | 1.026 | 10.750 | 1.200 | 0 | 0.337 | 2.408 |
| K-only | 0.452 | 0.400 | 1.935 | 13.020 | 0 | 1.121 | 12.760 | 0.450 | 0 | 0.105 | 0.731 |
| K–R | 0.197 | 0.108 | 1.934 | 1.880 | 2 | 1.242 | 12.780 | 0.540 | 3 | 0.028 | 0.139 |

Table S-6. Extended Metrics: S6: Computational Latency

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 2.558 | 2.234 | 4.699 | 14.980 | 33.100 | 22.160 | 25 | 33.950 | 42 | 0.585 | 4.448 |
| LQR | 1.355 | 1.137 | 3.017 | 14.960 | 21.400 | 20.238 | 25 | 47.840 | 56 | 0.298 | 2.287 |
| MPC | 1.780 | 1.428 | 3.896 | 14.980 | 15.800 | 1.006 | 1.660 | 0.160 | 9 | 0.374 | 3.401 |
| L1 | 1.801 | 1.509 | 5.556 | 14.940 | 26.500 | 22.134 | 25 | 44.490 | 53 | 0.395 | 2.949 |
| K-only | 1.355 | 1.137 | 3.017 | 14.960 | 21.400 | 20.238 | 25 | 47.840 | 56 | 0.298 | 2.287 |
| K–R | 1.916 | 1.641 | 3.631 | 14.980 | 29.500 | 21.050 | 25 | 39.790 | 47 | 0.430 | 3.354 |

Table S-7. Extended Metrics: S7: Multi-Axis Coupled Control

| Ctrl | RMSE(°) | MAE(°) | MaxErr(°) | Settle(s) | Over.(%) | CtrlRMS(°) | CtrlMax(°) | CtrlTV | Osc | IAE | ITAE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PID | 0.260 | 0.184 | 1.795 | 14.840 | 4.300 | 9.171 | 21.230 | 86.950 | 89 | 0.048 | 0.328 |
| LQR | 0.367 | 0.299 | 1.944 | 11.140 | 0.800 | 2.804 | 12.880 | 24.970 | 14 | 0.078 | 0.488 |
| MPC | 1.638 | 1.317 | 3.827 | 14.980 | 12.500 | 1.414 | 2.110 | 0.970 | 11 | 0.345 | 3.056 |
| L1 | 0.745 | 0.575 | 2.222 | 14.980 | 12.900 | 17.121 | 21.230 | 73.760 | 64 | 0.150 | 1.288 |
| K-only | 0.367 | 0.299 | 1.944 | 11.140 | 0.800 | 2.804 | 12.880 | 24.970 | 14 | 0.078 | 0.488 |
| K–R | 0.219 | 0.136 | 1.932 | 13.240 | 2.200 | 4.583 | 15.340 | 44.110 | 51 | 0.036 | 0.238 |

S-2. Controller Tuning Sweeps (Fairness Verification)

Each baseline controller’s key parameter was swept on scenario S4 (parameter drift) to verify that the reported comparison uses optimal or near-optimal settings for all methods. This addresses the reviewer question: “Did you tune baselines fairly?”

Table S-8. PID Gain Sweep (Ki=1.2, Kd=2.5 fixed) on S4

| Kp | RMSE (°) | Note |
| --- | --- | --- |
| 2 | 0.476 |  |
| 3 | 0.397 |  |
| 4 | 0.343 |  |
| 5 | 0.302 |  |
| 6 | 0.272 | Used in paper |
| 7 | 0.248 |  |
| 8 | 0.229 |  |
| 10 | 0.200 |  |
| 12 | 0.180 |  |
| 15 | 0.160 | Optimal (but fragile under latency) |

Note: Kp=15 achieves 0.160° on S4 but degrades to 4.2° under 100 ms latency (S6). Kp=6 provides robust performance across all scenarios.

Table S-9. LQR Cost Weight Sweep (Q_q=1, Q_θ=1, R=0.1 fixed) on S4

| Q_α | RMSE (°) | Note |
| --- | --- | --- |
| 10 | 1.792 |  |
| 30 | 1.382 |  |
| 50 | 1.199 |  |
| 80 | 1.038 |  |
| 100 | 0.966 | Used in paper |
| 150 | 0.841 |  |
| 200 | 0.759 |  |
| 300 | 0.653 |  |
| 500 | 0.536 | Optimal |

Table S-10. MPC Horizon Sweep on S4

| Horizon N | RMSE (°) | Note |
| --- | --- | --- |
| 3 | 1.224 | No horizon sensitivity (linearized model bottleneck) |
| 5 | 1.224 | No horizon sensitivity (linearized model bottleneck) |
| 8 | 1.224 | No horizon sensitivity (linearized model bottleneck) |
| 10 | 1.224 | Used in paper |
| 15 | 1.224 | No horizon sensitivity (linearized model bottleneck) |
| 20 | 1.224 | No horizon sensitivity (linearized model bottleneck) |

Table S-11. L1 Adaptive Gain Sweep (ωc=15 rad/s fixed) on S4

| Γ | RMSE (°) | Note |
| --- | --- | --- |
| 500 | 1.445 | Optimal |
| 1000 | 1.728 |  |
| 2000 | 2.064 |  |
| 5000 | 2.446 | Used in paper |
| 10000 | 2.367 |  |
| 20000 | 2.286 |  |
| 50000 | 2.277 |  |

Table S-12. K–R Forgetting Factor Sweep on S4

| λ | RMSE (°) | Note |
| --- | --- | --- |
| 0.980 | 0.372 |  |
| 0.990 | 0.258 |  |
| 0.993 | 0.257 | Optimal |
| 0.995 | 0.262 | Used in paper |
| 0.997 | 0.271 |  |
| 0.999 | 0.285 |  |
| 1.000 | 0.294 |  |

S-3. Generalization / Overfitting Test Details

R-weights were trained on Dryden severe turbulence (seed=100) for 12 s. The converged weights (||W||=0.599) were then transferred to three unseen disturbance profiles. Table S-13 reports the results.

Table S-13. Generalization Test: Train on Dryden → Test on Unseen Disturbances

| Test Condition | K-only | K–R fresh | K–R trained | Ratio |
| --- | --- | --- | --- | --- |
| Diff. turbulence seed | 0.269 | 0.198 | 0.182 | 0.921 |
| Discrete (1-cos) gusts | 0.270 | 0.198 | 0.182 | 0.920 |
| Adversarial resonant | 0.272 | 0.198 | 0.182 | 0.919 |

All ratios < 1.0: pre-trained K–R outperforms fresh K–R on every unseen disturbance type, confirming that the R-step learns the model mismatch structure rather than the specific disturbance realization.

S-4. Scalability Test Details

Table S-14. Scalability: 1-Axis to 3-Axis

| Configuration | K-only (°) | K–R (°) | Improvement |
| --- | --- | --- | --- |
| 1-axis (pitch, nominal) | 0.486 | 0.215 | +55.7% |
| 1-axis + model mismatch | 0.267 | 0.194 | +27.4% |
| 3-axis coupled (pitch+roll) | 0.407 | 0.232 | +42.9% |

K–R improvement is consistent (27–56%) across all system dimensions. No re-tuning of λ, λ_r, or W_max was required between configurations.

S-5. Extended Worst-Case Analysis (100 runs)

Table S-15. Full Worst-Case Statistics (100 Monte Carlo runs)

| Ctrl | Mean | Std | 95th% | 99th% | Worst | MaxErr | Div |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S1: Extreme Turbulence |  |  |  |  |  |  |  |
| PID | 0.145 | 0.000 | 0.145 | 0.146 | 0.146 | 1.879 | 0 |
| LQR | 0.286 | 0.002 | 0.289 | 0.291 | 0.292 | 1.936 | 0 |
| K–R | 0.179 | 0.000 | 0.179 | 0.180 | 0.180 | 1.936 | 0 |
| S4: Parameter Drift |  |  |  |  |  |  |  |
| PID | 0.274 | 0.002 | 0.277 | 0.278 | 0.279 | 1.666 | 0 |
| LQR | 0.965 | 0.009 | 0.981 | 0.983 | 0.983 | 1.633 | 0 |
| L1 | 2.446 | 0.028 | 2.498 | 2.507 | 2.512 | 4.474 | 0 |
| K–R | 0.260 | 0.001 | 0.263 | 0.264 | 0.264 | 1.589 | 0 |

S-6. Latency Crossover Data

Table S-16. RMSE vs Computational Delay

| Delay (ms) | K-only (°) | K–R (°) | Improvement | Winner |
| --- | --- | --- | --- | --- |
| 0 | 0.441 | 0.193 | +56.3% | K–R |
| 20 | 0.441 | 0.192 | +56.5% | K–R |
| 40 | 0.439 | 0.195 | +55.7% | K–R |
| 60 | 0.488 | 0.917 | −87.7% | K-only |
| 80 | 0.910 | 1.342 | −47.5% | K-only |
| 100 | 1.355 | 1.916 | −41.4% | K-only |
| 150 | 2.737 | 3.610 | −31.9% | K-only |
| 200 | 6.522 | 8.036 | −23.2% | K-only |

Crossover occurs at approximately 60 ms. Below this threshold, K–R’s online adaptation provides significant advantage (+56%). Above it, the RLS weight update introduces phase lag that compounds with transport delay.

S-7. Real-Time Timing Results

Table S-17. Computation Time per Controller (5000 calls, x86_64)

| Controller | Mean (µs) | Std (µs) | Max (µs) | % of 10ms | % of 1ms |
| --- | --- | --- | --- | --- | --- |
| K-only | 3.6 | 1.2 | 31 | 0.04 | 0.36 |
| LQR | 5.0 | 1.3 | 25 | 0.05 | 0.50 |
| PID | 9.8 | 223 | 15,811 | 0.10 | 0.98 |
| L1 | 10.4 | 96 | 6,810 | 0.10 | 1.04 |
| MPC | 19.9 | 111 | 7,862 | 0.20 | 1.99 |
| K–R | 34.0 | 126 | 6,799 | 0.34 | 3.40 |

F-16 dynamics evaluation: 30.4 µs/call. K–R total per step: 34.0 + 30.4 = 64.5 µs (0.64% of 10 ms cycle). Platform: Linux x86_64, Python 3.12, numpy 2.4.

S-8. Metric Definitions

| Metric | Definition |
| --- | --- |
| RMSE | Root mean square error: √(mean(e²)), where e = α_ref − α |
| MAE | Mean absolute error: mean(¦e¦) |
| MaxErr | Maximum absolute error: max(¦e¦) over entire simulation |
| Settle time | Time of last crossing of 2% error band around final reference |
| Overshoot | (max(α) − α_ref_final) / ¦α_ref_final¦ × 100% |
| Ctrl RMS | Root mean square of elevator deflection: √(mean(δe²)) |
| Ctrl Max | Maximum absolute elevator deflection |
| Ctrl TV | Total variation of control: Σ¦Δδe¦, measures control smoothness |
| Oscillations | Number of zero-crossings of tracking error e(t) |
| IAE | Integral of absolute error: ∫¦e¦ dt |
| ITAE | Integral of time-weighted absolute error: ∫ t¦e¦ dt |

S-9. Complete Reproducibility Information

A. Controller Parameters

Table S-18. All Controller Hyperparameters

| Controller | Parameters | Source |
| --- | --- | --- |
| PID | Kp=6.0, Ki=1.2, Kd=2.5, T_f=0.05 s (deriv. filter), int_max=±0.3 rad | [3] |
| LQR | Q=diag(100, 1, 10), R=0.1, states=[α, q, θ] | [4] |
| MPC | N=10, Q=diag(100,1,10), R=1.0, dt_mpc=0.05 s, SLSQP solver | [5] |
| L1 | Γ=5000, ωc=15 rad/s, σ_max=50 | [6] |
| K-only | Kα=5.0, Kq=2.0, Kθ=1.0 | [4] |
| K–R | Kα=5.0, Kq=2.0, Kθ=1.0, λ=0.993, λr=0.001, Wmax=10, Rmax=10°, nf=12 | This work |

B. Random Seeds

All simulations use deterministic seeding for exact reproducibility:

| Simulation | Seed Rule |
| --- | --- |
| Single-run results (Tables 3, S-1 through S-7) | seed = 42 |
| Monte Carlo campaigns (Table 4) | seed = 100 + run_index, run_index ∈ [0, N) |
| Worst-case analysis (Table 5, S-15) | seed = 5000 + run_index, run_index ∈ [0, 100) |
| Generalization test (Table S-13) | Training: seed=100. Test seeds: 999 (turb), N/A (gust), N/A (adv) |
| Fairness sweeps (Tables S-8 through S-12) | seed = 42 (all sweeps) |

C. Simulation Configuration

Integration method: 4th-order Runge-Kutta. Timestep: dt = 0.02 s. Duration: T = 15 s (all scenarios). State clamps: V ∈ [30, 300] m/s, α ∈ [−15°, 55°]. Actuator: position limit ±25° (default, scenario-specific in Table 2 of main manuscript), rate limit ±60°/s.

D. Software Environment

Language: Python 3.12.3. Dependencies: numpy 2.4.2, scipy 1.17.0. No proprietary toolboxes. Operating system: Linux (Ubuntu 24). Total wall-clock time for all simulations (7 scenarios × 6 controllers × single run + 4 Monte Carlo campaigns + all verification tests): approximately 5 minutes on standard hardware.

E. Code Repository Structure

| File | Description | Size |
| --- | --- | --- |
| fast_sim.py | Main simulation runner: all 7 scenarios, 6 controllers, Monte Carlo engine | 22 KB |
| core_tests.py | 7 verification tests: ablation, generalization, residual structure, convergence, disturbance estimation, scalability, linear sanity | 31 KB |
| final_checks.py | Worst-case analysis, fairness tuning sweeps, real-time timing, limitations | 24 KB |
| theory_suite.py | Theory verification: RLS convergence, Lyapunov, robustness margins, PE | 31 KB |
| models/f16_model.py | F-16 aerodynamic model with Stevens & Lewis lookup tables [1,8] | 26 KB |
| models/turbulence.py | MIL-F-8785C Dryden turbulence model with forming filters [9,11] | 11 KB |
| controllers/controllers.py | All 6 controller implementations with common interface | 26 KB |
| results/*.json | Raw numerical results in JSON format for independent verification | 32 KB |

F. How to Reproduce

Step 1: Install Python 3.10+ with numpy and scipy: pip install numpy scipy

Step 2: Run main results (reproduces Table 3): python fast_sim.py

Step 3: Run verification tests (reproduces Tables S-6, S-13, S-14): python core_tests.py

Step 4: Run worst-case + fairness (reproduces Tables S-8–S-12, S-15): python final_checks.py

Step 5: Regenerate figures: python generate_figures.py

All commands should complete within 5 minutes. Output JSON files in the results/ directory can be compared against the provided reference results for verification.