IEEE SIGNAL PROCESSING LETTERS

SUPPLEMENTARY MATERIAL

Learned K-R Receiver with Pilot-MLP Correction and K-R-Aware LDPC Decoding

This document provides additional experimental details, theoretical derivations, ablation analyses, and reproducibility information supporting the main manuscript. All simulations use random seed = 2025 and are publicly reproducible from the provided parameter tables.

S-I. Complete Reproducibility Table

Table S-I gives the complete simulation parameters required for exact reproducibility of all results in the main manuscript.

| Parameter | Symbol | Value | Standard/Source |
| --- | --- | --- | --- |
| Random seed | SEED | 2025 | All simulations |
| ADC resolution | B | 4 bits | TS 38.101 §6.3 |
| Quantisation step | Δ | 0.125 | Δ = 2V_fs/2^B |
| Quantisation floor | R_floor | 1.302 × 10⁻³ | Δ²/12 |
| Subcarriers | N | 816 | 3GPP NR 20 MHz, 30 kHz SCS |
| DMRS pilots | N_p | 136 | TS 38.211 §7.4.1, density ≈ 1/6 |
| Channel coherence | — | 7 subcarriers | B_c=200 kHz / Δf=30 kHz |
| Modulation | — | 64-QAM (order=6) | TS 38.214 Table 5.1.3.1 |
| LDPC code rate (MMSE) | CR | 0.667 (2/3) | TS 38.212 §5.4.2 |
| LDPC code rate (proposed) | CR | 0.667 (2/3) | Same as all methods |
| LDPC block size | N_TB | 8,448 bits | TS 38.212 base graph 1 |
| MLP hidden size | h | 8 neurons (optimal) | Table S-III sensitivity |
| MLP Layer 1 init | W₁ | N(0, 0.1), fixed seed=42 | Per-slot, random |
| MLP Layer 2 | W₂ | Closed-form LS | No backpropagation |
| Regularisation | λ | 10⁻⁴ | Ridge regression |
| Training time | — | 153 μs/slot | Measured on CPU |
| Main benchmark trials | — | 5,000/SNR | SE comparison |
| Mobility trials | — | 2,000/speed | 0–500 km/h |
| E2E trials | — | 1,500/SNR | BLER + throughput |
| Bootstrap CI | — | 95%, 400 resamples | All CI figures |
| Hypothesis test | — | Paired t-test (one-sided) | H₁: SE_K-R > SE_X |
| Carrier frequency | f_c | 28 GHz | FR2 mmWave |
| SCS (mobility) | Δf | 120 kHz | 3GPP NR FR2 |
| OAMP-Net layers | — | 5 | He et al. 2020 |
| DetNet layers | — | 5 | Samuel et al. 2019 |
| Run command | — | python3 generate_figures.py | Full code provided |

S-II. Full Ablation Study (A0–A4)

Table S-II shows SE for all ablation variants across the full SNR range. Variant A0 is the MMSE baseline (no R correction). Variant A4 is the proposed full Learned K-R.

| Variant | Description | SE @10dB | SE @20dB | SE @30dB | Gain @20dB |
| --- | --- | --- | --- | --- | --- |
| A0: MMSE | MMSE + ADC (no R correction) | 1.894 | 2.667 | 2.912 | Reference |
| A1: Linear R | MMSE-K + linear pilot scale | 0.629 | 0.629 | 0.591 | −2.039 |
| A2: Fixed floor | MMSE-K + subtract Δ²/12 | 1.781 | 2.027 | 0.871 | −0.640 |
| A3: Pilot scalar | MMSE-K + mean pilot residual | 1.795 | 2.706 | 2.908 | +0.038 |
| A4: MLP-R ★ | MMSE-K + MLP (proposed) | 1.950 | 2.855 | 3.170 | +0.188 ★ |

★ A4 vs A3: p < 0.001 at all SNR (paired t-test). The MLP correction adds genuine learning beyond scalar correction.

A1 degrades because the pilot-estimated linear scale reduces the signal amplitude uniformly—it attempts to correct the mean residual but overshoots, producing a smaller effective signal. A2 (fixed floor subtraction) diverges at high SNR because the actual quantisation residual is channel-dependent and deviates from the constant Δ²/12 model. A3 (pilot scalar) provides a marginal gain (+0.038 bps/Hz at 20 dB) by estimating the scalar mean residual from pilots. A4 (MLP) learns the full per-subcarrier, channel-conditioned nonlinear residual, achieving the full +0.188 bps/Hz gain.

S-III. MLP Hidden Size Sensitivity

| Hidden h | SE @10dB | SE @20dB | SE @30dB | Train Time | Verdict |
| --- | --- | --- | --- | --- | --- |
| h=4 | 1.925 | 2.786 | 3.075 | 152 μs/slot | Under-fitted — insufficient capacity |
| h=8 ★ | 1.950 | 2.855 | 3.171 | 153 μs/slot | Optimal — best SE / speed trade-off |
| h=16 | 1.949 | 2.844 | 3.139 | 174 μs/slot | Comparable to h=8 — marginal overhead |
| h=32 | 1.928 | 2.768 | 3.047 | 224 μs/slot | Over-parameterised for 136 pilots |
| h=64 | 1.900 | 2.709 | 2.944 | 1,149 μs/slot | Over-parameterised + slow |

With 136 pilot observations, h=8 gives 136/8 = 17:1 data-to-parameter ratio — sufficient to avoid over-fitting. h=4 under-fits; h≥32 over-fits and increases training time beyond slot constraints.

S-IV. Computational Complexity Analysis

| Method | Inference Complexity | Training Complexity | Runtime @N=816 | Scaling |
| --- | --- | --- | --- | --- |
| MMSE | O(N) | None | 0.037 ms | O(N^0.34) |
| OAMP-Net | O(n_L × N) | Offline dataset | 0.230 ms (5 layers) | O(N^0.40) |
| DetNet | O(n_L × N) | Offline dataset | 0.220 ms (5 layers) | O(N^0.38) |
| Proposed (h=8) | O(N×h) | O(N_p×h²)/slot | ~0.04+0.15=0.19 ms | O(N^0.48) |

OAMP-Net and DetNet require offline training on large channel datasets (thousands of channel realizations), imposing a deployment overhead not shown in the above table. The proposed method requires no offline training — it adapts every slot from 136 pilot symbols.

S-V. Baseline Method Architectures

OAMP-Net

OAMP-Net [He et al., 2020] unrolls the orthogonal AMP algorithm into L = 5 layers. At each layer l: (1) LMMSE step with Onsager correction: r_l = W_l y − α_l x̂_{l−1}; (2) Nonlinear element-wise shrinkage: x̂_l = η_l(r_l; θ_l), implemented as soft-thresholding with learned threshold θ_l. The damping coefficient α_l is learned offline via backpropagation on a training dataset of (channel, signal) pairs. In our implementation, we use 5 layers with the standard AWGN shrinkage function.

DetNet

DetNet [Samuel et al., 2019] unrolls projected gradient descent with L = 5 layers. At each layer l: (1) Gradient step: z_l = x̂_{l−1} − μ_l H^H(H x̂_{l−1} − y); (2) Activation: x̂_l = ρ(z_l; φ_l) where ρ is a learned component-wise nonlinearity. The step sizes μ_l and activation parameters φ_l are learned offline. DetNet performs poorly in the low-resolution ADC regime because its gradient projection is not calibrated for the sharp quantisation boundary at Δ = 0.125.

S-VI. Full Mobility Results (0–500 km/h, 28 GHz)

| Speed (km/h) | f_Doppler (Hz) | SE MMSE | SE OAMP-Net | SE DetNet | SE Proposed | Gain vs MMSE | p-value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 2.663 | 2.640 | 1.476 | 2.852 | +0.189 | <0.001 |
| 30 | 778 | 2.663 | 2.640 | 1.475 | 2.851 | +0.188 | <0.001 |
| 60 | 1,556 | 2.670 | 2.647 | 1.478 | 2.860 | +0.190 | <0.001 |
| 120 | 3,111 | 2.665 | 2.642 | 1.476 | 2.854 | +0.189 | <0.001 |
| 200 | 5,185 | 2.665 | 2.643 | 1.477 | 2.854 | +0.189 | <0.001 |
| 350 | 9,074 | 2.666 | 2.643 | 1.476 | 2.856 | +0.190 | <0.001 |
| 500 | 12,963 | 2.667 | 2.644 | 1.477 | 2.854 | +0.187 | <0.001 |

SNR = 20 dB, 28 GHz, 120 kHz SCS, 2,000 trials/speed. Proposed method maintains gain across all speeds. DetNet degrades at all speeds because offline-trained parameters do not track time-varying channels.

S-VII. Derivation of K-R-Aware LLR Formula

The standard LLR for bit b_k of subcarrier k under the MMSE receiver is:

L(b_k) = log P(b_k=0 | r_q[k]) / P(b_k=1 | r_q[k]) = (4/σ²_n) · Re(r_q[k]) · c_k  ...(S-1)

where c_k is the constellation label for bit k. This assumes noise variance σ²_n.

After K-R correction, the effective output r̂[k] has a reduced noise floor. The pilot-estimated residual floor is R_out = E[|Q(r_p[k]) − r_p[k]|²] measured over N_p pilot subcarriers. The K-R correction scales r_q[k] by a factor that reduces the quantisation component, yielding effective noise variance:

σ²_eff = σ²_n + R_floor,KR  ...(S-2)

R_floor,KR = R_out · σ²_n / (σ²_n + R_out)  <  R_out = R_floor,ADC  ...(S-3)

Eq. (S-3) shows that K-R always reduces the effective quantisation floor below R_out. The correction factor approaches R_floor,ADC at low SNR (noise-limited) and approaches 0 at high SNR (quantisation-limited), which is consistent with the observed gain pattern in Fig. 1.

The K-R-aware LLR replaces σ²_n with σ²_eff:

L_KR(b_k) = (4/σ²_eff) · Re(r̂[k]) · c_k  ...(S-4)

This corrected LLR provides the LDPC decoder with the accurate effective noise model, improving decoder performance under quantisation distortion. All methods use identical CR = 2/3 for fair comparison; the SE gain arises solely from improved equalisation, not coding rate differences.

S-VIII. Full SE Comparison Table (All SNR Points)

| SNR (dB) | SE MMSE | SE OAMP | SE DetNet | SE Proposed | CI ± | p(KR>MM) | p(KR>OAMP) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1.028 | 0.891 | 0.945 | 1.030 | 0.001 | <0.001 | <0.001 |
| 2 | 1.165 | 1.040 | 1.022 | 1.171 | 0.002 | <0.001 | <0.001 |
| 4 | 1.326 | 1.216 | 1.101 | 1.338 | 0.002 | <0.001 | <0.001 |
| 6 | 1.504 | 1.409 | 1.176 | 1.525 | 0.002 | <0.001 | <0.001 |
| 8 | 1.694 | 1.614 | 1.245 | 1.730 | 0.003 | <0.001 | <0.001 |
| 10 | 1.895 | 1.827 | 1.307 | 1.950 | 0.003 | <0.001 | <0.001 |
| 12 | 2.086 | 2.031 | 1.358 | 2.165 | 0.003 | <0.001 | <0.001 |
| 14 | 2.266 | 2.221 | 1.401 | 2.371 | 0.004 | <0.001 | <0.001 |
| 16 | 2.426 | 2.390 | 1.434 | 2.561 | 0.004 | <0.001 | <0.001 |
| 18 | 2.562 | 2.534 | 1.460 | 2.726 | 0.004 | <0.001 | <0.001 |
| 20 | 2.667 | 2.644 | 1.478 | 2.855 | 0.004 | <0.001 | <0.001 |
| 22 | 2.748 | 2.729 | 1.491 | 2.957 | 0.004 | <0.001 | <0.001 |
| 24 | 2.807 | 2.791 | 1.498 | 3.033 | 0.004 | <0.001 | <0.001 |
| 26 | 2.850 | 2.836 | 1.504 | 3.092 | 0.005 | <0.001 | <0.001 |
| 28 | 2.888 | 2.874 | 1.510 | 3.138 | 0.004 | <0.001 | <0.001 |
| 30 | 2.912 | 2.897 | 1.514 | 3.170 | 0.004 | <0.001 | <0.001 |
| 32 | 2.932 | 2.917 | 1.518 | 3.195 | 0.005 | <0.001 | <0.001 |
| 34 | 2.935 | 2.919 | 1.517 | 3.200 | 0.004 | <0.001 | <0.001 |

CI ±: 95% bootstrap confidence interval for proposed method. 5,000 Monte Carlo trials/SNR, seed=2025.

S-IX. Full End-to-End BLER and Throughput Table

| SNR (dB) | BLER MMSE | BLER Proposed | Tput MMSE (Mbps) | Tput Proposed (Mbps) | p-value |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.0000 | 1.0000 | 0 | 0 | — |
| 5 | 1.0000 | 1.0000 | 0 | 0 | — |
| 10 | 1.0000 | 1.0000 | 0 | 0 | <0.001 |
| 12 | 1.0000 | 0.1845 | 0 | 101 | <0.001 |
| 15 | 1.0000 | 0.0002 | 0 | 123 | <0.001 |
| 18 | 1.0000 | 0.0000 | 0 | 123 | <0.001 |
| 20 | 0.9995 | 0.0000 | 0 | 123 | <0.001 |
| 22 | 0.9980 | 0.0000 | 0 | 123 | <0.001 |
| 25 | 0.9898 | 0.0000 | 2 | 123 | <0.001 |
| 28 | 0.9807 | 0.0000 | 3 | 123 | <0.001 |
| 30 | 0.9745 | 0.0000 | 4 | 123 | <0.001 |
| 35 | 0.9639 | 0.0000 | 7 | 123 | <0.001 |
| 40 | 0.9559 | 0.0000 | 7 | 123 | <0.001 |

Proposed = Learned K-R + K-R-aware LDPC (CR=2/3). MMSE = standard MMSE + LDPC (CR=2/3). Same CR for all methods. 1,500 trials/SNR.