Significantly reducing the impact of light: a novel framework for smartphone to predict water turbidity

[Supplementary material]

Lingyan Qi a, b, Kejia Zhang a, Mingzhu Guo a, Xinzhe Jiang a, Weiwei Zhao a, Jiefa Cai a, Yaqin Duan a, Jiacong Huang c, *

a School of Geography and Tourism, Anhui Normal University, Wuhu 241002, China

b Engineering Technology Research Center of Resources Environment and GIS, Anhui Province, Wuhu 241002, China

c Key Laboratory of Watershed Geographic Sciences, Nanjing Institute of Geography and Limnology, Chinese Academy of Sciences, Nanjing 210008, China

* Corresponding author.

E-mail: jchuang@niglas.ac.cn (J. Huang)

- 1. Convolutional neural networks used to estimate turbidity

Convolutional neural networks were developed with the idea of local connectivity. Each node is connected only to a local region in the input. The input is usually taken to be three-dimensional: the height, weight, and number of channels. The equations were displayed as follows Borovykh (2017).

Where  is the input value of the basic feedforward neural network, while are referred to as the weights and  as the biases.  represents the number of nodes in each hidden layer of a feedforward neural network.

Where  and ,  is a differentiable nonlinear activation function, while  is the layer  of the basic feedforward neural network.

Where  and ,  is the outputs from the previous layer are again linearly combined and passed through the nonlinearity.

Where  is a discrete convolution of two one-dimensional signals. A convolution at point i is computed by shifting the signal g over the input f along j and computing the weighted sum of the two.

Suppose and , the convolution of the two is given by.

Where  and ,  is a one-dimensional input of size N with no zero padding. The output feature map from the first layer is then given by convolving each filter  for h = 1, ..., M1. Note that since the number of input channels in this case is one, the weight matrix also has only one channel. Like the feedforward neural network, this output is then passed through the non-linearity  to give .

In each subsequent layer  = 2, ..., the input feature map, , where  is the size of the output filter map from the previous convolution with , is convolved with a set of  filters , h = 1, ..., , to create a feature map , and  is the filter size parameter, thus controls the receptive field of each output node. Without zero padding, the convolution output in every layer has width  for l = 1, ..., L.

- 2. Metrics’ equations to evaluate model fits

,

,

,

,

Where  and  represent the average values of predicted  and measured , respectively.

Fig. S1. Representative images of water sample collection for the 653 paired records used to develop and validate the framework, showing only one sampling site per city. Each panel includes the location, waterbody type, date, weather, and smartphone used.

Fig. S2. 653 preprocessed water-surface images generated by the smartphone framework, arranged by increasing turbidity values (NTU). Each image represents a single water sample processed under consistent imaging conditions.

Table S1. Studies deducing indicators of turbidity. R²=coefficient of determination, RMSE=root means square error, MAE=mean absolute error, GOCI= geostationary ocean color imager, UAV= unmanned aerial vehicle, MAPE= Mean Absolute Percentage Error, Bias=mean error, NS=Nash–Sutcliffe efficiency, PLS=Partial Least Squares, SLR=Stepwise Linear Regression, Resnet=Residual Network.

| Techniques | Sensor | Range | Accuracy | Study area | Time | Process | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Empirical regression models | Landsat 8 | 2.3–107.02 NTU | R² = 0.97 RMSE = 1.414 NTU | Tennessee River (USA) | 2 December 2018, 13 February 2019, 15 August 2019 | River turbidity estimation using Landsat 8 OLI red band nonlinear regression | Hossain et al. (2021) |
| Neural network approach | GOCI | 4.66-71.46 NTU | R² = 0.84; RMSE = 58.8 NTU; MAE = 25.1 NTU | Yellow River Estuary, Yangtze River Estuary, Hangzhou Bay (China) | March–April 2018 | Retrieval of turbidity in turbid estuaries using GOCI data and neural network, analysis of turbidity-tide relationship | Feng et al. (2020) |
| Partial Least Squares regression | UAV hyperspectral imager (DJI M600 pro) | 3.2–56.37 NTU | R² = 0.72, MAPE=19.35% | Aquaculture ponds, irrigation canals, and reservoir in Dawa District, Panjin (China) | September 2019; October 2020 | Retrieval of turbidity in small water bodies using UAV hyperspectral data and PLS model, mapping spatial distribution | Cui et al. (2022) |
| Empirical regression model | Landsat 8 | 0.21-3.46 FTU | R² = 0.84, RMSE=0.28 FTU | Cam Ranh Bay and Thuy Trieu Lagoon (Vietnam) | 2013–2016 | Retrieval of turbidity using Landsat 8 OLI red band, analysis of spatiotemporal variation and controlling factors | Quang et al. (2017) |
| Stepwise Linear Regression, Polynomial Kernel Regression | Sentinel-2 | 0-80 NTU | SLR: R²=0.736, RMSE=2.03 NTU, MAE=1.39 NTU Kernel: R²=0.725, RMSE=2.07 NTU, MAE=1.40 NTU | North Tyrrhenian Sea (Italy) | 2015 to 2021 | Retrieval of seawater turbidity using Sentinel-2 MSI Level-1C TOA reflectance with machine learning methods, generation of turbidity maps | Magrì et al. (2023) |
| Ordinary least squares regression model | Landsat 8 | 12.3-177.5 NTU | R² = 0.87 | Lake Tana (Ethiopia) | June 2020 | Retrieval and spatial distribution mapping of lake turbidity using Landsat 8 OLI imagery and OLS regression | Ahmad et al. (2025) |
| Single-band turbidity algorithm | Landsat 5 | 1–70 NTU (normal range) >100 NTU (extreme events) | R²=0.70±0.15，RMSE=7.78±2.59 NTU | Apalachicola Bay (USA) | 2004 to 2011 | Complex interactions of physical forcings and their effects on water dynamics have been discussed | Joshi et al. (2017) |
| Random forest model | Sentinel-2 | 0.5–15 NTU | R² = 0.632 | Los Angeles (USA) | July 2021 to March 2024 | Integrating satellite data with machine learning | Kong et al. (2025) |
| Statistical regression-based turbidity retrieval | GF-1 WFV, HJ-1A/1B CCD | 1.5–140 NTU | GF-1 WFV: R²=0.79 HJ-1A/1B CCD: R²=0.91 | Weishan Lake, Shandong Province (China) | July 2014 to June 2015 | Retrieval of lake turbidity using GF-1 and HJ-1 imagery with statistical regression models, analysis of spatiotemporal variation | Yin et al. (2019) |
| Gradient Boosting Regression | Sentinel-2 | 0–2200 FNU | R²=0.90, RMSE=116.62 FNU, MAE=43.24 FNU, Bias=1.32 FNU | 17 countries globally (68 lakes, 2 rivers, 2 estuaries, 11 coastal oceans) | 2005–2021 | Development and application of a global turbidity retrieval model using Sentinel-2 imagery and Gradient Boosting Regression | Chowdhury et al. (2025) |
| Empirical algorithm | MODIS | 1.8–160 FTU | R²=0.87 | Black Sea (Romania) | 2013–2014 | Retrieval of turbidity using MODIS data and analysis of spatiotemporal dynamics of the Danube River plume | Constantin et al. (2016) |
| Empirical multivariate regression | Sentinel-2, Landsat-8 | 1.36–7.42 NTU | Sentinel-2: R²=0.80 Landsat-8: R²=0.81 | Chebara Dam (Kenya) | January 2019 | Retrieval of reservoir turbidity using Sentinel-2A and Landsat-8 imagery, comparison of sensor performance | Ouma et al. (2020) |
| Artificial neural network | OPPO Reno3 | 6.53–91.10 NTU | NSE=0.89, RMSE=4.96, MAE=4.29 | Yangtze River Delta (China) | May to November 2022 | Retrieval of turbidity in small water bodies using smartphone imagery | Gan C (2023) |
| Radiometric turbidity retrieval method using smartphone camera and gray card | Smartphone cameras (iPhone 4/5, iPod touch, HTC One M8, Nexus 5, Galaxy S5, etc.) | 0.5–80 NTU | R²=0.93 | Columbia River mouth, Maine coast, Georgia coast, Washington State, San Francisco Bay (USA) | 2013–2017 | Development and validation of a smartphone-based turbidity measurement application using camera and 18% gray card | Leeuw and Boss (2018) |
| Bayesian modeling | Smartphone | 1.09-216 NTU | NS = 0.89 | Various water bodies across China | 2020 to 2021 | Predict turbidity from smartphone images using Bayesian model and field data | Huang et al. (2021) |
| Deep learning | Hikvision RGB surveillance camera | 0-58.3 NTU | R2 = 0.89 | Qiangyong Co Lake, Tibetan Plateau (China) | May to June 2023 | Turbidity retrieved from RGB images via lake segmentation and deep learning regression | Lu et al. (2025) |
| Convolutional Neural Network | Samsung Galaxy Ultra S21 smartphone camera | 0–40 NTU | Formazine sample classification accuracy 98.7% Kaolin clay sample classification accuracy 90.9% | Laboratory | 2024 | Study on drinking water turbidity classification using smartphone camera and CNN | Jantarakasem et al. (2024) |
| convolutional neural network-random forest model | Sentinel-2 | 0.16–192.49 NTU | R²=0.9, RMSE=11.698 NTU | Xingxingshao Reservoir, Chagan Lake, Erlong Lake, Jingpo Lake (China) | 2018–2024 | Retrieval of lake and reservoir turbidity using Sentinel-2 imagery and weighted blending deep learning model based on optical water type classification | Ma et al. (2025) |
| Gradient boosting (CatBoost, LightGBM, XGBoost) and convolutional neural network | USGS 14206950, USGS 14211720 | 0.3–170 FNU | USGS 14206950: XGBoost model R=0.951, NSE=0.903, RMSE=3.552 FNU, MAE=1.816 FNU USGS 14211720: CatBoost model R=0.961, NSE=0.920, RMSE=2.502 FNU, MAE=1.219 FNU | Oregon (USA) | 2014–2022 | Estimation of river turbidity using machine learning and deep learning models based on hydrological and water quality variables | Gharehbaghi et al. (2025) |
| Resnet | Network camera | 0-100 FNU | Good light: R²=0.96, MAE=5.86 FNU Low light: R²=0.90, MAE=6.09 FNU Shadow: R²=0.97, MAE=6.20 FNU | Montgomery County, Missouri, USA (Site 06934500) Douglas County, Nebraska, USA (Site 06800500) | 2022–2023 | Non-contact river turbidity monitoring using video images, addressing data imbalance and lighting conditions | Zhou et al. (2024) |

- References

Ahmad, I., Alasgah, A. A., Zelenakova, M., Dar, M. A., Damtie, M., Berhan, M., 2025. Quantifying turbidity dynamics in lake water using OLS regression: A landsat 8 OLI-based remote sensing approach. Journal of Hydrology: Regional Studies 60. https://doi.org/10.1016/j.ejrh.2025.102523

Borovykh, A., Bohte, S., Oosterlee, C.W., 2017. Conditional time series forecasting with convolutional neural networks. arXiv: Machine Learning 5, 1–22. https://doi.org/https://doi.org/10.48550/arXiv.1703.04691

Chowdhury, M., de la Calle, I., Laiz, I., Ruescas, A. B., 2025. Near-Real-Time Turbidity Monitoring at Global Scale Using Sentinel-2 Data and Machine Learning Techniques. Remote Sensing 17(22). https://doi.org/10.3390/rs17223716

Constantin, S., Doxaran, D., Constantinescu, Ș., 2016. Estimation of water turbidity and analysis of its spatio-temporal variability in the Danube River plume (Black Sea) using MODIS satellite data. Continental Shelf Research 112, 14–30. https://doi.org/10.1016/j.csr.2015.11.009

Cui, M., Sun, Y., Huang, C., Li, M., 2022. Water Turbidity Retrieval Based on UAV Hyperspectral Remote Sensing. Water 14(1). https://doi.org/10.3390/w14010128

Feng, J., Chen, H., Zhang, H., Li, Z., Yu, Y., Zhang, Y., Bilal, M., Qiu, Z., 2020. Turbidity Estimation from GOCI Satellite Data in the Turbid Estuaries of China’s Coast. Remote Sensing 12(22). https://doi.org/10.3390/rs12223770

Gan C, L. C. W., Zhao X M, Zhang S Q, Liu X M, Xia Y Q, 2023. Research on the inversion of water quality parameters in small and

micro water bodies based on smartphone and artificial neural network. Research of Agricultural Modernization 44(5), 892–902. https://doi.org/10.13872/j.1000-0275.2023.0087

Gharehbaghi, A., Heddam, S., Mehdizadeh, S., Kim, S., 2025. Development of interpretable intelligent frameworks for estimating river water turbidity. Engineering Applications of Computational Fluid Mechanics 19(1). https://doi.org/10.1080/19942060.2025.2511886

Hossain, A. K. M. A., Mathias, C., Blanton, R., 2021. Remote Sensing of Turbidity in the Tennessee River Using Landsat 8 Satellite. Remote Sensing 13(18). https://doi.org/10.3390/rs13183785

Huang, J., Qian, R., Gao, J., Bing, H., Huang, Q., Qi, L., Song, S., Huang, J., 2021. A novel framework to predict water turbidity using Bayesian modeling. Water Research 202. https://doi.org/10.1016/j.watres.2021.117406

Jantarakasem, C., Sioné, L., Templeton, M. R., 2024. Estimating drinking water turbidity using images collected by a smartphone camera. AQUA — Water Infrastructure, Ecosystems and Society 73(6), 1277–1284. https://doi.org/10.2166/aqua.2024.085

Joshi, I. D., D’Sa, E. J., Osburn, C. L., Bianchi, T. S., 2017. Turbidity in Apalachicola Bay, Florida from Landsat 5 TM and Field Data: Seasonal Patterns and Response to Extreme Events. Remote Sensing 9(4). https://doi.org/10.3390/rs9040367

Kong, Y., Jimenez, K., Lee, C. M., Winter, S., Summers-Evans, J., Cao, A., Menczer, M., Han, R., Mills, C., McCarthy, S., Blatzheim, K., Jay, J. A., 2025. Monitoring Coastal Water Turbidity Using Sentinel2—A Case Study in Los Angeles. Remote Sensing 17(2). https://doi.org/10.3390/rs17020201

Leeuw, T., Boss, E., 2018. The HydroColor App: Above Water Measurements of Remote Sensing Reflectance and Turbidity Using a Smartphone Camera. Sensors 18(1). https://doi.org/10.3390/s18010256

Lu, Z., Zhu, X., Li, J., Li, M., Wang, J., Wang, W., Zheng, Y., Zhang, Q., 2025. Detecting glacial lake water quality indicators from RGB surveillance images via deep learning. International Journal of Applied Earth Observation and Geoinformation 136. https://doi.org/10.1016/j.jag.2025.104392

Ma, Y., Chen, Q., Song, K., Yang, Q., Zheng, Q., Ma, Y., 2025. An Optical Water Type-Based Deep Learning Framework for Enhanced Turbidity Estimation in Inland Waters from Sentinel-2 Imagery. Sensors 25(20). https://doi.org/10.3390/s25206483

Magrì, S., Ottaviani, E., Prampolini, E., Besio, G., Fabiano, B., Federici, B., 2023. Application of machine learning techniques to derive sea water turbidity from Sentinel-2 imagery. Remote Sensing Applications: Society and Environment 30. https://doi.org/10.1016/j.rsase.2023.100951

Ouma, Y. O., Noor, K., Herbert, K., 2020. Modelling Reservoir Chlorophyll-a, TSS, and Turbidity Using Sentinel-2A MSI and Landsat-8 OLI Satellite Sensors with Empirical Multivariate Regression. Journal of Sensors 2020, 1–21. https://doi.org/10.1155/2020/8858408

Quang, N., Sasaki, J., Higa, H., Huan, N., 2017. Spatiotemporal Variation of Turbidity Based on Landsat 8 OLI in Cam Ranh Bay and Thuy Trieu Lagoon, Vietnam. Water 9(8). https://doi.org/10.3390/w9080570

Yin, C. A. O., Yuntao, Y. E., Hongli, Z., Yunzhong, J., Hao, W., 2019. Remote sensing of total suspended matter concentration and turbidity in a macrophytic lake. National Remote Sensing Bulletin 23(6), 1253–1268. https://doi.org/10.11834/jrs.20198144

Zhou, N., Chen, H., Chen, N., Liu, B., 2024. An Approach Based on Video Image Intelligent Recognition for Water Turbidity Monitoring in River. ACS ES&T Water 4(2), 543–554. https://doi.org/10.1021/acsestwater.3c00596