Table S1 Detailed scanning parameters of MRI

| Parameter Type | Parameter Details |
| --- | --- |
| Equipment | Siemens 3.0 T superconducting MRI machine |
| Scanning Position | Closed-mouth: Maximum intercuspal position (FH plane parallel to ground) Wide-open mouth: ≥35 mm (with calibrated bite block) |
| Scanning Plane | Oblique sagittal (parallel to condylar long axis) Oblique coronal (perpendicular to condylar long axis) |
| T1WI Sequence | TR = 680 ms; TE = 9.5 ms; Matrix = 192 × 144; FOV = 120 × 120 mm |
| T2WI Sequence | TR = 2400 ms; TE = 35 ms; Matrix = 320 × 320; FOV = 120 × 120 mm |
| PDWI Sequence | TR = 2500 ms; TE = 30 ms; Matrix = 192 × 192; FOV = 120 × 120 mm |
| Slice Setting | Slice number: 12–18 (dynamic adjustment) Slice thickness: 2 mm Slice gap: 0.2 mm |
| Image Format | DICOM |

Abbreviations: TR = Repetition time; TE = Echo time; FOV = Field of view; T1WI = T1-weighted imaging; T2WI = T2-weighted imaging; PDWI = Proton-density-weighted imaging

Table S2 Detailed scanning parameters of CBCT

| Parameter Type | Parameter Details |
| --- | --- |
| Equipment | NewTom 5G CT system (QR srl, Verona, Italy) |
| Scanning Position | Closed-mouth: Maximum intercuspal position Wide-open mouth: With calibrated bite block |
| Positioning Standard | FH plane parallel to ground; Midsagittal plane perpendicular to horizontal plane (laser-guided) |
| Scanning Parameters | FOV = 180 × 160 mm (unilateral TMJ coverage) Tube voltage: 110 kV Tube current: 5 mA Isotropic voxel size: 0.25 mm |
| Reconstruction Protocol | Multiplanar reorientation (axial view to define condylar long axis) Oblique sagittal (parallel to condylar axis) Oblique coronal (perpendicular to condylar axis) |
| Image Format | DICOM |

Abbreviations:  CBCT = Cone-Beam Computed Tomography; MRI = Magnetic Resonance Imaging; DICOM = Digital Imaging and Communications in Medicine; FH = Frankfurt horizontal

Fig. S1 Flowchart of the study design and model development