Appendix A:

A post-hoc compensation of the measured ADC was done to investigate the impact of perfusion on measured diffusion using the perfusion fraction (f) described for TP and TRA (fTP = 0.084, fTRA = 0.040) in Kim et al.24. The compensated ADC was calculated by subtracting -ln⁡(1-f)b1000 from the measured ADC. This recalculation was based on the following equation described to calculate the ADC:

ADC=-1b×ln⁡Sb1000Sb0

With b being the highest b-value used, Sb1000 being the signal at the highest b-value, which in this case was b1000. Sb0 was the signal measured at b = 0.

Using the recalculated values, the following results were retrieved:

| Metric | Original value | Recalculated value | Difference |
| --- | --- | --- | --- |
| Mean ADC in FTBhigh | 1.320 mm2/s | 1.247 mm2/s |  |
| Mean ADC in FTBlow | 1.444 mm2/s | 1.371 mm2/s |  |
| AUROC logistic model | 0.72 | 0.74 | Non-significant (p = 0.21, via bootstrapping with 2000 iterations) |

As with the original measurements, the mean ADC differed significantly between the FTB-classes (p < 0.01).

The usage of this post-hoc calculation illustrated that the impact of perfusion on our measured diffusion is likely limited, as the primary outcomes of this manuscript did not change. The usage of a b>200 value instead of the b0 used in this manuscript for future research is however still recommended.