# Supplementary material:

# Table 1: Scanner protocols used for the CT-abdomens

| Manufacturer | Siemens Healthineers, Munich, Germany | Siemens Healthineers, Munich, Germany |
| --- | --- | --- |
| Scanner Type | SOMATOM Definition AS | SOMATOM Force |
| Reconstruction kernel | Br 38 | Br 40 |
| Slice thickness (mm) | 3.0 | 3.0 |
| Iterative reconstruction algorithm | Sinogram affirmed iterative reconstruction (SAFIRE), level 3 | Advanced modelled iterative reconstruction (ADMIRE), level 3 |
| Tube current-time product (mAs) | 190 | 150 |
| Tube voltage (kV) | 120 | 120 |
| Dose optimization | Optimized for parenchymal scanning | Optimized for parenchymal scanning |
| Rotation time | 0.5 seconds | 0.5 seconds |
| Contrast medium | Ultravist 300 | Ultravist 300 |
| Maximum flow rate | 4.4 mL/s | 4.4 mL/s |
| Injection duration (s) | 30 | 30 |
| Total contrast dose | 4 mL/kg, maximum 133 mL | 4 mL/kg, maximum 133 mL |

Abbreviations: Br: body kernel, kg: kilograms, kV: kilovolts, mAs: milliampere seconds mL: millilitres, mm: millimetres, s: seconds