Neuro-Fuzzy Enhanced Deep Reinforcement Learning for Adaptive Urban Traffic Signal Control

| Michael Bamidele Soroyewun Computer Science Department, Ahmadu Bello University, Zaria, Kaduna State, Nigeria delemike@gmail.com, mbsoroyewun@abu.edu.ng ORCID ID - 0000-0001-9795-5564 | Afolayan Ayodele Obiniyi Computer Science Department, Federal University Lokoja, Lokoja, Kogi State, Nigeria aaobiniyi@gmail.com |
| --- | --- |
| Sahalu Balarabe Junaidu Computer Science Department, Ahmadu Bello University, Zaria, Kaduna State, Nigeria sahalu@abu.edu.ng | Emmanuel Adewale Adedokun Computer Engineering Department, Ahmadu Bello University, Zaria, Kaduna State, Nigeria wale@abu.edu.ng |

Corresponding author: M. B. Soroyewun

Email: mbsoroyewun@abu.edu.ng

Abstract

Deep Reinforcement Learning (DRL) has become a leading paradigm for adaptive traffic signal control, yet baseline implementations suffer from eight structural limitations spanning action discretization, reward noise, state interpretability, phase rigidity, undirected exploration, Q-value uncertainty, experience replay weighting, and sensor robustness. This paper proposes NF_PN_D3QN, a Neuro-Fuzzy enhanced extension of the Prioritized Noisy Dueling Double Deep Q-Network (PN_D3QN), which addresses all eight gaps through a unified neuro-fuzzy framework incorporating a Fuzzy Feature Extractor, Mamdani Reward Shaper, Green Duration FIS (Fuzzy Inference System), Phase Urgency scorer, Exploration Policy, softmax-entropy Q-Confidence gate, Fuzzy Priority PER (Prioritized Experience Replay), and Fuzzy Sensor Model. A methodological contribution is the documented three-version evolution of the Q-confidence gate: absolute gap thresholding produced 97% FIS dominance and no effective learning; relative gap normalization reduced but did not eliminate FIS persistence at convergence; softmax entropy correctly identified genuine network uncertainty, allowing FIS deferral to decline naturally from 11% at episode 1 to 0% by episode 30. Experiments across five traffic scenarios in Simulation of Urban Mobility (SUMO) show that NF_PN_D3QN achieves a last-25-episode mean waiting time of 6.40 seconds, statistically equivalent to PN_D3QN's converged 6.07 seconds, confirming that all DRL methods share a common performance ceiling well below Max-Pressure's 13.0 seconds. NF_PN_D3QN's primary advantage is sample efficiency: deployable performance is reached in 15 to 20 episodes versus 80 to 90 for PN_D3QN and 150 or more for D3QN, a four to eight times improvement with direct implications for live deployment where poor early decisions affect real commuters.

Keywords: adaptive traffic signal control; deep reinforcement learning; neuro-fuzzy systems; softmax entropy confidence; prioritized experience replay.

Competing Interests: The authors declare that they have no competing interests, financial or otherwise, related to the work submitted for publication.

Acknowledgments

The authors acknowledge the support of Ahmadu Bello University, Zaria, Nigeria, and the open-source SUMO traffic simulation community.