Supplementary Information of

FSBS and PLMF Pulses Enable High-Performance Distributed Acoustic Sensing

Yanbo Xiao1,2,#, Zengling Ran1,2,#,*, Qingqiang Zhu1, Mengyue He1, Junhao Luo1, Yunfei Chen1, Qiaoling Chen1, Haifeng Chen1, Shujie An1,2, Yu Wang2, Wei Zhou2,Yunjiang Rao1,2

1The Key Laboratory of Optical Fiber Sensing and Communication, University of Electronic Science and Technology of China, Chengdu 611731, China

2Optical Science and Technology Ltd., China National Petroleum Corporation, Chengdu 611731, China

Co-first Authors: Yanbo Xiao, Zengling Ran

Corresponding author: ranzl@uestc.edu.cn

This Supplementary Information consists of the following sections:

S1. Theoretical analysis

S2. Demodulation process for the phase

Supplementary note S1: Theoretical analysis

S1.1 Passive chirp suppression on the basis of FSBS

According to Henry’s linewidth-enhancement model, the instantaneous frequency deviation can be expressed as ∆ft=-α4π1PtdPdt , where Pt represents the optical power and α represents the SOA linewidth-enhancement factor1. Consequently, the SOA-modulated pulses exhibit negative chirp on the rising edge and positive chirp on the falling edge.

The pulse output by the SOA can be represented as Eint=Ptexpj[ϕin(t)]=Ptexpj[2πf0t+ϕ0+ϕSOA(t)], , where Pt represents the amplitude envelope, f0 represents the frequency of the laser, ϕ0 represents the initial phase, and ϕSOA(t)=2π-∞t∆fτdτ. The chirp spectrum of an SOA pulse is given by ∆fint=12πdϕin(t)dt=2πf0+∆ft. The term 2πf0  represents the central frequency of the chirp, whose magnitude does not affect the chirp range and is therefore neglected in the subsequent derivation. Because the FSBS process is dominated by guided acoustic modes, the susceptibility can be expressed as a function of frequency as χBnf=11+jf-fanΓn, which provides a more intuitive representation of the process. Consequently, the analysis is conducted in the frequency domain.

The SOA-induced chirp on the rising edge can be derived by linearizing the logarithm of the optical power around the average power P0. We let Pt=P0+∆Pt. Approximating lnPt≈lnP0+∆PtP0 (∆Pt≪P0), the chirp introduced by the SOA can be expressed as follows:

∆Finf=F[∆ft]≈j2πfCα∆Pf                     (S1)

where F represents the Fourier transform function and Cα=-α4πP0<0. The FSBS-induced phase response at position z is expressed as

∂ΦFSBSz,f∂z=K(z)Hfe-az∆Pf                          (S2)

where Hf≃Re[χBf] and K(z) represents the optoacoustic coupling coefficient, which is related to the temperature, stress and coating. The phase induced by FSBS at a propagation length of L is

ΦFSBSf;L=0LK(z) Hfe-azdz∆Pf                       (S3)

The FSBS-induced contribution to the instantaneous optical frequency is given by the time derivative of the phase

∆FFSBSf=j2πfΦFSBS=j2πfK0Hf1-e-aLa∆Pf              (S4)

By considering the broadband transmission properties of the optical fiber and the contribution from FSBS, the output is

∆Foutf≈∆Finf+∆FFSBSf=1+K0CαHf1-e-aLa∆Finf          (S5)

Within the acoustic band around fa, Hf>0, and the factor K0CαHf1-e-aLa<0. As a result, ∆FFSBSf and ∆Finf have opposite signs in the complex field, which leads to coherent cancellation of the edge-induced chirp.

S1.2 Validation of chirp suppression using pulse self-interference

The chirp introduced by SOA modulation can be regarded as a superposition of multiple linear chirps. For simplicity, it is represented as a linear chirp pulse Et=Aejω0t+12Ct2, where A represents the pulse amplitude, ω0 represents the optical carrier frequency, and C represents the chirp coefficient. After passing through an unbalanced interferometer with a delay τ, the detected intensity becomes  It=Et+Et-τ2=2A2+2A2cos⁡(ω0τ+Ctτ-12Cτ2). When the pulse contains chirp (C≠0), the term Ctτ induces a time-varying fringe phase, which results in intensity fluctuations within the interference window. After FSBS-based suppression, C decreases, and It becomes constant, which corresponds to a stable self-interference intensity curve. In our experimental setup, the SOA generates 50 ns optical pulses with a rising edge of 8 ns. The difference in path length between the two fiber arms is 2 m, which corresponds to a time delay of τ≈10 ns.

Supplementary note S2: Demodulation process for the phase

S2.1 Strain resolution, ultralow-frequency response, and sensing range

After coherent signal acquisition, the detailed data processing process is as follows:

P1: The signal of each frequency component is separated by a bandpass filter.

P2: Because there are time delay signals between the different frequency components in one period, this time delay needs to be compensated for in the calculation process. After bandpass filtering and time delay compensation, the signal of the i-th frequency component is as follows:

Imi=xmi                                 (S6)

P3: The Hilbert transform or I/Q transform is performed on the signals of each frequency component to convert each real signal into a complex signal:

Emi=xmi+jymi                          (S7)

where xmi represents the real component and ymi represents the imaginary component.

P4: A vector rotation operation is performed on the complex signals:

Rmik&=xmi+jymi⋅x1i-jy1ix1i2+y1i2&=x'mi+jymi'                      (S7)

P5: Multiple components in a multifrequency pulse period are synthesized:

Am=i=0n-1Rmi                                (S9)

where n represents the number of pulses in the multifrequency pulse period.

P6: Furthermore, according to Am, the phase distribution of the multifrequency pulse period along the fiber is calculated.

S2.2 Spatial resolution

To improve the bandwidth of the detection pulse, 60 pulses are used in one period. According to the experimental scheme in the main text, the coherent process for incident pulse signals and backscattered signals with local oscillator light can be completed in the process of one acquisition. The detailed data processing process is as follows:

P1: The 60 pulses in a multifrequency pulse period are divided into 6 subgroups, as shown in Fig. 5a.

P2: For subgroups of backscattered signals, matched filtering is applied to each subgroup of backscattered light.

P3: Then, the signal obtained by matching filtering in P2 can be calculated as described in S2.1.