Supplementary Materials

For

Cross-Platform Experimental Validation of Multi-Stage Adaptive Gate Driving for MOSFET Switching Loss Reduction in Transformer Boost Circuits

This supplementary material documents the original MS-AGD implementation and its validation on a 5 W, 24–800 V SiC MOSFET flyback prototype, supporting the cross-platform adaptation presented in the main manuscript.

S1. Original Platform Specifications

S1.1 Converter design

The prototype targets portable high-step-up applications. Design parameters: input 24 V (±20%), output 800 V, rated 5 W, switching frequency 100 kHz, DCM operation, step-up ratio 33:1.

S1.2 Power stage

Transformer: EFD15 core, 1:40 turns ratio. Output stage: 2.2 μF/1500 V film capacitor with resistive load. Main switch: Wolfspeed C3M0065090D SiC MOSFET (900 V, 65 mΩ,Qg≈60nC), rated to sustain the 800 V output stress plus leakage-inductance spikes.

S1.3 Instrumentation

Power measurement: Yokogawa WT310E (±0.1%, 100 kHz). Waveform capture: Tektronix MDO3104 (1 GHz, 2.5 GS/s) with P6139A voltage probe and TCP0030A current probe. Thermal measurement: PT1000 (±0.05°C) bonded to the MOSFET package. Ambient control: programmable chamber at 16°C with ±1.1 m°C stability over 240 min.

Fig. S1. Flyback prototype PCB with integrated MS-AGD module (52 mm × 26 mm)

S2. Fixed Gate Resistor Trade-off

S2.1 Governing relation

During the Miller plateau, the drain–source slew rate is set by the gate loop impedance:

dVDSdt=-Vdrive-VmpRg×CgdVDS#S1

The strong nonlinearity of CgdVDS causes dVDSdt to accelerate 3–5× as VDS​ decreases, even under constant Ig​.

S2.2 Quantitative trade-off

For the original platform (VDD=24V,Cgd0=80pF):

Rg=2Ω,tmp≈9ns, peak dVDSdt≈2.67Vns—loss minimized, EMI exceeds regulatory limits.

Rg=10Ω,tmp≈45ns; Stage-III energy Estage3≈0.54μJ, accounting for approximately 87% of total turn-on loss.

Rg=20Ω,tmp≈90ns; EMI compliant but switching loss rises 3–4× relative to the 2 Ω case.

S2.3 Input voltage sensitivity

Since tmp∝VDD​, a 12–48 V input range induces a 4× variation in tmp​. Any fixed Rg​ optimized at one operating point therefore deviates from optimum at voltage extremes, precluding single-parameter optimization.

S3. MS-AGD Architecture

S3.1 Topology

Fig. S2. Complete MS-AGD schematic. Upper dashed region: three cascode current-mirror groups (mirror_1–mirror_3), each comprising four transistors and an enable switch (S1–S3), sharing low-voltage supply vcc_L. Lower-left: differentiating network (C2 with resistor divider). Lower-right: comparison stage. Central region: flyback power stage (T1, Q0, D1, C3), supplied by vcc_P.

The driver integrates three functional blocks: programmable current mirrors for stage-wise gate drive, a passive differentiator for dVDSdt extraction, and a dual-threshold comparator pair for plateau boundary identification.

S3.2 Four-stage piecewise-linear drive profile

The MS-AGD replaces the conventional rectangular PWM profile with four independently configured current segments:

Stage 1 (turn-on delay): low Ig​ suppresses gate-trace EMI during sub-threshold charging.

Stage 2 (current rise): elevated Ig​ minimizes VDS-ID​ overlap loss.

Stage 3 (Miller plateau): moderate Ig​ decouples plateau dVDSdt (EMI determinant) from total switching energy.

Stage 4 (enhanced conduction): high Ig​ drives VGS​ to steady state, minimizing RDS(on)​ settling time.

This stage-wise current allocation decouples loss optimization (Ig​ amplitude) from EMI control (dVDSdt).

S3.3 Cascode current mirror

A simple current mirror exhibits an output impedance of approximately 50 kΩ, insufficient for the 0–15 V VGS​ swing. The cascode topology raises output impedance above 5 MΩ, limiting current deviation to below 5% across the full swing.

S4. Detection Circuit Design

S4.1 Differentiator sizing

The time constant τdiff=Rdiff×Cdiff satisfies τdiff≪tmp​ (preserving differentiation mode) and τdiff≫tnoise (suppressing noise amplification). The original platform uses τdiff=5ns,(Rdiff=1KΩ,Cdiff=5pF). The peak differentiator output is:

Vpeak=Rdiff×Cdiff×VDDtmp#S2

Since tmp∝VDD​, the ratio VDDtmp​ is approximately invariant. Measured Vpeak=2.6±0.3V across 12–48 V input, confirming calibration-free operation.

S4.2 Dual-threshold discrimination

Two TLV3601 comparators (2.5 ns propagation delay) implement threshold discrimination:

Miller_Start: Vthhigh=0.9×Vpeak​ (approximately 2.4 V)—delays triggering until dVDSdt is fully established, suppressing false triggers from Stage 1–2 noise.

Miller_End: Vthlow=0.1×Vpeak​ (approximately 0.27 V)—extends detection into the dVDSdt decay tail, ensuring plateau-stage drive persists until VDS​ settles.

S4.3 Timing budget

Combined differentiator response, comparator propagation, and switch-gate delay yield a total detection delay below 50 ns, substantially shorter than the minimum observed tmp​ across all operating conditions.

S5. Discrete Implementation

S5.1 BOM and footprint

The MS-AGD uses only general-purpose discrete components, with no ICs, MCUs, or programmable logic. Single-unit BOM cost is $0.48, versus approximately $15.70 for a comparable digital closed-loop driver. The cascode current mirror module measures 13 mm × 14 mm (182 mm²), verified by caliper measurement.

Fig. S3. MS-AGD current-mirror PCB with dimensional reference.

S5.2 PWM interface

The MS-AGD accepts a single enable signal from a standard PWM controller, requiring no firmware modification. Internal comparators autonomously handle stage transitions based on real-time dVDSdt sensing.

S5.3 Static consumption

Total static power consumption is approximately 8 mW, compared with approximately 185 mW for the digital closed-loop baseline—a primary contributor to the light-load efficiency advantage documented in Section S6.

S6. Original Platform Validation

S6.1 Four-stage VGS​ waveform

Fig. S4. Measured VGS​ at Vin=24V,Pout=5W,fsw=100kHz

Stage 1 (0–155 ns): 0 → 3.04 V at 20 mV/ns, yielding Ig≈48mA (design 50 mA, <5% deviation).

Stage 2 (155–278 ns): 3.04 → 4.39 V at 36 mV/ns.

Stage 3 (278–790 ns): plateau at 4.33–4.39 V for 512 ns.

Stage 4 (790–1300 ns): 4.39 → 11.43 V.

S6.2 Thermal performance

Fig. S5. MOSFET temperature rise ΔT vs. fsw​ (20–125 kHz)

At 125 kHz, the MS-AGD reduces ΔT by 0.79°C relative to the Rg=10Ω baseline (1.13°C vs. 1.92°C). Averaged across the frequency range, the MS-AGD yields a 30% reduction in switching loss, with ΔT held within 1.05–1.20°C against a 1.47–1.92°C baseline range.

S6.3 Conversion efficiency

Fig. S6. Efficiency vs. Pout (0.5–5 W), fixed Rg​ vs. MS-AGD

Full load (5 W): 91.9% → 94.5% (+2.6 pp).

Light load (0.5 W): 85.6% → 88.5% (+2.9 pp; reported as 3.3–3.4 pp in the main manuscript due to rounding across repeated measurements).

The elevated light-load gain reflects the MS-AGD's suppression of gate-charge-dependent losses that fixed Rg​ driving cannot mitigate.

S6.4 Input voltage robustness

Fig. S7. Efficiency and detected Vmp​ vs. input voltage (10–50 V)

Across the 5× input range, conversion efficiency remains within 92–94.5% (peak near Vin=25V), and the detected Miller plateau voltage fluctuates within 4.21–4.24 V (peak-to-peak 30 mV), confirming the calibration-free detection predicted in Section S4.

S6.5 Device compatibility

Fig. S8. Turn-on waveforms on three MOSFETs (Vds=100V)

Device A (C3M0065090D SiC): Qg=38nC,tmp≈200ns.

Device B (Si MOSFET): Qg=65nC,tmp≈488ns.

Device C (200 W-class MOSFET): Qg=140nC,tmp≈900ns.

The plateau voltage held at Vmp≈4.18V across all three devices, and the four-stage profile exhibited no oscillation or overshoot. The MS-AGD resolved plateau boundaries across a 4.5× variation in tmp​ without recalibration.

S6.6 Applicability boundaries

Detection floor: inherent circuit delay 28–30 ns. For Qgd<10nC devices (e.g., 30 V GaN HEMTs), tmp​ may fall below 15 ns, exceeding the circuit's time resolution.

Power range: 1–50 W direct application. Above 50 W, paralleled current mirrors are required; below 1 W, the 8 mW static consumption erodes the efficiency advantage.

Operating-point specificity: current setpoints optimized for the nominal operating point; performance degrades at range extremes.