Supplementary material of ‘Controlling the dynamics of electric field driven droplet on a lubricant-infused surface’

Geng Wang1,2,3, Junyu Yang4, Timan Lei5, Jin Chen2, Halim Kusumaatmaja4*, Kai Li6,1,3*, Kai H. Luo3*

- National Microgravity Laboratory, Institute of Mechanics, Chinese Academy of Sciences, Beijing 100190, China

- Department of Mechanical Engineering, University College London, Torrington Place, London WC1E 7JE, UK

- School of Engineering Science, University of Chinese Academy of Sciences, Beijing 100049, China

- Institute for Multiscale Thermofluids, School of Engineering, The University of Edinburgh, Edinburgh, UK

- Chair of Building Physics, Department of Mechanical and Process Engineering, ETH Zürich (Swiss Federal Institute of Technology in Zürich), Zürich 8092, Switzerland

- School of Fundamental Physics and Mathematical Sciences, Hangzhou Institute for Advanced Study, UCAS, Hangzhou, 310024, China

* Corresponding Authors:

halim.kusumaatmaja@ed.ac.uk (H. K.)

likai@imech.ac.cn (K. L.)

k.luo@ucl.ac.uk (K. H. L.)

Mesh sensitivity assessment

A mesh resolution sensitivity analysis is conducted in this section. The validation is performed from two aspects, which are ternary multiphase flow and ternary flow coupled with an electric field. First, similar to the case in Fig. 3(b), different mesh resolutions are used to simulate the droplet deceleration during sliding on a lubricant-infused pillar. For all cases, the droplet radius is R0=0.5 mm and the initial velocity is V0=0.4 m/s. Three mesh resolutions dx=R0/35, dx=R0/70 and dx=R0/105 are examined. The quantitative comparison of dimensionless droplet displacement S* is shown in Fig. S1 (right axis). It can be seen that when the mesh resolution is increased from R0/35 to R0/70 or R0/105, the maximum error in S* remains within 5%. In addition, Fig. S1(b) presents a qualitative comparison of the droplet morphology at t/tc=24 for the three mesh resolutions. The droplet shapes are highly consistent, with only minor differences observed in the thickness of the lubricant cloak layer.

Figure S1. For cases with different mesh resolutions, (a) shows a quantitative comparison of S* for sliding deceleration (right axis) and EF driven (left axis) droplet on LIS. (b) is a qualitative comparison of the sliding droplet morphology at t/tc=24.

Then, similar to Fig. 4, the above simulations are extended to EF driven droplet motion on LIS. The results of droplet displacement S* obtained using resolutions of R0/35 and R0/70 are compared. In this simulation, we set S0=9.3R0 and E0=0.6Ec. It is observed that at the end of the simulation (t/tc=24), the maximum error in S* between the two resolutions is also approximately 5%. Considering the computational complexity of the present model and the large number of three-dimensional simulations performed, a mesh resolution of R0/35 is adopted. This resolution ensures that the key features are accurately captured while maintaining a reasonable computational cost.

Correlation analysis of Cα

In this section, we perform a correlation analysis of the constant parameter Cα appearing in Eq. (1) of the main text. In particular, we examine the effects of the simulation configuration (3D or Q3D), the initial lubricant thickness, and Ios(w)/Is. Figure S2 plots the relationship between Ios(w)/Is and Cα for different hoil with both 3D and Q3D simulations, where:

Cα=Cdrag,sim1+1.5IoswIs2μμ0.#S1

Figure S2. Cα as a function of Ios(w)/Is for different hoil for 3D and Q3D simulations. The lines represent the best fitted values for Cα that we employ for further calculations in the main text.

As shown in Fig. S2, the values of Cα mainly related to the simulation configuration (3D or Q3D) and hoil when Ios(w)/Is increases from 0 to 0.8, while it is nearly independent of Ios(w)/Is. The lines represent the best-fitted values of Cα for different hoil in the 3D and Q3D cases.

Validation of drag force

Figures S3(a) and S3(b) show the evolution of the velocity V and the normalised displacement S* for droplets with R0=0.5mm decelerating on LIS at different initial velocities. The symbols represent the LBM simulation results, while the solid lines denote the analytical solutions obtained by solving the droplet force balance equation, in which Fdrag is given by equation (1) in the main text.

Fdrag=-mdVdt=-md2Sdt2.#S2

Figure S3(c) presents the comparison between the LBM results and the analytical solutions for different R0. It can be seen that equation (1) in the main text accurately describes Fdrag for different initial velocities and droplet radius, and the LBM results are in excellent agreement with the analytical solutions.

Figure S3. Transient evolution of (a) the velocity V and (b) the normalised displacement S*during droplets sliding deceleration on LIS for with different initial velocities. (c) is the transient evolution of S* for droplets with different initial radius. Symbols in the figures represent the LBM simulation results and the lines denote the analytical solutions.

Figure S4(a) shows the evolution of S* for droplets with different θws and θos, with R0=0.5mm, μo=1cP, and V0=0.4 m/s. It can be observed that, depending on the value of Ios(w)/Is, the droplet motion can be classified into three different regimes. For cases with Ios(w)/Is>0.8, the droplet is almost stationary due to pinning on the micropillar surface. For cases with Ios(w)/Is<0, the evolution of S* for cases with different θws is nearly identical and follows the analytical solution with Ios(w)/Is=0 (black solid line in Fig. S4(a)). When 0<Ios(w)/Is<0.8, the droplet evolution can be described by solving the analytical solution in Eq. (S2).

Figure S4(b) shows the evolution of S* for droplets with different μo. It can be seen that the LBM results are in good agreement with the analytical solutions. However, for cases with larger μo, the LBM results show a slight decrease in S* after reaching the maximum value. This is because the high-viscosity lubricant accumulates at the meniscus at the front of the droplet during motion. After the initial kinetic energy of the droplet is dissipated, the accumulated lubricant pushes the droplet backwards, leading to a small recoil of S*.

Figure 4. Transient evolution of S*during droplet sliding deceleration on LIS with (a) different θws and θos and (b) lubricant viscosity μo. Symbols represent the LBM simulation results, and the lines denote the analytical solutions.

Derivation of the electric-field force

In this section, the derivation of the analytical solution for the electric field force is presented and validated through comparison with LBM results. For the electric field far from the droplet, the electric potential φ(x,y) satisfies the Laplace equation:

∂2φ∂x2+∂2φ∂y2=0.#S3

In our simulation, the bottom boundary y0 is set as a grounded boundary. The top boundary yLy and left boundary x0 are insulating boundaries. The right boundary xLx can be recognized as a mixed boundary condition, with a driven electrode segment y∈(yc,Ly held at φ0 and an insulating segment y∈[0,yc with ∂φ/∂x=0. The above boundary conditions can be summarized as:

φx,0=0,  ∂φ∂yx,Ly=0,  ∂φ∂x(0,y)=0,

φLx,y=φ0 y>yc,  ∂φ∂xLx,y=0y<yc.#S4

To satisfy φx,0=0 and ∂φ/∂y(x,Ly)=0, φ is expanded in the orthogonal basis sin⁡(αny), with transverse wavenumbers:

αn=2n+1π2Ly,n=0,1,2,⋯#S5

By adopting separation of variables solution for Eq. (S3), a solution satisfies ∂φ/∂x(0,y)=0 can be written as:

φx,y=n=0∞Ancoshαnxsinαny.#S6

Imposing the mixed boundary condition at x=Lx using a weighted-residual projection onto sin⁡(αny) yields a scalar relation for each mode. Defining the segment integrals:

JD,n=ycLysin2αnydy,    JN,n=ycLysin2αnydy,#S7
the modal amplitude can be expressed compactly as:

An=φ0cosαnyc αncoshαnLxJD,nαnsinhαnLxJN,n.#S8

Figure S5. (a) Qualitative illustration of the simulation domain and the spatial electric field strength distribution for the validation case. (b) Quantitative comparison of the electric potential obtained from LBM simulations (symbols) and the analytical solution (lines) at different heights

In the semi-infinite-strip approximation (i.e.,Lx→ ∞), the hyperbolic factor reduces to an exponential decay (e-αn(Lx-x)) away from the driven boundary.  Thus Eq. (S6) has:

φx,y=n=0∞Bne-αnLx-xsinαny,#S9

This form automatically satisfies Laplace’s equation and (Lx-x)→ ∞, where Bn can be achieved by subscribing Lx→ ∞ into Eq. (S8):

Bn=φ0cosαnycαnJDαn+αnJNαn,#S10

considering sin2(αy)=(1-cos⁡(2αy))/2 we have:

sin⁡2(αy) dy=y2-sin⁡(2αy)4α,#S11

according to Eq. (S5), we have 2αnLy=(2n+1)π, hence:

JD,n=Ly-yc2+sin⁡(2αnyc)4αn,JN,n=yc2-sin⁡(2αnyc)4αn,#S12

For simplicity, we assume that the mode n=0 is dominant. Therefore, Eq. (S9) can be simplified as:

φx,y=B0e-α0Lx-xsinα0y.#S13

To validate the above equation, the LBM results are compared with the analytical solution in Eq. (S13). In this simulation, we set Lx=500, Ly=150, yc=lc=18, and φ0=19.99 (all in lattice units). The simulation setup is also shown in Fig. S5(a). Figure S5(b) shows the comparison between the electric potential obtained from the LBM simulations and the analytical solution (Eq. (S13)) at different height y. It can be seen that the analytical results are in very good agreement with the LBM results, with only slight deviations observed in the region near the electrode (i.e., x=Lx and y=2Ly/3).

We then derive the expression for the electric force in the point dipole dielectrophoretic (DEP) model. The electric field is defined as E=-∇φ, according to Eq. (S13), we have:

Ex=-∂ϕ∂x=α0B0e-α0dsinα0y,Ey=-∂ϕ∂y=-α0B0e-α0dcosα0y.#S14

Defining the squared electric field magnitude along the direction of motion as:

Ex2≡1Ly0LyEx2+Ey2 dy=α02B02 e-2α0Lx-x,#S15

and

dEx2dx=2α03B02e-2α0(Lx-x),#S16

substituting Se=(Lx-x), yc=le, and Ly=(he+le) into Fele (Eq. (2) in main text), we obtain the same form as the analytical expression for the electric force given by Eqs. (4) and (5) in the main text.

Fele =KeCelee-2α0Se,#S17

and

Cele=2α0φ02cos2α0le JDα0+α0 JNα02,α0=π2he+le,JDα0=he2+sin2α0le4α0,JNα0=le2-sin2α0le4α0.#S17