Supporting Information

Effect of copper on bioconcentration of benzotriazole ultraviolet stabilizers (BUVSs) in common carp (Cyprinus carpio)

Dandan Zhaoa, Tadiyose Girma Bekelea, Hongxia Zhaoa, *

a Key Laboratory of Industrial Ecology and Environmental Engineering (Ministry of Education), School of Environmental Science and Technology, Dalian University of Technology, Linggong Road 2, Dalian, 116024, China

Corresponding Author

*Prof. Hongxia Zhao, phone: +86-411-8470 7965, E-mail: hxzhao@dlut.edu.cn

Contents

Text S1: GC-MS conditions

Text S2: Water sample treatment

Table S1: Abbreviations, names, molecular formulas, and properties of the six BUVSs standards involved in the present study

Table S2: Chemical Structure of the six BUVSs standards involved in the present study

Table S3: Qualitative, quantitative ions and retention time for 6 BUVSs

Table S4: Method detection limits (MDL), method quantification limits (MQLs), spiked average recoveries and relative standard deviations (RSD, n = 6) for each BUVSs

Table S5: Uptake and depuration phase duration, mortality, and hepatosomatic index (HSI) for exposed and control common carp

Table S6: No significant difference in the HSIs was found between the treatment groups and the control group (Paired Student’s t-test; P > 0.05)

Table S7: The concentration of BUVSs in water samples (n=2)

Figure S1: Uptake and depuration curves of BUVSs in tissues of common carp under different Cu concentrations.

Text S1: GC-MS conditions

Analytes were determined by GCMS-QP2010 SE. Separations were carried out in a Agilent DB-5MS type column (30 m × 0.25× 0.25 µm), operated at a constant helium flow of 1.0 ml·min−1. The GC oven was programmed as follows: 180 °C (held for 1 min), rate 10 °C min−1 to 290 °C (held for 10 min). The temperature of the injector was maintained at 290 °C, and samples were injected in the splitless mode. The mass spectrometer was operated in the electron impact (EI) ionization mode (70 eV). The acquisition was performed in the selected ion monitoring mode (SIM) to obtain the highest possible selectivity. Qualitative, quantitative ions and retention time for each analyte were listed in Table S3.

Text S2: Water sample treatment

The BUVSs in the water were extracted by hydrophilic-lipophilic balance solid-phase extraction (C18 SPE) column using an improved procedure described by Kameda et al. (2011). In detail, the C18 column were pre-washed using 10 mL dichloromethane followed by 10 mL methanol and 10 mL ultra-pure water. The surrogate spiked (Allyl-bzt) water sample was transferred to the cartridge with a flow-rate of approximately 1.5 mL/min. Following that, the cartridge was dried and 10 mL dichloromethane was used to elute the target BUVSs. The elutes were evaporated to near dryness under a gentle nitrogen stream and re-dissolved in 1 mL n-hexane before GC-MS analysis.

| Chemical Name | Abbreviations | CAS Number | Molecular Formula | logKowa |  |
| --- | --- | --- | --- | --- | --- |
| 2-(2-Hydroxy-5-tert-butylphenyl)-2H-benzotriazole | UV-PS | 3147-76-0 | C13H11N3O |  | 4.36 |
| 2-(2'-Hydroxy-3',5'-di-tert-butylphenyl)- benzotriazole | UV-320 | 3846-71-7 | C20H25N3O |  | 7.21 |
| 2-T-butyl-6-(5-chloro-2H-benzotriazol-2-yl)-4-methylphenol | UV-326 | 3896-11-5 | C17H18ClN3O |  | 5.55 |
| 2-(5-chloro-2h-benzotriazol-2-yl)-4,6-bis(1,1-dimethylethyl)-phenol | UV-327 | 3864-99-1 | C20H24ClN3O |  | 6.91 |
| 2-(3,5-Di-tert-amyl-2-hydroxyphenyl)- benzotriazole | UV-328 | 25973-55-1 | C22H29N3O |  | 7.25 |
| 2-(2h-benzotriazol-2-yl)-4-(1,1,3,3-tetramethylbutyl)-phenol | UV-329 | 3147-75-9 | C20H25N3O |  | 6.21 |
| 2-(2H-benzotriazol-2-yl)-4-methyl-6-(2-propenyl)phenol | Allyl-bzt | 2170-39-0 | C16H15N3O |  | 5.3 |

Table S1 Abbreviations, names, molecular formulas, and properties of the six BUVSs standards involved in present study

a Values were obtained from EPI Suite．

Table S2 Chemical Structure of the six BUVSs standards involved in present study

| Abbreviation | Chemical Structure | Abbreviation | Chemical Structure |
| --- | --- | --- | --- |
| UV-PS |  | UV-327 |  |
| UV-320 |  | UV-328 |  |
| UV-326 |  | UV-329 |  |
| Allyl-bzt |  |  |  |

Table S3 Qualitative, quantitative ions and retention time for 6 BUVSs

| Compound | Retention time (min) | Qualitative ion (m/z) | Quantitative ion (m/z) |
| --- | --- | --- | --- |
| UV-PS | 8.645 | 252.00 | 267.00 |
| UV-320 | 10.250 | 308.00 | 323.00 |
| UV-326 | 10.850 | 300.00 | 315.00 |
| UV-327 | 11.665 | 342.00 | 357.00 |
| UV-328 | 11.605 | 322.00 | 323.00 |
| UV-329 | 11.055 | 252.00 | 253.00 |
| Allyl-bzt | 9.200 | 265.00 | 250.00 |

Table S4 Method detection limits (MDL), method quantification limits (MQLs), spiked average recoveries and relative standard deviations (RSD, n = 6) for BUVSs

| Compound | MDLsa (ng/g ww) | MQLsa (ng/g ww) | Spiked levels (ng/g) | Recoveryb (%) | RSD (%) |
| --- | --- | --- | --- | --- | --- |
| UV-PS | 0.05 | 0.15 | 500 | 104.67 | 13.30 |
|  |  |  | 50 | 97.18 | 23.48 |
|  |  |  | 5 | 86.42 | 14.74 |
| UV-320 | 0.35 | 1.16 | 500 | 92.70 | 6.87 |
|  |  |  | 50 | 91.19 | 13.14 |
|  |  |  | 5 | 85.59 | 16.38 |
| UV-327 | 0.34 | 1.12 | 500 | 99.92 | 7.40 |
|  |  |  | 50 | 106.59 | 15.39 |
|  |  |  | 5 | 105.43 | 16.59 |
| UV-326 | 0.81 | 2.7 | 500 | 109.04 | 12.26 |
|  |  |  | 50 | 103.77 | 13.96 |
|  |  |  | 5 | 96.61 | 18.66 |
| UV-329 | 0.46 | 1.52 | 500 | 111.18 | 19.99 |
|  |  |  | 50 | 100.38 | 22.79 |
|  |  |  | 5 | 98.02 | 15.11 |
| UV-328 | 0.05 | 0.16 | 500 | 97.68 | 8.94 |
|  |  |  | 50 | 97.60 | 13.91 |
|  |  |  | 5 | 89.70 | 15.79 |

aThe method detection limit (MDL) was calculated as three times the standard deviation (SD) of the mean procedural blanks (n= 6), while method quantification limit (MQLs) was set as ten times the SD of mean procedural blanks.

bThe recovery of each BUVSs was determined by analysis of six replicates of aquarium raised fish (i.e., non-contaminated biotic matrix) spiked with a mixture of BUVSs standards.

Table S5 Uptake and depuration phase duration, mortality, and hepatosomatic index (HSI) for exposed and control common carp

| Group | Number of fish |  | Number of dead fish | Uptake period (d) | Depuration period (d) | Mean initial fish mass (g) | Mortality (%) | Average liver weight | HSI (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Control | 60 |  | 1 | 28 | 20 | 3.37 ± 0.61 | <2 | 0.060 ± 0.006 | 1.78%±0.05% |
| Cu-blank |  | 60 | 3 | 28 | 20 | 3.14 ± 0.45 | 5 | 0.041 ± 0.007 | 1.31%±0.16% |
| Cu-low | 60 |  | 2 | 28 | 20 | 3.43 ± 0.77 | <4 | 0.053 ± 0.009 | 1.58%±0.20% |
| Cu-high | 60 |  | 2 | 28 | 20 | 3.22 ± 0.87 | <4 | 0.037 ± 0.01 | 1.10%±0.27% |

Table S6 No significant difference in the HSIs was found between the treatment groups and the control group (Paired Student’s t-test; P > 0.05)

| Group | N | Paired correlation | significance |
| --- | --- | --- | --- |
| Control & Cu-blank | 14 | 0.214 | 0.057 |
| Control & Cu-low | 14 | -0.250 | 0.587 |
| Control & Cu- high | 14 | 0.447 | 0.603 |
| Cu-blank & Cu-low | 14 | -0.070 | 0.310 |
| Cu-blank & Cu- high | 14 | 0.451 | 0.175 |
| Cu-low & Cu-high | 14 | 0.053 | 0.339 |

Table S7 The concentration of BUVSs in water samples (n=2)

| Compound | Group | 7 day (μg/L) | 28 day (μg/L) |
| --- | --- | --- | --- |
| UV-PS | Cu-blank | 12.339±1.21 | 10.021±0.44 |
|  | Cu-low | 13.187±2.12 | 12.150±1.39 |
|  | Cu-high | 15.289±1.22 | 13.722±1.27 |
| UV-320 | Cu-blank | 10.274±1.29 | 15.213±2.33 |
|  | Cu-low | 12.963±3.28 | 12.764±0.98 |
|  | Cu-high | 14.134±2.12 | 13.713±0.03 |
| UV-327 | Cu-blank | 15.543±1.10 | 12.829±1.22 |
|  | Cu-low | 14.739±0.67 | 12.315±0.72 |
|  | Cu-high | 12.271±2.09 | 14.503±0.57 |
| UV-326 | Cu-blank | 16.287±0.89 | 11.086±1.28 |
|  | Cu-low | 13.322±2.11 | 10.991±0.97 |
|  | Cu-high | 15.692±1.73 | 11.124±1.01 |
| UV-329 | Cu-blank | 11.562±2.73 | 11.466±0.77 |
|  | Cu-low | 13.783±3.23 | 11.223±0.56 |
|  | Cu-high | 11.562±2.72 | 12.178±1.29 |
| UV-328 | Cu-blank | 12.976±2.13 | 15.642±0.76 |
|  | Cu-low | 14.231±1.78 | 12.176±1.75 |
|  | Cu-high | 13.256±3.12 | 14.219±0.22 |

Fig S1 Uptake and depuration curves of BUVSs in tissues of common carp under different Cu concentrations.

References

Kameda Y, Kimura K, Miyazaki M. 2011. Occurrence and profiles of organic sun-blocking agents in surface waters and sediments in japanese rivers and lakes. Environmental Pollution 159:1570-1576. DOI:10.1016/j.envpol.2011.02.055.