Supplementary Information for

Ultrasound phase microscopy for rapid label-free super-resolution vascular imaging

Supplementary Discussion 1: Theoretical resolution limit analysis of ultrasound phase microscopy

In this part, the theoretical resolution limit of UPM will be derived.

The far field approximated response of the apodization from a non-zero constant dc offset γ can be written as

γAkx=γNsinNd2kxsind2kx #1

Where N is the number of elements in the ultrasound array, d is the element spacing or the pitch of the ultrasound array, and kx=2πλsinθ is the lateral wavenumber. As the non-zero constant apodization is an even function, it is far field approximated response is pure real.

The far field approximated response of Apodzm can be written as

-jBkx=-j2Nsin2Nd4kxsind2kx #2

As Apodzm is an odd function, its far field approximated response is pure imaginary.

Then the far field approximated response of Apoddc1 and Apoddc2 can be written as

Sdc1=γA-jB #3

Sdc2=γA+jB #4

To ease the analysis, the responses are converted to polar coordinate, and the angle component can be expressed as

Φdc1kx=tan-1-BγA=tan-1-2γsin2Nd4kxsinNd2kx#5

As sinNd2kx=2sinNd4kxcosNd4kx,

We can rewrite it as

Φdc1kx=tan-1-BγA=tan-1-1γtankxNd4#6

Then we have

Φdc1kx≈-kxγNd4 #7

Similarly

Φdc2kx≈kxγNd4 #8

To ease the analysis, we define Φkx=kxγNd4. Then we have the polar coordinate expression of three apodizations’ response as

Sdc1≈Me-jΦkx #9

Sdc2≈Me+jΦkx #10

Szm=Bejπ2 #11

Where M is the magnitude.

If we assume the scatter moves in axial direction has an initial phase of φ and a relative phase offset ∆φ=4πfcvc*PRF is generated between two consecutive frames. Then the beamformed signals can be expressed as

Sdc1n=Me-jΦkxejφ #12

Sdc2n=Me+jΦkxejφ #13

Szmn=Bejπ2ejφ #14

Sdc1n+1=Me-jΦkxej(φ+∆φ) #15

Sdc2n+1=Me+jΦkxej(φ+∆φ) #16

Szmn+1=Bejπ2ej(φ+∆φ) #17

Then we have

P1=argSdc1n-Sdc2n+1*=arg-M2e-j2Φkxe-j∆φ#18

P1=wrapπ-2Φkx-∆φ #19

Similarly

P2=wrapπ+2Φkx-∆φ #20

We also have

P3=argSdc1nSzmn+1*=argMBe-jΦkxe-jπ2e-j∆φ#21

P3=wrap(-Φkx-π2-∆φ)#22

Similarly, we have

P4=wrapΦkx+π2-∆φ #23

Then we have PreA=P1+P2 and PreB=P3+P4 expressed as

PreA=wrapπ-2Φkx-∆φ+wrapπ+2Φkx-∆φ #24

PreB=-2∆φ #25

Because of the existence of wrap function, there are two cases for PreA. When we have 2Φkx≤∆φ, PreA=2π-2∆φ, and PUPM=2π. When we have 2Φkx>∆φ, PreA=-2∆φ, and PUPM=0. As we have Φkx≈kxγNd4, kx=2πλsinθ, and L=Nd, PUPM equals to 2π when

sinθ<γ∆φπλL #26

And equals to 0 otherwise.

From this derivation, UPM algorithm functions as a spatial gate that outputs 2π within a very narrow angular limit. And the width of the spatial gate is linear to the relative phase offset which means it is also linear to the scatter velocity.

The width of the spatial gate can be modified in post processing by γ. However, the spatial resolution cannot improve unlimitedly by reducing γ. From the derivation below, we can find out that the signal to noise ratio (SNR) determines the ultimate resolution that UPM algorithm could achieve.

With γ, we can estimate the effective SNR near the center as

SNReff≈γ2SNR0 #27

where SNR0 is the original SNR.

This is because when we are looking at the signals near the center, kx≈0, B(0)≈0. Then the effective signal power is λ2A2. Because Apodzm is orthogonal to the non-zero constant dc offset apodization. The effective noise power is σzm2+γ2σones2. As γ is always much smaller than 1. The effective noise power can be approximated as σzm2.

Then the standard deviation of the phase noise σΦ can be estimated as1

σΦ=12SNReff=1γ2SNR0 #28

If we define the minimum resolvable angle of UPM algorithm as δθUPM, then we can derive δθUPM according to the phase noise and the spatial phase gradient which can be expressed as

δθUPMδΦδθ=σΦ #29

δΦδθ≈πL2γλ #30

And we have

δθUPM≈2πλL1SNR0 #31

where γ is canceled out.

From this derivation, the resolution of UPM algorithm is no longer diffraction limited (λL). Instead, the resolution of UPM is limited by SNR. We can also derive the optimal γopt to use in UPM given SNR.

We assume when θ=0, the scatter is right in front of the array, Φ0=N(0,σΦ). If we want to keep signals within two standard deviations, we have

2σΦ≤∆φ #32

2γSNR0≤∆φ #33

Then we have

γ≥2∆φSNR0 #34

To achieve the optimal spatial resolution, smaller w is preferable. Then we have

γopt=2∆φSNR0 #35

From this expression, we can observe that the γopt is determined by two variables, ∆φ and SNR. When we have higher SNR or faster flow, we can use smaller γ to achieve a narrower spatial gate.

To summarize, the theory behind UPM is derived. Different from conventional beamforming whose resolution limit is determined by the wavelength or diffraction, UPM’s spatial resolution limit is also determined by the SNR. The higher the SNR, the higher the improvement UPM could provide. Besides, the optimal dc offset γopt is also derived. Though we have the expression of γopt, we still use a constant empirical γ because the flow velocity is not a prior in the beamforming process.

Supplementary Discussion 2: Computational Cost Analysis of Ultrasound Phase Microscopy

In this section, we analyze the computational cost of the UPM algorithm and demonstrate its efficiency compared to conventional DAS used in CFI.

Although the UPM process theoretically requires three distinct apodization weights to generate the desired phase responses, we can factorize these apodizations to optimize computational efficiency. We define two orthogonal apodizations, Apodleft and Apodright. For Apodleft, the weights on the left half of the sub aperture are set to 1, while the right half is 0. Conversely, Apodright, has 0 on the left and 1 on the right.

The three apodizations we need in UPM (Apodzm, Apoddc1, and Apoddc2) can be synthesized from Apodleft and Apodlight as follows:

Apodzm=Apodleft-Apodright #36

Apoddc=Apodleft+Apodright #37

Apoddc1=γApoddc+Apodzm #38

Apoddc2=γApoddc-Apodzm #39

where Apoddc corresponds to the uniform rectangular apodization conventionally used in standard DAS beamforming.

Because the beamforming process is fundamentally a linear spatial filtering process, we do not need to apply three apodizations to the raw channel data independently. Instead, we can first compute the beamformed outputs using only ApodLeft and Apodright and use Eq. 44-47 to achieve the desired beamformed data. As a result, UPM’s beamforming cost can be reduced from 3*D*N*F to D*N*F.

Assuming the ultrasound array has N elements, each acquisition has D samples per element, and the sub aperture size is F. The total floating-point operations (FLOPs) required for conventional DAS beamforming per acquisition is proportional to

OperationDAS=D*N*Apoddc-1=D*N*(F-1) #40

For UPM

OperationUPM=D*N*ApodLeft+ApodRight-2+D*N*5= D*N*(F+3)#49

Then it is obvious that in the beamforming stage, UPM only needs  D*N*4 extra operations (Eq. 44-47) compared to DAS. Given that the sub aperture size F is significantly larger than 4, the extra computational overhead of UPM is negligible. After the beamforming, UPM requires four relative phase offset calculations and a final phase summation on each pixel to finalize the processing. As these operations are performed on the beamformed pixels, the computational costs of these operations are all proportional to D*N instead of D*N*F. Therefore, UPM can achieve super-resolution imaging with a computational cost at the same order as that of standard DAS, providing real-time imaging potential.

Supplementary Figure 1 | GPU acceleration of UPM beamforming.  The raw ultrasound channel RF data are first interpolated by a factor of 4 to improve the delay accuracy. A low pass filter is applied after the interpolation to remove the spectral images. Then the raw channel data is multiplied with the beamforming matrix to complete the delay-and-sum process. A complex carrier is then multiplied with the beamforming RF data to demodulate the beamformed RF signal to baseband IQ signal. Then a low pass filter is applied before the decimation to prevent aliasing.

|  | Verasonics Vantage | Verasonics NXT |
| --- | --- | --- |
| Sampling Frequency | 62.5 MHz | 125 MHz |
| Quantization Depth | 14 -bit | 16-bit |
| Buffer Size | 16GB | 64GB |
| Bandwidth | 2 MHz to 42 MHz | 1 MHz to 60 MHz |
| Data Type | 16-bit integer | 16-bit float |

Supplementary Tabel 1 | System specifications for both new generation and previous generation ultrasound imaging research platforms.  New generation provides higher sampling frequency, bandwidth, buffer size, and quantization depth compared to the previous generation.

|  | MS200 | MS550D | MS550S | MS700 |
| --- | --- | --- | --- | --- |
| Bandwidth | 9-18 MHz | 20-55 MHz | 32-56 MHz | 30-70 MHz |
| Pitch | 125 µm | 55 µm | 55 µm | 38 µm |
| Voltage | 30 V | 25 V | 25 V | 20 V |

Supplementary Tabel 2 | Ultrasound transducer array specifications.  The bandwidth and pitch come from the manufacturer provided information. The voltage is the actual voltage used in data acquisition.

| Batch Size | 8 frames | 16 frames | 32 frames |
| --- | --- | --- | --- |
| Mean | 2.3232 s | 2.2133 s | 2.2438 s |
| STD | 0.0945 s | 0.0951 s | 0.1031 s |

Supplementary Tabel 3 | Beamforming time measurements with GPU acceleration.  The UPM beamforming time of each functional ultrasound acquisition (640 frames) was measured by repeatedly running the GPU beamforming program 100 times.

Captions for Supplementary Videos

Supplementary Video 1 | 50 MHz UPM images of label-free mouse brain.

Supplementary Video 2 | 40 MHz UPM images of label-free mouse brain.

Supplementary Video 3 | 30 MHz UPM images of label-free mouse brain.

Supplementary Video 4 | 30 MHz UPM images of label-free rat brain.

Supplementary Video 5 | 30 MHz UPM images of label-free rat spinal cord segment 1.

Supplementary Video 6 | 30 MHz UPM images of label-free rat spinal cord segment 2.

References

1.	Czarske, J. W. Statistical frequency measuring error of the quadrature demodulation technique for noisy single-tone pulse signals. Meas. Sci. Technol. 12, 597–614 (2001).